3GPP TSG RAN WG1 Meeting #98	R1-1909375

Prague, CZ, 26th -30th October 2019

Source:	Ericsson

Title:	On additional SRS symbols for DL efficiency enhancements

Agenda Item:	6.2.3.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#97, the following was agreed:

Agreement

A guard period can be configured for frequency hopping and antenna switching of additional SRS symbols.

- If guard period is configured, it is 1 OFDM symbol

- FFS: Guard period for frequency hopping and/or antenna switching is always configured when intra-subframe repetition is not configured,

Working Assumption

When intra-subframe frequency hopping/repetition and intra-subframe antenna switching are concurrently configured, frequency hopping should be performed before antenna switching.

Conclusion

Legacy SRS symbols follow the legacy configuration

Agreement

Aperiodic additional SRS can only be triggered for transmission in any subframe belonging to the legacy UE-specific SRS subframe configuration

Agreement

At least for the case where there no legacy SRS transmission on the subframe, at least independent open loop power control of additional SRS symbols from legacy SRS symbols is supported.

- Further study the power control for SRS symbols when additional and legacy SRS symbols are transmitted in the same subframe.

- FFS: independent closed loop

Agreement

For the sequence generation of additional SRS symbols:

where  is the absolute symbol index within the slot  and Nsymb is the number of OFDM symbols per slot.

- FFS: c_init

Agreement

For the handling of collision of SRS and PUCCH/PUSCH transmission, downselect from the following in RAN1#98

- Alt1: Use sPUSCH and/or sPUCCH

  - Introduce sPUSCH and/or sPUCCH capability for Rel-16 UEs to enable sPUSCH and/or sPUCCH transmission in the SRS subframe.

  - Multiplexing of sTTI on same subframe and same PRBs with additional SRS on symbols where SRS is not transmitted is supported

- Alt2: UE drops or delays SRS transmission in the additional symbols if the SRS collides with PUCCH/PUSCH/PRACH in the same carrier

  - Choose between drop and delay

- Alt3: UE does not expect to be triggered with aperiodic SRS in the additional symbols of the SRS collides PUCCH/PUSCH/PRACH in the same carrier

  - FFS: Collision on interband-CA, intraband-CA

- Alt4: It is up to eNB/UE implementation

Additionally, RAN1 sent a LS to RAN4 in RAN1#96bis, requesting information about the RAN4 assumption on transient period due to intra-subframe frequency hopping or antenna switching of SRS, and received the following reply in R1-1907980:

•	The current TS 36.101 lists the transient period for power change and frequency hopping for SRS as -20 to +20 usec w.r.t symbol boundary. However, starting rel 15, transient period for power change and frequency hopping in general for sTTI (not specifically for SRS) has been reduced to -10 to +10 usec. For UEs supporting rel 15 and above it assumes that maximum duration of transient period for power change and frequency hopping for SRS is -20 to +20 usec. The same range could also be used for SRS Antenna hopping as well.

•	However, some UEs may require smaller transient period than that mentioned above but no requirement for such UEs has been studied yet in RAN4.

In this contribution, we present our view on the remaining open issues.

# Guard period for antenna switching

It has been agreed that a guard period of 1 OFDM symbol may optionally be configured when frequency hopping and/or antenna switching is applied for additional SRS symbols in normal UL subframe. Additionally, it was taken as working assumption that frequency hopping should be applied before antenna switching when they are both concurrently configured to apply within the same subframe.

However, the sole motivation for the design choice according to the working assumption was that it was assumed that a GP symbol would be needed after antenna switching but not after frequency hopping, as is illustrated in Figure 1a). In that case, it makes sense to perform the frequency hopping first in order to minimize the number of GP symbols required. However, according to RAN4 LS reply, there is no differentiation with respect to the transient period required between frequency hopping and antenna switching. Thus, the same GP needs to be applied after frequency hopping as well, according to Figure 1b).

According to RAN4 LS reply, the required transient period is the same for both frequency hopping and antenna switching

| - FH precedes AS with no GP after FH | - FH precedes AS with GP after FH |
| --- | --- |
| - AS precedes FH with GP after FH |  |

Figure 1: Illustration of different SRS mapping strategies

Thus, given this assumption, the benefit with performing frequency hopping before antenna switching vanishes and instead it proves to be an inferior solution compared to first performing antenna switching and then frequency hopping, according to Figure 2c), since the phase drift between the channel of different antenna ports at a certain PRB is larger due to their larger time separation, which degrades the DL precoding derived from the SRS channel estimates. Therefore, the working assumption should be rejected and the strategy according to Figure 2c) should be adopted.

When intra-subframe frequency hopping/repetition and intra-subframe antenna switching are concurrently configured, antenna switching should be performed before frequency hopping. I.e. the working assumption at RAN1#97 is rejected.

Another issue for further study is if the eNB always should be required to configure the one-symbol GP if the additional SRS symbols are not configured with intra-subframe repetition. The stated motivation being that with just one repetition (i.e. R=1), the degradation of an SRS symbol due to power ramping in the transient period would be too severe so that the SRS channel estimate would be unusable. However, whether the channel estimates are useable or not is at the discretion of the eNB and we should remember that more stringent UE power mask requirements than ± 20μs are already in place for sTTI capable UEs, and even further stringent requirements may be considered in the future. Furthermore, certain UE implementations may be more capable than what is imposed by the RAN4 requirements. From that perspective, there is no need for any restriction on GP configuration from RAN1/RAN2 perspective in our view.

Rule that “guard period for frequency hopping and/or antenna switching is always configured when intra-subframe repetition is not configured” is not needed from RAN1/RAN2 perspective

# Power control for additional and legacy SRS symbols

The legacy SRS power control mechanism in LTE is defined by the expression:

[dBm]

That is, the open-loop power control parameters P0 and α as well as the closed-loop PC state fc(i) follows that of PUSCH. Since additional SRS symbols are introduced with the purpose of sounding the UL channel for DL CSI acquisition utilizing reciprocity, it is not obvious that the power control for additional SRS symbols should follow that of PUSCH. Accordingly, it was agreed in RAN1#97 that at least the open-loop PC parameters are independent for the additional SRS symbols compared to the legacy SRS, however it was left FFS whether this applied to the closed-loop PC parameters and how to handle the case when both additional SRS and legacy SRS symbols are present in a subframe.

Regarding the second issue, if SRS transmitted in two adjacent symbols are transmitted using widely different power levels, there could potentially be a power ramping issue. Thus, for simplicity it is preferred that the same power level is used for all SRS symbols of the subframe. That is, in case both additional SRS and legacy SRS is present in the subframe, the same power control (i.e. the same set of open and closed loop power control parameters) should be used for both SRS transmissions. In our view, the legacy SRS power control should be applied on all SRS symbols in this case in order to not interfere with the PUSCH link adaptation.

If both legacy SRS and additional SRS symbols are present in a subframe, the power control for all SRS symbols follows that of legacy SRS

Additionally, in our view, using the same closed-loop power control offset fci for additional and legacy SRS symbols but different open-loop power-control reference levels makes little technical sense. For instance, for additional SRS symbols, the open-loop reference level might be set too low and a positive TPC command is required to be sent whereas for legacy SRS, the open-loop reference level might be set too high and a negative TPC command is required to be sent. It is therefore proposed that, similarly as for SRS on carriers without PUCCH/PUSCH, an independent closed-loop PC offset is used for additional SRS symbols.

The closed-loop power control offset of additional SRS symbols is independent from that of legacy SRS symbols

It has been agreed to re-use the Rel-15 DCI formats which can trigger SRS for the triggering of Rel-16 SRS by changing the interpretations of the codepoints of the SRS request field to enable triggering of both additional SRS symbols only, or joint triggering of additional and legacy SRS.

In LTE, SRS transmission can be triggered by both DL assignments (e.g. DCI Formats 1A/2B/2C/2D) as well as UL grants (e.g. DCI formats 0/4). While the DL DCI formats include a TPC field for PUCCH power control, it cannot be used to change the closed-loop power level for the triggered SRS, while the UL DCI formats contain a TPC field used for changing the power control level for both PUSCH and SRS (since the same PC state is used). It is not advised to change the behavior of this TPC field, i.e. it should only apply to the PUSCH/legacy SRS power control loop and not be modified to alternatively or additionally apply to the additional SRS power control loop under certain conditions. Naturally, this should also apply to PUCCH/PUSCH TPC commands available via DCI Format 3/3A.

Additionally, DCI Format 3B can be used to send SRS TPC commands to multiple UEs. In our view, it is enough that the closed-loop power control offset for additional SRS symbols can be controlled by the TPC command carried by DCI Format 3B, no additional mechanism needs to be considered. That is, if the SRS request field of DCI Format 3B indicates triggering of additional SRS symbols only, the TPC command applies to the additional SRS power control offset. IF both legacy and additional SRS is triggered, the TPC command applies to the legacy SRS (since it is proposed that the legacy SRS PC is used if the additional and legacy SRS symbols are present in the same subframe).

The TPC command in DCI formats carrying UL grant only impacts the closed-loop PC parameter of PUSCH/legacy SRS

Closed-loop PC of additional SRS symbols is only achieved when DCI Format 3B triggers only additional SRS symbols to the UE

# Discussion on periodic additional SRS

One of the remaining issues is if periodic additional SRS should be supported in LTE Rel-16 in addition to aperiodic SRS.  For periodic multi-symbol SRS, the extent of UL throughput loss for legacy UEs depends on the periodicity of multi-symbol SRS transmissions.

A longer periodicity for multi-symbol SRS transmissions means that the UL channel is sounded less frequently, and the possibility of the estimated channel being outdated which may impact performance of Rel-16 UEs supporting multi-symbol SRS enhancement.

A longer periodicity for multi-symbol SRS transmission would then translate to lower impact on UL throughput of legacy UEs.  On the other hand, a shorter periodicity for multi-symbol SRS transmissions would lead to more accurate channel estimation for Rel-16 UEs at the expense of higher impact on the UL throughput of legacy UEs.

Hence, if periodic multi-symbol SRS enhancement is to be specified in Rel-16, the trade-off between channel sounding accuracy from multi-symbol SRS transmissions of Rel-16 UEs and UL throughput impact on legacy UEs needs to be been evaluated.

However, given that the benefits of periodic SRS is very uncertain and no strong arguments have been expressed that it is needed we don’t see the need for this feature. Note that this issue has been known and discussed for a several meetings, and there is no consensus among companies to support periodic additional SRS, we propose to not support periodic additional SRS in LTE Rel-16.

Periodic additional SRS is not supported in LTE Rel-16

# Collision handling

The following four alternatives for collision handling was agreed for further downselection in RAN1#97:

Agreement

For the handling of collision of SRS and PUCCH/PUSCH transmission, downselect from the following in RAN1#98

- Alt1: Use sPUSCH and/or sPUCCH

  - Introduce sPUSCH and/or sPUCCH capability for Rel-16 UEs to enable sPUSCH and/or sPUCCH transmission in the SRS subframe.

  - Multiplexing of sTTI on same subframe and same PRBs with additional SRS on symbols where SRS is not transmitted is supported

- Alt2: UE drops or delays SRS transmission in the additional symbols if the SRS collides with PUCCH/PUSCH/PRACH in the same carrier

  - Choose between drop and delay

- Alt3: UE does not expect to be triggered with aperiodic SRS in the additional symbols of the SRS collides PUCCH/PUSCH/PRACH in the same carrier

  - FFS: Collision on interband-CA, intraband-CA

- Alt4: It is up to eNB/UE implementation

The WI objectives clearly defines that no enhancements to PUSCH or PUCCH are in scope. I.e. no special PUSCH design for rate matching around additional SRS symbols should be considered in the WI. One of the motivations for this restriction was that e.g. slot-based PUSCH have already been introduced in the Rel-15 sTTI work and there is no need to reinvent the wheel in the MIMO session. Thus, it is clear that sPUSCH and additional SRS could be present in the same subframe (in a non-overlapping manner) if the UE has capability for both these kinds of transmissions. However, it is unclear if a separate sPUSCH/sPUCCH capability needs to be introduced specifically for SRS multiplexing purpose, it should be discussed further at the end of the release as part of the general Rel-16 capability discussion.

Separate sPUCCH/sPUSCH capability can be addressed as part of the general Rel-16 capability discussion

The remaining alternatives are three different options for handling collision with additional SRS symbols and other UL channels, and hence one of the alternatives must be selected. In our view, since additional SRS is only supported for aperiodic triggering, the eNB is in full control of the transmission occasion of the SRS and can by implementation avoid triggering an SRS that would collide with other UL transmission by the UE. Hence, there is no need to introduce specification support for a collision handling mechanism. The UE can simply consider collision between SRS and other UL signals an error case.

For the handling of collision of SRS and PUCCH/PUSCH transmission, support Alt 4

# Conclusion

In this paper, we provide our views on some remaining issues of Rel-16 LTE SRS enhancements.  We make the following proposals:

Proposal 1	When intra-subframe frequency hopping/repetition and intra-subframe antenna switching are concurrently configured, antenna switching should be performed before frequency hopping. I.e. the working assumption at RAN1#97 is rejected.

Proposal 2	Rule that “guard period for frequency hopping and/or antenna switching is always configured when intra-subframe repetition is not configured” is not needed from RAN1/RAN2 perspective

Proposal 3	If both legacy SRS and additional SRS symbols are present in a subframe, the power control for all SRS symbols follows that of legacy SRS

Proposal 4	The closed-loop power control offset of additional SRS symbols is independent from that of legacy SRS symbols

Proposal 5	The TPC command in DCI formats carrying UL grant only impacts the closed-loop PC parameter of PUSCH/legacy SRS

Proposal 6	Closed-loop PC of additional SRS symbols is only achieved when DCI Format 3B triggers only additional SRS symbols to the UE

Proposal 7	Periodic additional SRS is not supported in LTE Rel-16

Proposal 8	For the handling of collision of SRS and PUCCH/PUSCH transmission, support Alt 4