3GPP TSG-WG1 Meeting #98	R1-1909376

# Prague, Czech Republic, 26- 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of power headroom computation in NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NB-IOT UE reports power headroom (PH) in DPR, which is sent in Msg3 together with a CCCH SDU, i.e., random access procedure in idle mode. In the idle mode RA procedure, the UE transmit power for MSG3 is computed based on “NPUSCH (re)transmissions corresponding to the random access response grant then j=2.” Thus, the power headroom computation should refer to the computaion of MSG3 transmit power. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To change the variables in power headroom computation from PO_NPUSCH, c1 and αc1 to PO_NPUSCH, c2 and αc2. Impact analysis Impacted functionality: Resource Schedulling Inter-operability: If the UE is implemented according to the CR and the NW is not, there is no inter-operability foreseen. If the NW is implemented according to the CR and the UE is not, there is no inter-operability foreseen. Implementation of this CR from Rel-13 will not cause interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Power headroom computation is not precise. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

##### 16.2.1.1.2	Power headroom

If the UE transmits NPUSCH in NB-IoT UL slot  for serving cell , power headroom is computed using

PHci=PCMAX, ci-{PO_NPUSCH, c12+αc12∙PLc } [dB]

where, , PO_NPUSCH, c12, αc12, and, are defined in Subclause 16.2.1.1.1.

The power headroom shall be rounded down to the closest value in the set [PH1, PH2, PH3, PH4] dB as defined in [10] and is delivered by the physical layer to higher layers.