3GPP TSG RAN WG1 Meeting #98			     R1-1909388

Prague, Czech Rep., August 26th – 30th, 2019.

Agenda Item:	7.2.4.1

Source: 	KT Corp.

Title:	Considerations on PSFCH Design for NR V2X

Document for:	Discussion and Decision

Introduction

In RAN1 #97[2], some progresses for the design of NR sidelink PSFCH was made as follows:

Agreements:

A sequence-based PSFCH format with one symbol (not including AGC training period) is supported.

This is applicable for unicast and groupcast including options 1/2.

Sequence of PUCCH format 0 is the starting point.

FFS: 1 PRB or multiple PRBs is/are used for this PSFCH format

FFS: feasible number of HARQ-ACK bits, mapping of HARQ-ACK bit

FFS whether to support the following formats

X-symbol PSFCH format with a repetition of the one-symbol PSFCH format (not including AGC training period).

E.g. X=2

A PSFCH format based on PUCCH format 2

A PSFCH format spanning all available symbols for sidelink in a slot

To establish reasonable and efficient design criteria, considering procedures operating HARQ and corresponding PSFCH is required. Some agreements about procedures of HARQ and PSFCH for NR sidelink in RAN1 #96b[1] are as follows:

Agreements:

It is supported, in a resource pool, that within the slots associated with the resource pool, PSFCH resources can be (pre)configured periodically with a period of N slot(s)

N is configurable, with the following values

1

At least one more value >1

FFS details

The configuration should also include the possibility of no resource for PSFCH. In this case, HARQ feedback for all transmissions in the resource pool is disabled

HARQ feedback for transmissions in a resource pool can only be sent on PSFCH in the same resource pool

The agreements are about the period of PSFCH, leaving the possibility of the support of the period with larger than the corresponding PSSCH. Other progresses about this issue was made in RAN1 #97[2] as follows:

Agreements:

For the period of N slot(s) of PSFCH resource, N=2 and N=4 are additionally supported.

Agreements:

For a PSSCH transmission with its last symbol in slot n, when the corresponding HARQ feedback is due for transmission, it is expected to be in slot n+a where a is the smallest integer larger than or equal to K with the condition that slot n+a contains PSFCH resources.

FFS details of K

Agreements:

At least for the case when the PSFCH in a slot is in response to a single PSSCH:

Implicit mechanism is used to determine at least frequency and/or code domain resource of PSFCH, within a configured resource pool. At least the following parameters are used in the implicit mechanism:

Slot index (FFS details) associated with PSCCH/PSSCH/PSFCH

Sub-channel(s) (FFS details) associated with PSCCH/PSSCH

Identifier (FFS details) to distinguish each RX UE in a group for Option 2 groupcast HARQ feedback

FFS detailed applicability of the above parameters

FFS: Other parameters (e.g. SL-RSRP/SINR, Layer-1 source ID, location information, etc.)

Conclusion:

Study further whether/how to handle/avoid the following cases for PSFCH transmission and reception:

Case 1 (PSFCH TX/RX overlap): A UE transmitted a PSSCH and received SCI scheduling another PSSCH where PSFCH resources corresponding the two PSSCHs appear in the same slot.

Case 2 (PSFCH TX to multiple UEs): A UE received SCI from different UEs and the associated PSFCHs appear in the same slot.

Case 3 (PSFCH TX with multiple HARQ feedback to the same UE): A UE received multiple SCI from the same UE and the associated PSFCHs appear in the same slot.

As a result of RAN #97, periods longer than 1 slot are supported. This indicated a problem stated in the Conclusion that indicates demand of the method of sharing PSFCH resources. This contribution gives some views and considerations for the shared PSFCH resources, mainly focusing the appropriate design approach for the Tx signal of HARQ feedback.

PSFCH Collision Case

The previous conclusion indicates three cases of PSFCH resource collision. For each case, UE need to perform:

- Case 1: The UE need to perform Tx and Rx at the same time, with the same frequency region.

- Case 2: The UE need to multiplex Tx signals at the same time and frequency region.

- Case 3: The UE need to receive Rx signal excluding interfered signals from different sources.

To solve each case, we observed the simplest methods for Case 1 as follows.

Observation 1. For Case 1, A UE can implicitly struggle to avoid such case, and regard the receiving Rx as NACK for unavoidable situations.

For Case 2 and 3, some code-domain multiplexing for each PSFCH signal need to be applied using orthogonal, at least near-orthogonal sequences. The independent sequences can be arranged in time with perfect synchronization for Case 2, however, it is never be assured at Case 3.

Observation 2. Individual PSFCHs from different users are not time-aligned.

Simulation Results

To obtain enough Rx performance with reasonable complexity, it can be considered that reusing NR pseudo-random or low-PAPR sequences. Considering its functionality, it seems reasonable that designing PSFCH for HARQ feedback starts from adopting the structure of PUCCH format 0. TS38.211 Table 5.1.2.2-2 gives sequences based on QPSK for PUCCH format 0. Choosing one from 30 sequences in the table by u determined by the UE hopping parameters and cell ID, {0,6} (or {0,3,6,9}) cyclic shifts for them are used for each HARQ feedback {0,1} (or {00,01,10,11}). To make UEs to give different sequences the easiest way is giving different u or different cyclic shift to each corresponding PSCCHs. We assumed giving different u to make consistency of former criteria about HARQ-bits to cyclic-shift mapping.

As a control group for comparison, we considered sequences of length 24 in TS38.211 Table 5.1.2.2-3 and cyclic-shifted set of ZC sequences of length 11 and 23 for zero cross-correlation model. We assumed the number of UEs accessing same time and frequency region of sidelink resource as 4, so three independent interfering signals received at the same time and frequency region. We also considered variation of arrival time and received power of the interference signals.

In addition to this, we’ve considered an “expanded” CP for PSFCH like PRACH for the Case 3. The simplest idea we made to achieve this is to use 2 consecutive symbols with the whole range of the former symbol as CP of the latter. As a result, the CP ratio becomes 53%. Compare this with the normal CP ratio 7%. To give proper comparison of SINR, we’ve made repetition for the normal CP case. Despite of this, the SINR does not changed by the repetition since all the interference signals are repeated also with exactly same pattern.

Figure 1 and 2 show the simulation result. Expressions in the legends shows the type of sequence, adoption of expanded CP, and amplitude variance. For example, a legend “11e, av0,8” means the type of sequence is length 11 ZC sequence (having zero cross-correlation property), expanded CP applied, and the interference signal received with power amplified randomly between 1.0±0.8, that is 0.2 to 1.8. We assumed the perfect channel, with no AWGN, Doppler, and multipath fading (flat fading is considered).

Figure 1. SINR of PSFCH for the CDM case (1RB).

Figure 2. SINR of PSFCH for the CDM case (2RBs).

From these results, we made the following observations and a proposal.

Observation 3: Conventional NR short-length low-PAPR sequences cannot give enough performance for CDMA operation of PSFCH.

Observation 4: Sequences with zero cross-correlation shows significantly better SINR, at least for the time variance 0 reflecting Case 2.

Proposal 1: Adopt sequence set with low cross-correlation for the cover code of PSFCH transmission at least for Case 2.

The expanded CP is highly resilient for the time error among UEs, however, too much relaxation may have interference from outer group V2X signals. This indicates the followings.

Observation 5: The model applying expanded CP gives significantly better SINR with the situation of large time variance and using sequences with zero cross-correlation. On the other hand, there is no significant merits for the expanded CP with sequences having random-like correlation property, like the conventional NR short length low-PAPR sequences.

Proposal 2: Consider adopting expanded CP for PSFCH when adopting low cross-correlation sequence.

Summary

Following is the summary of our observations and proposals.

Observation 1. For Case 1, A UE can implicitly struggle to avoid such case, and regard the receiving Rx as NACK for unavoidable situations.

Observation 2. Individual PSFCHs from different users are not time-aligned.

Observation 3: Conventional NR short-length low-PAPR sequences cannot give sufficient performance for CDMA operation of PSFCH.

Observation 4: Sequences with zero cross-correlation shows significantly better SINR, at least for the time variance 0 reflecting Case 2.

Observation 5: The model applying expanded CP gives significantly better SINR with the situation of large time variance and using sequences with zero cross-correlation. On the other hand, there is no significant merits for the expanded CP with sequences having random-like correlation property, like the conventional NR short length low-PAPR sequences.

Proposal 1: Adopt sequence set with low cross-correlation for the cover code of PSFCH transmission at least for Case 2.

Proposal 2: Consider adopting expanded CP for PSFCH when adopting low cross-correlation sequence.

References

[1] RAN1 Chairman’s note (final), 3GPP RAN1 #96bis, Xi’an, China, April 8-12, 2019.

[2] RAN1 Chairman’s note (final), 3GPP RAN1 #97, Reno, USA, May 13-17, 2019.

Appendix

Simulation Assumptions

- Signal selection: Randomly choose from all possible, not duplicated cyclic shifts for ZC sequence, from u of 30 kinds for conventional NR short-length low-PAPR sequence

- Sampling Rate: 10000 for each OFDM symbol

- SINR calculation: dB of arithmetic mean value of signal power/interference power of 1000 independent simulations.

- Amplitude variation: Power amplify by uniformly chosen from [1-av, 1+av]

- Time variation: Time shift of interference signals determined by uniformly chosen delay from [-Tτ, +Tτ], where T is the length of single received OFDM symbol excluding CP, that is, T=2048∙(κ/2μ)Tc.