3GPP TSG RAN WG1 Meeting #98 	R1-1909409

Prague, Czech Republic, 26th – 30th August, 2019

Agenda Item: 6.2.2.2

Source: MediaTek Inc.

Title: Early transmission in preconfigured UL resources in NB-IoT

Document for: Discussion and Decision

# Introduction

A new RAN1-led Working Item on Rel-16 enhancements for NB-IoT was approved at RAN Plenary #80 [1].  In RAN#82, the scope of the WID was extended to include Connection to 5GC. In RAN#83, a further WID extension was approved to include  support for quality reporting in connected mode for anchor and non-anchor carriers.

In RAN1#96bis, the following agreements and working assumptions were made

Agreement

- In idle mode, a UE can be configured such that TA is always valid within a given cell.

  - Up to RAN2 how to implement

    - e.g. PUR Time Alignment Timer or NRSRP Threshold = infinity

- The value(s) of NRSRP threshold(s) is UE specific

- The UE monitors the NPDCCH for at least a time period after a PUR transmission.

  - FFS: Details of the time period

  - FFS: UE behaviour if nothing is received in that time period.

  - FFS: If and how often UE monitors NPDCCH after a PUR allocation in which it has not transmitted

- Reuse existing field(s) of DCI format N0 to convey the dedicated PUR ACK

- After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect an UL grant for retransmission on NPDCCH. Other behaviors are FFS.

- For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

Working Assumption#1

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

  - FFS: Which PUR configurations and PUR parameters will be signaled via L1

  - FFS: Definition of PUR configurations and PUR parameters

- The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

Working Assumption#2

- For dedicated PUR

  - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

    - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived

  - FFS if the UE monitors any additional RNTI which may be shared with other UEs.

  - Note: The same RNTI may be used over non-overlapping time and/or frequency resources

Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed. (LS is endorsed in R1-1905570)

In RAN1#97, the following agreements were made

- For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission

  - FFS: Value of x, and if x is fixed or signaled

  - FFS: Support for monitoring of PUR SS Window before PUR transmission

Note: The PUR SS Window is the time period where the UE monitors the NPDCCH for at least a time period after a PUR transmission

- NPDCCH candidates are determined by USS like search space

  - FFS: Other details on the USS like search space

    - Type2-CSS can also be discussed as part of the FFS

In RAN2#106bis, the following agreements and aspects requiring further study were made:

- RRC response message needs to be supported by the UE and could be used in all cases.

- For some cases L1 signalling is sufficient to acknowledge, i.e. RRC response message is not needed.

- RAN2 assumes the L1 signalling for acknowledgement is sent only after the eNB determines there is no pending downlink data or signalling.

- RAN2 assumes that the configuration for D-PUR provided by RRC signalling is not updated via L1 signalling, and should check with RAN1 what parameters are planned to be (re)configured using DCI.

- It is feasible to provide the UE with a UE-specific RNTI for D-PUR. Common or shared RNTI is also feasible.

- The RNTI used for D-PUR is signalled together with other D-PUR configuration.

This contribution aims to discuss support of early transmission in preconfigured UL resources in NB-IoT.

# Dedicated PUR

L1 ACK Information

RAN2#106bis made agreement that For some cases L1 signalling is sufficient to acknowledge, i.e. RRC response message is not needed. Hence, working assumption#1 made in RAN1#96bis applies

Proposal 1: confirm working assumption #1

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

  - FFS: Which PUR configurations and PUR parameters will be signaled via L1

  - FFS: Definition of PUR configurations and PUR parameters

We have preference to include other parameters in the L1 PUR ACK in case no further L2/L3 signaling needed following L1 ACK. It seems beneficial if gNB indicates Timing advance adjustment to maintain UL timing alignment and UE TX power adjustment to improve reliability depending on power headroom.

Proposal 2:  The PUR L1 ACK can include Timing advance ajustment and UE Tx power adjustment.

Whether NPUSCH Repetition adjustment for new data transmissions could be indicated in the L1 PUR ACK should be further discussed in RAN2, as it was agreed in RAN2#106bis that RAN2 assumes that the configuration for D-PUR provided by RRC signalling is not updated via L1 signalling.

Observation 1: RAN2 assumes that the configuration for D-PUR provided by RRC signalling is not updated via L1 signalling.

Assuming a 1-bit field is used to differentiate PUR ACK or UL grant in format N0, all the remaining bits in format N0 could be re-interpreted for additional data in L1 PUR ACK. This could allow up to 21 spare bits.

Proposal 3: A 1-bit field is used to differentiate PUR ACK or UL grant in format N0, remaining 21 bits in format N0 can be re-interpreted for additional data in L1 PUR ACK.

HARQ for D-PUR

RAN1#95 agreed that In idle mode, only one HARQ process is supported for dedicated PUR. In case the PUR L1 ACK is not received, the UE can assume implicit NACK. RAN1#96bis agreed that After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect an UL grant for retransmission on NPDCCH.The PUR retransmission assuming could be done either on

- Next most immediate dedicated PUR: simple way, may result on longer latency to transmit old data, or transmit new data in next available dedicated PUR. This can be handled by D-PUR configuration assuming a reasonable BLER target and also at the application layer assuming packet drop probability is reasonable (e.g. old data may not be re-transmitted, prioritize new data for tracking device).

- Dynamically: already supported based on RAN1#96bis agreement. It is simpler and more efficient way for D-PUR configuration that allows old data to be re-transmitted before the next D-PUR period.

Proposal 4: HARQ re-transmission based on UL grant indication on NPDCCH only is supported in D-PUR.

PUR search space monitoring

RAN2#106bis made agreement that The RNTI used for D-PUR is signalled together with other D-PUR configuration. There seems no need for the UE to monitor any additional RNTI which may be shared with other UEs in D-PUR. Hence we have preference to confirm working assumption #2 as follows:

Proposal 5: Working assumption #2 is confirmed as follows

- For dedicated PUR

  - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

RAN1#97 made agreement that  	For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission. It is for further study the value of x, and if x is fixed or signaled, and whether to support for monitoring of PUR SS Window before PUR transmission.

The PUR SS Window is the time period where the UE monitors the NPDCCH for at least a time period after a PUR transmission. It is up to the eNB scheduler when to schedule L1 PUR ACK or UL grant for re-transmission via DCI on Format N0. Since the PUR SS is to receive DCI format N0 following UL transmission on D-PUR on PUSCH and since a UE-specific RNTI is used, the operations involved in UE can be expected to be similar to that following UL A/N on PUCCH after DL data transmission on PUSCH in connected mode. Recall that in this case, a minimum scheduling delay of 12 slots is specified before a DCI on Format N1 can be received to indicate DL assignment for new DL data transmission or re-transmissions.

Proposal 6: The value of x is fixed and set to 12.

# Conclusion

In this contribution, we discussed support of early transmission in preconfigured UL resources in NB-IoT.

Proposal 1: confirm working assumption #1

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

  - FFS: Which PUR configurations and PUR parameters will be signaled via L1

  - FFS: Definition of PUR configurations and PUR parameters

Proposal 2:  The PUR L1 ACK can include Timing advance ajustment and UE Tx power adjustment.

Observation 1: RAN2 assumes that the configuration for D-PUR provided by RRC signalling is not updated via L1 signalling.

Proposal 3: A 1-bit field is used to differentiate PUR ACK or UL grant in format N0, remaining 21 bits in format N0 can be re-interpreted for additional data in L1 PUR ACK.

Proposal 4: HARQ re-transmission based on UL grant indication on NPDCCH only is supported in D-PUR.

Proposal 5: Working assumption #2 is confirmed as follows

- For dedicated PUR

  - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

Proposal 6: The value of x is fixed and set to 12.

# References

- RP-181451, New WID on Rel-16 enhancements for NB-IoT, Ericsson, TSG RAN#80, June 2018

- RP-182292, WID revision: Additional enhancements for NB-IoT, Huawei, TSG RAN#82, December 2018

- RP-190337, WID revision: Additional enhancements for NB-IoT, Huawei, TSG RAN#83, March 2019