3GPP TSG-RAN WG1 Meeting #98	R1-1909417

# Prague, CZ, August 26th – 30th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | x | Radio Access Network |  |  |  |  | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Aperiodic SRS trigger list correction in 38.212 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2019-03-29 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-15 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |
| Reason for change: |  | R2-1810868 corrected TS 38.331 to reflect the RAN1 agreement from RAN1#93 that “Up to ‘maxNrofSRS-TriggerStates-1’ SRS triggering states can be configured for one SRS resource set”. The field aperiodicSRS-ResourceTriggerList-v1530 was added in SRS-Config, which provides a list of SRS request field values to the single value in aperiodicSRS-ResourceTrigger. Therefore, when multiple SRS triggering states are used for one SRS resource set, both aperiodicSRS-ResourceTriggerList-v1530 and aperiodicSRS-ResourceTrigger are provided according to TS 38.331. This introduction of aperiodicSRS-ResourceTriggerList has not propagated to TS 38.212, which presently only refers to the aperiodicSRS-ResourceTrigger. Consequently, TS 38.214 does not reflect that multiple SRS triggering state(s) can be associated with an SRS resource set. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | AperiodicSRS-ResourceTriggerList is used to list the SRS request field values used to trigger the SRS resource set in addition to the field values in AperiodicSRS-ResourceTrigger. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the network configures aperiodicSRS-ResourceTriggerList and aperiodicSRS-ResourceTrigger and the UE follows 38.212, the UE will not transmit an SRS resource set when triggered with a value in aperiodicSRS-ResourceTriggerList. However, if UE follows 38.331, it will transmit an SRS resource set when triggered with values in aperiodicSRS-ResourceTrigger and in aperiodicSRS-ResourceTriggerList. Therefore, the network may need to choose between getting inaccurate SRS measurements when SRS is not triggered and not configuring multiple trigger values for an SRS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | X |  |  | Other core specifications |  |  |  |  |  |  | TS 38.214 Draft CR R1-1909418 |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact to aperiodic SRS transmission for UEs configured to transmit aperiodic SRS, and corrects TS 38.212 to align with 38.331. Interoperability analysis: - If the network is implemented in accordance to the CR and the UE is not, the network expects SRS transmission when the UE may not transmit it, leading to degraded channel measurements and UL and DL capacity. - If the UE is implemented in accordance to the CR and the network is not, the network may expect the UE to not transmit an SRS resource set if that set is configured with more than one trigger values, thereby constraining network implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

<--------------------------------Unchanged Text Omitted------------------------------>

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 1_0, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList-v1530 set to 1 | SRS resource set(s) configured with higher layer parameter SRS-SetUse set to 'antenna switching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList-v1530 set to 2 | SRS resource set(s) configured with higher layer parameter SRS-SetUse set to 'antenna switching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList-v1530 set to 3 | SRS resource set(s) configured with higher layer parameter SRS-SetUse set to 'antenna switching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |