3GPP TSG RAN WG1 #98	R1- 1909425

Prague, CZ, August 26th – 30th 2019

Source:	Ericsson

Title:	UL Reference Signals for NR Positioning

Agenda item:	7.2.10.2

Document for:	Discussion and decision

Introduction

During RAN1#97, the discussion on enhancements to the SRS for positioning resulted in agreements regarding the number of configurable symbols, comb size and staggered pattern. Each of these agreement carried further possible enhancements that are discussed in this paper.

UL SRS design for positioning

SRS configuration

The SRS configuration for UL positioning should follow previous LTE implementation. Similarly to the DL PRS, the SRS configuration supported by a cell is reported via NRPPa to the location server. If a UE is requested to perform e.g. UTDOA or RTT with SRS transmissions, the location server then informs the neighbor cells of the SRS configuration. which in turn, informs the other cells Config via RRC/NRPPa

For Periodic positioning, the configuration is straightforward. The serving cell signals to the location server the configuration(s) it may use for SRS for positioning. Then, when the location server signals to the cell that a UE in this cell (hence, a serving cell) should use SRS, the serving cell configures the UE with SRS resources for positioning via RRC. During the same procedure, the location server should inform non-serving neighbor cells of the SRS configuration. Then the location server requests measurements and obtain reports from the UE via LPP.

The SRS for positioning is configured by the UE serving cell via RRC

The SRS for positioning   UE configuration is communicated to the UE neighbor (non-serving) cell via NRPPa from the location server

SRS pattern for positioning and resource allocation

During RAN1#97, the following agreement was reached:

Agreement: SRS transmissions for positioning are realized with staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource - FFS: construction of the pattern inside the SRS resource structure

There are two ways to construct an SRS pattern within a resource. For full flexibility, one could configure each symbol of the pattern with a specific comb offset. This allows the flexibility to have e.g. a full staggered pattern or another pattern if deemed suitable (for example, the comb offset may be repeated in some or all symbols).  Such an approach has obvious advantages in terms of flexibility but cost in configuration overhead.

On the other side a possible configuration is to create a fixed pattern for each comb factor. This pattern could then be truncated or cyclically repeated based on the number of symbols configured.  This is a very compact and efficient way of configuring the SRS.

The patterns proposed allow to fully cover the frequency range over multiple symbols. If the configured number of symbols is less than the pattern size, the appropriate number of symbol are transmitted. The pattern is such that if the transmitted number of symbols is less than the comb size, the transmitted symbols cover the frequency range as evenly as possible.

The SRS configuration for the positioning pattern follows a fixed pattern for each comb size, with a configurable comb offset and number of symbols

The fixed pattern proposed are as follow. Comb 6 and comb 12 are subject to a further agreement:

Figure 1: SRS transmission pattern for comb 2,4 and 6

Figure 2: SRS transmission patterns for comb 8 and 12. Note that comb 8 pattern spans to two RBs.

## Number of symbols in an SRS resource and comb size

The number of symbols in an SRS resource is currently limited to 1,2 or 4 symbols in specifications. During RAN1#96b and RAN1#97, multiple contributions have proposed a higher comb number, with up to comb 12 being proposed. During the RAN1#97 meeting, the following agreements were reached:

Agreement: For positioning, the number of consecutive OFDM symbols in an SRS resource is configurable with one of the values in the set {1, 2, 4, 8, 12} - FFS: Other values including 3,6,14 - Note: Values of 1, 2 and 4 within an SRS resource can already be configured in Rel-15 Agreement: For positioning, the SRS comb size set is extended from {2,4} to {2,4,8} - FFS: Additional comb sizes: 1, 6, 12 - Note: For the comb sizes of 6 and 12, the number of PRBs may be restricted if currently defined sequences are to be used - FFS: Maximum number of cyclic shifts for the different comb sizes (cyclic shifts for comb sizes of 2 and 4 already exist in Rel-15)

Adding symbols to the SRS resource is useful if

- there is a need to accumulate more energy in order to reliably receive the SRS.

- When combed transmission is used, it may also be useful to have additional symbols to expand the TOA range (by using multiple, staggered comb offsets in the resource)

Based on this, it is difficult to motivate the use of additional symbol value beside 6 symbols, which could be useful to allow a full-range transmission with comb-6 if comb-6 is agreed.    Moreover, including an additional value in the list does not cost any additional signaling overhead as the list already requires 3 bits.

Comb-1 is of interest when the scenario does not allow any multiplexing within a symbol in either the comb dimension or in cyclic shifts. UEs allocated with comb 1 would then be time multiplexed in different symbols.   This is the case when the UE speed is such that the channel is not coherent over multiple symbols, and the range over which the UE position should be estimated is about the duration of the symbol. Other cases can be resolve with a combination of comb-based and CS-based multiplexing. Hence, one could see the use of comb-1 as a corner case.

It should be noted that a comb 1 can be realized with the current specification, by specifying two resources with the parameter SRS-SpatialRelationInfo of  the resources involved in creating the comb1 set to the resource ID of one of the SRS resources in the comb.

comb-1 transmission is only required in cases where the UE needs a large TOA range and has a very short coherence time (high speed)

Comb-1 transmission can already be supported by specification

During RAN1#96b, the extension to comb 6 and comb 12 was also discussed. Comb 6 may be of interest in order to schedule multiple resources over time in a slot. Compared to comb-8, comb-12 would be interesting to add capacity in number of multiplexed UE per symbols. Also, as mentioned in our contribution on DL PRS, higher comb values are more overhead efficient. For the same amount of UEs to schedule, and the same target accuracy and TOA range, a larger comb will yield less overhead.

One concern was that the current description of the SRS sequence is not compatible with these comb sizes for the case of low SRS bandwidth, as shown in table 1:

Table 1 comb factor size and minimum PRB bandwidth

| Comb factor | Sequence length for min PRB bandwidth of 4 PRBs | Minimum PRB bandwidth for existing sequence length (sequence length in parenthesis) |
| --- | --- | --- |
| 1 | 48 | 4PRBs (sequence according to 5.2.2.1 in 38.211) – no need for CGS |
| 2 | 24 | 4PRBs (CG sequence according to table 5.2.2.2-4 in 38.211) – CGS needed up to 4PRBs, sequence according to 5.2.2.1 in 38.211 afterward. |
| 4 | 12 | 4PRBs (CG sequence according to table 5.2.2.2-2 in 38.211) – CGS needed up to 8PRBs, sequence according to 5.2.2.1 in 38.211 afterward. |
| 6 | 8 | 12 PRBs (CG sequence according to table 5.2.2.2-4 in 38.211) – no CGS available for 16 PRBs - sequence according to 5.2.2.1 in 38.211 afterward. |
| 8 | 6 | 4PRBs (CG sequence according to table 5.2.2.2-1 in 38.211) |
| 12 | 4 | 12 PRBs (CG sequence according to table 5.2.2.2-2 in 38.211). afterward, CG sequences available for size of 24PRBs, then from 36PRBs according to 5.2.2.1 in 38.211 |

From table 1, it can be seen that comb 6 and comb 12 can be used for larger bandwidth than the current minimum bandwidth of 4 PRBs. It is therefore proposed to allow comb 6 and 12, with the following conditions:

It is possible to use comb 6 for SRS transmission, with the minimum SRS bandwidth to be a multiple of 12 PRBs up to 20 PRBs, and 4PRBs afterward

It is possible to use comb 12 for SRS transmission, with the minimum SRS bandwidth to be a multiple of 12 PRBs up to 24PRBs, and 4 PRBs afterward.

Support comb-6 for SRS transmission

The number of symbols currently supported for transmission of an SRS resource is sufficient for the IOO and UMi channel scenarios

the number of SRS symbols per resource is extended to   1,2,4,6 and 8 and 12, if comb-6 is to be supported.

The minimum SRS bandwidth is set to 4 ressource blocks except

  - For comb size 6, the SRS bandwidth shall be a multiple of 12 RBs up to 20PRBs, and 4PRB afterward

  - For comb size 12, the SRS bandwidth shall be a multiple of 12 RBs up to 24 PRBs and 4 PRBs after that

Number of cyclic shifts for SRS

In the uplink, each UEs is assigned a specific SRS resource for transmission. In order to accommodate as many UEs as possible in the shortest possible time, it is thus of interest to increase the multiplexing of UEs over a single symbol. This can be of interest especially in industrial indoor scenarios where the deployment is favorable to using relatively high combs and number of cyclic shifts and many UEs will share the resources. In a typical 300sqm hall, it is not unreasonable to expect hundred to thousands of UEs managed. In order to allow an efficient use of the time frequency resource, the positioning reference signals cannot take too much of the resource allocation, and therefore multiplexing should be considered when possible.

Comb-based and cyclic shift-based transmission essentially do the same thing – separating the UEs by allocating a certain portion of the SRS symbol transmission time.    For indoor industrial scenarios at least, a short channel spread is expected so that a large amount of cyclic shift/combs could be used without interfering signals between UEs.  In our downlink contribution [2], we discuss the issue of cyclic shifts and combs for DL PRS. The same discussion could be done for UL SRS. The number of UL SRS cyclic shift is specified to be up to 8 for comb-2 and 12 for comb 4, respectively, based on a certain delay spread assumption and cell size as well as the combed symbol unaliased range. Based on the previous discussion, most use cases of interest to SRS-based positioning are indoor, that is to say with a small cell size and delay spread. In this case a tighter spacing of cyclic shifts could be realized, with the largest amount of available cyclic shift being when a full range symbol (i.e. comb-1 or full staggered comb pattern) is available, in which case the cyclic shifts could be distributed over the full symbol range.

Table 1 from [2] is reproduced below for convenience and shows the number of orthogonal cyclic shift signals tolerating a certain delay for different numerologies, based on a full-range symbol availability. Patterns with M<N symbols for a comb-N SRS resource will have a subset of these cyclic shifts available, due to the reduced unaliased symbol duration.

As seen in table 3, the available number of cyclic shifts, at least for FR1, is much larger than the currently configurable values for SRS.   Note that the table consider a comb-1 signal where the whole signal duration can be exploited. Currently, specifications limit the number of cyclic shifts to 8 in comb 2 (i.e. up to 16 UE can be potentially multiplexed within the SRS symbol duration) and 12 for comb 4 (48 UEs multiplexed in the SRS). In our calculation, considering the delays occurring in an indoor scenario, up to about 130 UEs could be multiplexed in a single symbol (at SCS 15kHz). This could be achieved with e.g comb 8 and 24 cyclic shifts (where only a subsets of shifts could be used, up to 133 out of 192), comb 4 and 48 shifts, comb 2 and 96 shifts, etc. Hence depending on comb value, the Maximum number of cyclic shifts nSRScs,max should be increased accordingly to reach the maximum available number of CS for the delay incurred by the scenario. Moreover, in order to avoid issues with legacy, the new maximum number of cyclic shifts should be a multiple of the legacy. Thus it is proposed to increase nSRScs,max possible values to also include 24 and 48.

The  possible values for the maximum number of cyclic shifts for SRS is increased to [8,12,24,48]

In the current specification, nSRScs,max is hardcoded to the value of the comb size. While that may be valid for other use, for the purpose of positioning, the connection  between the comb size and the maximum number of cylic shift could be removed and instead nSRScs,max should be configured as part of the resource configuration.

Maximum number of cyclic shifts nSRScs,max and comb size should be independently configured at the resource set level.

The actual cyclic shift of a SRS resource should  be configured by the parameter nSRScs∈0,1,2,…,nSRScs,max-1.

Table 3 Number of orthogonal cyclic shift signals tolerating a certain delay for different numerologies.

|  |  | Number of orthogonal cyclic shift signals tolerating a given delay (assuming comb-1, full bandwidth SRS) |  |  |
| --- | --- | --- | --- | --- |
| Subcarrier spacing | OFDM symbol length without CP | Max 0.5 µs delay (150m) | Max 1 µs delay (300m) | Max 1.5 µs delay (450m) |
| 15 kHz | 66.7 µs | 133 | 67 | 44 |
| 30 kHz | 33.3 µs | 67 | 33 | 22 |
| 60 kHz | 16.7 µs | 33 | 17 | 11 |
| 120 kHz | 8.3 µs | 17 | 8 | 6 |

## SRS usage

During the RAN1 97 meeting, an agreement was reached to have a new usage for positioning in SRS. The pre-existing (rel-15) usages for positioning are as follow:

- nonCodebook is aimed, as the name suggests, at enabling non-codebook based PUSCH transmission. This SRS usages is restricted to a single SRS resource set made of up to 4 SRS resources. This SRS configuration is aiming at giving the possibility for the network to confirm or revise the choice of PUSCH precoding by the UE, so that the network can respond by downselecting some of the layers (precoders) selected by the UE via the SRI field in DCI.

- Codebook: is aimed at enabling codebook-based transmission of PUSCH. In this usage the SRS is transmitted for reciprocity-based channel sounding, and the network responds to the SRS transmission by sending the suitable precoding matrix to the UE.  Only a single resource set may be configured with up to two SRS resources.

- beamManagement: is aimed at identifying suitable beam candidates. In this usage, only one resource per resource set may be used at a given time instant.

- Antenna switching: is aimed at reciprocity-based DL CSI acquisition via SRS carrier switching.

The SRS for positioning will use most likely use multiple resource sets, with power control configurations targeting different cells based on the path loss between the UE and the measuring cell. Within the resource set, at least in FR2, several resources (beams) may be configured to align with the measuring cell.

Define a SRS usage for positioning where

  - The UE may be configured with more than 1 resource set, each with more than one resource

    - FFS maximum amount of resource sets

  - Same Resource may be allocated to more than one resource set

  - SRS transmission follow the agreed patterns for positioning

Conclusions

In the previous sections we made the following observations:

Observation 1	comb-1 transmission is only required in cases where the UE needs a large TOA range and has a very short coherence time (high speed)

Observation 2	Comb-1 transmission can already be supported by specification

Observation 3	It is possible to use comb 6 for SRS transmission, with the minimum SRS bandwidth to be a multiple of 12 PRBs up to 20 PRBs, and 4PRBs afterward

Observation 4	It is possible to use comb 12 for SRS transmission, with the minimum SRS bandwidth to be a multiple of 12 PRBs up to 24PRBs, and 4 PRBs afterward.

Observation 5	The number of symbols currently supported for transmission of an SRS resource is sufficient for the IOO and UMi channel scenarios

And the following proposals:

Proposal 1	The SRS for positioning is configured by the UE serving cell via RRC

Proposal 2	The SRS for positioning   UE configuration is communicated to the UE neighbor (non-serving) cell via NRPPa from the location server

Proposal 3	The SRS configuration for the positioning pattern follows a fixed pattern for each comb size, with a configurable comb offset and number of symbols

Proposal 4	Support comb-6 for SRS transmission

Proposal 5	the number of SRS symbols per resource is extended to   1,2,4,6 and 8 and 12, if comb-6 is to be supported.

Proposal 6	The minimum SRS bandwidth is set to 4 ressource blocks except

o	For comb size 6, the SRS bandwidth shall be a multiple of 12 RBs up to 20PRBs, and 4PRB afterward

o	For comb size 12, the SRS bandwidth shall be a multiple of 12 RBs up to 24 PRBs and 4 PRBs after that

Proposal 7	The  possible values for the maximum number of cyclic shifts for SRS is increased to [8,12,24,48]

Proposal 8	Maximum number of cyclic shifts nSRScs,max and comb size should be independently configured at the resource set level.

Proposal 9	The actual cyclic shift of a SRS resource should  be configured by the parameter nSRScs∈0,1,2,…,nSRScs,max-1.

Proposal 10	Define a SRS usage for positioning where

o	The UE may be configured with more than 1 resource set, each with more than one resource (note: may not have a spec impact)

i.	FFS maximum amount of resource sets

o	Same Resource may be allocated to more than one resource set

o	SRS transmission follow the agreed patterns for positioning

References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019

- R1- 1909425  DL Reference Signals for NR Positioning, Ericsson, RAN1#97 Reno, USA, 2019