3GPP TSG RAN WG1 #98	R1- 1909426

Prague, CZ, August 26th – 30th 2019

Source:	Ericsson

Title:	UE and gNB measurements for NR positioning

Agenda Item:	7.2.10.3

Document for:	Discussion, Decision

# Introduction

The following were measurement related agreement in RAN WG1 #96 and #97 meetings.

RAN1 #97: Agreement: Select one (or more) of the following options for the definition of the RSTD - Option 1: RSTD is defined as the time difference with respect to the subframe timings associated with the different TPs; - Option 2: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resource sets of different TPs - Option 3: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resources of different TPs - Note: Definition of the RSTD applies at least to the RSTD measurements obtained from DL PRS. It does not preclude the RSTD obtained from other DL RS. Agreement: - For FR1, the reference point of the RSTD measurement is the Rx antenna connector of the UE - For FR2, RAN1 has considered the following options as the reference point of the RSTD measurement. The final decision is up to RAN4 on how to define the reference point of the RSTD measurement. - Option 1: the combined signal from antenna elements corresponding to a given receiver - Option 2: the Rx antenna of the UE Agreement: DL PRS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port that carry a DL PRS resource configured for RSRP measurements within the considered measurement frequency bandwidth - FFS: DL PRS-RSRP for 2 ports, if 2 port transmission of a DL PRS is agreed. Agreement: - For FR1, the reference point for the DL PRS-RSRP is the Rx antenna connector of the UE; - For FR2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch Agreement: When receiver diversity is in use by the UE to obtain the DL PRS-RSRP measurement, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches Agreement: Support inter-frequency DL PRS-RSRP measurements for NR positioning in Rel-16 when the UE is in RRC_CONNECTED state. Agreement: UE can be indicated to report the DL PRS Resource ID(s) or the DL PRS Resource set ID(s) associated with the DL PRS Resource ID(s) or the DL PRS Resource set(s), which are used in determining the UE measurements (DL RSTD, UE Tx-Rx time difference, DL PRS-RSRP) in UE measurement reporting. Agreement: - Support reporting separate metric(s) corresponding to quality of each of the following measurements - RSTD - UE Rx-Tx time difference - UL-RTOA - gNB Rx-Tx time difference - UL angle of arrival, including the azimuth of arrival (AoA) and the zenith of arrival (ZoA) - FFS: The details of these quality metrics RAN1 #96: Agreement:​ Intra-frequency and inter-frequency DL RSTD measurements (currently supported to be based at least on DL PRS) are supported in RRC_CONNECTED mode ​ ​Agreement:​ At least intra-frequency DL PRS-RSRP measurements are supported in RRC_CONNECTED mode​ - FFS: Inter-frequency DL PRS-RSRP measurements in RRC_CONNECTED mode​ Agreement:​ - The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements. ​ - A DL PRS Resource ID ​ - A subset of DL PRS Resource IDs from a single DL PRS Resource set​ - A DL PRS Resource set​ Agreement:​ - The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference.

The work item description for NR positioning support mandates RAN1 centric objectives for NR positioning. One of the objectives is to define UE and gNB based measurements for positioning to support RAT dependent positioning methods in different scenarios. These measurements are done over the reference signals that are defined (or will be defined) in NR. The NR positioning measurements will be defined over resources and  resource sets that map to occasions.

In this contribution, we present our view on positioning measurements that are required to be done either by a UE or a base station depending on the positioning method being employed.

# 2 UE measurements

UE measurements for positioning in LTE are handled either via LPP or via RRC+LPPa, and a similar architecture can be anticipated also for NR. Some UE measurements already defined in Rel 15 are also relevant for positioning, while we will also agree on and define new measurements for positioning in Rel. 16. Therefore, this section is split into two subsections, with the first one addressing existing Rel. 15 UE measurements, and the second discusses new measurements in Rel. 16

## 2.1 Rel. 15 UE NR measurements

There are several positioning related NR measurements among the ones defined in Rel. 15 [4].  These include:

- SS-RSRP, SS-RSRQ and SS-SINR of serving and intra-frequency neighbor cells

- SS-RSRP, SS-RSRQ and SS-SINR of inter-frequency cells

- SS-RSRPB of intra-frequency cells (only FR2)

- CSI-RSRP, CSI-RSRQ and CSI-SINR of intra- and inter-frequency cells

These are similar to the ones defined for LTE as part of enhanced cell ID (E-CID).

Existing Rel. 15 UE NR measurements includes positioning relevant measurements that are suitable for NR E-CID.

Support UE reporting of the Rel. 15 NR measurements for positioning

Send an LS to RAN2 to inform about the agreed reporting of Rel 15 UE NR measurements

## 2.2 Rel. 16 UE NR Measurements

### 2.2.1 UE DL ToA measurement

Several UE measurements are based on time of arrival (ToA) measurements. These are typically related to the start of a received slot, subframe, radio frame etc. The UE will use configured DL signals to estimate the ToA, but the estimated ToA is not reported to any network node.

The ToA measurement is only discussed as a means to discuss and align definitions of UE measurements that the UE will be configured to report.Here, we define ToA as the timing of a subframe i from a cell, defined by the first detected path in time. The path can be associated to an uncertainty as well as a received power level etc. Other detected paths due to multipath propagation from the same cell constitute the additional paths associated to the cell. Each of these paths are associated to a relative time difference (RTD) to the first detected path, and can also be associated to an uncertainty, a power level, etc. See Figure 1 for an example of the definition.

Figure 1. Example of a ToA measurement with (P-1) additional paths.

### 2.2.2 UE RSTD and RTD measurements

Reference Signal Time Difference (RSTD) is one of the major measurements that UE may report to support multitude of positioning methods in NR. In this section, we elaborate on the definition of RSTD measurement and how UE should report this measurement.

The UE RSTD and RTD measurements can be used in methods like DL-TDOA, single base station based positioning, E-CID etc. As in LTE, a single RSTD measurement in NR can also be defined as relative timing difference between the neighbour cell j and the reference cell i, defined as TsubframeRxj – TsubframeRxi, where TsubframeRxj is the time when the UE receives the start of one subframe from cell j, TsubframeRxi is the time when the UE receives the corresponding start of one subframe from E-UTRAN cell i that is closest in time to the subframe received from cell j. The reference point for the observed subframe time difference shall be the antenna connector of the UE [38.215]. Moreover, as in LTE, the RSTD measurements can include one or several paths of the received DL signals to provide the multipath timing profile. The measurements provide the multipath timing profile at individual base stations, RTD and the relative timing between base stations (RSTD) as shown in Figure 1. The peaks shown in the figure are the channel impulse response (CIR) peaks obtained from the received signal.

As shown in the Figure 2, measurements can be defined as RSTD measurements and RTD measurements.

- The RSTD measurements are time difference measurements between first path of a neighbour cell and first path of the reference cell. The number of detected neighbour cells N can be further restricted by a configured max number of neighbours.

- RTD measurements are time difference measurements of the multipath paths from any specific cell. With P paths detected, there can be up to P additional paths per cell considered, unless there is a maximum number of additional paths configured.

Figure 2 Relative time difference (RTD) from additional paths and RSTD measurements

Large number of paths considered for RTD measurements, P, would be part of the rich reporting which can be used for positioning methods based on analyzing multipath components for UE positioning. The values of N and P can be decided based on the UE capabilities.

Figure 3. Rich reporting from UE to location server for robust position estimation

The figure 3 shows the information flow in a typical positioning framework. As shown in the picture, to compute position of a UE in network-based approach, two processing/estimating engines are needed. One on the UE side for estimating TOA and another on the network side to estimate UE position. For robust and precise positioning estimation, information additional to the TOA estimates from the UE will be required, shown as robustness information in the Fig.3.  Such information from the UE is useful in weighing TOA estimates from the UE with several base stations. A few such information can be

- If the UE is in line-of-sight (LOS) or non-line-of-sight (NLOS) with a base station.

- Power levels of the peaks in the channel impulse response used for computing TOA estimates by the UE as well as the noise/interference level.

- Magnitude and timings of multiple peaks.

Figure 4 Positioning estimation comparison of one vs. multiple RSTD reporting for LTE from [5].

The benefit of reporting multiple paths was evaluated already for LTE for OTDOA based positioning e.g. in [5] and was seen to give significant gains in positioning accuracy (see Fig.4). Here we want to stress the particular importance of also reporting the strongest peak. For weak signals from distant transmission points it’s often impossible to detect the first peak reliably while the strongest peak can still be detected. In such cases the distance to the transmission point can’t really be estimated but the location of the strongest peak still holds important information. The line of sight distance to the transmission point can’t be larger than the propagation delay for the strongest peak times the speed of light. This information can be used to reject time of arrival estimates that are in contradiction to this inequality or in other words to reject outliers. In Fig.5 we see significant improvement in positioning accuracy from reporting of the estimated first peak along with the strongest peak as compared to only reporting the detected first peak. We propose that measurement and reporting of the strongest peak should be configurable and indicated in the UE measurement report.

UE should be configured to measure and report the position of strongest peak along with the detected first peak. The RSTD measurement report should indicate which of the reported peaks is the strongest peak.

Figure 5 Numerical validation of the positioning estimation accuracy when first peak + strongest peak is reported by the UE and when only the first peak is reported by the UE.

The resolution of the RSTD measurements by UE has to be specified. Precision of RSTD measurements w.r.t. the true values of these measurements should be set as requirement by the network. The network can configure number of strongest beams (Q) for which above measurements should be reported. Along with the beam/resource IDs.

Figure 6 Finer RTD around the desired peak

It should be possible to focus the RTD measurements around certain peaks, as shown in the Fig.6. As is well known that for positioning the first peak is of most importance. Hence finer RSTD measurements around the first peak would contribute more to the positioning accuracy than RSTD measurements of later peaks. The network should be able to configure finer RSTD measurements of the CIR at the UE side.

The measurements and measurement reporting rates should also be configured by the location server. This can be done on a scenario basis. For example, the location server may require higher rates of measurement reporting to be able to provide measurements to a moving vehicle.

The resolution of RSTD measurement reporting has to be specified.

UE reports RSTD between neighbour and reference cells, and additional paths for neighbour and reference cells if detected.

The network configures values N, P and Q for the measurements to be performed and reported by the UE

The maximum value for the number of cells in an RSTD measurement N , number of measured paths per cell P and number of reported paths per cell Q are a the UE capability .

Value of P should be decided based on number of peaks required for positioning method such as, single base station based positioning, fingerprinting, multipath based positioning.

Network should be able to configure finer RSTD measurements in CIR around any peak.

The measurement rates and measurement reporting rates should be configured by the network.

Requirements on the measurement precision (RSTD precision) should be set by the network based on the required time of arrival accuracy.

### 2.2.2	UE RSRP measurements

The Reference Signal Received Power (RSRP) measurements are used for positioning methods like E-CID based positioning, received signal strength (RSS) based positioning etc. RSRP measurements over different reference signals provide measures of signal strength and quality for communication purposes. For positioning, the RSRP measurements can also provide measure of timing measurements. Power levels of every path being reported would help the network in improving the position estimation of the UE.

RSRP measurements should be defined over the specified NR-PRS signal and will be called NRPRS-RSRP. It could be defined as the linear average over the power contributions (in [W]) of the resource elements that carry PRS reference signals configured for positioning measurements within the considered measurement frequency bandwidth in the configured PRS occasions.

The RSRP measurements should be specified for every RSTD measurement corresponding to a peak in the CIR. The values of the peaks could be defined as the power contributions (in [W]) of the PRS resource elements for a certain delay value. As shown in the Fig.7, the power peaks can be reported as [h1 h2 … hp hp-1]. In LTE, additional path is reported by the UE which includes the RSTD values of additional path and the path quality measure [2].

In NR, in addition, dynamic range of every path should also be reported. The dynamic range of every path can be defined as the difference of the peak power to the noise floor of the correlation output. The noise floor of the correlation output consists of thermal noise, diffuse scattering components, contribution due to nonlinearity, aliasing etc. So, it is important to know how clear is a peak with respect to the noise floor around it.

Dynamic range of peaks in the power delay profile should be reported along with the RSTD measurements on these peaks.

Figure 7 A typical CIR and the power measurements at every peak.

### 2.2.3	UE RX-TX time difference measurements

In order to support positioning method that rely on round trip time (RTT), UE Rx-Tx measurement would be required.

Figure 8 UE Rx-Tx and gNB Rx-Tx measurements

The UE Rx – Tx time difference can be defined as TUE-RX – TUE-TX, where TUE-RX is the UE received timing of downlink radio frame i from the serving cell, defined by the first detected path in time and TUE-TX is the UE transmit timing of uplink radio frame i. The reference point for the UE Rx – Tx time difference measurement shall be the UE antenna connector. UE Rx-Tx and gNB Rx-Tx measurements can be seen in the Fig.8.

In RTT methods, systematic delay errors at the UE affects the positioning estimation more than in TDOA based methods. Similarly, knowledge of the delay distribution at the UE side can be used in the position estimation algorithms to improve the positioning accuracy. Hence, a measure of UE Rx-Tx measurement should also be reported along with the measurements. The measure can be the distribution of the UE Rx-Tx measurement.

A quality measure of UE Rx-Tx measurement should be reported by the UE along with the UE Rx-Tx measurements.

Figure 9  Performance of a RTT method as a function of errors in Rx-Tx measurements in IOO scenario.

Higher variance of Rx-Tx measurements error affects the positioning performance as shown in Fig.9. In RAN#97 quality of Rx-Tx measurement was mentioned as FFS. As shown in the Fig.9, Standard deviation of Rx-Tx measurements should be reported as quality measure of these measurements. While reporting the Rx-Tx measurement. The signal configuration over which these measurements are done should also be reported.

Standard deviation of UE Rx-Tx measurements in terms of Ts should be reported.

The UE Rx – Tx time difference can also be used to calculate the RTT based on DL PRS RSTD measurements and UL SRS TOA measurements. For this to work the UE Rx – Tx time difference measurements should be based on the first reported peak of the reference cell in the DL PRS RSTD measurement. It therefore makes sense to include the UE Rx – Tx time difference relative to the first reported peak of the reference cell in the DL PRS RSTD measurement report.

Include the UE Rx – Tx time difference relative to the first reported peak of the reference cell in the DL PRS RSTD measurement report.

The accuracy of an RTT estimate calculated based on DL PRS RSTD measurements including UE Rx – Tx time difference and UL SRS TOA measurements will be impacted by UE clock drift in between the DL PRS reception and the SRS transmission. Since the UE oscillator is tuned towards the serving cell RX frequency the UE clock will be impacted by doppler shifts and during DRX it will be impacted by UE oscillator frequency drift. A doppler shift results in a maximum time drift ∆t=vc∙t while a UE oscillator frequency drift results in a maximum time drift ∆t=p∙t22, where p is the maximum frequency drift. If the DL PRS and the UL SRS are scheduled at a large time distance from each other this time drift will impact the RTT and positioning accuracy. This can potentially be a problem when targeting high positioning accuracy when the UE velocity is very high. Typically the time drift is however small enough to allow very good RTT and positioning accuracy.

## 2.2.5 	UE measurement processing

TOA estimates in the UE are typically based on the estimated (bandwidth-limited) channel impulse response (CIR) of the received PRSs. At a basic level, such a CIR estimate is naturally obtained with integer-sample resolution, which limits the achievable TOA accuracy to about +/- 0.5 sample, if interpolation is not used in addition. However, by interpolation of the integer-resolution CIR all intermediate points of the time-continuous CIR can in principle be found with very high accuracy. This can be achieved using well-known quasi-perfect interpolation schemes such as with an appropriately-designed FIR filter. In the absence of noise, these “true” values can then be found with arbitrary precision.

In this way the TOA of an isolated single path, without noise, may be estimated with extremely high accuracy by identifying the timing of the impulse peak. Of course, with noise (and interference) the achievable TOA accuracy is limited by this noise, so there is in practice a limit to how much increasing the interpolation resolution “pays off” in terms of in worthwhile increased TOA accuracy – obviously a higher TOA accuracy gain is achievable by interpolation at higher SINRs than at lower SINRs.

There is also an issue when there are several paths that are closely spaced in time, in which case there are typically limits how well these may be discriminated, which introduces an additional source of inaccuracy for the TOA estimation.

From a complexity perspective it should be noted that only the time zones of interest, around the relevant candidate TOAs, need to be interpolated, i.e. the full CIR does not need to be interpolated. This reduces very significantly the required complexity.

In Fig.10 an example is shown of the very significant positioning gain obtained using an interpolation factor 4 for the IOO scenario.

Fig. 10 – Example of increased positioning accuracy achieved by using a factor 4 interpolation compared to using only integer sample resolution

By using interpolation, the accuracy of the estimated TOA, and associated positioning, may be very significantly improved, compared to using no interpolation, i.e. based on integer sample resolution.

It is well-known that the accuracy of TOA estimates is fundamentally important for the resulting positioning accuracy. It is also the case that by using interpolation these TOA estimates may, in many cases, be substantially improved. It is therefore desirable that UEs implement appropriate strategies for improved TOA accuracy. The potential of such improved TOA accuracy needs to be considered by RAN4 when specifying requirements for RSTD measurements accuracy, or any other measurements that also depend on TOA accuracy.

Future RAN4 requirements on RSTD, and other TOA-based measurements, need to take into account the performance achievable when interpolation is used.

RAN1 to inform RAN4 about the benefits of using interpolation in connection with TOA estimation.

# 3	gNB measurements

To facilitate UL based positioning solutions, gNB needs to do the required time and angle measurements by using the UL reference signals. In [3] we have presented candidate reference signals that are suitable for UL NR positioning. In this section, we will elaborate on our view on measurements that gNB can perform within a positioning occasion for UE positioning.

## 3.1 gNB Rx-Tx time difference

The gNB Rx-Tx time difference measurement facilitates positioning solutions based on UL RTT exploiting either a single or multiple base stations. The gNB Rx-Tx time difference defines the time difference between when the gNB detects the first path in time of the UL radio frame with UL positioning reference signal and the timing when gNB transmit DL radio frame with DL positioning reference signal.

## 3.2 Timing advance (TADV)

Timing advance is one of the time measurements in UL direction that defines gNB Rx-Tx time difference with or without UE Rx-Tx time difference. Depending on the configuration, TADV can be defined/calculated as

TADV = gNB Rx-Tx time difference + UE Rx-Tx time difference

or

TADV = gNB Rx-Tx time difference.

## 3.3 Angle of Arrival (AoA)

In a beamformed UL positioning reference signal transmission, the gNB in addition to first path detection of the radio frame containing the UL positioning reference signal can also estimate the angle of arrival (AoA) of such signals. The AoA measurement is done by the gNB with respect to a reference direction. In positioning solutions like uTDOA, additional information based on AoA can enhance the TOA measurement accuracy. Moreover, a single base station based positioning solution can also exploit this info for UE localization.

## 3.4 UL Relative Time of Arrival (TUL-RTOA)

The UL relative time of arrival is a time measurement that can be done at gNB for the time difference between beginning of subframe i containing the UL positioning reference signal and the beginning of subframe j with UL positioning reference signal. This sort of measurement is required when network based positioning method is being employed.

Define gNB Rx-Tx time difference, timing advance, AoA and UL RTOA as gNB measurements for NR positioning.

# Conclusions

In the previous sections we made the following observations:

Observation 1	Existing Rel. 15 UE NR measurements includes positioning relevant measurements that are suitable for NR E-CID.

Observation 2	The resolution of RSTD measurement reporting has to be specified.

Observation 3	By using interpolation, the accuracy of the estimated TOA, and associated positioning, may be very significantly improved, compared to using no interpolation, i.e. based on integer sample resolution.

Observation 4	Future RAN4 requirements on RSTD, and other TOA-based measurements, need to take into account the performance achievable when interpolation is used.

And the following proposals:

Proposal 1	Support UE reporting of the Rel. 15 NR measurements for positioning

Proposal 2	Send an LS to RAN2 to inform about the agreed reporting of Rel 15 UE NR measurements

Proposal 3	UE should be configured to measure and report the position of strongest peak along with the detected first peak. The RSTD measurement report should indicate which of the reported peaks is the strongest peak.

Proposal 4	UE reports RSTD between neighbour and reference cells, and additional paths for neighbour and reference cells if detected.

Proposal 5	The network configures values N, P and Q for the measurements to be performed and reported by the UE

Proposal 6	The maximum value for the number of cells in an RSTD measurement N , number of measured paths per cell P and number of reported paths per cell Q are a the UE capability .

Proposal 7	Value of P should be decided based on number of peaks required for positioning method such as, single base station based positioning, fingerprinting, multipath based positioning.

Proposal 8	Network should be able to configure finer RSTD measurements in CIR around any peak.

Proposal 9	The measurement rates and measurement reporting rates should be configured by the network.

Proposal 10	Requirements on the measurement precision (RSTD precision) should be set by the network based on the required time of arrival accuracy.

Proposal 11	Dynamic range of peaks in the power delay profile should be reported along with the RSTD measurements on these peaks.

Proposal 12	A quality measure of UE Rx-Tx measurement should be reported by the UE along with the UE Rx-Tx measurements.

Proposal 13	Standard deviation of UE Rx-Tx measurements in terms of Ts should be reported.

Proposal 14	Include the UE Rx – Tx time difference relative to the first reported peak of the reference cell in the DL PRS RSTD measurement report.

Proposal 15	RAN1 to inform RAN4 about the benefits of using interpolation in connection with TOA estimation.

Proposal 16	Define gNB Rx-Tx time difference, timing advance, AoA and UL RTOA as gNB measurements for NR positioning.

# References

- RP-182155, Study on NR positioning support, RAN1, Intel Corporation, Ericsson, June 2019.

- 3GPP TS 36.355 V14.2.0, LTE Positioning Protocol (LPP) (Release 14), June 2017

- R1-19NNN, DL and UL Reference Signals for NR Positioning, RAN1. Ericsson, April 2019.

- 3GPP TS 38.215 V.15.5.0, NR; Physical layer measurements

- R1-167663, Multipath RSTD reporting per cell for enhanced indoor positioning, RAN1, Ericsson, August 2016