3P TSG RAN WG1 Meeting #97																R1- 1909427

Prague, Czech Republic, August 24th – 29th, 2019

Source:	Ericsson

Title:	Physical-layer procedures in NR positioning

Agenda item:	7.2.10.4

Document for:	Discussion and decision

Introduction

In this contribution, we address the remaining issues related to the physical layer procedures in NR positioning.

Procedures for positioning

## DL PRS related procedures

the following agreement have been made in previous meeting regarding the DL PRS procedures:

Agreement: For intra-frequency measurements, when measurement gaps are configured, the UE is expected to measure the DL PRS resource outside the configured (active and inactive) BWPs (Option 2 in the related Agreement from RAN1 #96-Bis). Note: The UE has flexibility to measure only on a part of the DL PRS configured bandwidth as long as measurement requirements are met. Agreement: For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported (options are from previous related agreement in RAN1#96bis): - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts.

In addition to QCL type D relation, the remaining QCL types A,B and C could also be considered. It is of interest for the UE to know the relation between the DL PRS and the cell other reference signals. The TOA estimation will benefit from knowledge of the doppler shift, doppler spread, average delay and delay spread information.  In order to establish the PRS search window, QCL information can give input to the UE in terms of coherence time and delay distribution (doppler spread, average delay. It is thus propose to extend the QCL information for PRS in relation to the SSB and CSI RS to the other QCL types:

For positioning purposes, to assist UE to acquire the PRS TOA, the DL PRS QCL types A,B and C relation to DL Reference Signals from a serving or neighboring cell can be reported to the UE in the PRS configuration

  - The reference signals QCL status to be reported can be at least SSB, CSI-RS

  - FFS other reference signals

UL SRS related procedures

Transmission timing and timing adjustments

UL timing is controlled via a TA adjustment to ensure all uplink transmission in a cell arrive aligned at the gNB. There has been discussion on whether to have separate TA values for the neighbor cell, independent of the serving cell, for the purpose of positioning. The issue with such methods is that  a separate TA value is of interest only if the TA required exceeds the CP length. However, at these values the transmitted SRS is no longer orthogonal in the serving cell, where the path loss to the gNB is the lowest and therefore the impact of loss of orthogonality will be perceived the most. Conversely, a serving cell-based TA that results in a  misaligned SRS in the neighboring cell will experience a comparatively large path loss, so that the added interference is low compared to the one a serving cell would experience with a neighbor cell-based TA.

TA for SRS is following the serving cell timing.

Power control

The previous meetings  yielded the following options for power control of the UL SRS in positioning:

RAN1#96b: Agreement For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified: - Option 1: The UL SRS Tx power is constant (i.e., no power control is supported). - FFS: The value of the constant Tx power and how it is determined - Option 2: The UL SRS Tx power is based on the existing power control procedure. - Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example: - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - FFS: which power control modes are supported (e.g., open loop only, or open loop and closed loop).

RAN1#97 Agreement: For positioning purposes, with regards to the UL SRS transmission power, support at least the following in addition to the existing Rel-15 behaviour: - Support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - Study further the UE configuration signalling and procedure. - FFS on a fallback procedure if the UE is not able to obtain the pathloss reference. - FFS on number of measurements for pathloss.

The SRS is RRC configured by the UE serving cell. To configure power control, the following RRC configuration is in place at  the resource set level:

alpha Alpha 																						OPTIONAL, -- Need S

p0 INTEGER (-202..24) 																			OPTIONAL, -- Cond Setup

pathlossReferenceRS CHOICE {

ssb-Index SSB-Index,

csi-RS-Index NZP-CSI-RS-ResourceId

} 																										OPTIONAL, -- Need M

The estimation of the neighbouring cell path loss can be based on CSI-RS or SSB, and potentially from DL-PRS on the neighbor cells, based on previous agreement. To allow this, the location server shall communicate the resource IDs for the agreed reference signals from the neighbor cell via NRPPa to the serving cell so that the RRC config of SRS can be completed in the serving cell.

To support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control, The neighbor cell reference signals resources IDs are communicated to the serving cell in NRPPa.

As can be seen there is no additional impact on the RRC configuration of SRS to support the configuration of a neighbor cell reference signal. However, measuring the path loss in neighbor cell may pose a hearability problem for neighbor cells that are far away from the UE. In this case the path loss estimate may be such that the derived power will reach the maximum output power of the UE. In order to create a manageable level of interference, it may be of interest to cap the transmitted power to a value in relation to the power that the UE would have used to reach the serving nodeB.

When the SRS transmission is configured with a neighbor cell RS for PL calculation, the UE maximum transmission power may not exceed a value of P_SRS_offset dB compared to computed power required with the serving cell path loss.

    - FFS value of P_SRS_offset

Conclusions

In the previous sections we made the following proposals:

Proposal 1	For positioning purposes, to assist UE to acquire the PRS timing, the DL PRS QCL type C relation to DL Reference Signal from a serving or neighboring cell can be reported to the UE in the PRS configuration

o	The reference signals QCL status to be reported can be at least SSB, CSI-RS

FFS other reference signals

o

Proposal 2	TA for SRS is following the serving cell timing.

Proposal 3	To support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control, The neighbor cell reference signals resources IDs are communicated to the serving cell in NRPPa.

Proposal 4	When the SRS transmission is configured with a neighbor cell RS for PL calculation, the UE maximum transmission power may not exceed a value of P_SRS_offset dB compared to computed power required with the serving cell path loss.

i.	FFS value of P_SRS_offset

References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019

- R1- 1907508 DL Reference Signals for NR Positioning, Ericsson, RAN1#97 Reno, USA, 2019