3GPP TSG RAN WG1 #98	R1-1909457

Prague, Czech, Aug 26th – 30th, 2019

Agenda Item:	7.2.9.1

Source:	Apple Inc.

Title:	PDCCH based power saving channel design for UE power saving

Document for:	Discussion, Decision

# Introduction

During the RAN1#97, good amount of progress on power saving signal/channel were made with focus on

- Power saving signal/channel monitoring occasion

- Search space set

- Power saving techniques inside and outside Active Time

In this contribution, we discuss the remaining design details on power saving signal/channel for those issues. We further discuss about power saving signal/channel transmission from gNB and UE’s behavior when UE fails to detect such signal. Lastly, as a further extension of power saving signa inside Active Time, we provide evaluation results of PDCCH skipping on top of short DRX and R16 features such as wake up signal and cross slot scheduling.

# Power Saving Signal/Channel Outside Active Time

## PDCCH Monitoring Occasion

During RAN1#97, we made following agreements on monitoring of power saving signal outside Active Time.

Agreements: The monitoring occasion(s) of the power saving signal/channel outside the Active Time is “indicated” to the UE by the gNB with an offset before the DRX ON - “Indicated” implies the explicit signaling by higher layer signaling or implicit through the CORESET/search space - FFS: The value and the range of offset Agreements: Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time - Following the principle of Rel-15 search space configuration - FFS: the corresponding UE behaviour in monitoring the power saving signal/channel outside Active Time - FFS whether UE can further monitor the search space set(s) inside Active Time Agreements: Outside Active Time, the PDCCH-based power saving signal/channel is configured for triggering UE to or not to monitor the subsequent ON duration(s) - FFS a single vs. multiple durations, particularlly checking consistency with RAN2 agreements - …. Agreements: One or more PDCCH monitoring occasion of PDCCH-based power saving signal/channel is supported outside Active Time - FFS: Whether the monitoring occasions of the PDCCH-based power saving signal/channel in the same slot or not for multiple monitoring occasions - FFS: whether the monitoring occasions are via search space set(s), CORESET(s), a combination thereof, etc.

Explicit/Implicit

The offset between power saving signal monitoring occasion and DRX ON duration could be either explicitly indicated or implicitly indicated based on CORSET / search space configuration.

- For “explicit” case, RRC signaling could be used to configure offset directly.

- For “implicit” case which is used when there is no RRC signaling, the default offset value could be used.

The offset could be given in slots. Since the DRX offset is UE specific value, the monitoring occasion determined by implicit value is also UE specific. If the DRX offset is well distributed across different UEs in time such that it minimizes the chance of using the same DRX offset values across different UEs, the chance of two UEs having the same monitoring occasion for power saving signal is also minimized. This method of using offset w.r.t ON duration will automatically distribute UEs across different monitoring occasions.

If network want to multiplex multiples UEs in the same DCI with slightly different DRX offsets to improve resource efficiency, the explicit configuration could be used.

Proposal 1

- If the offset for monitoring occasion for power saving signal outside Active Time is explicitly configured by RRC, UE use the configured value. Otherwise, UE use default offset value and DRX offset to determine the monitoring occasion of power saving signal.

Range

The offset of power saving signal monitoring occasion from the start of DRX ON duration could range from X to Y slots. The offset needs to be large enough to allow UE to prepare PDSCH reception in case the UE is indicated to wake up.

- Small offset : If offset is too small, UE may not fully take advantage of two steps of power consumption level (power level for PDCCH only reception followed by different power level for receiving both PDCCH and PDSCH).

- Large offset : The Y value cannot be too large since then traffic arrival could happen between power saving signal monitoring occasion and start of DRX ON duration. This could introduce unnecessary delay of one DRX cycle for traffic arriving after monitoring occasion. Thus too large offset should be also avoided.

Proposal 2

- NR supports that power saving signal monitoring offset ranges from X to Y ms, where

  - X=[1/16]

  - Y=[5]

Consideration of DRX cycle and ON Duration

There is also impact from DRX parameters. If DRX cycle is very short, e.g., 2ms (short DRX cycle), then it may not be good idea to configure power saving signal for wake up purpose since anyway UE monitors PDCCH. In this case, the power saving gain is marginal due to too short cycle. In general, when CDRX cycle is very short, it is highly likely that ON duration is also very short and the gain of wake up signal gets marginal. So, in case short DRX is configured, depending on the short DRX cycle and ON duration value, UE may or may not monitor power saving signal.

Proposal 3

- If a UE is configured with short DRX with drx-ShortCycle < X and/or drx-onDurationTimer < Y and configured to monitor power saving signal channel outside Active Time, the UE skips monitoring power saving signal channel before ON duration when UE is operating in short DRX mode.

Offset and Dedicated Search Space for Monitoring

Once power saving signal monitoring offset is determined, the dedicated search space for the power saving signal should be also configured such that it is aligned with potential ON duration(s) with the given offset. The UE should monitor only those monitoring occasion(s) which is separated by configured offset from the corresponding ON duration as shown in Figure 1. If the monitoring occasion occurs during Active Time, then the monitoring of power saving signal is skipped. Before inactivity timer expires, it is hard to predict whether the inactivity timer would be extended with new packet arrival or not. So in this case (during Active Time) UE should not monitor power saving signal.

Figure 1 Configured search space set and actual monitoring with short CDRX

Proposal 4

- Dedicated search space for monitoring power saving signal outside Active Time is configured such that it is aligned with potential DRX ON duration based on current DRX configuration (including both short and long).

- There is a configured offset between configured search space and a corresponding potential ON duration.

- Only the power saving monitoring occasions associated with its ON duration is monitored by UE.

One-to-one association between Power saving signal and ON duration

RAN2 conclusion is that “PDCCH-based power saving signal/channel for wake-up purpose is used to indicate to the UE to wake up to monitor the PDCCH during the next occurrence of the drx-onDurationTimer.” The “next” implies the one-to-one association between a power saving signal/channel and ON duration. Since the DRX cycle is determined to guarantee certain level of delay performance, allowing power saving signal to skip multiple ON durations may defeat the purpose of configured DRX cycle. So, the relation should be one-to-one.

Proposal 5

- A power saving signal transmitted outside Active Time is associated only one next ON duration.

## Search Space Set

Regarding the search space and the new DCI monitored outside Active Time, RAN1 made following agreement in RAN1#97.

Agreements: For power saving signal/channel configured outside Active Time, introduce a new DCI format for a UE, where the UE is configured to monitor the DCI format, with the power saving information for the UE in the DCI configurable by RRC - FFS whether the DCI is in UESS or CSS or both - FFS detailed configuration of the power saving information - FFS the new DCI format - Note: the same DCI may carry power saving information for one or more UEs Agreements: A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured - FFS how to use the PS-RNTI for scrambling of the PDCCH-based power saving signal/channel

CSS, PS-RNTI, DCI Format

The first issue to address is whether the DCI is monitored in UESS or CSS or both. The power saving signal channel supports the multiplexing of power saving information for multiple UEs. And, to support a group of UEs, a new PS-RNTI is agreed to be supported. So, it is very natural to consider using common search space set for power saving signal monitoring purpose in which UE monitors a new PDCCH format (say DCI Format 2_4) for power saving. PS-RNTI is assigned to a set of UEs, and the CRC of the new DCI is scrambled with the assigned PS-RNTI. Note that the configured new search space is to be monitored only outside Active Time.

Proposal 6

- Define a new common search space type Type4-PDCCH CSS set for power saving signal monitoring.

- Define a new DCI format DCI Format 2_4 for power saving signal.

- UE can be configured to monitor DCI Format 2_4 with CRC scrambled with PS-RNTI in Type4-PDCCH CSS set.

- The configured search space Type4-PDCCH CSS is monitored only outside Active Time.

## CORESET

Regarding the CORESET for monitoring, outside Active Time, RAN1 made following agreement in RAN1#97.

Agreements: - The CORESET for the PDCCH-based power saving signal/channel outside Active Time can be configured to index to at least one of the CORESET(S) configured for other PDCCH monitoring - FFS whether the indexed CORESET can be exclusively used by the PDCCH-based power saving signal/channel (i.e., not be used for other PDCCH monitoring) - FFS whether or not to increase the number of CORESETs relative to that in Rel-15 - FFS whether or not a BWP is dedicated for PDCCH-based power saving signal/channel

NR R15 supports up to 3 CORESETs. In R16, it has been agreed to increase it to 5 for multi-panel support. The 5 CORESETs is good enough to support multi beam based power saving signal/channel in FR2. Whether a CORESET is used exclusively for power saving signal/channel or not is up to network configuration. And, whether a BWP is dedicated or not for power saving signal/channel is also up to network configuration.

## Power Saving Signal Transmission at gNB

Regarding UE behavior and understanding on power saving signal transmission (especially wake up signal) we have following agreement.

Agreements: Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time - Following the principle of Rel-15 search space configuration - FFS: the corresponding UE behaviour in monitoring the power saving signal/channel outside Active Time - FFS whether UE can further monitor the search space set(s) inside Active Time

There could be three ways of supporting wake up signal in gNB side.

- The first approach is to always send wake up signal on configured wake up signal transmission occasion and indicate whether to wake up or not. This approach is based on explicit signaling and UE, once configured, is expected to receive signal during every monitoring occasion and will decide what to do after receiving the signal. So, the signal should include both “wake-up”, “sleep”, and possibly other indications. Note that “sleep” indication makes UE skip corresponding DRX ON duration. In terms of resource usage, it is not so efficient since the power saving signal needs to be sent always every monitoring occasion.

- The second approach is to send wake up signal only when gNB wants to wake UE up. This approach is more efficient in terms of resource usage. Note that the signal transmission itself indicates wake-up. The absence of signal could indicate something else, which is discussed below.

- The third approach is to send go-to-sleep signal only when gNB wants to make UE skip upcoming DRX ON duration. If UE detects no signal, then default UE behavior is to wake up and monitor PDCCH.

From radio resource utilization perspective, the first approach is the worst choice since it requires gNB to sends PS signal before every DRX ON duration. The second approach is resource efficient since it requires no signaling during CDRX operation when there is no active data transfer.

Proposal 7

- If wake up signal is introduced in power saving signal, the wake up signal is sent only when network wants to wake UE up.

## UE Behavior when No PS signal is Detected

Under the second approach in previous section, when UE detects no PS signal before DRX ON duration, UE could behave two different ways as follows.

- Option 1) UE stays outside Active Time (DRX OFF) until next DRX ON duration start time. UE is not supposed to transmit SRS/CSI during this time.

- Option 2) UE skips PDCCH monitoring only. This allows UE to perform RRC configured activities such as SRS/CSI transmission.

Clearly Option 1 has higher impact to R15 UE behavior. Option 2) has no impact to those scheduled activities but still allow UE to take advantage of power saving opportunity. If UE has no RRC configured activity and receives no wake up signal, then UE can enter sleep state while it is in Active Time. Thus, Option 2 has no impact to DRX operation.

Proposal 8

- If UE detects no PS signal before DRX ON duration, UE skips PDCCH monitoring during DRX ON duration and performs RRC scheduled activities such as CSI/SRS transmission, if needed.

## PS Techniques Outside Active Time

Regarding the PS techniques carried in power saving signal outside Active Time, RAN1 made following agreement during RAN1#97. Based on recent agreements, power saving signal/channel transmitted outside Active Time could carry at least wake up signal to trigger UE to monitor PDCCH or not during subsequent DRX ON duration(s).

Agreements: Outside Active Time, the PDCCH-based power saving signal/channel is configured for triggering UE to or not to monitor the subsequent ON duration(s) - FFS a single vs. multiple durations, particularlly checking consistency with RAN2 agreements Further study in the new DCI how to potentially indicate at least the following techniques (subject to further WID update): - Indicating UE to use the aperiodic RS - Aperiodic CSI-RS - Aperiodic SRS - TRS - Triggering aperiodic CSI report - Cross-slot scheduling - Rel-15 DCI-based BWP switching The power saving techniques can be explicitly included in the DCI contents or implicitly indicated by other techniques (e.g., BWP switching).

With wake up signal in use, in case there are many ON durations skipped, network may need adaptation to time varying channel status. This could potentially introduce the reduction of time for data transfer, which is translated to lower power consumption. However, this kind of fast link adaptation (aperiodic RS indication) may or may not be required depending on traffic activity. If traffic arrival rate is high and if UE is less likely to enter OFF state, then channel status must be well tracked while inactivity timer is running. So, the reserved bits for aperiodic RS indication/reporting related may not be even needed. If it is really needed, as a workaround network could start a little bit conservative MCS from there and learn accurate MCS after initial transmission. So, regarding those additional techniques related to RS, they might be beneficial in very limited cases (with very sparse data arrival) only and there is work around for those cases. So, they may not be needed in most cases. Further study is needed from power saving perspective.

## DCI Information Encoding

Wake up indication in the new DCI monitored outside Active Time could include at least one bit indicating “wake up” and “sleep”. The inclusion of “sleep” is necessary because the DCI could be carrying signal to multiple UEs.

| Field value | Skipping duration (RRC configurable) |
| --- | --- |
| 0 | UE monitors PDCCH according to search space configuration (UE moves to Active Time) |
| 1 | Skip PDCCH monitoring for upcoming DRX ON duration (UE stays outside Active Time) |

Proposal 9

- Wake up signal outside Active Time can indicate one of following for next ON duration

  - skip PDCCH monitoring

  - Monitor PDCCH

# Power Saving Signal/Channel Inside Active Time

## Cross Slot Scheduling

Inside Active Time, scheduling DCI is reused and additional field could be configured to support power saving signal. Based on the agreement, at least cross slot scheduling related signal could be included. The cross slot related DCI information is further discussed in our companion paper [1].

## PDCCH Skipping

Although PDCCH skipping has not been explicitly included in WID scope, we think there are clear benefits to support this feature in R16.

- It is expected that 5G handsets would potentially suffer from higher power consumption compared to handset with LTE only mode due to much higher data rate and operating bandwidth, which will require extra engineering design considerations to handle the potential thermal issues.

- Increased power consumption of 5G handset might require larger battery size leading to larger form factor.

- PDCCH skipping provides further power saving gain on top of CDRX (short + long), wake up signal, and cross slot scheduling technique.

### PDCCH Skipping with Short CDRX

We first show that PDCCH skipping could provide additional gain on top of CDRX. NR CDRX is very flexible mechanism. The choice of good parameter set for DRX could provide fairly good power saving gain. However, even with a lot of potential, it has not been fully utilized so far in live networks since choosing right parameter sets for current application is not an easy task. The only exception is VoIP application. In this case, traffic pattern is fairly predictable, so, network could use fairly optimized CDRX parameters e.g., 40ms of CDRX cycle with 4ms for ON duration, etc. In other cases, always fixed parameter set is used (e.g., LDC=160ms, ITA=100ms, ODT=10ms). To optimize power consumption, these parameters needs to be adjusted according to application and even short CDRX could be considered. Although the use of short CDRX is not that popular in live networks, it has been shown that there is a potential power saving gain of using short CDRX. So, we compared following three cases to see the additional gain of PDCCH skipping on top of CDRX mechanism.

- Long CDRX only

- Long CDRX + short CDRX

- Long CDRX + short CDRX + PDCCH skipping

| Parameter | Value |
| --- | --- |
| Frequency | FR1 |
| Long CDRX only | LDC=160ms, IAT=100ms, ODT=8ms |
| Long CDRX & Short CDRX | set D1) LDC=160, IAT=40, ODT=8, SDC=20ms, SDCT=3 set D2) LDC=160, IAT=5, ODT=8, SDC=20ms, SDCT=5 |
| PDCCH skipping | duration = 10ms PDCCH skipping is sent only with scheduling DCI when DL buffer gets empty after current transmission. |
| Traffic | FTP3 with 2.5MB set T1) packet size = 0.5MB, mean inter arrival time = 0.2sec set T2) packet size = 0.1MB, mean inter arrival time = 0.04sec |
| Parameters not shown here follows baseline configuration given in 38.840. |  |

CDRX Parameter Set D1) LDC=160, IAT=40, ODT=8, SDC=20ms, SDCT=3

The evaluation results are shown in Figure 2 for two different traffic set T1 and set T2. The DRX parameter set D1. For traffic set T1 (left), the additional power saving gain of PDCCH skipping is 16.9% compared to Long & short CDRX. For traffic set T2(right) which is smaller packet size and mean smaller inter-arrival time, the power saving gain is 34.5% on top of short CDRX.

Figure 2 Power saving gain of PDCCH skipping compared to CDRX (Long + Short) with DRX parameter set D1 (LDC=160ms, IAT=40ms, ODT=8ms, SDC=20ms, SDCT=3). Note that the power consumption reduction in (%) is computed w.r.t Long + Short DRX case.

CDRX Parameter set D2) LDC=160, IAT=5, ODT=8, SDC=20ms, SDCT=5

For further optimization, we reduce inactivity timer (ITA) to 5ms and increase the short DRX cycle timer to 5 in D2. This CDRX configuration has very short ITA and longer short CDRX cycle timer, so it could be understood a version of CDRX for replacing continuous monitoring period (from long inactivity timer) with short periodic monitoring (by short cycle). This configuration, as shown below, could further reduce power consumption. However, considering that inactivity timer (ITA) is determined considering scheduling flexibility of network and the burstness of traffic, the choice of too short ITA value could have negative impact on network scheduler giving restriction on scheduling and potential decrease of system throughput. Figure 3 shows the results with DRX parameter set D2. The power consumption of short CDRX is reduced, but still PDCCH skipping could provide good power saving gain for traffic set T1 and T2. The gain for traffic T2 is large since smaller packet size with more frequent arrival makes UE stay longer in DRX ON state, where PDCCH skipping has more chance to make the UE skip PDCCH monitoring.

Figure 3 Power saving gain of PDCCH skipping compared to CDRX (Long + Short) with DRX parameter set D2 (LDC=160ms, IAT=5ms, ODT=8ms, SDC=20ms, SDCT=6). Note that the power consumption reduction in (%) is computed w.r.t Long + Short DRX case

Observation 1

- PDCCH skipping can provide additional power saving gain of 5.2%~34.5% on top of Long+Short CDRX.

### PDCCH Skipping with R16 Features (Wake Up Signal, Cross Slot Scheduling)

R16 has two power saving features already in its WI scope : wake up signal(WUS) and cross slot scheduling (CS). Wake up signal is useful when traffic arrival is sporadic and UE spends large amount of time in alternating CDRX ON/OFF state. In such case UE could effectively skip ON durations without grant arrival. Cross slot scheduling technique is useful when traffic arrival rate is low resulting in PDCCH only monitoring in many slots. It has been shown throughout the SI that there are fair amount of power saving gain from these two techniques. Question is how much additional power saving gain can PDCCH skipping bring on top of these two. In this evaluation, we show the PDCCH skipping can provide still good power saving gain on top of R16 features and both long and short CDRX.

Long DRX only

Figure 4 shows the power consumption and power saving gain with long DRX only case. The additional PS gain of PDCCH skipping ranges from 5.8% to 25.6% depending on the skip duration and traffic pattern (mostly packet inter arrival time). It is observed that gain increases in the traffic pattern with higher arrival rate (0.1MB/0.04sec).

Figure 4 Power saving gain of PDCCH skipping compared to R16 features (wake up signal(WUS), cross slot scheduling(CS)) with Long DRX only  (LDC=160ms, IAT=100ms, ODT=8ms). Note that the power consumption reduction in (%) is computed w.r.t WUS+CS case.

Long DRX + Short DRX

Figure 5 shows the power consumption and power saving gain with long+short DRX case. The additional PS gain of PDCCH skipping ranges from 7.8% to 28.2% depending on the skip duration and traffic pattern (mostly packet inter arrival time). Similarly, it is observed that gain increases in the traffic pattern with higher arrival rate(0.1M/0.04sec).

Figure 5 Power saving gain of PDCCH skipping compared to R16 features (wake up signal(WUS), cross slot scheduling(CS)) with Long DRX + short DRX (LDC=160ms, IAT=40ms, ODT=8ms, SDC=20ms, SDCT=3ms). Note that the power consumption reduction in (%) is computed w.r.t WUS+CS case.

Observation 2

- PDCCH skipping can provide additional power saving gain of 5.8%~28.2% on top of short CDRX and R16 features (wake up signal and cross slot scheduling).

Proposal 10

- NR supports PDCCH skipping indication in power saving signal inside Active Time.

# Conclusions

In this contribution, we have provide detailed design of power saving signal/channel including offset, search space, coreset, gNB power saving signal transmission, UE behavior, and DCI content for supporting power saving signal. Based on discussion we make following observation / proposals.

Proposal 1

- If the offset for monitoring occasion for power saving signal outside Active Time is explicitly configured by RRC, UE use the configured value. Otherwise, UE use default offset value and DRX offset to determine the monitoring occasion of power saving signal.

Proposal 2

- NR supports that power saving signal monitoring offset ranges from X to Y ms, where

  - X=[1/16]

  - Y=[5]

Proposal 3

- If a UE is configured with short DRX with drx-ShortCycle < X and/or drx-onDurationTimer < Y and configured to monitor power saving signal channel outside Active Time, the UE skips monitoring power saving signal channel before ON duration when UE is operating in short DRX mode.

Proposal 4

- Dedicated search space for monitoring power saving signal outside Active Time is configured such that it is aligned with potential DRX ON duration based on current DRX configuration (including both short and long).

- There is a configured offset between configured search space and a corresponding potential ON duration.

- Only the power saving monitoring occasions associated with its ON duration is monitored by UE.

Proposal 5

- A power saving signal transmitted outside Active Time is associated only one next ON duration.

Proposal 6

- Define a new common search space type Type4-PDCCH CSS set for power saving signal monitoring

- Define a new DCI format DCI Format 2_4 for power saving signal.

- UE can be configured to monitor DCI Format 2_4 with CRC scrambled with PS-RNTI in Type4-PDCCH CSS set.

- The configured search space Type4-PDCCH CSS is monitored only outside Active Time.

Proposal 7

- If wake up signal is introduced in power saving signal, the wake up signal is sent only when network wants to wake UE up.

Proposal 8

- If UE detects no PS signal before DRX ON duration, UE skips PDCCH monitoring during DRX ON duration and performs RRC scheduled activities such as CSI/SRS transmission, if needed.

Proposal 9

- Wake up signal outside Active Time can indicate one of following for next ON duration

  - skip PDCCH monitoring

  - Monitor PDCCH

Observation 1

- PDCCH skipping can provide additional power saving gain of 5.2%~34.5% on top of Long+Short CDRX.

Observation 2

- PDCCH skipping can provide additional power saving gain of 5.8%~28.2% on top of short CDRX and R16 features (wake up signal and cross slot scheduling).

Proposal 10

- NR supports PDCCH skipping indication in power saving signal inside Active Time.

# References

- R1-1909059 Cross slot scheduling design for UE power saving, Apple, Aug 2019

- R1-1909061 Traffic aware UE power saving techniques with UE assistance, Apple, Aug 2019