3GPP TSG RAN WG1 #98		                               			R1-1909481

Prague, Czech Republic, August 26th-30th, 2019

Agenda Item:	7.2.8.3

Source:	Apple Inc.

Title:	Feature lead summary on previous agreements and open issues on L1-SINR and SCell BFR

Document for:	Discussion/Decision

# Introduction

In this summary, the agreements made on Rel.16 NR_eMIMO L1-SINR and SCell BFR are compiled and systematically arranged. The main purpose is to identify missing parts, if any, as well as to aid specification editors. Therefore:

- Only agreements pertinent to specification are included.

- For a given topic/component, the captured agreement represents the most mature form. Redundancies (including most FFS items that have been agreed) are removed.

- Open issues (including missing items) are identified with yellow highlight

# Agreements on L1-SINR based beam selection

## Beam reporting content

It has been agreed that in a beam reporting instance, gNB can configure UE to report up to N reported SSBRI/CRIs as well as corresponding L1-SINR values.

The variable N is configured by RRC signaling with candidate values of {1, 2, 3, 4}.

Agreement At least support gNB can configure UE to report up to N reported SSBRI/CRIs defined in Rel-15 and corresponding L1-SINR values for in a beam reporting instance - N is configured by RRC signaling with candidate values of {1, 2, 3, 4}

## Beam reporting format

The detail beam reporting format has not been discussed, and it is still one open issue.

## Control signaling framework

It has been agreed that gNB can configure Channel Measurement Resource (CMR) only or CMR as well as Interference Measurement Resource (IMR) for L1-SINR measurement.

- CMR can be configured based on SSB or CSI-RS for beam management.

  - If IMR is not configured and CMR is based on CSI-RS, only CSI-RS with density = 3 REs/RB can be configured

- Interference can be measured based on ZP-IMR, NZP-IMR as well as both ZP-IMR and NZP-IMR.

  - If gNB configures UE to measure L1-SINR based on both ZP-IMR and NZP-IMR, only 1 ZP IMR resource can be configured

  - If IMR is configured based on NZP-IMR only, the NZP-IMR could only be based on CSI-RS with density = 3 REs/RB

  - FFS: when IMR is configured based on ZP-IMR only, whether interference measurement can be performed using CMR additionally

In addition, the association between CMR and IMR is also one open issue.

Agreement - When dedicated IMR is not configured, - If CMR is based on CSI-RS, when L1-SINR is configured, and interference measurement is performed using CMR with CSI-RS only with density 3 REs/RB for 1-port CSI-RS is used - Spec does not require UE to use SSB for interference measurement - Note: CSI-RS above is CSI-RS for BM - When dedicated IMR is configured, - NW can configure interference measurement for L1-SINR with either of the following options - ZP-IMR only - NZP-IMR only - (WA) ZP-IMR and NZP IMR (interference measurement is taken on both) - Maximum Number of ZP IMR is 1 - If IMR is configured based on NZP IMR only, when L1-SINR is configured, interference measurement is performed only with density 3 REs/RB CSI-RS - If IMR is configured based on ZP IMR only, when L1-SINR is configured, interference measurement is performed using ZP IMR - FFS: interference measurement is performed using CMR additionally - Note: CSI-RS above is CSI-RS for BM

## UE capability on L1-SINR

It has been agreed that the support of L1-SINR based beam selection is optional.

FFS: whether support of NZP-IMR and ZP-IMR based L1-SINR are separate UE capabilities.

Agreement - Support of L1-SINR is optional - FFS: Support of NZP IMR and ZP IMR are separate UE capabilities

# SCell BFR

## Beam failure detection

It has been agreed that beam failure detection can be based on 1-port periodic CSI-RS. The CSI-RS for BFD can be configured explicitly by RRC or implicitly by TCI state for a CORESET in SCell. The explicitly configured CSI-RS for SCell BFD should be transmitted within this SCell.

- Maximum number of RS for BFD per BWP is one open issue

SCell BFD is measured based on hypothetical BLER.

- Detail value for BLER threshold for SCell BFD is one open issue

Details on association between beam failure detection and beam failure event is one open issue.

Agreement - SCell BFD is based on periodic 1-port CSI-RS, which can be configured explicitly by RRC or implicitly by TCI state. - SCell BFD is measured based on hypothetical BLER Agreement At least for explicit configuration, downlink RS for BFD is in current CC Agreement When SCell BFD RS is configured in an implicit manner, BFD RS can be transmitted in active BWP of either current CC or another CC.

## New beam identification

It has been agreed that new beam identification can be performed based on SSB and CSI-RS for beam management. The SSB/CSI-RS can be transmitted within the SCell or another CC within the same band. The gNB can configure N SSB/CSI-RS resources per BWP for new beam identification.

- The range of N is one open issue

The threshold of new beam identification is based on L1-RSRP.

- Detail value for the threshold is one open issue

Agreement Downlink RS for new beam identification can be based on SSB and CSI-RS for BM Agreement Downlink RS for new beam identification can be transmitted in active BWP of the CC which is configured to be monitored for BFR or another CC within the same band Agreement New beam identification threshold is based on L1-RSRP

## Beam failure recovery request

It has been agreed that after UE declares beam failure, it shall transmit BFRQ.

For SCell with downlink only, the BFRQ can be conveyed by two steps:

Step 1 is carried by a dedicatedly configured SR-like PUCCH in PCell or PSCell to inform gNB beam failure happens

- Details for this PUCCH is one open issue

Step 2 is carried by a MAC CE to report detail information, e.g. failed CC index and new beam index

- If new candidate beam RS and corresponding threshold is configured

  - If at least one new beam is identified (L1-RSRP is higher or equal to the threshold)

    - UE reports 1 new beam index and failed CC index

  - Else

    - UE reports no new beam identified

- Else

  - Detail is one open issue

Details on BFRQ for SCell with both uplink and downlink is one open issue.

Agreement - For SCell BFR, BFRQ shall be conveyed if UE declares beam failure - UE shall convey new beam information during BFR procedure if new candidate beam RS and corresponding threshold is configured and at least if channel quality of new beam is above or equal to threshold Agreement During a BFRQ procedure, UE reports only 1 beam with corresponding beam index only per SCell Agreement On BFRQ procedure for SCell - Step 1 can be carried by at least a dedicated SR-like PUCCH resource for BFR over PCell or PSCell - (Working Assumption) Step 2 is carried by MAC CE Above applies at least for SCell with downlink only Agreement - When SCell BFR is configured and RS for new beam identification is configured, the threshold for new beam identification should be always configured - If a SCell has failed, when there is no new beam with L1-RSRP higher than configured threshold for SCell BFR, for new beam information reporting, UE reports that there is no new beam identified for the SCell

## Beam failure recovery response

Details on BFRR is still one open issue.

## Number of SCells for BFR

It has been agreed that gNB can configure UE to perform BFR for any configured SCells, and the maximum number of SCells for BFR is one UE capability.

Agreement A UE can be configured to perform BFR for any configured SCells - The maximum number of SCells for which the UE performs BFR is a UE capability