3GPP TSG RAN WG1 #98		                               			R1-1909482

Prague, Czech Republic, August 26th-30th, 2019

Agenda Item:	7.2.8.3

Source:	Apple Inc.

Title:	Feature lead summary on L1-SINR and SCell BFR

Document for:	Discussion/Decision

# Introduction

In this summary, issues related to L1-SINR based beam selection and SCell beam failure recovery are provided.

# L1-SINR

## Beam reporting format

It has been agreed that in a beam reporting instance, gNB can configure UE to report up to N reported SSBRI/CRIs as well as corresponding L1-SINR values, where N is configured by RRC signaling with candidate values of {1, 2, 3, 4}. Then one open issue is the reporting format.

Issue 2.1:

Down-select one of the following alternatives for L1-SINR based reporting format

- Alt 1: L1-SINR is reported based on the following table

  - The SINR is quantized based on what is specified in 38.133.

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1, if reported |
|  | CRI or SSBRI #2, if reported |
|  | CRI or SSBRI #3, if reported |
|  | CRI or SSBRI #4, if reported |
|  | SINR #1, if reported |
|  | Differential SINR #2, if reported |
|  | Differential SINR #3, if reported |
|  | Differential SINR #4, if reported |

- Alt 2: L1-SINR is reported based on the following table

  - The SINR is quantized based on what is specified in 38.133.

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1, if reported |
|  | CRI or SSBRI #2, if reported |
|  | CRI or SSBRI #3, if reported |
|  | CRI or SSBRI #4, if reported |
|  | SINR #1, if reported |
|  | SINR #2, if reported |
|  | SINR #3, if reported |
|  | SINR #4, if reported |

Offline proposal:

- When gNB configures UE to report SSBRI/CRI and corresponding L1-SINR, the following report format is supported.

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1, if reported |
|  | CRI or SSBRI #2, if reported |
|  | CRI or SSBRI #3, if reported |
|  | CRI or SSBRI #4, if reported |
|  | SINR #1, if reported |
|  | Differential SINR #2, if reported |
|  | Differential SINR #3, if reported |
|  | Differential SINR #4, if reported |

  - FFS: range and step size of differential SINR

    - Strive to define the same number of bits as differential RSRP

  - Differential SINR #N is determined based on the difference between measured SINR corresponding to the CRI/SSBRI #N and the measured SINR corresponding to CRI/SSBRI #1

  - The SINR #1 is the largest SINR among reported SINRs

  - The range of SINR is [-23, 40] dB

    - The SINR is quantized based on what is specified in 38.133

Companies’ comments and views

| Company | Comment |
| --- | --- |
| Intel, Spreadtrum, Apple | Support Alt1 |
| Nokia | Support Alt-1 |
| Ericsson | Support Alt-1 |
| Samsung | Support Alt-1 |
| NEC | Support Alt1 |
| Sony | Support Alt-1 |
| Docomo | Differential SINR reporting can be supported to reduce the reporting overhead. However, whether additional reporting content is supported or not is still under discussion. It may be too early to agree the reporting format shown in tables above. |
| ZTE | Support Alt1. The differential step should be further studied, e.g., configurable or determined based on the reference value of SINR #1 (non-differential one). |
| CMCC | Differential SINR reporting is supported. For the reporting format, whether SSBRI/CRI implies a CMR/IMR combination and whether to support gNB to configure UE to report [IMR index] and RSRP additionally in a beam reporting instance is still in FFS. The SSBRI/CRI in the above alternatives is unclear. |
| OPPO | Share similar view as DOCOMO. It is too early to discuss the reporting format now. We shall decide the reporting content first before discussing this. |
| MTK | Support Alt-1 |
| Huawei/HiSilicon | Similar view as DCM, CMCC, OPPO. |
| vivo | Similar view as DCM, CMCC, OPPO, Huawei/HiSilicon |
| Qualcomm | Support Alt2, i.e. report non-differential SINR value, First need to converge on # of bits on resolution/range of the differential report before deciding to support it or not. |

## Working Assumption on IMR configuration

In last meeting, there is one working assumption to support dedicated IMR to be configured based on both ZP-IMR and NZP-IMR, where the maximum number of ZP-IMR is 1.

- When dedicated IMR is configured, - NW can configure interference measurement for L1-SINR with either of the following options - ZP-IMR only - NZP-IMR only - (WA) ZP-IMR and NZP IMR (interference measurement is taken on both) - Maximum Number of ZP IMR is 1

Issue 2.2:

- Alt 1: confirm the working assumption on both ZP-IMR and NZP-IMR based interference measurement agreed in RAN1 #97

- Alt 2: Do not confirm the working assumption (please specify the potential issue)

FL recommendation: online discussion.

Companies’ comments and views

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon, vivo, Fujitsu, NEC, China Telecom, Spreadtrum, Docomo, Apple, Sony, Lenovo/Motorola Mobility, ZTE/Sanechips, MTK | Support Alt1 |
| Samsung | Support Alt2 “Concurrent use of NZP CSI-RS (CMR or NZP IMR) and ZP IMR is plagued with the following issues: 1) unclear benefit and use cases; 2) unclear or potentially demanding specification effort; 3) unclear or potentially demanding UE complexity impact, either for interference measurement or, if applicable, interference emulation” |
| Nokia/NSB | Support Alt2 “The current WA, i.e. ZP-IMR and NZP IMR (interference measurement is taken on both) introduces ambiguity about UE processing with network leading to the degradation of the overall system performance. Furthermore, the WA increases signalling and resource overhead as well as standardization efforts (e.g. different RAN4 requirements at UE-side).” |
| Ericsson | Support Alt2 “Interference estimation based on the combination of ZP-IMR and NZP-IMR leads to increased overhead and planning effort compared to NZP-IMR. The accuracy is similar to ZP-IMR only. Also, the RAN4 test design is complicated.” |
| LGE | Alt2. Share similar views with Samsung, Nokia/NSB, and Ericsson in terms of usage and performance gain compared to the increased UE complexity. |
| OPPO | The use case for ZP IMR and NZP IMR can be that ZP is used to measure ‘background’ interference and NZP is used to measure one particular ‘companion interference’ beam. However, there is one concern on “maximum number of ZP IMR is 1”. For each of N configured CMRs, the UE shall use the corresponding Rx beam to measure both CMR and IMR. If only one ZP IMR is configured, then (1) for P or SP RSs, the UE can only use one ZP IMR occasion to measure interference for one CMR and totally needs N occasion to complete the measurement for all N CMRs. (2) For AP RSs, it would not work because the UE can only measure ZP IMR for one CMR. |
| CATT | Alt.2. The gain of joint ZP+NZP operation compared to NZP or ZP alone should be clarified first. |
| Huawei/HiSilicon | Support Alt 1 |
| Qualcomm | Support Alt2. No strong use case for ZP+NZP. For MU-MIMO scheduling, it should be sufficient with “ZP only”, which can estimate both intra and inter cell interference at low complexity. As shown in our results, inter-beam interference has at most 0.6 dB impact on total SINR if gNB schedules UEs with 15 degree separation. So additional NZP for intra cell interference is not needed if gNB chooses UEs with sufficient angular separation. Additional UE behaviour needs to be defined if the combination is supported |

## Control signaling framework

It has been agreed that gNB can configure Channel Measurement Resource (CMR) only or CMR as well as Interference Measurement Resource (IMR) for L1-SINR measurement.

- CMR can be configured based on SSB or CSI-RS for beam management.

  - If IMR is not configured and CMR is based on CSI-RS, only CSI-RS with density = 3 REs/RB can be configured

- Interference can be measured based on ZP-IMR, NZP-IMR as well as both ZP-IMR and NZP-IMR.

  - If gNB configures UE to measure L1-SINR based on both ZP-IMR and NZP-IMR, only 1 ZP IMR resource can be configured

  - If IMR is configured based on NZP-IMR only, the NZP-IMR could only be based on CSI-RS with density = 3 REs/RB

Then the open issue is how to configure CMR/IMR, e.g. whether the CMR/IMR association is based on 1-to-1 mapping, 1-to-N mapping or N-to-1 mapping.

Issue 2.3:

For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured, down-select at least one of the following resource configuration schemes:

- Option 1: CMR and IMR are 1-to-1 mapped

- Option 2: 1 CMR can be mapped to 1 or more than 1 IMRs

  - Interference is accumulated on configured IMRs

  - 1 resource set for CMR and another resource set for IMR

- Option 3: 1 IMR can be mapped to 1 or more than 1 CMRs

- Option 4: More than 1 IMRs can be mapped to more than 1 CMRs.

Offline Proposal:

For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured to be based on ZP-IMR only, down-select at least one of the following resource configuration schemes:

- Option 1: CMR and IMR are 1-to-1 mapped from signaling perspective

- Option 3: 1 IMR can be mapped to 1 or more than 1 CMRs

For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured to be based on NZP-IMR only, down-select at least one of the following resource configuration schemes:

- Option 1: CMR and IMR are 1-to-1 mapped

- Option 2: 1 CMR can be mapped to 1 or more than 1 IMRs

  - Interference is accumulated on configured IMRs

  - 1 resource set for CMR and another resource set for IMR

  - CMR and associated IMR are QCLed with TypeD

  - FFS: whether configuration of QCL for IMR or association between CMR and IMR is supported

- Option 3: 1 IMR can be mapped to 1 or more than 1 CMRs

- Option 4: More than 1 IMRs can be mapped to more than 1 CMRs.

Companies’ comments and views

| Company | Comment |
| --- | --- |
| Vivo, ZTE | Support option 1, 2 and 3 |
| Lenovo/Motorola | Support option 2. We think allowing multi-IMR to multi-CMR mapping increases the IMR/CMR pairing flexibility, so this can be supported as option 4. |
| LGE | Support option 1 as a default operation and option 3 to save IMR overhead for P-2 operation. |
| Nokia | Support option 1 and option 2 The benefits of option 3 remain unclear (reporting not supported in P-3 operation). |
| Ericsson | We are somewhat confused: the CSI framework allows configuration of pairs of (CMR,IMR), and the UE reports the index into that list of pairs. The list can contain repeated CMRs and/or IMRs, so we get full freedom simply by reusing that framework. |
| Samsung | Apart from the issue in section 2.2 (whether a mixed use of ZP and NZP IMR is supported), our understanding is similar to Ericsson. Option 1 and 3 can be supported with the CSI framework. Option 2 can also be supported with the CSI framework, perhaps with some constraint(s), e.g. not allowing the mixed case of NZP+ZP (depending on section 2.2), max. number of IMRs (UE capability). |
| Intel | Option 1 |
| Sony | Given the fact that companies may have divergent interference measurement schemes, it would be better to maintain the flexibility of CMR/IMR pair. In addition, as mentioned by Samsung and Ericsson, current Spec may already support some Options. |
| Docomo | Support Option 1 or Option 2. Option 1 is similar as Rel-15 CSI framework. In Option 2, a CMR can be associated with more than one IMRs. In this case, IMR index reporting can be further considered. |
| ZTE2 | Option2 and option 3 can NOT be achieved through Rel-15 framework. - In Option2, one SINR value is determined according to CMR and associated multiple IMR (as multiple interference beams in MU-MIMO). If going with Rel-15 design, we can configured repeated CMR for these multiple IMR, but interference measurement should be performed per IMR, rather than combined one from multiple IMR. In Option3, the case that multiple CMR, multiple NZP-IMR and one ZP-IMR in WA still can NOT be achieved in Rel-15 CSI framework. Besides, it can well save IMR overhead, as LGE mentioned, e.g., ZP-IMR for P-2. |
| CMCC | L1-SINR can be used to represent the inter-cell interference or inter-beam interference, which can be realized by different configuration schemes between CMR and IMR. The flexible N-to-M mapping between CMR and IMR should be supported. |
| OPPO | Support option 1 and 2. Option 3 is not doable for L1-SINR beam reporting: the issue is if same IMR is mapped to K CMRs, then CMR with best SINR is also the CRM with best RSRP. Therefore, no justification to measure and report L1-SINR; Option 4: it could result in too complicated UE behavior and configuration. |
| CATT | Option 1. Prefer to use existing framework. |
| Huawei/HiSilicon | For inter-beam interference measurement with NZP IMR, support option 2 and option 4, as multiple IMRs should be measured for each CMR to identify the weakest/strongest interfering beam(s) for the CMR. This can provide insights that are useful for MU pairing. For inter-cell interference measurement with ZP IMR, support option 3 to reduce the overhead of IMR. |
| vivo | Similar understanding as ZTE that on what is supported in Rel15 interference calculation behavior may not be enough for L1-SINR calculation, e.g. accumulation of interference over NZP-IMR. |
| Qualcomm | Support option 1. CMR and IMR are 1-to-1 mapped to be consistent as CSI report frame work. . First, need to converge on QCL assumption to decide if 1-to-N and N-to-1 are to be supported Second, 1-to-N mapping can be achieved by 1-to-1 mapping by redefining the N individual resources as a single resource. Similarly, N-to-1 mapping can be achieved by 1-to-1 mapping by assigning different resource IDs to same actual resource |

## Use case for L1-SINR based beam reporting

According to the assumption of L1-SINR based beam reporting in previous agreements, it could be used at least for non-group based beam reporting. Then one open issue is whether it can be used for group based beam reporting.

Issue 2.4:

- Alt 1: L1-SINR can only be used for non-group based beam reporting.

  - Supported:

- Alt 2: L1-SINR can be used for both non-group based and group based beam reporting.

  - Supported: ZTE, Apple, Sony, Docomo, MTK, Huawei, HiSilicon

Proposal 2.4:

- Support gNB to configure L1-SINR based beam report for both non-group based and group based beam reporting.

Companies’ comments and views

| Company | Comment |
| --- | --- |
| Samsung | No strong preference, for consistency Alt 2 makes more sense |
| Sony | Support Alt 2 which is consistent with group-based L1-RSRP reporting. |
| Docomo | Support Alt. 2. |
| ZTE | We share the same views with Samsung, Sony and Docomo. |
| CMCC | Support Alt. 2. |
| OPPO | Support Alt1. The major use case for group-based beam reporting is multi-TRP/panel and multi-panel UE. Further study is needed on how and what to enhance group-based beam reporting. Therefore, suggest not consider L1-SINR for group based for now. |
| CATT | No strong view but can support alt-2 for progress. |
| Huawei/HiSilicon | Support Alt 2 |
| Qualcomm | Support Alt1. Need to first define L1-SINR for group based report, e.g. whether the interference for 2 reported beams should be measured simultaneously or TDMed. |

## Others

There are some proposals on other issues that have been discussed multiple times, e.g. to report L1-RSRP in addition to L1-SINR, or related to UE capability, or related to implementation complexity, e.g. CPU occupancy, timing requirement for report. These issues can be discussed at next meetings after more decisions for L1-SINR based beam selection are made.

# SCell BFR

## Beam failure detection

It has been agreed that beam failure detection can be based on 1-port periodic CSI-RS. The CSI-RS for BFD can be configured explicitly by RRC or implicitly by TCI state for a CORESET in SCell. Maximum number of RS for BFD per BWP is one open issue. In addition, SCell BFD is measured based on hypothetical BLER. Detail value for BLER threshold for SCell BFD is another open issue. Further details on association between beam failure detection and beam failure event is one open issue.

Issue 3.1-1:

The maximum number of RS for BFD per BWP is:

- Alt 1: 2

  - Supported: Ericsson, LGE (2 for explicit configuration, 3 for implicit configuration), Samsung, NEC, Panasonic, Docomo

- Alt 2: 3

  - Supported: Apple, Sony, ZTE, OPPO, CATT, APT

- Alt 3: 5

  - Supported:

- Alt 4: other (please specify)

  - Qualcomm: based on UE capability

Issue 3.1-2:

Down-select one of the following alternatives on SCell BFD threshold configuration

- Alt 1: The BLER threshold for SCell BFD is configured by rlmInSyncOutOfSyncThreshold.

  - Supported: Apple

- Alt 2: The BLER threshold for SCell BFD is the same as the default value of rlmInSyncOutOfSyncThreshold.

  - Supported: Ericsson, Samsung, NEC, Panasonic, Docomo, Lenovo/Motorola Mobility, ZTE, CMCC, OPPO, MTK, CATT, Huawei, HiSilicon, Qualcomm

Issue 3.1-3:

Down-select one of the following alternatives on SCell beam failure detection procedure,

- Alt 1: For SCell BFD procedure, reuse the beam failure detection procedure specified in section 5. 17 in 38.321 in Rel-15.

  - Supported: Apple, Docomo, ZTE(per SCell) , CMCC, OPPO, MTK, CATT, Qualcomm

- Alt 2: SCell beam failure detection procedure is based on that specified in section 5. 17 in 38.321 in Rel-15 with an enhancement. (Please specify what enhancement is needed.)

  - Supported: Intel

  - Panasonic: Specify the BFD/BFR procedure for multiple SCells based on group basic scenario

FL recommendation: offline discussion on issue 3.1-1 and issue 3.1-3. The starting point of issue 3.1-3 is as follows:

- For SCell BFR, UE shall declare beam failure for a SCell after detecting N consecutive beam failure indication for the SCell.

  - Reuse the same beam failure indication framework specified for PCell/PSCell in 38.213.

  - N is configured by RRC

Proposal 3.1:

- Support that the BLER threshold for SCell BFD is the same as the default value of rlmInSyncOutOfSyncThreshold.

Companies’ comments and views

| Company | Comment |
| --- | --- |
| Ericsson | Issue 3.1-1: Alt1 for explicit configuration and Alt2/Alt3 (FL: which one?) for implicit configuration (number of BFD RS equals to the number of configured CORESTs with active TCI states) Actually, we consider the choice of 3 CORESETs for the SpCell as an oversight, and would be OK to derive a reduction rule similar to what we have for RLM. However, the SpCell solution is the baseline, and RAN2 urgently needs info on the explicit configuration. Maybe we should agree on that first? Issue 3.1-2: Keep the SpCell behaviour For issue 3.1-3: Beam failure detection is performed per SCell, the UE declares beam failure per SCell, and reports per SCell |
| LGE | Similar view with Ericsson: 2 for explicit configuration and 3 for implicit configuration. |
| Nokia | Issue 3.1-3: MAC layer detection can be reused but UE should be able to determine failure for multiple SCells based on one of the SCells in the group. This would limit the monitoring complexity for BFD in case of multiple SCells. Grouping should be up to network. |
| Intel | Issue 3.1-1: We think that the total number of BFD RS across all CCs (including PCell) and RLM RS are more important to define than per BPW BFD Issue 3.1-3: For this case, we are ok with re-using RAN2 procedure, but such decision should be up to RAN2. However, RAN2 process depends on BFD declaration from layer 1. This procedure should be further discussed with following alternatives: Alt. 1 – BFD is performed per-SCell and UE declares BFD when all BFD RS corresponding to SCell has failed. Alt. 2 – BFD is performed on a group of SCells and BFD can be declared when BFD RS (or a subset thereof) corresponding to a group of SCells fails (e.g. corresponding to the same band). |
| Panasonic | Issue 3.1-1: Support Alt 1 for a unique solution in the explicit RS configuration which is the same as that of PCell/PsCell specified in Rel-15. Issue 3.1-2: Support Alt 2. Issue 3.1-3: We think that it is beneficial to enhance the SCell BFD/BFR procedure to support the multiple SCells. Particularly, with FR2 operation, there are the situations that groups of SCells share similar failure condition properties. To reduce the BFD measurement effort for the UE and simplify the BFR procedure, the beam failure for a group of SCells can be obtained based the beam failure of a SCell. Therefore, we would like to support Alt. 2 with following modification - Alt 2: SCell beam failure detection procedure is based on that specified in section 5. 17 in 38.321 in Rel-15 with an enhancement. (Please specify what enhancement is needed.) - Specify the BFD/BFR procedure for multiple SCells based on group basis scenario Moreover, in our understanding, it is possible to configure an indicated SCell for each individual group with the BFD reference signal(s) (i.e., not all SCells within the group) when the group of SCells have a similar failure condition. Therefore, only the indicated SCell is configured to perform BFD and report for each individual group. By doing so, the downlink BFD RS could be reduced and other BFR resources could also be saved. Similarly, the gNB can also configure SCell BFD RS over more than one SCell within SCell group and the configuration information can indicate which SCell to perform BFD and report based on the UE capability and/or channel condition. - For the group-based SCell BFR, support to configure at least an indicated SCell with the BFD reference signal(s) for each individual group pointed by the configuration information |
| Sony | Issue 3.1-1: up to 3 BFD RSs per BWP corresponding to maximal 3 CORESETs per BWP Issue 3.1-3: agree with Nokia that BFD of multiple SCells can be carried out by detecting one of the SCells, assuming these SCells are in QCL-TypeD relation. |
| Docomo | Issue 3.1-1: Alt. 1: The same as Rel-15 for SpCell. Issue 3.1-2: Alt. 2. The same as Rel-15 for SpCell. Issue 3.1-3: Alt. 1: Can be the same as Rel-15. |
| ZTE | Issue 3.1-1: We share the same views with Apple and Sony. Rel-15 solution is still incomplete as a remaining issue without doubts. Why we need to let SCell-BFR in the same situation alive again in Rel-16 (e.g., it is very confusing of “2 for explicit and 3 for implicit”) even when we know this is wrong? BTW, it is SCell not PCell, where there is no CORESET#0 or CORESET-BFR. Issue 3.1-2: Keep the same UE behavior as in Rel-15. Further enhancement may be unnecessary. For issue 3.1-3: Beam failure detection is performed per SCell, and the UE declares beam failure event per SCell. But, in MAC procedure, the BFR-dedicated SR (i.e., PUCCH-BFR) and recovery reporting related MAC-CE format can be used for all cells configured with BFR procedure. |
| CMCC | Issue 3.1-2: support Alt 2. Issue 3.1-3: support Alt 1. |
| OPPO | Issue 3.1-1: share same views as Apple, Sony and ZTE. There is not CORESET#0 or CORESET-BFR. The beam failure is defined as when all the PDCCHs fail. So the UE shall monitor up to 3. Issue 3.1.-2: No need to change the beam failure threshold. Issue3.1.-3: the beam failure detection and recovery procedure shall be conducted per SCell, which is the most general method that is applicable to all deployment scenarios. The gNB can determine beam failure of other SCells based on the beam failure reporting for one SCell by implementation. No specification is needed for that. |
| CATT | Issue 3.1-1: alt-2 Issue 3.1-2: alt-2 Issue 3.1-3: alt-1 |
| Huawei/HiSilicon | Issue 3.1-1: In Rel-15, it was agreed first that a CORESET may be dedicatedly configured for receiving BFR response, but eventually not reflected in specs. To avoid repeating such history/ambiguity, we suggest clarify whether a CORESET may be dedicatedly configured for receiving SCell BFR response (thus no TCI state configured) first. This will impact the maximum number of BFD RS(s) that can be implicitly derived from activated TCI states for CORESETs. Issue 3.1-2: Support Alt 2. Issue 3.1-3: It is a bit strange for RAN1 to discuss whether to reuse RAN2 procedure. Similar to what other companies are already doing, we suggest the FL to write down the procedures or principles that we are discussing, instead of simply referring to RAN2 specs, which are ambiguous to us at least. |
| vivo | Issue 3.1-1: share same views as Apple, Sony, ZTE and OPPO. Sightly different understanding: there could be CORESET #0 on SCell. Issue 3.1-2: Support Alt 2. Issue 3.1-3: Per SCell seems to be easiest way. But we also think it is valid option to detect BFR for multiple SCells within a band. |
| Qualcomm | Issue 3.1-1: Support max BFD RS # indicated by UE capability, which can be less than max # of CORESETs per PDCCH config indicated by UE capability Issue 3.1.2: Support Alt2, i.e. BLER threshold same as PCell BFR. Issue 3.1.3: Support Alt1, i.e. BFD procedure same as PCell BFR |
| APT | Issue 3.1-1: share similar view as Apple, Sony, ZTE, OPPO, vivo. Since there is no CORESET-BFR agreed, 3 BFD RS per BWP is preferred. Issue 3.1-3: beam failure recovery is performed per SCell. Determining beam failure state of a SCell without beam failure recovery configuration can be gNB implementation. |

## New beam identification

It has been agreed that new beam identification can be performed based on SSB and CSI-RS for beam management. The SSB/CSI-RS can be transmitted within the SCell or another CC within the same band. The gNB can configure N SSB/CSI-RS resources per BWP for new beam identification. The range of N is one open issue. In addition, the threshold of new beam identification is based on L1-RSRP. Thus, detail range for the threshold is another open issue.

Issue 3.2-1:

The maximum number of RS for new beam identification per BWP is:

- Alt 1: 63

  - Supported: Apple (one state is reserved for no new beam indication), Sony

- Alt 2: 64

  - Supported: Nokia, Ericsson, Samsung, Intel, Panasonic, Docomo, ZTE, OPPO, MTK, CATT, Huawei, HiSilicon, Qualcomm

- Alt 4: other (please specify)

Issue 3.2-2:

The range of threshold for new beam identification is:

- Alt 1: based on RSRP-Range

  - Supported: Apple, Ericsson, Samsung, Panasonic, Sony, Docomo, Lenovo/Motorola Mobility, ZTE, CMCC, OPPO, CATT, Huawei, HiSilicon, Qualcomm

- Alt 2: other (please specify)

  - Supported: Intel

Proposal 3.2:

- For SCell BFR, support maximum number of RS for new beam identification per BWP to be 64.

- The range of threshold for SCell new beam identification is based on RSRP-Range.

Companies’ comments and views

| Company | Comment |
| --- | --- |
| LGE | Issue 3.2-1 seems correlated with Issue 3.6-2, i.e., how to report ‘no new beam found’ status. Our preference is Alt 2 by assuming that ‘no new beam found’ is reported by a separated field or resource. |
| Nokia | 3.2-1: up to 64 RS was agreed for BFR (RAN1#96bis). Adopt the same for SCell BFR. Separate indication of “no candidate” should be used in a MAC CE. |
| Ericsson | Issue 3.2-1: support alt2. Issue 3.2-2: support alt 1 |
| Intel | Issue 3.2-2: Support Alt. 2. Preference is to support NBI based on RSRP Threshold. |
| Panasonic | Issue 3.2-1: support Alt 2 Issue 3.2-2: support Alt 1 |
| Sony | Issue 3.2-1: support Alt 1 which removes the need to design another MAC CE to conduct the information of “no new beam” to NW. Issue 3.2-2: support Alt 1. |
| Docomo | Issue 3.2-1: Alt 2: 64. The same as SpCell. Issue 3.2-2: Alt. 1: based on RSRP-Range. The same as SpCell. |
| Lenovo/Motorola Mobility | Issue 3.2-2: this should be consistent with R15 new beam selection criteria, i.e. use L1-RSRP as threshold. |
| ZTE | Keep the same UE behavior as in Rel-15. Further enhancement may be unnecessary. |
| CMCC | Issue3.2-1: We think whether a separate indication of “no candidate beam” is needed should be first discussed. If the answer is no, and then if a default value of new beam index indicates no new beam is needed, we support Alt 1(one state is reserved for the failure beam indication). But if a dedicated SR-sequence detection is used to implicitly indicate whether new beam is identified or not, then we support Alt 2. Issue3.2-2: support Alt 1. |
| Huawei/HiSilicon | Issue 3.2-1: Support Alt 2. In RAN1#96bis, it has been agreed up to 64 candidate beams for BFR per BWP can be configured by RRC signaling. The total number of RSs for new beam identification is part of UE capability. Issue 3.2-2: Support Alt 1. |
| vivo | Ok with majority view. |
| Qualcomm | Issue 3.2-1: Support Alt2, i.e. max 64 candidate beams per BWP to be consistent with the agreement below. This also matches max 64 SSBs. Agreement Support the configuration of up to 64 candidate beams for BFR by RRC signalling, without introducing additional MAC CE signalling for down-selecting a subset of beams. - The total number of RSs for new beam identification and layer 1 RSRP measurement are part of UE capability signaling This applies per BWP. Issue 3.2-2: Support Alt1, i.e. based on RSRP range, same as PCell BFR |

## UE behavior when no new beam RS is configured

It has been agreed that when new beam RS is configured, gNB shall configure corresponding threshold for new beam identification. Then, UE shall report the new beam index if it identifies a new beam, or report no new beam identified, otherwise. However, the UE behavior when no new beam RS is configured is still one open issue.

Issue 3.3:

Down-select one of the following alternatives on UE behavior when no new beam RS is configured:

- Alt 1: UE shall expect gNB configures the new beam RS if BFR for corresponding SCell is configured

  - Supported: ZTE, CATT, Qualcomm

- Alt 2: If new beam RS is not configured, UE shall report failed CC index only

  - Supported: CMCC, OPPO

- Alt 3: other (please specify)

  - Panasonic, LGE, Ericsson, Docomo, MTK: If new beam RS is not configured, all SSBs are considered as new beam RS candidates.

  - Huawei, HiSilicon (If candidateBeamRSList is not configured, a default RS set should be used for new beam identification, FFS which default RS set.)

FL recommendation: offline discussion.

Companies’ comments and views

| Company | Comment |
| --- | --- |
| LGE | If no RS is configured for new beam candidates by network, all SSBs can be considered as new beam RS candidates as those SSBs will cover the entire cell. |
| Nokia | Issue 3.3, In any case UE should report at least failed CC index/indices. |
| Ericsson | Support Alt3. |
| Samsung | Agree with Nokia. We can further discuss if Alt3 is a sensible choice for the “default” set of beam RS candidates. |
| Intel | OK with Alt. 1 |
| Panasonic | Support Alt 3 |
| Docomo | Alt. 3 |
| ZTE | Support Alt.1. But, we can live with Alt2. The benefits of Alt3 is unclear. If going with Alt2, the SCell BFR procedure is accelerated compared with Alt-3 when new beam RS is not configured. |
| CMCC | Support Alt 2 |
| Lenovo/Motorola Mobility | Support LGE. |
| OPPO | Support Alt2. New RS beam is not necessary for SCell BFR. All our discussion and agreement in RAN1 so far are based on new beam can be configured or not configured. Alt1 imposes restriction on gNB implementation and Alt3: the benefit is not clear. |
| CATT | Alt-1. Can accept alt-3 as a 2nd choice (pending details). In our view NBI RS is necessary to reconfigure SCell CORESET beams. Alt-2 implies other implementation-based remedies which go against the principle of the WID. |
| Huawei/HiSilicon | Issue 3.3, if RS for new beam identification is not configured, a default RS set can be used for new beam identification. Which RS set is selected as the default one can be further discussed. |
| vivo | People seem to like to reuse Rel-15 behavior that is still working. We support to reuse Rel-15 behavior for this issue. |
| Qualcomm | Support Alt1. Candidate beam RS should be always configured to avoid extra UE behavior. |
| APT | Alt1. Candidate beam RS should be always configured to avoid ambiguity. Do not see much benefit by introducing extra behavior here other than Rel-15. |

## BFRQ procedure for SCell with both UL/DL

The two-steps for SCell with DL only have been agreed, where the BFRQ can be conveyed by two steps:

- Step 1 is carried by a dedicatedly configured SR-like PUCCH in PCell or PSCell to inform gNB beam failure happens

- Step 2 is carried by a MAC CE to report detail information, e.g. failed CC index and new beam index

But the procedure for SCell with both UL and DL is still an open issue.

Issue 3.4:

Down-select one of the following alternatives for BFRQ procedure for SCell with both UL and DL:

- Alt 1: Reuse the same procedure as SCell with DL only

  - Supported: Samsung, Lenovo/Motorola, Docomo, Nokia/NSB, Ericsson, Apple, LGE, nokia, NEC, Intel (without any restriction on MAC-CE transmission over PCell/SCell), Panasonic, Sony, Docomo, OPPO, MTK, Qualcomm, APT

- Alt 2: Reuse the same CBRA based procedure as PCell/PSCell

- Alt 3: Reuse the same CFRA based procedure as PCell/PSCell

- Alt 4: Reuse both CBRA based and CFRA based procedure as PCell/PSCell

  - Supported: ZTE

- Alt 5: other (please specify)

Proposal 3.4:

- For SCell with both UL and DL, reuse the same BFRQ procedure as SCell with DL only.

  - Note: whether to support CBRA based BFRQ for both scenarios is a separate issue.

  - FFS: whether PUCCH-BFR and MAC CE for BFR can be transmitted in other non-failed SCells

Companies’ comments and views

| Company | Comment |
| --- | --- |
| LGE | Support Alt1 |
| Nokia | Same procedure should be used for SCell with DL and UL. UE can send the SR and/or send the MAC CE to a non-failed cell that has UL grant available. |
| Intel | MAC CE in step 2 can be sent over PCell or SCell (if uplink is available) |
| Panasonic | Support Alt 1 |
| Docomo | Alt. 1. |
| ZTE | Support Alt4. In beam correspondence case, the Alt4 has a less latency than Alt1. |
| OPPO | Support alt1. Do not see any motivation to support different mechanisms for DL only and DL/UL. From the perspective of specification effort and simpler system implementation, re-using the same procedure for all the SCell scenarios is the best choice. |
| MTK | Agree with Nokia |
| Huawei/HiSilicon | We suggest clarifying whether CBRA is supported on SCell or not first. |
| vivo | Similar view as Huawei. |
| Qualcomm | For BFRQ for SCell with DL + UL, support at least Alt1. Note that Alt1 assumes that PCell is working. A remaining issue is that if PCell fails, can PCell be recovered via SCell by switching the role? |

## Details for Step 1 PUCCH

It has been agreed that step 1 PUCCH (PUCCH-BFR) is based on a SR-like PUCCH. After transmitting PUCCH in PCell/PSCell, UE could deliver some other information via a MAC CE. Then how many PUCCH resources can be configured as PUCCH-BFR could be one issue. Another issue is the PUCCH format used for PUCCH-BFR. There are some other proposals that are correlated to the two issues, e.g. how to multiplex PUCCH-BFR with other signals. Such issues can be discussed after decision for the two issues are made.

Issue 3.5-1:

With regard to maximum number of PUCCH resources configured for step 1 BFRQ (PUCCH-BFR):

- Alt 1: UE can be configured up to 1 PUCCH-BFR resource

  - Supported: Lenovo/Motorola, Apple, Nokia, Ericsson, Samsung, NEC, Panasonic, Sony, Docomo, ZTE, CMCC, MTK, CATT, Qualcomm, APT

- Alt 2: UE can be configured up to N (N>1) PUCCH-BFR resources, where each PUCCH-BFR corresponds to SCells within a cell group

  - Supported: LG

- Alt 3: Others (please specify)

  - PUCCH for BFR can be configured for SCell with uplink, Nokia

  - Intel: UE can be configured up to N (N=2 or 3) PUCCH-BFR resources, where each PUCCH-BFR corresponds to SCells group

  - OPPO: normal SR is used for step 1.

Issue 3.5-2:

The PUCCH-BFR can be configured based on:

- Alt 1: PUCCH format 0 and PUCCH format 1

  - Supported: LG, Ericsson, Sony, Docomo, ZTE/Qualcomm (FFS: Rules of multiplexing/colliding), MTK, Huawei, HiSilicon, APT

- Alt 2: others (please specify)

Proposal 3.5:

- Support to configure up to 1 PUCCH-BFR resource per UE.

- Support PUCCH-BFR to be configured to be based on both PUCCH format 0 and PUCCH format 1

  - FFS: details when PUCCH-BFR is collided with other uplink signal(s).

Companies’ comments and views

| Company | Comment |
| --- | --- |
| LGE | For Issue 3.5-1, a PUCCH can be configured on a PSCell as well as PCell for BFRQ in case of dual connectivity, i.e., one PUCCH per cell group. For Issue 3.5-2, I slightly modified the wording because this issue is only about the PUCCH resource dedicatedly configured for BFRQ. In our understanding, the BFRQ message can still be transmitted on a PUCCH other than PUCCH-BFR if the PUCCH-BFR is collided with another PUCCH. |
| Nokia | Issue 3.5-1: one PUCCH BFR per PCell. In case for SCell with DL/UL there can be PUCCH for BFRQ per SCell. So in general the could be one PUCCH BFR resource per serving cell. |
| Ericsson | In our understanding, “per cell group” is implicitly understood. For PUCCH formats, SR-like would imply PUCCH formats 0 and 1 |
| Intel | General comment on Issue 3.5: On FFS point on Step 1, support case for skipping Step 1 i.e., step 2 only with transmission of MAC-CE. Issue 3.5-1: PUCCH-BFR resource is used to indicate failed SCell group (e.g. corresponding to the same band). |
| Panasonic | We share the same view with Nokia in term of issue 3.5-1. |
| Sony | Issue 3.5-1: support Alt 1. Issue 3.5-2: agree with Ericsson that Step 1 SR-alike BFRQ (small payload) at least implies PUCCH format 0 and 1. |
| Docomo | Issue 3.5-1: Alt. 1 Issue 3.5-2: Alt. 1, and agree with LG’s comment |
| ZTE | Regarding “cell-group”, we share the same views with Ericsson. If not explicit explanation, all discussion here is based on a cell group, e.g., MCG or SCG. Issue 3.5-1: Considering that the failed CC index(es) are carried by MAC-CE, the PUCCH BFR configuration per cell is not necessary. Issue 3.5-2: Support Alt1. Meanwhile, the multiplexing rules, e.g., PUCCH-BFR (i.e., dedicated SR) + normal-SR/HARQ/CSI, and the colliding rules, e.g., PUCCH-BFR vs ap-SRS, should be further considered. |
| CMCC | Issue 3.5-1: Support Alt 1. |
| Huawei/HiSilicon | Issue 3.5-1: We assume ‘cell group’ in Alt 2 corresponds to either MCG or SCG. Under this assumption, it seems natural to support Alt 2, considering DC case. If the intention of Alt 1 is that even for DC case, there can be only one PUCCH-BFR, we believe this is overly restrictive. Issue 3.5-2: Support Alt 1. |
| vivo | Issue 3.5-1: Same view as Nokia and Panasonic. Issue 3.5-2: Majority view. |
| Qualcomm | For Issue 3-5.1: Support Alt1, i.e. 1 PUCCH-BFR resource to minimize overhead For Issue 3-5.2: All PUCCH formats can be allowed, if the info carried by BFR-SR can also be multiplexed with A/N and CSI in all formats if collided. |

## Details for Step 2 MAC CE

It has been agreed that the MAC CE in step 2 can be used to report failed CC index as well as new beam index. How to transmit the step 2 MAC CE could be one issue. In addition, it has been agreed to report no new beam identified, then how to report this information via MAC CE is another open issue. A third issue is when multiple SCells fail, how to report the step 2 MAC CE.

Issue 3.6-1:

Down-select one of the following alternatives for step 2 MAC CE transmission:

- Alt 1: Step 2 MAC CE can be carried by a dynamic-grant based PUSCH only

  - Supported: Huawei, HiSilicon

- Alt 2: Step 2 MAC CE can be carried by any PUSCH

  - Supported: vivo, OPPO, Apple, Nokia, Ericsson, Samsung (this may be up to RAN2), NEC, Intel, Panasonic, Sony, Docomo, Lenovo/Motorola Mobility, ZTE, CMCC, OPPO, MTK, Qualcomm

- Alt 3: Others (please specify details)

Issue 3.6-2:

Down-select one of the following alternatives on how to report no new beam identified

- Alt 1: one field in MAC CE is reserved to indicate whether a new beam is identified or not

  - Supported: Huawei/HiSilicon, LGE, Nokia, Ericsson, Intel (details up to RAN2), Panasonic, Docomo, ZTE (details up to RAN2), MTK

- Alt 2: a default value of new beam index indicates no new beam is identified

  - Supported: NEC, Xiaomi, Apple, Sony, Lenovo/Motorola Mobility, CMCC, Qualcomm

- Alt 3: whether new beam is identified or not can be carried by PUCCH-BFR resource index

  - Supported:

- Alt 4: Others (please specify details)

  - Supported: Samsung (this issue should be left to RAN2 to decide), NEC

Issue 3.6-3:

When more than 1 SCells fail,

- Alt 1: UE can report 1 MAC CE in step 2 to indicate the failed CC indexes

  - Supported: ZTE, LG, Nokia, Panasonic, Sony, Docomo, OPPO, MTK, Huawei, HiSilicon, Qualcomm

- Alt 2: UE can report multiple MAC CEs, where one MAC CE corresponds to one SCell

  - Supported: Apple, Ericsson, Lenovo/Motorola Mobility

- Alt 3: Others (please specify details):

  - Supported: Samsung (this issue should be left to RAN2 to decide), NEC

Proposal 3.6:

- For SCell BFR, support to transmit the MAC CE for BFR in any PUSCH.

- The detail format for MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2

- Send a LS to RAN2

Companies’ comments and views

| Company | Comment |
| --- | --- |
| LGE | Issue 3.6-1: Does this issue require RAN1 agreement? Alt2 seems the baseline as long as a MAC-CE is used for step2, but this issue may be handled by RAN2. Issue 3.6-2: Support Alt 1 |
| Nokia | Issue 3.6-1: RAN1 can indicate its preference that SCell BFRQ can be multiplexed on any PUSCH. Issue 3.6-2: This relates to discussion on number of candidate RS. “no candidate indication” should a field in a MAC CE. Issue 3.6-3: Design is eventually up to RAN2 but RAN1 can indicate that MAC CE should be able to carry indication of multiple failed SCells (and candidate indexes. |
| Ericsson | Issue 3.6-1; agree with LG, leave to RAN2 Issue 3.6-2: could also be left to RAN2 Issue 3.6-3: could be left to RAN2. The intuition behind Alt2 is that it is extremely unlikely that 2 Scells will fail at the same time, so we should not design MAC CE based on that assumption. But is not an important issue. |
| Samsung | Overall, we more or less agree with LGE, Nokia, and Ericsson that these identified issues should be left to RAN2 to decide. For 3.6-1, however, RAN1 can state our preference that the MAC CE used for step 2 can be carried via any PUSCH transmission. It is perhaps helpful to send an LS to RAN2 so some bandwidth can be allocated in RAN1#98B. |
| NEC | We agree with Samsung. MAC-CE design should be left to RAN2. |
| Intel | Issue 3.6-1: Do not support RAN1 restriction on choice of PUSCH to transmit MAC-CE. Agree with LGE and Ericsson. Issue 3.6-2, 3: Details can be left to RAN2 |
| Panasonic | Issue 3.6-1: We share same view with Nokia as RAN1 indicates its preference and details up to RAN2. Issue 3.6-2: We support Alt 1 and let RAN2 decide this issue in detail. Issue 3.6-3: Share the same view with Nokia. As there are the situations that groups of SCells can share similar failure condition properties, the BF report content includes at least the group identifier for each individual group. |
| Sony | Issue 3.6-1: support Alt 2. It’s not necessary to constrain the PUSCH which carries the Step 2 MAC CE. Issue 3.6-2: support Alt 2. Issue 3.6-3: support Alt 1. |
| Docomo | Issue 3.6-1: Alt. 2. Issue 3.6-2: Alt. 1. Issue 3.6-3: Alt. 1. One MAC CE with variable size can indicate BFR for multiple SCells. |
| Lenovo/Motorola Mobility | 3.6-1: To expedite the beam recovery procedure, MAC-CE should be transmitted in the earliest PUSCH, triggered by the dedicated SCell beam recovery SR or not. |
| ZTE | Issue 3.6-1: Support Alt1. Also we share the same views with Nokia, Samsung and Panasonic that RAN1 shall indicate its preference that SCell BFRQ can be multiplexed on any PUSCH. Besides, Rules of colliding with ap-SRS is also needed to be considered. Issue 3.6-2: Support Alt1. The details are up to RAN2. Issue 3.6-3: Support Alt1. Alt-1 has lower MAC-CE overhead, i.e., MAC-CE header, compared with Alt2. Also, if going with Alt-2, the priority rules of multiple MAC-CE transmission, e.g. higher priority with lowest CC ID, should be specified, which does not seem to be necessary. |
| CMCC | Issue 3.6-1: We think that if there is no available MAC-CE resource, it is preferred that MAC-CE should be carried by a dynamic-grant PUSCH and then UE can report detail information on the scheduled resource. But if the MAC-CE resource is available, the failed CC index(es) and new beam information (if present) can be transmitted using the available MAC-CE resource carried by any PUSCH. So Alt 2 is supported. Issue 3.6-2: we don’t support current version of Alt 3. We support Alt 2 or Alt 3 with the modified wording as: whether new beam is identified or not can be indicated implicitly by dedicated SR-sequence detection. |
| OPPO | These three issues shall be treated by RAN2. However, it might be helpful if RAN1 can discuss them and feedback some information to RAN2. |
| MTK | Issue 3.6-1: We don’t need to restrict the transmission of BFRQ MAC CE on specific PUSCH. It should be transmitted on any available PUSCH to reduce the latency. Issue 3.6-2: We support the separate field for no new beam. Issue 3.6-3: To reduce the overhead, it is better to transmit one MAC CE which includes all information for failed CCs I agree with Ericsson. Actually, these issues should be handled by RAN2. |
| CATT | Prefer to leave these issues to RAN2. |
| Huawei/HiSilicon | Issue 3.6-1: Assuming RAN2 will define minimum delay between MAC-CE-BFR retransmissions, a large recovery latency may be incurred if the UE decides to carry MAC-CE-BFR over PUSCH that is not triggered by PUCCH-BFR (for example, the MAC-CE-BFR may even be transmitted on failed SCell). For this reason, we support Alt 1, where after receiving PUCCH-BFR, NW can be more cautious on scheduling PUSCH resources (e.g., low code rate) and also provide dedicated indication requesting UE to send in MAC-CE-BFR (e.g., a new RNTI). Issue 3.6-2: Support Alt 1. Issue 3.6-3: Support Alt 1. |
| vivo | We are open on whether to discuss these issues in RAN1 or RAN2. Our preference is as following: Issue 3.6-1: Alt 2; Issue 3.6-2: Alt 1; Issue 3.6-3: Alt 2. This issue might be entangled with 3.6-1. The corresponding timeline for different PUSCH may be different. The MAC CE design should take these timeline into account. |
| Qualcomm | For Issue 3-6.1: Support MAC-CE can be directly carried in a PUSCH without Step 1 to save latency/overhead. For Issue 3-6.2: Support Alt2, i.e. use a reserved value of new beam index in MAC-CE to indicate no new beam found For Issue 3-6.3: Support Alt1, i.e. 1 MAC CE can indicate multiple failed SCells to save overhead |
| APT | In general, we agree with Samsung that these issues can be up to RAN2 design. For issue 3-6.3, we see that multiple MAC-CEs with same LCID in one transmission is not common case, and thus prefer Alt 1. |

## Beam failure recovery response

The details for beam failure recovery response (BFRR) is still one open issue. There are some proposals to define a timer for UE to monitor the BFRQ response. When the timer expires, UE can consider BFRQ is not successfully delivered. A second issue is how to transmit the BFRQ response, e.g. whether the BFRQ response is carried by a dedicated CORSET as Rel-15 or not.

Issue 3.7-1:

Down-select one of the following alternatives on UE monitoring behavior for BFRR.

- Alt 1: UE starts a timer after the transmission of step 1 PUCCH

  - Supported: Apple, ZTE(Prohibit Timer), CATT

- Alt 2: UE starts a timer after the transmission of step 2 MAC CE

  - Supported: Huawei, HiSilicon, NEC, ZTE(Prohibit Timer), CMCC

- Alt 3: others (please specify)

  - Intel: RAN1 should only clarify to RAN2 the post step 2 procedure between CORESET BFR monitoring or legacy MAC CE activation/RRC reconfiguration of TCI states. Timers should be up to RAN2.

Issue 3.7-2:

Down-select one of the following alternatives on BFRR to step 1 PUCCH

- Alt 1: The BFRR to step 1 PUCCH is carried by a normal uplink grant with C-RNTI/MCS-C-RNTI

  - Lenovo/Motorola Mobility, OPPO (that is a natural choice with least specification effort), APT

- Alt 2: The BFRR to step 1 PUCCH is carried by an uplink grant with a dedicated RNTI for SCell BFR

  - Supported: Huawei, HiSilicon, Apple, Sony

- Alt 4: The BFRR to step 1 PUCCH is carried by a dedicated SS/CORESET

  - Supported: Fujitsu,

- Alt 5: others (please specify)

  - BFRR to step 1 PUCCH is not supported: ZTE

Issue 3.7-3:

Down-select one of the following alternatives on BFRR to step 2 MAC CE

- Alt 1: The BFRR to step 2 is carried by a dedicated SS/CORESET

  - Supported: Huawei, HiSilicon, Sony

- Alt 2: The BFRR to step 2 is carried by a MAC CE

  - Supported: ZTE, Lenovo/Motorola Mobility, OPPO, APT

- Alt 3: The BFRR to step 2 is carried by an uplink grant with a dedicated RNTI for SCell BFR

  - Supported: ZTE

- Alt 4: The BFRR to step 2 is a normal uplink grant to scheme a new transmission for the same HARQ process as step 2 MAC CE

  - Supported: Ericsson, Apple, Intel (w/o HARQ process), OPPO, APT

- Alt 5: others (please specify)

FL recommendation: offline discussion

Companies’ comments and views

| Company | Comment |
| --- | --- |
| LGE | In general, existing SR procedure specified in TS38.321 should be a starting point for Issue 3.7-1/2/3. For Issue 3.7-1, BFRR monitoring timer seems not needed as there is no such timer for SR (re)transmission. |
| Nokia | Response discussed when BFRQ details are agreed. |
| Ericsson | Existing procedures can be reused: there is already a timer after which the UE repeats SR. The UE would retransmit the MAC CE if instructed by the NW – HARQ. The MAC CE will have an identifier so that the NW knows that beam failure has happened: there is no need for any special RNTI. |
| Samsung | At this point we agree with LGE and Ericsson. We do not see any reason not to reuse the existing SR procedures. |
| NEC | Agree with LGE, SR procedure is enough for sending MAC-CE. But considering there is no ACK/NACK for uplink transmission, we may need to clarify when UE can declare a successful BFR for Scell. |
| Panasonic | Share the same view with Nokia |
| Sony | Sorry for the misleading proposal in our Tdoc. For Issue 3.7-2, we believe that Alt.2 is our original intention to support. For Issue 3.7-3, support Alt 1. which is somehow aligned with Rel.15 BFR for PCell. |
| ZTE | Issue 3.7-1: Support Alt1 and Alt2, where the prohibit timer are both required for Step-1 and Step-2 procedure, specifically. - The timer for step-1 is to prevent the frequent re-transmission of PUCCH-BFR after SCell fails but not receiving gNB response for step2 - The timer for step-2 is to prevent the frequent re-transmission of MAC-CE for recovery reporting Issue 3.7-2: Support Alt5 “BFRR to step 1 PUCCH is not supported”. The cancelation for pending dedicated BFRQ/SR is performed, once the Step-2 MAC-CE carrying the failed SCell is re-transmitted. It is similar with SR procedure. Issue 3.7-3: Support Alt2 and Alt3, but we can live with Alt4 (but with the same HARQ process). |
| CMCC | Issue 3.7-1: we support Alt 2 considering the Step 2 MAC CE can be carried by any PUSCH and it is not always carried by a dynamic-grant based PUSCH only case. |
| Lenovo/Motorola Mobility | Issue 3.7-1: We should reuse the existing SR procedure. There is no need to define a new timer. Issue 3.7-2: Since the signaling and transmission of step 2 MAC-CE is carried in the Pcell which is working, MAC-CE can be signaled like normal PUSCH by PUCCH with C-RNTI. There is no need to define a separate procedure. |
| OPPO | We prefer the BFRR to be discussed after BFRQ details is settled down. |
| MTK | Issue 3.7-1: I don’t think we need specific new timer for BFRQ transmission. We can reuse the existing SR / MAC CE retransmission procedure. Issue 3.7-2: BFRR to step1 PUCCH is unnecessary. Issue 3.7-3: Agree with Nokia. We can discuss this after finalizing BFRQ transmission. |
| Huawei/HiSilicon | Issue 3.7-1: Support Alt 2. Issue 3.7-2: Support Alt 2. Issue 3.7-3: Support Alt 1. |
| vivo | We may first need to clarify the intention for these discussions. The intention raised by ZTE is fine to us, but solution-wise we are have different views. - to prevent the frequent re-transmission of PUCCH-BFR after SCell fails but not receiving gNB response for step2 - to prevent the frequent re-transmission of MAC-CE for recovery reporting |
| APT | Issue 3.71-1: for step-1, reuse SR prohibit timer similar to Rel-15. For step-2, it is not necessary to set a timer to constrain the transmission frequency. Since step-2 is MAC-CE based, its retransmission is controlled by HARQ mechanism. However, we do see benefit to set a timer to decide unsuccessful SCell BFR in case BFRR for step-2 is not received. Issue 3.7-2: same view as MTK. BFRR to step-1 is not necessary. Issue 3.7-3: prefer Alt 2 or alt 4. |

## Others

There are some other proposals on UE capability, multiplexing of PUCCH-BFR and other signals. As these could be correlated to some issues above, e.g. PUCCH format. They could be handled after the relevant decisions have been made.

# Reference

- R1-1908067	Enhancements on multi-beam operation	Huawei, HiSilicon

- R1-1908167	Further discussion on Multi-beam operation	vivo

- R1-1908192	Enhancements on Multi-beam Operation	ZTE

- R1-1908233	Views on Panel Activation and Deactivation	InterDigital, Inc.

- R1-1908324	Enhancements on multi-beam operation	Fujitsu

- R1-1908352	Discussion on Multi-beam Operation Enhancements	OPPO

- R1-1908380	Enhancements on Multi-beam Operation	MediaTek Inc.

- R1-1908427	Discussion of Beam Failure Recovery for SCell	China Unicom

- R1-1908502	Enhancements on multi-beam operation	Samsung

- R1-1908603	On multi-beam enhancements	CATT

- R1-1908654	Discussion on multi-beam enhancements	Intel Corporation

- R1-1908700	Discussion on multi-beam based operations and enhancements	LG Electronics

- R1-1908721	Discussion of multi-beam operation	Lenovo, Motorola Mobility

- R1-1908784	Enhancements on multi-beam operation	Sony

- R1-1908857	Discussion on multi-beam operation	NEC

- R1-1908870	Enhancements on multi-beam operation	CMCC

- R1-1908886	Enhancements on multi-beam operation	China Telecommunications

- R1-1908928	Discussion on Multi-beam Operations	Asia Pacific Telecom co. Ltd

- R1-1908959	Discussion on multi-beam operation	Spreadtrum Communications

- R1-1908973	Enhancements on Multi-beam Operation	Fraunhofer IIS, Fraunhofer HHI

- R1-1908975	On enhancements for multi-beam operations for NR MIMO in Rel. 16	Panasonic

- R1-1909048	Remaining Issues for Multi-beam opeartion	Apple Inc.

- R1-1909076	Enhancements on multi-beam operation	AT&T

- R1-1909126	Enhancements on beam management	Beijing Xiaomi Electronics

- R1-1909202	Discussion on multi-beam enhancement	NTT DOCOMO, INC.

- R1-1909210	Enhancements on Multi-beam Operation	Nokia, Nokia Shanghai Bell

- R1-1909225	Enhancements to multi-beam operation	Ericsson

- R1-1909273	Enhancements on Multi-beam Operation	Qualcomm Incorporated

- R1-1909382	Enhancements on multi-beam operation  	KDDI Corporation