3GPP TSG-RAN WG1 Meeting #98	R1-1909485

Prague, Czech Republic, 26th – 30th August 2019

Source:	Ericsson

Title:	FL summary#0 for physical layer control procedures for NTN

Agenda Item:	7.2.5.2

Document for:	Discussion and Decision

# Introduction

This document contains a summary of the contributions under AI 7.2.5.2 at RAN1#98. Discussions are summarized in Section 2 along with proposals.  The observations and proposals from individual contributions are listed in Section 4.

# Summary

The main topics in the contributions submitted to AI 7.2.5.2 are:

- UL power control

- AMC/CSI feedback

- Satellite beam switching

- Uplink transmission timing

The discussions under each of these topics are summarized below.

## UL power control

In RAN1#97, the following agreement related to UL power control was made:

Agreement:

The need and the applicable scenarios for potential enhancements (with respect to the power control schemes in NR Rel-15) for both open-loop and closed-loop power control for NTN are to be studied.

The proposals related to UL power control (ULPC) from contributions submitted to RAN1#98 are given in Section 4.1 of this paper.  Similar to RAN1#97, most discussions revolve around the use of open-loop power control (OLPC) versus closed-loop power control (CLPC).  Some companies such as MediaTek, ZTE, and PCL express doubts on the feasibility of CLPC due to the long RTT and propose to focus on OLPC.  On the other hand, Ericsson, Nokia, Intel, and Panasonic propose to study both CLPC and OLPC for different NTN scenarios.  It should be noted that whether to study CLPC, OLPC or both is more or so already captured in the Agreement from RAN1#97 above.  Hence, no further agreements are needed for this part.

However, there are some additional aspects highlighted by some companies which are summarized below:

- Distance based ULPC:  CATT/Samsung propose distance-based ULPC for some NTN scenarios such as LEO.  Nokia proposes that ULPC without UE location information should be the starting point for study.

- Extension of ULPC parameter ranges:  Intel propose that the range of OLPC parameters (e.g. P0) and the range of absolute power adjustment command for CLPC may need to be revised for NTN

- Impact of transparent satellite UL and DL gains: Nokia proposes to study whether gNB can use information on transparent satellite UL and DL gains to perform ULPC.

It should be noted that despite the discussions in the last two meetings, evaluation results are lacking for ULPC for NTN.  Hence, to make better progress and to study the need for different ULPC and potential enhancements, companies are encouraged to evaluate the relevant schemes until next RAN1 meeting.  Companies are welcome to provide their additional views below:

| Company | Comments and Views |
| --- | --- |

## AMC/CSI feedback

In RAN1#97, the following agreement related to AMC/CSI feedback was made:

Agreement:

Study the performance of AMC in NTN considering at least the following solutions (some solutions may have no specification impact):

- Prediction-based link adaptation with prediction confidence level

- AMC with CQI reflecting only long-term fading

- Additional BLER targets for CQI reporting to limit number of retransmissions and latency

- CQI offset applied by gNB

- Finer granularity of CQI

- Prediction based CQI reporting

The proposals related to AMC/CSI feedback from contributions submitted to RAN1#98 are given in Section 4.2 of this paper.  Majority of the proposals are to study the different schemes which are already captured in the above agreement.

In the contributions submitted to RAN1#98, only very limited preliminary evaluation results are available on AMC/CSI feedback for NTN.

Based on preliminary results presented in R1-1909401, ZTE observed performance losses in case of scheduling with conservative ACM.

In R1-1909507, Ericsson provides preliminary evaluation results and observe the following:

- There is throughput loss with increasing feedback delay.  However, the throughput loss saturates beyond a certain feedback delay depending on the UE speed.

- At 3 km/hr UE speed, channel averaging to capture long-term fading does not improve throughput compared to the case with no channel averaging.  At 60 km/hr UE speed, averaging channel likely results in pessimistic CSI and thus throughput degrades compared to the case with no averaging.

However, it should be noted that the observations in R1-1909401 and R1-1909507 are for a limited number of schemes and are not enough to make general conclusions on the performance of certain schemes.  Hence, to make better progress and to study the need for potential enhancements on AMC/CSI feedback, companies are encouraged to evaluate the relevant schemes until next RAN1 meeting.  Companies are welcome to provide their additional views below:

| Company | Comments and Views |
| --- | --- |

## Satellite Beam switching

The proposals related to Satellite beam switching are given in Section 4.3 of this paper.  The proposals can be summarized as follows:

- ZTE propose to consider the NR Rel-15 beam management framework for Tx beam switching in NTN.  However, ZTE further proposes to consider timer based switching for BM to reduce signaling overhead and latency.

- Qualcomm proposes to consider additional BM CSI-RS configurations to support different satellite implementation needs

- CATT and Asia Pacific Telecom propose to consider BWP switching mechanisms to support fast satellite beam switching.

Based on the  summarize, the following proposal is made:

Offline Proposal 1: Study beam switching mechanism for NTN

- NR Rel-15 beam management and bandwidth part switching are the starting points for discussions and whether further enhancements are needed is FFS.

Companies are welcome to provide their views on Proposal 1 below:

| Company | Comments and Views |
| --- | --- |

## Uplink Transmission timing

The proposals related to uplink transmission timing are given in Section 4.4 of this paper.  The proposals are summarized below:

- UL HARQ ACK on PUCCH:  MediaTek and Ericsson propose to add an offset value to the transmission timing of UL HARQ ACK on PUCCH.  According to the proposal, for a scheduling DCI received in slot n, the slot in which to transmit HARQ ACK on PUCCH is given by n + K1 + K1,offset.  The value of K1,offset is configurable depending on RTT associated with different NTN scenarios.

- UL transmission on PUSCH:  MediaTek and Ericsson propose to add an offset value to the transmission timing of PUSCH.  According to the proposal, for a scheduling DCI received in slot n, the slot in which to transmit PUSCH is given by n∙2μPUSCH-μPDCCH+K2+K2,offset.  The value of K2,offset is configurable depending on RTT associated with different NTN scenarios.

- CSI transmission on PUSCH: Ericsson proposes to add an offset value to the transmission of CSI on PUSCH.  According to the proposal, for a DCI requesting CSI in slot n, the slot in which to transmit CSI on PUSCH is given by n +K+Koffset, where K is selected by the DCI.  The value of Koffset is configurable depending on RTT associated with different NTN scenarios.

In addition to the above companies, CATT and Huawei also propose to add an additional offset to the UL transmission timing.  With regards to how the above offset values are signaled, some companies propose to broadcast these offset (e.g., via SIB) while others propose these offsets to be UE specifically configured.   In addition, some companies also propose that the different offsets can have the same value.  Based on the above summary, the following proposal is made:

Offline Proposal 2: For UL transmission timing, introduce the configurable offset Koffset  for NR NTN.

- For UL HARQ-ACK on PUCCH, where HARQ ACK on PUCCH is transmitted on slot n + K1 + Koffset when a scheduling DCI is received in slot n.

- For UL transmission on PUSCH, where PUSCH is transmitted on slot n∙2μPUSCH-μPDCCH+K2+Koffset when a scheduling DCI is received in slot n.

- For CSI transmission on PUSCH, where CSI on PUSCH is transmitted on slot n +K+Koffset, when the DCI with CSI request is received in slot n and K is selected by the DCI.

- For a CSI report in uplink slot n’, the CSI reference resource is given in downlink slot n-nCSI_ref-Koffset, where  and nCSI_ref is as defined in 38.214.

- Koffset  is signaled by higher layers per beam or per-cell to the UE

Companies are welcome to provide their views on Proposals 2 and 3 below:

| Company | Comments and Views |
| --- | --- |

# References

RP-190710, “SID NR-NTN solutions_v61”.

# Appendix: Proposals in the summarized contributions

## Proposals on UL power control

| Company | Proposal |
| --- | --- |
| CATT, CAICT (R1-1908590) | Proposal 3: Support distance based UL power control in LEO case. |
| Huawei, HiSilicon, CAICT (R1-1908048) | Proposal 2: The existing NR Rel-15 power control scheme can be reused for NTN. |
| Intel Corporation (R1-1908642) | Range of parameters of UL power control such as UE-specific and cell-specific power offset P0 for open-loop power control, range of absolute power adjustment command for closed-loop power control might need to be revised for NTN |
| MediaTek Inc. (R1-1908014) | Observation 4: UE transmission power headroom limits effective closed-loop power control. Proposal 10: Study open-loop UL power control enhancements for NR NTN. |
| Nokia, Nokia Shanghai Bell (R1-1908251) | Proposal 1: UL power control without UE location information should be studied in the first step. Proposal 2: The performance of close loop power control should be studied under the different propagation delays for different NTN scenarios. Proposal 3: Power control should also be studied when HARQ is disabled. Proposal 4: RAN1 to agree how transparent satellite gain and transmit power is configured for service and feeder links. Observation 2: UE uplink transmit power control does not account for transparent satellite service-to-feeder link and feeder-to-service link gains. Proposal 5: RAN1 to study whether gNB can use information on transparent satellite uplink and downlink gains to adjust UE uplink transmit power control, for multiple channels, through broadcast or dedicated signaling. |
| Panasonic (R1-1908938) | Proposal 1: For NTN, open-loop power control and closed-loop power control as in Rel.15 should be considered, at least for GEO satellite communication. Proposal 2: For NTN, accuracy of open-loop power control should be studied. |
| PCL (R1-1908128) | Proposal 1: In power control procedure of NTN, the open loop method should be used. Proposal 2: The open loop procedure used for power control should be conservative to guarantee the robustness of transmission. |
| Samsung (R1-1908487) | Proposal 3: Open loop power control should be further studied in NTN and UE should be allowed to predict its own transmission power not only based on DL measurement, e.g., pathloss measurement but also other available information, such as gNB ephemeris and UE trajectory. Proposal 4: Closed loop power control should be studied in NTN and group power control can be considered. Proposal 5: Physical layer procedures for bent-pipe architecture need to be further studied taking both the link between the UE and the satellite and the link between the satellite and the gNB on ground into account. |
| Sony (R1-190877) | Proposal 1: Power control procedure based on using the using satellite ephemeris should be studied. |
| ZTE (R1-1909399) | Proposal 1: The open-loop power control mechanism should be prioritized for NTN. Proposal 2: Power head room reporting can be considered for NTN in case of VSAT terminal. |
| Ericsson (R1-1909507) | Observation 4: In addition to open loop power control, closed loop power control can be relevant in cases where the pathloss drastically changes in which case it is important to ensure that the UE can quickly adapt its UL transmit power. Proposal 6: Study whether enhancements to Rel-15 open-loop and/or closed-loop power control features are needed in NTN scenarios. |

## Proposals on AMC/CSI feedback

| Company | Proposal |
| --- | --- |
| Asia Pacific Telecom (R1-1908934) | Proposal 1: For CSI report, configurable BLER targets shall be considered. Proposal 2: For RACH, configurable MCS tables shall be considered. Proposal 3: For CSI report, polarization mismatch on CQI calculation shall be considered. Proposal 4: For CSI report, specification on the long-term CQI measurement shall be considered. |
| CATT, CAICT (R1-1908590) | Proposal 3: Support additional BLER target to reduce the retransmission latency Proposal 4: Consider finer CQI granularity for NTN case. |
| Huawei, HiSilicon, CAICT (R1-1908048) | Proposal 1: Prediction-based AMC scheme can be considered in certain scenario. |
| Intel Corporation (R1-1908642) | Proposal 1: Study solutions for fast CQI feedback during initial access and handover |
| MediaTek Inc. (R1-1908014) | Proposal 8: Study the performance of AMC including potential solutions/enhancements in NTN as follows: - AMC with DCI-triggered aperiodic CQI reflecting change in LOS between the satellite and the UE and long-term fading - CQI offset is applied by gNB to avoid mismatch between CSI report and the channel condition at the time of the transmission - Lower BLER target for CQI reporting for a small number of retransmissions to avoid excessive latency Proposal 9: Study DCI trigger for aperiodic CSI report. |
| Nokia, Nokia Shanghai Bell (R1-1908251) | Proposal 6: CQI offset applied by gNB can be used for UL/DL MCS selection in NTN without specification impact. Proposal 7: Prediction-based link adaptation can be implemented at the network side, and UE is expected to report what UE measures. Proposal 8: Short-term CQI reporting might be better than long-term CQI reporting in LEO scenario. Proposal 9: Performance evaluation is needed if introducing new BLER target for MCS selection and CQI reporting. |
| Panasonic (R1-1908938) | Proposal 3: For NTN, CQI table with narrower range of SINR for long term averaging for GEO and relatively short term averaging for LEO and finer granularity within that range should be studied and evaluated. |
| PCL (R1-1908128) | Proposal 3: Adopting the CSI inference based on CSI report may improve the level of matching between CSI and ACM scheme. |
| Qualcomm (R1-1909260) | Proposal 1: Consider increased CQI levels for NTN. Proposal 2: Consider additional target BLER other than 1e-1 and 1e-5 for CQI reporting for NTN. |
| Samsung (R1-1908487) | Proposal 1: Large propagation delays in NTNs have impact on AMC and following solutions can be considered • gNB applies AMC offset; • AMC based on long term channel; • Prediction with confidence level. Proposal 2: The feasibility of both long term and short term CSI and RRM measurement and feedback should be further investigated. |
| Sony (R1-190877) | Proposal 2: RAN1 should support prediction-based link adaptation using information related to the position of gNB and UE. Proposal 3: Enhancement of CQI reporting and/or MCS should be supported. Proposal 4: Repetition transmission should be studied. |
| ZTE (R1-1909399) | Observation 1: Significant performance loss will occurs in case of scheduling with conservative ACM. Proposal 3: Prediction based CSI report can be considered for NTN. |
| Ericsson (R1-1909507) | Observation 2: There is throughput loss with increasing feedback delay. However, the throughput loss saturates beyond a certain feedback delay depending on the UE speed. • At UE speed of 3 km/hr, the throughput loss saturates beyond 40 ms feedback delay. • At UE speeds of 30 km/hr and 60 km/hr, the throughput loss saturates beyond 5 ms feedback delay. • At 60km/h the throughput loss is negligible. Observation 3: At 3 km/hr UE speed, channel averaging to capture long-term fading does not improve throughput compared to the case with no channel averaging. At 60 km/hr UE speed, averaging channel likely results in pessimistic CSI and thus throughput degrades compared to the case with no averaging. |

## Proposals on Satellite Beam Switching

| Company | Proposal |
| --- | --- |
| Asia Pacific Telecom, MTI (R1-1908934) | Proposal 5: For L1-mobility, the association between satellite beams and BWPs shall be considered |
| CATT, CAICT (R1-1908590) | Proposal 1: Support BWP switching enhancement to reduce overhead and latency of satellite beam switching. |
| Qualcomm (R1-1909260) | Proposal 3: Consider additional BM CSI-RS configurations to support different satellite implementation needs. |
| ZTE (R1-1909399) | Proposal 4: Beam management based on current specification can be considered for NTN to achieve the Tx beam switch at gNB side as the baseline. Proposal 5: Additional enhancement, e.g., timer based switching, can be considered for BM to reduce the signaling overhead and latency. |

## Proposals on Uplink Transmission Timing

| Company | Proposal |
| --- | --- |
| CATT, CAICT (R1-1908590) | Proposal 5: When configuring scheduling timing, additional TA offset can be derived from RTT indication in the SIB signalling. |
| Huawei, HiSilicon, CAICT (R1-1908048) | Proposal 3: A constant offset in transmission related timing can be considered in NTN. |
| MediaTek Inc. (R1-1908014) | Observation 1: The TA_ntnOffset, K1_ntnOffset, and K2_ntnOffset have same duration in time. Proposal 1: The UL transmission timing offset corresponds to the common propagation delay between edge of the beam spot and the gNB to accommodate satellite RTT on the access link. Proposal 2: The UE transmits with UL timing based on DL timing of the reference cell – (NTA + NTA_offset) x Tc. Proposal 3: The gNB adjusts scheduling delay to accommodate satellite RTT for - UL HARQ ACK on PUCCH by n + K1 + K1_ntnOffset slots to accommodate satellite RTT. - UL data transmission on PUSCH by {⌊n∙2^(μ_PUSCH-μ_PUCCH )⌋} + K2 + K2_ntnOffset slots Proposal 4: TA_ntnOffset, K1_ntnOffset, and K2_ntnOffset are broadcast by gNB on SIB. |
| ZTE (R1-1909399) | Observation 2: UL scheduling error will occur due to the limited supported value of k2. Proposal 6: Extension of the scheduling time interval for uplink transmission should be considered for NTN with potential consideration on signaling overhead reduction. |
| Ericsson (R1-1909507) | Proposal 1: For HARQ-ACK transmission timing with small TA and large TA, either extend the range of K1 values or introduce a configurable offset to compensate for differential RTT and full RTT, respectively. Proposal 2: For aperiodic CSI transmission timing with large TA, either extend the range of slot offset values or introduce a configurable offset to compensate for full RTT. Proposal 3: For PUSCH transmission timing with large TA, either extend the range of K2 values or introduce a configurable offset to compensate for full RTT, respectively. Proposal 4: For small TA, study signaling methods for reference RTT. Proposal 5: Study the impact of large propagation delays on CSI reference resource, and revisit CSI reference resource definition for NTN scenarios with large propagation delays. |