3GPP TSG-RAN WG1 Meeting #98	R1-1909489

Prague, Czech Rep., August 26th – 30th 2019

Agenda Item:	6.2.2.1

Source:	Ericsson

Title:	Feature lead summary of UE-group wake-up signal in NB-IoT

Document for:	Discussion, Decision

# Introduction

One of the objectives with the Rel-16 WID for additional enhancements for NB-IoT is to specify [1]:

Improved DL transmission efficiency and/or UE power consumption: - … - Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4]

Related to the above objective, the following agreements were made for group WUS in RAN1 #97 [2]:

Working Assumption At least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to wgroup(m’) = w(m’) exp(j2πgm/G) - FFS: Details of g and G Agreement If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, the common WUS sequence for all the group WUS UEs in the same WUS resource can be configured to be the Rel-15 WUS sequence or a Rel-16 WUS sequence. Working Assumption The UE may assume that the transmit power for Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence. Working Assumption The maximum WUS duration for the Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence. Agreement Each UE monitors up to X WUS sequences. - Value of X will be selected between 2 and 3 - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

This feature lead summary is based on the contributions in [1]-[11].

# Discussion

## Configuration

What is the maximum number of configurable groups per WUS resource?

>6:				Sony

8:				Qualcomm

≥8:				Huawei

The more the better:	Ericsson

The maximum number of UE groups per WUS resource is 8. FFS whether this includes a mobility group or if that is in addition to the 8.

Should group configuration be uniform for all resources or done per resource?

Uniform configuration for all resources:	Ericsson

Separate configuration for all resources:	Nokia

No consensus yet regarding UE resource configuration.

Should weighting of the UE distribution be implemented?

Yes:	LGE, Nokia, Qualcomm

No:	Ericsson, ZTE

No consensus yet regarding weighting of the UE distribution.

Should separate paging carriers be introduced for Rel-16 (WUS) UEs?

Yes:	Ericsson, Qualcomm, Sony

Send LS to RAN2 asking them to introduce separate paging carriers for Rel-16 WUS UEs.

Should it be possible to configure the UE to alternate between WUS resources?

Yes:	Huawei, ZTE

FFS:	Ericsson

## Sequence design

How to separate different UE groups within a resource?

Working assumption

At least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to w_(group)(m’) = w(m’) exp(j2πgm/G)

Confirm WA: Ericsson, Qualcomm

Revert WA:

Confirm working assumption that at least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to w_(group)(m’) = w(m’) exp(j2πgm/G)

What are the parameters G and g for the phase shift?

G = 132, g = 2+13*UE_group, 0 ≤ UE_group ≤ 10:		Ericsson

G = 132, g = 14(UE_group+1), 0 ≤ UE_group ≤ 7:		Qualcomm

G=132, g = 12*UE_group, 0 ≤ UE_group ≤ 10:		Huawei

G=132, g = 13*UE_group, 0 ≤ UE_group ≤ 10: 		Huawei

G = 131 or 132, g = 2+13*UE_group, 0 ≤ UE_group ≤ 10:	MediaTek

TBD until #groups are agreed:					ZTE

G = 132 and g = [2+13*UE_group OR 14*(UE_group+1)]

What sequence is common WUS?

UE_group = 10 (g=0) if not shared with legacy and configurable to UE_group 9 or 10 if shared:																Ericsson

g = 126:													Qualcomm

The common WUS sequence is given by the outcome of Proposal 4.

Should different sequences be used for each WUS resource?

Yes: Ericsson, Huawei, LGE, [Nokia], Qualcomm

No:

Different sequences are used for different WUS resources.

How should the different sequences for different resources be defined?

Use different scrambling initialization: Ericsson, Qualcomm

Different WUS resources use different scrambling initialization, c_init.

## Multilevel grouping

Number of simultaneous sequences to detect

2:	Ericsson, NTT DoCoMo, Qualcomm, Sony

3:	ZTE

If more than X groups per resource: 	Nokia

A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs.

## Power and duration

Use same power and max duration as for Rel-15

Confirm WAs:	Ericsson, NTT DoCoMo, Qualcomm, Sony

Revert WAs:		ZTE

Working Assumption

The UE may assume that the transmit power for Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

Working Assumption

The maximum WUS duration for the Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

Confirm the following working assumptions:

  - The UE may assume that the transmit power for Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

  - The maximum WUS duration for the Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

# Conclusion

In the previous sections we made the following observations:

Observation 1	No consensus yet regarding UE resource configuration.

Observation 2	No consensus yet regarding weighting of the UE distribution.

Observation 3	The common WUS sequence is given by the outcome of Proposal 4.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The maximum number of UE groups per WUS resource is 8. FFS whether this includes a mobility group or if that is in addition to the 8.

Proposal 2	Send LS to RAN2 asking them to introduce separate paging carriers for Rel-16 WUS UEs.

Proposal 3	Confirm working assumption that at least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to w_(group)(m’) = w(m’) exp(j2πgm/G)

Proposal 4	G = 132 and g = [2+13*UE_group OR 14*(UE_group+1)]

Proposal 5	Different sequences are used for different WUS resources.

Proposal 6	Different WUS resources use different scrambling initialization, c_init.

Proposal 7	A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs.

Proposal 8	Confirm the following working assumptions:

a.	The UE may assume that the transmit power for Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

b.	The maximum WUS duration for the Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

# References

RP-191576, “WID revision: Additional enhancements for NB-IoT,” Futurewei, RAN #84, Newport Beach, USA, June 2019.

R1-1907638, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT,” Futurewei, RAN1 #97, Reno, USA, May 2019.

R1-1908027, “UE-group wake-up signal in NB-IoT,” Ericsson, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1908086 “UE-group wake-up signal,” Huawei, HiSilicon, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1908264, “Discussion on Wake-up signal for NB-IoT,” ZTE, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1908300, “UE group wake-up signal for NB-IoT,” Nokia, NSB, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1908528, “Discussion on UE-grouping wake up signal in NB-IoT,” LG Electronics, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1908747, “UE-Group WUS in NB-IoT,” MediaTek, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1908761, “UE group wake up signal for NB-IoT,” Sony, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1908834, “UE-group wake-up signal,” Qualcomm, RAN1 #98, Prague, Czech Rep., August 2019.

R1-1909169, “Discussion on UE-group wake up signal in NB-IoT,” NTT DoCoMo, RAN1 #98, Prague, Czech Rep., August 2019.