3GPP TSG RAN WG1 #98		R1-1909501
Prague, CZ, August 26th – 30th, 2019

Agenda item:	7.2.13.2

Source:	Apple Inc.

Title:	FL Summary of remaining issues on enhancements to single Tx switched uplink for EN-DC

Document for:	Decision

# Introduction

In this document, we summarize the open issues and topics discussed in related contributions [1]-[9] to RAN1 #98 under agenda item 7.2.13.2.

# Some previous agreements

In RAN #81 [14], it was further agreed to include the single Tx enhancements for the R16 work item on “MR-DC and CA enhancements”[15]. The WID was also updated in RAN #84 [16] as follows:

- Study and, if found beneficial over the existing single Tx switched uplink solution, specify enhancements to single Tx switched uplink solution for EN-DC, such as allowing all DL and UL subframes for data transmission for both NR and LTE. [RAN1]. Note: UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG - Enable the Release 15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” to apply to dual uplink EN-DC, possibly including any conclusions on the previous objective (6) for LTE FDD Pcells [RAN1].

Later, in RAN1 Ad-Hoc #1901[16], the following agreement was made:

Agreements: - It is concluded that it is beneficial to specify enhancements to single Tx switched uplink solution for EN-DC over the existing single Tx switched uplink solution, e.g. via introducing a new reference HARQ configuration using NR/LTE Rel-15 or LTE eIMTA solution, etc.

In RAN1 #96, a similar “SUO case 1” with DL-reference HARQ timing as in the EN-DC with LTE FDD PCell was agreed for EN-DC with LTE TDD PCell [11]:

Agreements: For single tx switched UL in EN-DC with TDD PCell, the LTE PCell can be configured with DL-reference UL/DL config - For DL HARQ timing, the DL-reference UL/DL configuration is applied - Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference - For UL HARQ timing is the same as without reference configuration for PCell’s UL/DL configuration other than TDD pattern 0/6, - FFS: whether/how to support TDD pattern 0/6 for LTE PCell - UE is not expected to transmit on the MCG and SCG simultaneously - For type 2 UE (i.e., UE without dynamic power sharing capability): - UE is allowed to transmit LTE PUSCH only in the UL subframes designated as UL in the DL-reference configuration - For type 1 UE (i.e., UE with dynamic power sharing capability): - The UE should not assume that LTE PUSCH is only scheduled in the UL subframes associated configured by the DL-reference configuration - The UE should not assume that NR PUSCH is only scheduled in the remaining UL subframes other than those configured by the DL-reference configuration - If there is a collision, - In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH - In other UL subframes, UE behaviour to be expected to specified with details FFS - E.g., drop LTE PUSCH, drop NR PUSCH, etc. - FFS for the case of NR SRS & NR PRACH - Note: the impact of switching time (if non-zero) will be further studied. - Note: the above does not assume any restriction between gNB and eNB (e.g., tight coordination) - FFS: whether/how to support HARQ-offset (similar as in SUO case 1 in EN-DC with LTE FDD PCell)

In RAN1#96bis

Agreements: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following clarifications are agreed: - For the LTE TDD PCell: use the PDSCH ACK timeline as in Table 10.1.3.1-1 in 36.213. - For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell - For the LTE FDD SCell: use the PDSCH ACK timeline defined for SCell as in case of LTE FDD- TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - FFS - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) Agreements: For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration, - Support PUCCH formats 3/4/5, PUCCH fallback operation based on implicit resource indication is not supported. - FFS PUCCH format and resource determination when the UE only receives DAI=1 - FFS whether or not there are any additional issues related to SPS

From the above agreement, there are at least the following open issues to be further concluded for SUO case 1 in EN-DC with LTE TDD PCell:

- DL HARQ timing for LTE TDD SCells

- Supportable LTE TDD patterns, i.e., are pattern 0 & 6 supported?

- How to handle the case of “collision” between UL transmissions over MCG and SCG

- LTE PUCCH format to use when DAI=1

- Support of HARQ-offset as in single Tx for EN-DC with FDD PCell.

# Clarification on DL HARQ timing for “SUO case1” in EN-DC with TDD PCell

The main open question is: What is the DL HARQ timing for LTE TDD SCell with SUO case1 in EN-DC with LTE TDD PCell?

The main aspect for consideration is the fact that the TDD SCell can have same or different UL/DL configuration (as in SIB) from the TDD PCell, where adopting the same DL-ref HARQ timing as the PCell may not always work. A few different solutions were proposed in the contributions to RAN1 #98.

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 8: For LTE DL CA, the DL HARQ timing of TDD SCell with TDD PCell: - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) |
| Apple | Proposal 1: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell: - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) |
| ZTE | Proposal 4: Confirm the following FFS - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) |
| LGE | Proposal #4: Determine HARQ-ACK timing for LTE TDD Scell as for the case of Pcell with UL/DL configuration 2/4/5 in TDD CA with different UL/DL configuration |
| Samsung | Proposal 3: For a LTE TDD SCell with a same or different UL/DL configuration as the LTE TDD PCell, the LTE TDD SCell uses the same DL-reference UL/DL configuration as the PCell in the PDSCH ACK timeline defined for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e., Table 10.1.3A-1 in 36.213). |
| Nokia | Proposal 4: For LTE DL CA, the Sell uses the same DL-reference UL/DL configuration as the PCell - For the LTE TDD SCell with different UL/DL configuration as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213) - For the LTE TDD SCell with the same UL/DL configuration as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213) |
| Qualcomm | Proposal 5: For a UE configured with EN-DC and more than one DL LTE serving cell, and for a TDD secondary cell with different SIB1 UL/DL configuration compared to the TDD Pcell reference HARQ configuration, the downlink HARQ timing reference configuration is given by the pair of its own SIB1 DL/UL configuration and the reference DL HARQ configuration of the Pcell similar to the LTE TDD-TDD CA with different DL/UL configurations |

5 out of 7 companies proposed use PDSCH ACK timing of LTE TDD-FDD CA with TDD PCell for the case where LTE TDD SCell has different DL/UL configuration from the LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213).

Potential agreement: Confirm the FFS part in the agreement from RAN1 #96bis

For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following clarifications are agreed:

- For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell (already agreed in RAN1 #96bis)

  - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

  - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

# Reuse single UL mechanism for dual Tx UE to reduce DL de-sensing from harmonics

Main open issue: What are the needed specification changes for  applying existing single UL mechanism to reduce DL des-ensing issue from Harmonics.

Background: The WID for MR-DC and CA enhancements [16] was updated in RAN #84 to include additional objective of enabling R15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” to apply to dual uplink EN-DC.

The fundamental mechanism is the same as the handling of single Tx for EN-DC, where the network limits the LTE UL transmissions to only a subset of the available LTE UL subframes. Therefore, same or similar mechanism and signaling could potentially be applied for the harmonic issue as well, except that existing R15 specification limit the usage of the corresponding RRC signaling to a few scenarios. Specifically, the signaling defined in R15 includes the following:

- tdm-PatternConfig, as defined in 36.331: “UL/DL reference configuration indicating the time during which a UE configured with EN-DC is allowed to transmit. This field is used when power control or IMD issues require single UL transmission as specified in TS38.101-3 [85] and TS 38.213”

- tdm-Pattern, as defined in 38.306: “Indicates whether the UE supports the tdm-PatternConfig for single UL-transmission associated functionality, as specified in TS 36.331 [17]. Support is conditionally mandatory for UEs that do not support dynamic power sharing and for UEs that indicate single UL transmission for any BC, and optional otherwise”

- singleUL-Transmission, as defined in 38.306: “Indicates that the UE does not support simultaneous UL transmissions as defined in TS 38.101-3 [4]. The UE may only include this field for certain band combinations defined in TS 38.101-3 [4]. If included for a particular band combination, the field applies to all fallback band combinations of this band combination that are defined in TS 38.101-3 [4] as being allowed to include this field and does not apply to any other fallback band combinations defined in TS 38.101-3 [4]”

Below is a summary of companies’ views/proposals from the contributions to RAN1 #98:

| Company | proposal |
| --- | --- |
| Huawei | Observation 1: The functionality and configuration of Case 1 HARQ timing does not result in any restriction on NR uplink transmission for EN-DC UEs. Observation 2: Release 15 specifications do not preclude Case 1 HARQ timing from being applied to dual uplink EN-DC UEs. Proposal 11: RAN1 makes a conclusion that Release 15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” has been applied to dual uplink EN-DC. No RAN1 specification change is needed. Send an LS to inform RAN2 of the RAN1 conclusion. |
| ZTE | Proposal 1: In order to reduce the implementation impact of the ongoing Rel-15 SUO for EN-DC with LTE FDD PCell, it’s desirable to reuse the Rel-15 SUO mechanism for dual Tx UEs. Proposal 2: Both Case 1 and Case 2 HARQ timing are supported for dual Tx UEs to realize single uplink in Rel-16. Network configures either Case 1 or Case 2 HARQ timing for UE via RRC signaling. Proposal 3: Dual Tx UEs share the same UE capabilities (singleUL-Transmission and tdm-Pattern) with single Tx UEs. |
| Intel | Proposal 6: - Support the configuration of DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell to an EN-DC UE with dual uplink. Further study on two options, - Option 1: A dual uplink UE works as if a UE with single uplink. - Option 2: A dual uplink UE still works as dual uplink. |
| Samsung | Proposal 5: In EN-DC between LTE FDD PCell and NR TDD SCell, Case 1 HARQ timing and HARQ-offset can be configured for EN-DC UEs suffered from harmonic interferences. Proposal 6: In EN-DC between LTE FDD PCell and NR TDD SCell, Rel-15 spec. for UL HARQ timing to PUSCH and PUCCH formats is applied for EN-DC UEs suffered from harmonic interferences. |
| Nokia | Observation: - For a UE without dynamic power sharing capability, if UE is configured with tdm-pattern, dual uplink transmission was already supported in the subframe designated as LTE UL . - For a UE with dynamic power sharing capability, supporting tdm-pattern is an optional requirement. Proposal 6: The tdm-pattern is supported for a UE with dynamic power sharing capability. |
| Vodafone | Observation 1: The definitions of the UE Capability Parameters tdm-Pattern and tdm-PatternConfig imply that a UE would not enable DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell, when a UE is performing dual UL transmissions. Proposal 1: RAN1 should select Option B to avoid the need for bringing a CR to the existing RAN 2 specifications, 38.306 and 36.331. RAN1 should amend 38.213 to replace any reference to UE behaviours when configured with “tdm-Pattern” and “tdm-Pattern Config” with “tdm-Pattern or tdm-Pattern2” and “tdm-PatternConfig or tdm-PatternConfig2” respectively. Proposal 2: RAN1 should send a Liaison Statement to RAN2 to inform them of the decision and ask them to introduce the corresponding RRC signalling parameters to meet the WID Objective |
| Apple | Proposal 6: For purpose of configuring a DL-reference config to the LTE PCell to mitigate DL de-sensing, RAN1 to decide among the following two options: - Option 1: reuse existing “tdm-PatternConfig” defined in 36.331, but extend the description to also include the usage scenario of DL de-sensing from harmonics. - Option 2: introduce a new signal “tdm-PatternConfig2” for the usage scenario fo DL de-sensing from harmonics. Proposal 7: Modify the description of “tdm-Pattern” in 38.306 to include the usage scenario of DL de-sensing from harmonics. Proposal 8: Introduce a new per-BC capability bit for UE to indicate that DL de-sensing from Harmonic need to be handled by configuring DL-reference config to the LTE PCell and limit LTE UL transmissions to a subset of the subframes. |
| Ericsson | Observation - Case 1 HARQ timing procedures for LTE FDD Pcell in TS 36.213 (i.e., procedures related to the parameter subframeAssignment-r15) specify the behavior discussed in the newly added objective related to dual uplink EN-DC (i.e., “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell”). No changes to RAN1 specs are needed to enable these procedures for dual uplink EN-DC case |

Potential agreement: Rel-15 single Tx switched UL operation can be applied to EN-DC UE capable of dual UL Tx to mitigate DL de-sensing due to Harmonics:

- Support new RRC signaling “tdm-PatternConfig2” to configure dual Tx UE a DL-reference config to the MCG PCell

  - UL transmission on the SCG is independent of the DL-ref config “tdm-PatternConfig2”

- Reuse R15 UE capability signaling “tdm-Pattern”, with modification on description in 38.306 to include the usage scenario of DL de-sensing from harmonics.

- FFS: Introduce a new per-BC capability bit for UE to indicate that DL de-sensing from Harmonic need to be handled by configuring DL-reference config to the LTE PCell and limit LTE UL transmissions to a subset of the subframes

- Send LS to RAN2

# PUCCH formats

Main open issue: should we limit the supportable LTE PUCCH formats on the LTE TDD PCell? Is fallback allowed when DAI=1?

Background: an issue related to single Tx operation with case 1 DL HARQ timing is the potential PUCCH resource collision between legacy UE and single Tx UE, if fallback operation is triggered, i.e., DAI=1. When fallback to PUCCH format 1a/1b in LTE, the resource is implicitly mapped from CCE. As mapping for the legacy UE and single Tx UE follow different TDD configurations (i.e. one corresponds to “subframeAssignment” in SIB1, and the other corresponds to “tdm-PatternConfig-r15”), two PDSCH from different subframes may possibly map to the same PUCCH resource.

To avoid potential PUCCH resource collision and simplify implementation, we may consider to allow only PUCCH format 3/4/5 without supporting PUCCH fallback for UE configured with SUO case 1.

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 9: For single UL operation in EN-DC with LTE TDD PCell and UE is configured with DL-reference UL/DL configuration, only PUCCH format 3/4/5 are supported. When UE only receives DAI=1, PUCCH fallback operation is not supported and the default PUCCH format and resource can be used. |
| ZTE | Observation 1: - If only SPS transmission is received during the bundle window, PUCCH resource reserved for SPS is used to transmit HARQ-ACK; - If SPS transmission and DAI=1 are received during the bundle window, PUCCH resource reserved for DAI=1 is used to transmit HARQ-ACK. Proposal 5: For SUO in EN-DC with LTE TDD PCell and UE is configured with DL-reference UL/DL configuration, consider the following two solutions to determine PUCCH format and resource when the UE only receives DAI=1, - Configure a default ARI value, e.g., ARI=00. If UE only receives DAI=1, UE uses the default ARI value to determine the PUCCH format and resource. - Reinterpret the TPC field. When DAI=1, the TPC field is still used as the ARI. If UE only receives DAI=1, UE uses the TPC field to determine the PUCCH format and resource. |
| LGE | Proposal #3: Allocate an explicit PUCCH format 1a/1b resource for the fallback HARQ-ACK transmission corresponding to the PDCCH with DAI = 1 |
| Intel | Proposal 5: Using the PUCCH format3/4/5 resource with lowest index when UE receives one PDSCH with DAI=1 and/or SPS PDSCH on PCell only |
| Samsung | Proposal 4: For EN-DC between LTE TDD PCell and NR SCell, if the UE receives DAI=1 or only SPS related PDCCH/PDSCH, the UE uses one PUCCH resource value of four PUCCH resource values which are already RRC configured for the use of PUCCH format 3/4/5. |
| Nokia | Proposal 3: PUCCH fallback operation is not supported, DAI=1 only is always in the last subframe of DL bundle window, and specific PUCCH format 3/4/5 resource is used |
| Qualcomm | Proposal 6: For EN-DC with TDD-Pcell, only PUCCH formats 3/4/5 without the fallback operation are supported Proposal 7: Support an optional RRC parameter which configures a PUCCH resource when there is only one HARQ-ACK for DAI=1 or for SPS PDSCH to be transmitted; if the parameter is absent, when there is only one HARQ-ACK for DAI=1 or for SPS PDSCH, the UE transmits the HARQ-ACK on the PUCCH resource associated with ARI=0 |
| Apple | Proposal 4: For single UL operation in EN-DC with LTE TDD PCell, where UE is configured with DL-reference UL/DL configuration (i.e. SUO with case 1 HARQ timing), only LTE PUCCH formats 3/4/5 are supported - when DAI=1, using the PUCCH resource with the lowest index |

Based on the contributions, 7 out of 8 companies proposed to support only PUCCH formats 3/4/5 when DAI=1, and PUCCH fallback operation is not supported.

Potential agreement: For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration,

- Support only PUCCH formats 3/4/5 when DAI=1, i.e., PUCCH fallback operation is not supported.

  - FFS PUCCH resource determination when the UE only receives DAI=1

# Supportable LTE TDD Patterns with “SUO case1” in EN-DC with TDD PCell

Main open issue: whether to support LTE TDD Pattern 0 or 6 on LTE PCell together with SUO case1 in EN-DC with LTE TDD PCell?

According to the latest agreement from RAN1 #96, the LTE PCell’s UL HARQ timing (i.e. gap between PUSCH to PUSCH retransmission) is the same as without the reference configuration for PCell’s UL/DL configuration other than TDD pattern 0/6, while whether/how to support TDD pattern 0/6 is FFS.

However, for LTE TDD pattern 0 and 6, the PUSCH HARQ interval varies (e.g. 10, 11, 13, 14 ms) depending on the subframe number, as shown by Table 8-2 and Table 8.3-1 in TS 36.213. This makes it difficult to reuse the original PUSCH HARQ timing once a subset of UL subframes is configured using the DL-reference config. Meanwhile, there seems to be no existing commercial deployment using these two configurations.

Below is summary of companies’ positions/proposals:

| Company | Proposal |
| --- | --- |
| ZTE | Proposal 6: TDD configuration 0 and TDD configuration 6 are NOT supported for SUO in EN-DC with LTE TDD PCell. |
| LGE | Proposal #1: Apply UL HARQ RTT of 10 msec for Pcell UL/DL configuration 0/6 and determine UL HARQ timing based on UL-reference UL/DL configuration 1 |
| Intel | Proposal 1: TDD UL/DL configuration 0 and 6 are not supported for single UL operation enhancement for EN-DC in case of TDD PCell |
| Nokia | Proposal 1: Single uplink operation is not supported for TDD UL/DL configuration 0/6 as LTE Pcell in EN-DC |
| Apple | Proposal 2: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC with LTE TDD PCell, the case where LTE PCell’s TDD pattern (as configured in SIB1) being 0 or 6 is not supported in R16 |
| Qualcomm | Proposal 8: Support asynchronous UL HARQ for the case where TDD-PCell UL-DL configuration #0 or #6 for single tx switched UL in EN-DC with TDD-PCell as an optional feature |

Based on the contributions to RAN1 #98, 4 out of 6 companies proposed that TDD pattern 0 & 6 are not supported for SUO in EN-DC with LTE TDD PCell. 1 company proposed to consider applying extra UL-reference config to TDD pattern 0 & 6 to change the UL PUSCH HARQ RTT to 10ms. 1 company proposed to support async UL HARQ for pattern 0 & 6 as an optional feature.

Potential agreement: When UE is configured with DL-reference config on the LTE PCell to support single UL in EN-DC with LTE TDD PCell, the LTE PCell’s TDD pattern (as configured in SIB1) should not include TDD pattern 0 or 6 for R16.

Note: this does not preclude the consideration of TDD pattern 0 or 6 if commercial need arises in future.

# UE handling of “collision” between UL transmissions over MCG and SCG

Main open issue: for type 1 UE, if there is a collision in UL subframes other than those designated as UL in the DL-reference configuration, what is the UE behavior? Should it be specified or up to UE implementation?

Background:

- It was stated in WID [14] that “UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG”

- It was agreed in RAN1 #96 that “for type 1 UE, If there is a collision in the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH”

Below is summary of companies’ positions/proposals:

| Company | Proposal |
| --- | --- |
| Huawei | Proposal 3: For EN-DC UE configured with LTE TDD PCell, if there is a collision between LTE PUSCH and NR PUCCH/PRACH/PUSCH/SRS in UL subframes other than the subframes designated as UL by DL-reference TDD configuration, UE drops LTE PUSCH |
| ZTE | Observation 2: For SUO with EN-DC, always dropping NR PUSCH in case of collision will cause undesirable performance loss in NR side Proposal 7: For SUO with EN-DC in case of collision, if the NR PUSCH grant arrives later than the LTE PUSCH grant, a UE is expected to drop NR PUSCH; otherwise, the UE is expected to drop LTE PUSCH. Proposal 8: A UE is expected to drop LTE PUSCH if this LTE PUSCH collides with NR PUCCH resources configured by RRC even if there is no actual transmission in this PUCCH resources. |
| LGE | Proposal #2: Consider how to handle PUSCH retransmission purely depending on PHICH without detecting UL grant PDCCH in LTE side for handling of PUSCH collision between LTE and NR |
| Intel | Proposal 2: In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUCCH if collision occurs. Proposal 3: In the remaining UL subframes other than those configured by the DL-reference configuration, LTE PUSCH transmission is dropped if it collides with NR UL transmissions |
| Samsung | Proposal 1: NR PUSCH can be dropped if there is a collision between LTE PUSCH and NR PUSCH |
| Nokia | Proposal 2: If there is collision in the subframe which is not designated as UL in the DL-reference configuration, UE is expected to drop NR UL transmission |
| Qualcomm | Proposal 1: For single tx switched UL in EN-DC with TDD Pcell, the type 1 UE should consider LTE and NR uplink collisions an error case. Proposal 2: For single tx switched UL in EN-DC with TDD Pcell: - For type 2 UE (i.e., UE without dynamic power sharing capability), not only LTE PUSCH, but also any LTE UL transmissions should take place only in UL subframes designated as UL in the DL-reference UL-DL configuration. - For type 1 UE (i.e., UE with dynamic power sharing capability) not only LTE PUSCH but also other LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated as UL in the DL-reference UL-DL configuration. - For type 1 UE (i.e., UE with dynamic power sharing capability): - Semi-statically configured LTE UL transmissions are not transmitted in UL subframes not designated as UL in the DL-reference UL-DL configuration. In these UL subframes, the collision between LTE and NR transmissions is treated as an error case. |
| Ericsson | Proposal 2: For Case 1 HARQ timing via UE-specific reference TDD configuration for TDD Pcell, for subframes not indicated as ‘UL’ by the reference TDD configuration, the following default UE behavior is supported - if there are colliding LTE and NR UL transmissions in the same subframe, the UE is expected to transmit LTE and drop NR |
| Apple | Proposal 3: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, but scheduled with overlapping transmissions on MCG and SCG, the UE behavior is not specified at least for type 2 UE. RAN1 to further decide whether LTE should always be prioritized for type 1 UE in this case |

Based on companies’ proposals in the contribution and offline discussions, 4 out of 8 propose to drop NR transmission and transmit LTE when there is overlapping transmission in the subframe in the case of single-Tx  switched UL in EN-DC.

Potential agreement: for type 1 UE configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, if there is overlapping transmission in the subframe which is not designated as UL in the DL-reference configuration, UE is expected to drop NR UL transmission.

# Impact from UL switching time

Main open issue: does the spec need to put any explicit constraint on eNB/gNB UL scheduling, so as to minimize the impact from UL switching time? Or purely leave it for network implementation when such optimization is desired?

Background: The switching time was discussed in RAN4, and final agreement was captured in [12]. For intra-band EN-DC with single PA architecture, the maximal switching time can be up to 120us, which is close to 2 OFDM symbols in LTE. For certain TDD patterns with limited UL opportunities, such switching time may lead to non-negligible overhead.

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 2: For type 1 UE incapable of dual PA, LTE PUSCH can only be scheduled in the UL subframes designated as UL in the DL-reference configuration. |
| ZTE | Proposal 9: For type 1 UE incapable of dual PA, LTE PUSCH is only allowed in the UL subframes designated as UL in the DL-reference configuration. |

Potential agreement: For type 1 UE incapable of dual PA, LTE PUSCH can only be scheduled in the UL subframes designated as UL in the DL-reference configuration

# Support of HARQ Offset

Main open issue: whether a similar “HARQ-offset” as introduced in SUO case 1 for EN-DC with LTE FDD PCell should also be introduced for SUO case 1 for EN-DC with LTE TDD PCell?

Background: the following agreement was made in RAN1 #90bis for EN-DC with LTE FDD PCell:

Agreements: - In Case 1, LTE TDD UL HARQ timing is supported and the UE is allowed to transmit only in the subframes designated as UL in the reference TDD configuration. Additionally, a UE-specific HARQ subframe offset can be configured. - The offset (HARQ_offset) is in the range [0…9] - The offset doesn’t change any subframe or slot number - If, before applying the offset, the HARQ feedback for DL SF number m was mapped to UL SF number n, then after the offset, the HARQ feedback for DL SF number (m + HARQ_offset) is mapped to UL SF number (n + HARQ_offset) - If, before applying the offset, the PUSCH in SF number m was scheduled (by UL grant or PHICH) in SF number n, then after the offset, the PUSCH in SF number (m + HARQ_offset) is scheduled (by UL grant or PHICH) in SF number (n + HARQ_offset) - If, before applying the offset, the HARQ-ACK (if carried on PHICH) corresponding to PUSCH in SF number m was transmitted in SF number n, then after the offset, the HARQ-ACK (if carried on PHICH) corresponding to PUSCH in SF number (m + HARQ_offset) is transmitted in SF number (n + HARQ_offset) - SRS and PRACH transmission may only occur in subframes indicated as UL in the reference TDD configuration but shifted by the configured non-zero HARQ_offset

Below is summary of companies’ positions/proposals from contributions:

| Company | Proposals |
| --- | --- |
| Huawei | Proposal 6: For EN-DC with LTE TDD PCell, do not support HARQ-offset for type 1 UE configured with DL-reference TDD configuration. Proposal 7: If HARQ-offset is supported by type 2 EN-DC UE configured with LTE TDD PCell when the LTE PCell is configured with DL-reference TDD configuration, specification should guarantee to avoid possible error cases regarding configuration of HARQ-offset. |
| ZTE | Proposal 11: Do NOT support HARQ-offset for SUO for EN-DC with LTE TDD PCell in Rel-16. |
| LGE | Proposal #5: Support HARQ-offset in SUO case with LTE TDD Pcell in order for balancing UE load with PUCCH transmission among UL subframes |
| Intel | Proposal 4: - Support HARQ-offset for EN-DC with TDD PCell. |
| Samsung | Proposal 2: HARQ-offset indices corresponding to LTE UL subframes designated as UL in the DL-reference UL-DL configuration can be considered. |
| Qualcomm | Proposal 4: The HARQ offset should be configured for the users in case of EN-DC with TDD Pcell. The exact values and their applicability to different TDD DL/UL configurations should further be discussed |

FL suggestion:FFS whether support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell

# Configuration of RACH resource

Main open issue: should network assign RACH resource differently for dual Tx UE and UE operating in SUO case 1? Should type 1 and type 2 UE be treated differently?

Background: it was agreed in RAN1 #90bis that PRACH resource configuration and transmission should satisfy the following:

Agreement: - For Case 1, FDD timing configuration (i.e., periodicity and offset) is applied to LTE PRACH and SRS on LTE UL carriers. - The UE is not required to support transmission of LTE PRACH or SRS transmission which does not coincide with the configured HARQ-ACK transmission occasions

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 5: For UE configured with EN-DC and the LTE PCell is configured with DL-reference TDD configuration, the UE is allowed to transmit LTE PRACH in all uplink subframes regardless the configured DL-reference TDD configuration |
| ZTE | Proposal 10: For Rel-16 SUO for EN-DC with LTE TDD PCell, LTE PRACH and SRS transmission are only allowed to transmit in the subframes designated as UL in DL-reference configuration. |
| Ericsson | Proposal 1 - Following enhancement for PRACH should be supported - If a UE is configured with Case1 HARQ timing, UE’s PRACH transmissions are not limited to the UL subframes given by subframe-Assignment-r15 (i.e., for FDD, UE can transmit PRACH in any UL subframe). - If the UE’s LTE PRACH transmission collides with an NR UL transmission, PRACH transmission is prioritized. |
| Apple | Proposal 5: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config. - For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized - For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions |

Potential agreement: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

# Configuration of SRS resource

Main open issue: should network config SRS resource differently for dual Tx UE and UE operating in SUO case 1? Should type 1 and type 2 UE be treated differently?

Background: it was agreed in RAN1 #90bis that SRS resource configuration and transmission in R15 should satisfy the following:

Agreement: - For Case 1, FDD timing configuration (i.e., periodicity and offset) is applied to LTE PRACH and SRS on LTE UL carriers. - The UE is not required to support transmission of LTE PRACH or SRS transmission which does not coincide with the configured HARQ-ACK transmission occasions

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Huawei | Proposal 1: For single UL operation in EN-DC with TDD PCell and the PCell is configured with DL-reference TDD configuration, UE is allowed to transmit LTE SRS in subframes that designated as UL and special by DL-reference TDD configuration Proposal 4: For EN-DC UE configured with LTE TDD PCell and if the UE is capable of dual PA, LTE SRS and NR SRS transmissions are not restricted by DL-reference TDD configuration, i.e. the UE can transmit LTE SRS in UL or special subframes that designated as DL by DL-reference TDD configuration, and it can transmit NR SRS in subframes that designated as UL or special. Proposal 10: Allow EN-DC type 2 UE which is incapable of dynamic power sharing to transmit NR SRS overlapping in time with the last symbol of a LTE uplink subframe designated by subframeAssignment-r15 - The last LTE symbol thereof is reserved and not transmitted. |
| ZTE | Proposal 10: For Rel-16 SUO for EN-DC with LTE TDD PCell, LTE PRACH and SRS transmission are only allowed to transmit in the subframes designated as UL in DL-reference configuration. |
| Qualcomm | Proposal 2: For single tx switched UL in EN-DC with TDD Pcell: - For type 2 UE (i.e., UE without dynamic power sharing capability), not only LTE PUSCH, but also any LTE UL transmissions should take place only in UL subframes designated as UL in the DL-reference UL-DL configuration. - For type 1 UE (i.e., UE with dynamic power sharing capability) not only LTE PUSCH but also other LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated as UL in the DL-reference UL-DL configuration. - For type 1 UE (i.e., UE with dynamic power sharing capability): - Semi-statically configured LTE UL transmissions are not transmitted in UL subframes not designated as UL in the DL-reference UL-DL configuration. In these UL subframes, the collision between LTE and NR transmissions is treated as an error case. |

Potential agreement: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s LTE SRS resource configurations are not limited to the UL or special subframes designated as UL by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE SRS transmission collides with an NR UL transmission, LTE SRS transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE SRS transmission which are designated as DL by DL-reference configuration.

Potential agreement: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s NR SRS resource configurations are not limited to the UL or special subframes designated as DL the by DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s NR SRS transmission collides with an LTE UL transmission, LTE transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of NR SRS transmission which are designated as UL or special subframe by DL-reference configuration.

- UE can be configured to transmit NR SRS overlapping in time with the last symbol of UL subframe designated UL by the DL-reference config.

# Handling of LTE UL CA in the MCG

Main open issue: should R16 support an enhancement to allow for UL CA in MCG  for an U in EN-DC, when the single UL is applied? If so, what will be the available UL subframes for the LTE SCells?

Background: it was agreed that LTE UL CA would not be supported for SUO in R15, as shown in the following agreements in RAN1 #92bis.

Agreement: In Rel-15, Case 1 HARQ timing is supported for the LTE serving cells only for the following cases - A single FDD cell - DL CA with FDD-FDD CA and FDD-TDD CA, where the LTE PCell is FDD and there is only one UL carrier - The UE is not configured for cross-carrier scheduling Agreement: For a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell, the following is supported for the LTE serving cells - For an FDD SCell, the same DL HARQ timing as the PCell is applied for the FDD SCell, - For a TDD SCell, the same DL HARQ timing as the PCell is applied restricted to the set of DL/special subframes on the TDD SCell - The UL HARQ timing for the FDD PCell is the same for the CA and non-CA cases

For the cases where LTE UL CA is feasible or can be supported, there would be the following two scenarios:

- The LTE SCell’s UL is in the same band as the LTE PCell (i.e. intra-band LTE UL CA). In this case, the LTE SCell’s UL is also part of the same “difficult band combination”, and therefore it is desirable that the UL subframes are aligned between the LTE PCell and LTE SCell, and they also follow the same UL HARQ timing.

- The LTE SCell’s UL is in a different band as the LTE PCell (i.e. inter-band LTE UL CA)

  - If the LTE SCell is not in a difficult band combination with NR, then obviously all UL subframes in the LTE SCell can be used

  - If the LTE SCell is also in a difficult band combination with NR, then the handling could be more complicated and potentially require different handling depending on whether LTE SCell and NR PSCell are in TDD or FDD band, respectively. Therefore, it is desirable to avoid such band combination in real deployment, unless there is clear market need, where the handling should be evaluated case by case.

Below is summary of companies’ positions/proposals from contributions:

| Company | proposal |
| --- | --- |
| Nokia | Proposal 5: If Case 1 HARQ timing is configured, the UL subframe usage in a UL CA carrier is determined by whether its CA band is problematic band in EN-DC band combination. - For intra-band UL CA and inter-band CA with problematic CA band, the uplink subframes in Scell are aligned with the uplink subframe allocation in PCell - For inter-band CA with non-problematic CA band, all UL subframe in the carrier of that band can be scheduled |

Potential agreement:

# Appendix: Summary of proposals

[1] R1-1908077, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon, 3GPP RAN1#98

Proposal 1: For single UL operation in EN-DC with TDD PCell and the PCell is configured with DL-reference TDD configuration, UE is allowed to transmit LTE SRS in subframes that designated as UL and special by DL-reference TDD configuration.

Proposal 2: For type 1 UE incapable of dual PA, LTE PUSCH can only be scheduled in the UL subframes designated as UL in the DL-reference configuration.

Proposal 3: For EN-DC UE configured with LTE TDD PCell, if there is a collision between LTE PUSCH and NR PUCCH/PRACH/PUSCH/SRS in UL subframes other than the subframes designated as UL by DL-reference TDD configuration, UE drops LTE PUSCH.

Proposal 4: For EN-DC UE configured with LTE TDD PCell and if the UE is capable of dual PA, LTE SRS and NR SRS transmissions are not restricted by DL-reference TDD configuration, i.e. the UE can transmit LTE SRS in UL or special subframes that designated as DL by DL-reference TDD configuration, and it can transmit NR SRS in subframes that designated as UL or special.

Proposal 5: For UE configured with EN-DC and the LTE PCell is configured with DL-reference TDD configuration, the UE is allowed to transmit LTE PRACH in all uplink subframes regardless the configured DL-reference TDD configuration.

Proposal 6: For EN-DC with LTE TDD PCell, do not support HARQ-offset for type 1 UE configured with DL-reference TDD configuration.

Proposal 7: If HARQ-offset is supported by type 2 EN-DC UE configured with LTE TDD PCell when the LTE PCell is configured with DL-reference TDD configuration, specification should guarantee to avoid possible error cases regarding configuration of HARQ-offset.

Proposal 8: For LTE DL CA, the DL HARQ timing of TDD SCell with TDD PCell:

- For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

- For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

Proposal 9: For single UL operation in EN-DC with LTE TDD PCell and UE is configured with DL-reference UL/DL configuration, only PUCCH format 3/4/5 are supported. When UE only receives DAI=1, PUCCH fallback operation is not supported and the default PUCCH format and resource can be used.

Proposal 10: Allow EN-DC type 2 UE which is incapable of dynamic power sharing to transmit NR SRS overlapping in time with the last symbol of a LTE uplink subframe designated by subframeAssignment-r15

- The last LTE symbol thereof is reserved and not transmitted.

Observation 1: The functionality and configuration of Case 1 HARQ timing does not result in any restriction on NR uplink transmission for EN-DC UEs.

Observation 2: Release 15 specifications do not preclude Case 1 HARQ timing from being applied to dual uplink EN-DC UEs.

Proposal 11: RAN1 makes a conclusion that Release 15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” has been applied to dual uplink EN-DC. No RAN1 specification change is needed. Send an LS to inform RAN2 of the RAN1 conclusion.

[2] R1-1908243, “Discussion on single Tx switched uplink solution for EN-DC”, ZTE, 3GPP RAN1#98

Observation 1:

- If only SPS transmission is received during the bundle window, PUCCH resource reserved for SPS is used to transmit HARQ-ACK;

- If SPS transmission and DAI=1 are received during the bundle window, PUCCH resource reserved for DAI=1 is used to transmit HARQ-ACK.

Observation 2: For SUO with EN-DC, always dropping NR PUSCH in case of collision will cause undesirable performance loss in NR side.

Proposal 1: In order to reduce the implementation impact of the ongoing Rel-15 SUO for EN-DC with LTE FDD PCell, it’s desirable to reuse the Rel-15 SUO mechanism for dual Tx UEs.

Proposal 2: Both Case 1 and Case 2 HARQ timing are supported for dual Tx UEs to realize single uplink in Rel-16. Network configures either Case 1 or Case 2 HARQ timing for UE via RRC signaling.

Proposal 3: Dual Tx UEs share the same UE capabilities (singleUL-Transmission and tdm-Pattern) with single Tx UEs.

Proposal 4: Confirm the following FFS

- For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

- For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

Proposal 5: For SUO in EN-DC with LTE TDD PCell and UE is configured with DL-reference UL/DL configuration, consider the following two solutions to determine PUCCH format and resource when the UE only receives DAI=1,

- Configure a default ARI value, e.g., ARI=00. If UE only receives DAI=1, UE uses the default ARI value to determine the PUCCH format and resource.

- Reinterpret the TPC field. When DAI=1, the TPC field is still used as the ARI. If UE only receives DAI=1, UE uses the TPC field to determine the PUCCH format and resource.

Proposal 6: TDD configuration 0 and TDD configuration 6 are NOT supported for SUO in EN-DC with LTE TDD PCell.

Proposal 7: For SUO with EN-DC in case of collision, if the NR PUSCH grant arrives later than the LTE PUSCH grant, a UE is expected to drop NR PUSCH; otherwise, the UE is expected to drop LTE PUSCH.

Proposal 8: A UE is expected to drop LTE PUSCH if this LTE PUSCH collides with NR PUCCH resources configured by RRC even if there is no actual transmission in this PUCCH resources.

Proposal 9: For type 1 UE incapable of dual PA, LTE PUSCH is only allowed in the UL subframes designated as UL in the DL-reference configuration.

Proposal 10: For Rel-16 SUO for EN-DC with LTE TDD PCell, LTE PRACH and SRS transmission are only allowed to transmit in the subframes designated as UL in DL-reference configuration.

Proposal 11: Do NOT support HARQ-offset for SUO for EN-DC with LTE TDD PCell in Rel-16.

[3] R1-1908552, “HARQ related issues on single Tx-switched UL operation for EN-DC”, LG Electronics, 3GPP RAN1#98

Proposal #1: Apply UL HARQ RTT of 10 msec for Pcell UL/DL configuration 0/6 and determine UL HARQ timing based on UL-reference UL/DL configuration 1.

Proposal #2: Consider how to handle PUSCH retransmission purely depending on PHICH without detecting UL grant PDCCH in LTE side for handling of PUSCH collision between LTE and NR.

Proposal #3: Allocate an explicit PUCCH format 1a/1b resource for the fallback HARQ-ACK transmission corresponding to the PDCCH with DAI = 1.

Proposal #4: Determine HARQ-ACK timing for LTE TDD Scell as for the case of Pcell with UL/DL configuration 2/4/5 in TDD CA with different UL/DL configuration.

Proposal #5: Support HARQ-offset in SUO case with LTE TDD Pcell in order for balancing UE load with PUCCH transmission among UL subframes

[4] R1-1908665, “Further discussions on single uplink operation enhancement for EN-DC”, Intel, 3GPP RAN1#98

Proposal 1:

- TDD UL/DL configuration 0 and 6 are not supported for single UL operation enhancement for EN-DC in case of TDD PCell.

Proposal 2:

- In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUCCH if collision occurs.

Proposal 3:

- In the remaining UL subframes other than those configured by the DL-reference configuration, LTE PUSCH transmission is dropped if it collides with NR UL transmissions.

Proposal 4:

- Support HARQ-offset for EN-DC with TDD PCell.

Proposal 5:

- Using the PUCCH format3/4/5 resource with lowest index when UE receives one PDSCH with DAI=1 and/or SPS PDSCH on PCell only.

Proposal 6:

- Support the configuration of DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell to an EN-DC UE with dual uplink. Further study on two options,

  - Option 1: A dual uplink UE works as if a UE with single uplink.

  - Option 2: A dual uplink UE still works as dual uplink.

[5] R1-1908515, “Enhancements to single Tx switched UL for EN-DC”, Samsung, 3GPP RAN1#98

Proposal 1: NR PUSCH can be dropped if there is a collision between LTE PUSCH and NR PUSCH.

Proposal 2: HARQ-offset indices corresponding to LTE UL subframes designated as UL in the DL-reference UL-DL configuration can be considered.

Proposal 3: For a LTE TDD SCell with a same or different UL/DL configuration as the LTE TDD PCell, the LTE TDD SCell uses the same DL-reference UL/DL configuration as the PCell in the PDSCH ACK timeline defined for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e., Table 10.1.3A-1 in 36.213).

Proposal 4: For EN-DC between LTE TDD PCell and NR SCell, if the UE receives DAI=1 or only SPS related PDCCH/PDSCH, the UE uses one PUCCH resource value of four PUCCH resource values which are already RRC configured for the use of PUCCH format 3/4/5.

Proposal 5: In EN-DC between LTE FDD PCell and NR TDD SCell, Case 1 HARQ timing and HARQ-offset can be configured for EN-DC UEs suffered from harmonic interferences.

Proposal 6: In EN-DC between LTE FDD PCell and NR TDD SCell, Rel-15 spec. for UL HARQ timing to PUSCH and PUCCH formats is applied for EN-DC UEs suffered from harmonic interferences.

[6] R1-1908923, “Discussion on single uplink operation enhancement for EN-DC”, Nokia, Nokia Shanghai Bell, 3GPP RAN1#98

Proposal 1: Single uplink operation is not supported for TDD UL/DL configuration 0/6 as LTE Pcell in EN-DC.

Proposal 2: If there is collision in the subframe which is not designated as UL in the DL-reference configuration, UE is expected to drop NR UL transmission.

Proposal 3: PUCCH fallback operation is not supported, DAI=1 only is always in the last subframe of DL bundle window, and specific PUCCH format 3/4/5 resource is used.

Proposal 4: For LTE DL CA, the Sell uses the same DL-reference UL/DL configuration as the PCell

- For the LTE TDD SCell with different UL/DL configuration as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

- For the LTE TDD SCell with the same UL/DL configuration as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

Proposal 5: If Case 1 HARQ timing is configured, the UL subframe usage in a UL CA carrier is determined by whether its CA band is problematic band in EN-DC band combination.

- For intra-band UL CA and inter-band CA with problematic CA band, the uplink subframes in Scell are aligned with the uplink subframe allocation in PCell

- For inter-band CA with non-problematic CA band, all UL subframe in the carrier of that band can be scheduled

Proposal 6: The tdm-pattern is supported for a UE with dynamic power sharing capability.

[7] R1-1909005, “TDD Timing on FDD Bands for Dual Uplink EN-DC”, Vodafone, 3GPP RAN1#98

Observation 1: The definitions of the UE Capability Parameters tdm-Pattern and tdm-PatternConfig imply that a UE would not enable DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell, when a UE is performing dual UL transmissions.

Proposal 1: RAN1 should select Option B to avoid the need for bringing a CR to the existing RAN 2 specifications, 38.306 and 36.331. RAN1 should amend 38.213 to replace any reference to UE behaviours when configured with “tdm-Pattern” and “tdm-Pattern Config” with “tdm-Pattern or tdm-Pattern2” and “tdm-PatternConfig or tdm-PatternConfig2” respectively.

Proposal 2: RAN1 should send a Liaison Statement to RAN2 to inform them of the decision and ask them to introduce the corresponding RRC signalling parameters to meet the WID Objective.

[8] R1-1909143, “Enhancements to single-tx operation for EN-DC”, Ericsson, 3GPP RAN1#98

Proposal 1

- Following enhancement for PRACH should be supported

  - If a UE is configured with Case1 HARQ timing, UE’s PRACH transmissions are not limited to the UL subframes given by subframe-Assignment-r15 (i.e., for FDD, UE can transmit PRACH in any UL subframe).

    - If the UE’s LTE PRACH transmission collides with an NR UL transmission, PRACH transmission is prioritized.

Proposal 2

- For Case 1 HARQ timing via UE-specific reference TDD configuration for TDD Pcell, for subframes not indicated as ‘UL’ by the reference TDD configuration, the following default UE behavior is supported

  - if there are colliding LTE and NR UL transmissions in the same subframe, the UE is expected to transmit LTE and drop NR.

Observation

- Case 1 HARQ timing procedures for LTE FDD Pcell in TS 36.213 (i.e., procedures related to the parameter subframeAssignment-r15) specify the behavior discussed in the newly added objective related to dual uplink EN-DC (i.e., “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell”).

No changes to RAN1 specs are needed to enable these procedures for dual uplink EN-DC case

[9] R1-1909063, “Remaining issues on enhancements to Single Tx Switched Uplink solution for EN-DC”, Apple Inc., 3GPP RAN1#98.

Proposal 1: For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell:

- For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

- For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

Proposal 2: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC with LTE TDD PCell, the case where LTE PCell’s TDD pattern (as configured in SIB1) being 0 or 6 is not supported in R16.

Proposal 3: When UE is configured with DL-reference config on the LTE PCell to support single Tx switched UL in EN-DC, but scheduled with overlapping transmissions on MCG and SCG, the UE behavior is not specified at least for type 2 UE.

- RAN1 to further decide whether LTE should always be prioritized for type 1 UE in this case.

Proposal 4: For single UL operation in EN-DC with LTE TDD PCell, where UE is configured with DL-reference UL/DL configuration (i.e. SUO with case 1 HARQ timing), only LTE PUCCH formats 3/4/5 are supported

- when DAI=1, using the PUCCH resource with the lowest index.

Proposal 5: If a UE is configured with Case1 HARQ timing in EN-DC, UE’s PRACH resource configurations are not limited to the UL subframes given by the DL-reference config.

- For type 1 UE (i.e. with fast communication between LTE and NR modems): if the UE’s LTE PRACH transmission collides with an NR UL transmission, LTE PRACH transmission is prioritized

- For type 2 UE (i.e. without fast communication between LTE and NR modems): the UE is not required to support transmission of LTE PRACH transmission which does not coincide with the configured HARQ-ACK transmission occasions

Proposal 6: For purpose of configuring a DL-reference config to the LTE PCell to mitigate DL de-sensing, RAN1 to decide among the following two options:

- Option 1: reuse existing “tdm-PatternConfig” defined in 36.331, but extend the description to also include the usage scenario of DL de-sensing from harmonics.

- Option 2: introduce a new signal “tdm-PatternConfig2” for the usage scenario fo DL de-sensing from harmonics.

Proposal 7: Modify the description of “tdm-Pattern” in 38.306 to include the usage scenario of DL de-sensing from harmonics.

Proposal 8: Introduce a new per-BC capability bit for UE to indicate that DL de-sensing from Harmonic need to be handled by configuring DL-reference config to the LTE PCell and limit LTE UL transmissions to a subset of the subframes.

[10] R1-1909286, “Enhancements for EN-DC Single-Tx TDM Operation”, Qualcomm Inc., 3GPP RAN1#98

Proposal 1: For single tx switched UL in EN-DC with TDD Pcell, the type 1 UE should consider LTE and NR uplink collisions an error case.

Proposal 2: For single tx switched UL in EN-DC with TDD Pcell:

- For type 2 UE (i.e., UE without dynamic power sharing capability), not only LTE PUSCH, but also any LTE UL transmissions should take place only in UL subframes designated as UL in the DL-reference UL-DL configuration.

- For type 1 UE (i.e., UE with dynamic power sharing capability) not only LTE PUSCH but also other LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated as UL in the DL-reference UL-DL configuration.

- For type 1 UE (i.e., UE with dynamic power sharing capability):

  - Semi-statically configured LTE UL transmissions are not transmitted in UL subframes not designated as UL in the DL-reference UL-DL configuration. In these UL subframes, the collision between LTE and NR transmissions is treated as an error case.

Proposal 3: In Rel.16, for single tx switched UL in EN-DC with FDD Pcell, the specification should enable type 1 UE to transmit LTE PUSCH in the UL subframes other than the offset-UL subframes determined by subframeAssignment-r15 and harq-Offset-r15.

Proposal 4: The HARQ offset should be configured for the users in case of EN-DC with TDD Pcell. The exact values and their applicability to different TDD DL/UL configurations should further be discussed.

Proposal 5: For a UE configured with EN-DC and more than one DL LTE serving cell, and for a TDD secondary cell with different SIB1 UL/DL configuration compared to the TDD Pcell reference HARQ configuration, the downlink HARQ timing reference configuration is given by the pair of its own SIB1 DL/UL configuration and the reference DL HARQ configuration of the Pcell similar to the LTE TDD-TDD CA with different DL/UL configurations.

Proposal 6: For EN-DC with TDD-Pcell, only PUCCH formats 3/4/5 without the fallback operation are supported.

Proposal 7: Support an optional RRC parameter which configures a PUCCH resource when there is only one HARQ-ACK for DAI=1 or for SPS PDSCH to be transmitted; if the parameter is absent, when there is only one HARQ-ACK for DAI=1 or for SPS PDSCH, the UE transmits the HARQ-ACK on the PUCCH resource associated with ARI=0.

Proposal 8:  Support asynchronous UL HARQ for the case where TDD-PCell UL-DL configuration #0 or #6 for single tx switched UL in EN-DC with TDD-PCell as an optional feature.

# References

- R1-1908077, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1908243, “Discussion on single Tx switched uplink solution for EN-DC”, ZTE, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1908552, “HARQ related issues on single Tx-switched UL operation for EN-DC”, LG Electronics, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1908665, “Further discussions on single uplink operation enhancement for EN-DC”, Intel, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1908515, “Enhancements to single Tx switched UL for EN-DC”, Samsung, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1908923, “Discussion on single uplink operation enhancement for EN-DC”, Nokia, Nokia Shanghai Bell, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1909005, “TDD Timing on FDD Bands for Dual Uplink EN-DC”, Vodafone, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1909143, “Enhancements to single-tx operation for EN-DC”, Ericsson, 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1909063, “Remaining issues on enhancements to Single Tx Switched Uplink solution for EN-DC”, Apple Inc., 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- R1-1909286, “Enhancements for EN-DC Single-Tx TDM Operation”, Qualcomm Inc., 3GPP RAN1#98, Prague, Czech Republic, August 26th – 30th, 2019.

- “Chairman Note, RAN4#86bis”, 3GPP RAN4 #86bis, Melbourne, Australia, USA 16 - 20 April, 2018.

- “Chairman Notes, RAN1 #96”, 3GPP RAN1 Ad-Hoc #96, Athens, Greece, February 25th – March 1st, 2019.

- R4-1902496, “Draft CR for 38.101-3: Switching time for intra-band EN-DC upon dual PA UE capability”, Huawei, HiSilicon, 3GPP RAN4#90, Athens, Greece, February 25th – March 1st, 2019.

- RP-180680, “Solutions for single Tx TDM”, RAN #80, La Jolla, CA, U.S.A., June 11st-14th, 2018.

- “Chairman Note, RAN#81”, 3GPP RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- PR-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #81, Gold Coast, Australia, 10th – 13th September, 2018.

- PR-191600, “Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core)”, RAN #84, Newport Beach, U.S.A., 3rd – 6th June, 2019.

- “Chairman Notes, RAN1 Ad-Hoc #1901”, 3GPP RAN1 Ad-Hoc #1901, Taipei, Taiwan, 21st – 25th January 2019.