3GPP TSG RAN WG1 Meeting #98    	            R1-1909522

Prague, Czech, August 26th – 30th, 2019

Source:	Qualcomm Incorporated

Title:	Summary of Rel.15 maintenance for Scheduling and HARQ

Agenda Item:	7.1.3

Document for: 	Discussion and Decision

# Background

This contribution summarizes following 9 contributions submitted to AI 7.1.3.

| TDoc | Title | Source | Type |
| --- | --- | --- | --- |
| R1-1908197 | Discussion on UL cancellation timeline | vivo | discussion |
| R1-1908456 | Draft CR on description of PDSCH-to-HARQ_feedback timing indicator for TS38.213 | Samsung | draftCR |
| R1-1908560 | Correction on HARQ-ACK transmission | CATT | draftCR |
| R1-1908563 | Correction on time gap definition | CATT | draftCR |
| R1-1908564 | Correction on timing for MAC CE applicability with HARQ-ACK repetition in 38.213 | CATT | draftCR |
| R1-1908565 | Correction on timing for MAC CE applicability with HARQ-ACK repetition in 38.214 | CATT | draftCR |
| R1-1909115 | Draft TS38.213 CR on UL cancelation timeline | vivo | draftCR |
| R1-1909237 | Clarifications on NR time ambiguity | Qualcomm Incorporated | discussion |
| R1-1909238 | Draft CR on MAC-CE activation timing | Qualcomm Incorporated | draftCR |

# R1-1908197, 9115, 8563 (UL cancel time)

These draft CRs propose following:

- There are three timeline specifications where a UE is not expected to cancel a configured UL transmission by a DCI scheduling DL reception. Two of them uses Tproc,2, while the one uses N2. To be consistent, it is suggested to change N2 to Tproc,2 for the last one.

- For the same issue, one part assumes Tproc,2 with d2,1=1. To be consistent, it is suggested to apply d2,1=1 assumption for all three cases.

- R1-1908563 proposes to clarify how  for SCS is determined for Tproc,2 in the first places.

R1-1909115:

### 11.1.1 UE procedure for determining slot format < Unchanged parts are omitted > If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the subset of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming - the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols. […] For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 1_0, DCI format 1_1, or DCI format 0_1 - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 - the UE receives PDCCH as described in Subclause 10.1 - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE does not receive the PDSCH or the CSI-RS in the set of symbols of the slot - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot, the UE - does not transmit the PUCCH, or the PUSCH, or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that is a number of symbols equal to the PUSCH preparation time for the corresponding PUSCH timing capability assuming after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 - does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is a number of symbols equal to the PUSCH preparation time for the corresponding PUSCH timing capability assuming after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0

R1-1908563:

## 11.1 Slot configuration […] For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the subset of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding UE processing capability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH carrying the DCI format 1_0, DCI format 1_1 or DCI format 0_1 and the SCS configuration of the SRS, PUCCH, PUSCH or PRACH. - the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols […] ### 11.1.1 UE procedure for determining slot format […] If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the subset of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH carrying the DCI format 1_0, DCI format 1_1 or DCI format 0_1 and the SCS configuration of the SRS, PUCCH, PUSCH or PRACH. - the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols. […] For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 1_0, DCI format 1_1, or DCI format 0_1 - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 - the UE receives PDCCH as described in Subclause 10.1 - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE does not receive the PDSCH or the CSI-RS in the set of symbols of the slot - if the UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in the set of symbols of the slot, the UE - does not transmit the PUCCH, or the PUSCH, or the PRACH in the slot and does not transmit the SRS in symbols from the set of symbols in the slot, if any, starting from a symbol that isafter PUSCH preparation time for the corresponding PUSCH timing capability [6, TS 38.214] assuming after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0 - does not expect to cancel the transmission of the SRS, or the PUCCH, or the PUSCH, or the PRACH in symbols from the set of symbols in the slot, if any, starting before a symbol that is after the PUSCH preparation time for the corresponding PUSCH timing capability [6, TS 38.214] assuming after a last symbol of a CORESET where the UE is configured to monitor PDCCH for DCI format 2_0

Since the definition of  in Tproc,2 in TS38.214 Section 6.4 is only for PUSCH, it is good idea to clarify  for each case as proposed by CATT. The value of d2,1 is missing in the two places of the current spec; need to clarify it. Correction to Section 10.2 should be a separate issue and hence the CR should focus on 11.1 and 11.1.1. Based on this understanding, updated draft CR (Draft-CR1) is made and is attached. Note that in Draft-CR1, unlike R1-1908563, for the first change in Section 11.1.1, “DCI format 2_0” is included for determination of SCS.

Proposal:

- Adopt the attached draft CR on this aspect (Draft-CR1).

# R1-1908560 (Corrections to processing time)

This draft CR proposes various aspects. Following discusses one-by-one.

- Clarification on Type-1 HARQ-ACK codebook on PUSCH (issue 1)

The first change must be already common understanding. The second change addresses the UE behavior when a DL DCI and the UL DCI are partially overlapped in time, which must be a corner case. Therefore, it is not fundamentally necessary to approve these changes.

A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release that the UE detects a DCI format scheduling the PDSCH reception or SPS PDSCH release in a PDCCH monitoring occasion that starts after the first symbol of a PDCCH monitoring occasion where the UE detects a DCI format 0_0 or a DCI format 0_1 scheduling the PUSCH transmission.

- Clarification on time gap between BWP switching and a PUSCH (issue 2 and issue 5)

Following part includes two aspects: (1) change N2 to Tproc,2 with clarifying , and (2) including BWP-switching on UL BWP on the PCell. Both changes are OK.

A UE does not expect to detect a DCI format switching a DL BWP or an UL BWP on Pcell within prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where is defined in [6, TS 38.214] and corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH carrying the DCI format switching a DL BWP or an UL BWP on Pcell, the SCS configuration of the PDCCH carrying the DCI scheduling the PUSCH and the SCS configuration of the PUSCH.

- Clarification on time gap between BWP switching and a PUCCH (issue 3)

Following part includes three aspects: (1) change N3 to absolute time duration with clarifying  and Tc, (2) including BWP-switching on UL BWP on the PCell, and (3) clarifying the condition where the Cap#2 processing time is applicable. Same as above, (1) and (2) are reasonable clarifications. However, (3) is contradict to the existing specification when the Cap#2 is applicable (see below yellow-highlighted part). Therefore, it is suggested not to approve this.

Note that the following TP misses “or an UL BWP on Pcell” for  determination.

A UE does not expect to detect a DCI format switching a DL BWP or an UL BWP on Pcell within prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where κ and Tc are defined in subclause 4.1 of [4, TS 38.211] and corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI format switching a DL BWP and the SCS configuration of the PUCCH, and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the DCI format switching a DL BWP or an UL BWP on Pcell, for , for , for ; otherwise , for , for , for , for .

### 9.2.3 UE procedure for reporting HARQ-ACK […] If a UE detects a first DCI format 1_0 or DCI format 1_1 indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format 1_0 or DCI format 1_1 indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in subclause 4.1 of [4, TS 38.211] and corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, for , for , for ; otherwise , for , for , for , for .

- Addition of time gap between BWP switching and a PUCCH/PUSCH for Type-2 HARQ-ACK codebook (issue 4)

Following two paragraphs are proposed to be added under Type-2 HARQ-ACK codebook. Originally, N3/N2 time gap prior to HARQ-ACK transmission that prohibit BWP switching was introduced since BWP switching impacts HARQ-ACK codebook for Type-1 HARQ-ACK codebook. RAN1 needs to discuss whether these changes are needed for Type-2 HARQ-ACK codebook.

On PUCCH:

A UE does not expect to detect a DCI format switching a DL BWP or an UL BWP on Pcell within prior to a first symbol of a PUCCH transmission where the UE multiplexes HARQ-ACK information, where κ and Tc are defined in subclause 4.1 of [4, TS 38.211] and corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI format switching a DL BWP and the SCS configuration of the PUCCH, and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the DCI format switching a DL BWP or an UL BWP on Pcell, for , for , for ; otherwise , for , for , for , for .

On PUSCH:

A UE does not expect to detect a DCI format switching a DL BWP or an UL BWP on Pcell within prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where is defined in [6, TS 38.214] and corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH carrying the DCI format switching a DL BWP or an UL BWP on Pcell, the SCS configuration of the PDCCH carrying the DCI scheduling the PUSCH and the SCS configuration of the PUSCH.

- Clarification on time gap N3 (issue 6)

The following change re-defines the condition where Cap#2 processing time is available. This broaden the condition of Cap#2 applicability and hence NBC change, which should not be approved.

If a UE detects a first DCI format 1_0 or DCI format 1_1 indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format 1_0 or DCI format 1_1 indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the end the PDCCH reception that includes the second DCI format is not earlier than from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in subclause 4.1 of [4, TS 38.211] and corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH providing the second DCI format and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format, for , for , for ; otherwise , for , for , for , for .

- Clarification on HARQ-ACK timing (issue 7)

The timing aspects should be discussed overall, using R1-1909237. Regarding “With reference to slots for PUCCH transmissions”, this is a good clarification and can be included in alignment CR.

With reference to slots for PUCCH transmissions, for a SPS PDSCH reception ending in slot assuming TTA=0, where TTA is defined in [4, TS 38.211], the UE transmits the PUCCH in slot where is provided by the PDSCH-to-HARQ-timing-indicator field in DCI format 1_0 or, if present, in DCI format 1_1 activating the SPS PDSCH reception. With reference to slots for PUCCH transmissions, if the UE detects a DCI format 1_1 that does not include a PDSCH-to-HARQ-timing-indicator field and schedules a PDSCH reception or activates a SPS PDSCH reception ending in slot assuming TTA=0, where TTA is defined in [4, TS 38.211], the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot where is provided by dl-DataToUL-ACK. With reference to slots for PUCCH transmissions, if the UE detects a DCI format 1_0 or a DCI format 1_1 scheduling a PDSCH reception ending in slot assuming TTA=0 or if the UE detects a DCI format 1_0 indicating a SPS PDSCH release through a PDCCH reception ending in slot assuming TTA=0, where TTA is defined in [4, TS 38.211], the UE provides corresponding HARQ-ACK information in a PUCCH transmission within slot , where is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK. corresponds to the last slot of the PUCCH transmission that overlaps with the PDSCH reception or with the PDCCH reception in case of SPS PDSCH release assuming TTA=0, where TTA is defined in [4, TS 38.211].

Proposal:

- Correction to issue 1 is not adopted.

- Correction to issue 2 and issue 5 is adopted (Draft CR-2).

- Correction to issue 3 is updated; condition of Cap#2 applicability is based on existing spec, and “or an UL BWP on Pcell” is added for  determination (Draft CR-2).

- Correction to issue 4 is to be discussed during the meeting.

- Correction to issue 6 is not adopted.

- Correction to issue 7 is not adopted; Addition of “With reference to slots for PUCCH transmissions” is included in the alignment CR.

# R1-1908563 (Processing time for DL SPS release)

The draft CR proposes to clarify the UE processing time for SPS PDSCH release. The main changes are following.

- N is chosen based on the smallest SCS configuration among the SCS configurations of the PDCCH for SPS PDSCH release and that of the PUCCH carrying the HARQ-ACK for the SPS PDSCH release.

- Clarify that the processing time according to UE capability #2 is applied only if it is configured to the serving cell.

## 10.2 PDCCH validation for DL SPS and UL grant Type 2 […] A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after symbols from the last symbol of a PDCCH providing the SPS PDSCH release. . If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release in FR1 for , for , and for , otherwise, for , for , for , and for , wherein corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

The first clarification is reasonable. On the other hand, same as the change in 8560, the second point is contradicting with the following part of Section 9.2.3; the processing timeline according to UE capability #2 should be applicable only if it is configured over all the serving cells.

### 9.2.3 UE procedure for reporting HARQ-ACK […] If a UE detects a first DCI format 1_0 or DCI format 1_1 indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format 1_0 or DCI format 1_1 indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in subclause 4.1 of [4, TS 38.211] and corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, for , for , for ; otherwise , for , for , for , for .

Therefore, here it is suggested to take the first clarification while not to take the second clarification. Together with the changes described in the previous section, text proposal is included in the “Draft-CR2”.

Proposal:

- Correction is updated (Draft-CR2).

# R1-1908564, 8565, 9238 (MAC CE based activation with PUCCH repetition)

The CRs propose to clarify that the MAC-CE based activation actions are 3ms after the last slot where the ACK for the MAC-CE PDSCH is transmitted on, which removes the ambiguity of when activation is effective in case of PUCCH repetition is configured for HARQ-ACK for the MAC-CE.

For TS38.213, a new draft CR (Draft-CR3) is created to address “the last” slot throughout the spec, based on R1-1908564 and R1-1909238. Note that R1-1909238 addresses another issue (HARQ-ACK for MAC-CE based activation on PUSCH), which is to be handled in Section 6.

For TS38.214, R1-1908565 addresses the issue throughout the spec. This should be approved.

Proposal:

- Approve the draft CR (Draft-CR3) for TS38.213 to address “the last” slot throughout the spec.

- Approve R1-1908565 to address the same issue throughout the spec.

# R1-1909237 (overall timing aspects)

This is a discussion paper to clarify common understanding on the timing aspects. Companies are encouraged to check the understanding below.

In addition to the above, clarification of possible range of accumulated TA value is proposed.

Proposal:

- Companies are encouraged to check R1-1909237.

- Make a conclusion that the proposal in R1-1909237 is the RAN1 common understanding.

- Clarify following:

  - Cumulative TA is not less than zero, i.e. actual TA is not less than NTA offset

  - Cumulative TA is not more than 2ms/2μ, i.e. actual TA is not more than 2ms/2μ + NTA offset

  - The UE can consider it an error case if either of the above is not met.

# R1-1909238 (HARQ-ACK for MAC-CE activation)

This draft CR proposes, in addition to the clarification on MAC-CE activation time when PUCCH repetition is configured, to clarify that HARQ-ACK for the MAC-CE is transmitted either by using PUCCH or PUSCH (in the current spec, using PUSCH is missing).

### 7.2.1 UE behaviour If a UE transmits a PUCCH on active UL BWP of carrier in the primary cell using PUCCH power control adjustment state with index , the UE determines the PUCCH transmission power in PUCCH transmission occasion as [dBm] where - is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier of serving cell in PUCCH transmission occasion - is a parameter composed of the sum of a component , provided by p0-nominal, or dBm if p0-nominal is not provided, for carrier of primary cell and, if provided, a component provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP of carrier of primary cell , where . is a size for a set of values provided by maxNrofPUCCH-P0-PerSet. The set of values is provided by p0-Set. If p0-Set is not provided to the UE, , - If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot where is the last slot where the UE transmits HARQ-ACK information for the PDSCH providing the activation command and is the SCS configuration for the HARQ-ACK transmission - If the UE is not provided PUCCH-SpatialRelationInfo, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to 0 in p0-Set - is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion on active UL BWP of carrier of serving cell and is a SCS configuration defined in [4, TS 38.211] - is a downlink pathloss estimate in dB calculated by the UE using RS resource index as described in Subclause 7.1.1 for the active DL BWP of carrier of the primary cell as described in Subclause 12 - If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB - If the UE is provided a number of RS resource indexes, the UE calculates using RS resource with index , where . is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS - If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referencesignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referencesignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot where is the last slot where the UE transmits HARQ-ACK information for the PDSCH providing the activation command and is the SCS configuration for the HARQ-ACK transmission *** Unchanged text is omitted *** ### 9.2.2 PUCCH Formats for UCI transmission *** Unchanged text is omitted *** A spatial setting for a PUCCH transmission is provided by PUCCH-SpatialRelationInfo if the UE is configured with a single value for pucch-SpatialRelationInfoId; otherwise, if the UE is provided multiple values for PUCCH-SpatialRelationInfo, the UE determines a spatial setting for the PUCCH transmission as described in [11, TS 38.321]. The UE applies corresponding actions in [11, TS 38.321] and a corresponding setting for a spatial domain filter to transmit PUCCH in the first slot that is after slot where is the last slot where the UE transmits HARQ-ACK information with ACK value corresponding to a PDSCH reception providing the PUCCH-SpatialRelationInfo and is the SCS configuration for the HARQ-ACK transmission - If PUCCH-SpatialRelationInfo provides ssb-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a SS/PBCH block with index provided by ssb-Index for a same serving cell or, if servingCellId is provided, for a serving cell indicated by servingCellId - else if PUCCH-SpatialRelationInfo provides csi-RS-Index, the UE transmits the PUCCH using a same spatial domain filter as for a reception of a CSI-RS with resource index provided by csi-RS-Index for a same serving cell or, if servingCellId is provided, for a serving cell indicated by servingCellId - else PUCCH-SpatialRelationInfo provides srs, the UE transmits the PUCCH using a same spatial domain filter as for a transmission of a SRS with resource index provided by resource for a same serving cell and/or active UL BWP or, if servingCellId and/or uplinkBWP are provided, for a serving cell indicated by servingCellId and/or for an UL BWP indicated by uplinkBWP *** Unchanged text is omitted *** ## 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI state. For a CORESET with index 0, the UE expects that QCL-TypeD of a CSI-RS in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block - if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot where is the last slot where the UE transmits HARQ-ACK information for the PDSCH providing the activation command and is the SCS configuration for the HARQ-ACK transmission. The active BWP is defined as the active BWP in the slot when the activation command is applied. *** Unchanged text is omitted ***

Considering that the clarification for MAC-CE activation timing for PUCCH repetition is to be clarified in Section 4, the draft CR is updated as Draft-CR4.

Proposal:

- Approve the draft CR (Draft-CR4) for TS38.213 to address “HARQ-ACK for MAC-CE on PUSCH” throughout the spec.