3GPP TSG RAN WG1 #98			                                                                  R1-1909531

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.3.3

Source:	AT&T

Title:	Summary of 7.2.3.3 Mechanisms for resource multiplexing among backhaul and access links

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary of section 7.2.3.3 on potential enhancements to support multiplexing of access and backhaul links and provides proposals synthesized from the views expressed in contributions listed in the Appendix.

# Resource multiplexing among backhaul and access links

Observations and proposals in this section are primarily related to the following objectives from the IAB WID:

- Specification of mechanisms for resource multiplexing among backhaul and access links. This includes:

  - Specification of semi-static configuration for IAB-node/IAB-donor DU resources in case of TDM operation subject to half-duplex constraint. This shall be forward compatible to allow the support of half-duplex scenarios with FDM and SDM resource sharing among backhaul and access links.

  - Specification of time resource types for the DU’s child links: DL hard, DL soft, UL hard, UL soft, Flexible hard, Flexible soft, Not Available

  - Specification of dynamic indication (L1 signalling) of the availability of soft resources for a child IAB-node DU

  - Specification of required transmission/reception rules for IAB-nodes and associated behaviours regarding time resource utilization as discussed in TR 38.874 clause 7.3.3.

## Support for per-link IAB-node DU resource configuration

Background from RAN#97:

- The resources are configured on a per DU (cell) basis

  - FFS: indication of additional supplemental per-link resource configurations of child DUs

Company proposals:

| Intel | Proposal 1: Support additional supplemental per-link DU configuration. |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Proposal 2: Support per-link resource configurations of child DUs in the case of dual connectivity Proposal 3: Extend the current IAB resource configuration rules to support different dual connectivity scenarios. |
| Ericsson | Proposal 1 The DU configuration is not link specific at least for Rel-16. Proposal 2 If it should be agreed to include the support for link-specific DU configuration, more specifically, by allowing overriding the DU-specific H/S/NA base pattern for a subset of child links, it should only allow the Hard/Soft resource type overridden by NA, but not allow overriding from Hard to Soft, from Soft to Hard, nor from NA to Hard/Soft. |
| Qualcomm | Proposal 1: The DU is semi-statically configured with H / S / NA attributes for each resource. The NA attribute is optionally provided for each link to a child IAB node. |

Offline proposal 2.1-1:

Additional supplemental resource configurations which indicate NA resources on a per-link basis can be provided to an IAB-node DU.

## New slot formats for IAB-nodes

Background:

## Previous agreements:

- Indication of D/U/F resources in the semi-static DU resource configuration includes the following:

  - The flexibility to configure all of the slot patterns and formats supported by the existing Rel-15 TDD-UL-DL-Config RRC configurations and slot format table defined in Table 11.1.1-1 in TS38.213

    - FFS: additional pattern durations than supported in Rel-15

    - FFS: default resources or pattern

  - New slot formats defined only for IAB nodes (DU and MTs) which begin with uplink slots, uplink symbols, or flexible symbols.

    - Note: usage of these slot formats should be compatible with Rel-15 access UEs sharing the same link

    - FFS: whether these slot formats also need to be included in the MT RRC configuration and/or SFI carried on DCI Format 2_0

Company proposals:

| Vivo | Proposal 2: Slot format for semi-static backhaul link resource partition can be design based on the existing TDD-UL-DL-ConfigurationCommon, TDD-UL-DL-ConfigDedicated. And no restriction to always start from downlink and end with uplink for TDD-UL-DL-ConfigurationCommon, TDD-UL-DL-ConfigDedicated and slot format table defined in Table 11.1.1-1 in TS38.213. Proposal 3: Further down-select additional slot format for DU resource from Table 1 in R1-1908147 Table 1. Candidates for additional slot format for DU resource (total 105) Note 1: List of compatible Rel-15 access UEs slot format index as defined in Table 11.1.1-1 in TS38.213 Format; Symbol number in a slot; Note1. 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13. 0; U; D; D; D; D; D; D; D; D; D; D; D; D; D; 2 54. 1; U; F; D; D; D; D; D; D; D; D; D; D; D; D; 2 54. 2; U; F; F; D; D; D; D; D; D; D; D; D; D; D; 2 54. 3; U; F; F; F; D; D; D; D; D; D; D; D; D; D; 2 54. 4; U; F; F; F; F; D; D; D; D; D; D; D; D; D; 2 54. 5; U; F; F; F; F; F; D; D; D; D; D; D; D; D; 2 54. 6; U; F; F; F; F; F; F; D; D; D; D; D; D; D; 2 54. 7; U; F; F; F; F; F; F; F; D; D; D; D; D; D; 2 54. 8; U; F; F; F; F; F; F; F; F; D; D; D; D; D; 2 54. 9; U; F; F; F; F; F; F; F; F; F; D; D; D; D; 2 54. 10; U; F; F; F; F; F; F; F; F; F; F; D; D; D; 2 54. 11; U; F; F; F; F; F; F; F; F; F; F; F; D; D; 2 54. 12; U; F; F; F; F; F; F; F; F; F; F; F; F; D; 2 54. 13; U; F; F; F; F; F; F; F; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15 54. 14; U; U; D; D; D; D; D; D; D; D; D; D; D; D; 2 54. 15; U; U; F; D; D; D; D; D; D; D; D; D; D; D; 2 54. 16; U; U; F; F; D; D; D; D; D; D; D; D; D; D; 2 54. 17; U; U; F; F; F; D; D; D; D; D; D; D; D; D; 2 54. 18; U; U; F; F; F; F; D; D; D; D; D; D; D; D; 2 54. 19; U; U; F; F; F; F; F; D; D; D; D; D; D; D; 2 54. 20; U; U; F; F; F; F; F; F; D; D; D; D; D; D; 2 54. 21; U; U; F; F; F; F; F; F; F; D; D; D; D; D; 2 54. 22; U; U; F; F; F; F; F; F; F; F; D; D; D; D; 2 54. 23; U; U; F; F; F; F; F; F; F; F; F; D; D; D; 2 54. 24; U; U; F; F; F; F; F; F; F; F; F; F; D; D; 2 54. 25; U; U; F; F; F; F; F; F; F; F; F; F; F; D; 2 54. 26; U; U; F; F; F; F; F; F; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15 54. 27; U; U; U; D; D; D; D; D; D; D; D; D; D; D; 2 54. 28; U; U; U; F; D; D; D; D; D; D; D; D; D; D; 2 54. 29; U; U; U; F; F; D; D; D; D; D; D; D; D; D; 2 54. 30; U; U; U; F; F; F; D; D; D; D; D; D; D; D; 2 54. 31; U; U; U; F; F; F; F; D; D; D; D; D; D; D; 2 54. 32; U; U; U; F; F; F; F; F; D; D; D; D; D; D; 2 54. 33; U; U; U; F; F; F; F; F; F; D; D; D; D; D; 2 54. 34; U; U; U; F; F; F; F; F; F; F; D; D; D; D; 2 54. 35; U; U; U; F; F; F; F; F; F; F; F; D; D; D; 2 54. 36; U; U; U; F; F; F; F; F; F; F; F; F; D; D; 2 54. 37; U; U; U; F; F; F; F; F; F; F; F; F; F; D; 2 54. 38; U; U; U; F; F; F; F; F; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15 54. 39; U; U; U; U; D; D; D; D; D; D; D; D; D; D; 2 54. 40; U; U; U; U; F; D; D; D; D; D; D; D; D; D; 2 54. 41; U; U; U; U; F; F; D; D; D; D; D; D; D; D; 2 54. 42; U; U; U; U; F; F; F; D; D; D; D; D; D; D; 2 54. 43; U; U; U; U; F; F; F; F; D; D; D; D; D; D; 2 54. 44; U; U; U; U; F; F; F; F; F; D; D; D; D; D; 2 54. 45; U; U; U; U; F; F; F; F; F; F; D; D; D; D; 2 54. 46; U; U; U; U; F; F; F; F; F; F; F; D; D; D; 2 54. 47; U; U; U; U; F; F; F; F; F; F; F; F; D; D; 2 54. 48; U; U; U; U; F; F; F; F; F; F; F; F; F; D; 2 54. 49; U; U; U; U; F; F; F; F; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15 54. 50; U; U; U; U; U; D; D; D; D; D; D; D; D; D; 2 54. 51; U; U; U; U; U; F; D; D; D; D; D; D; D; D; 2 54. 52; U; U; U; U; U; F; F; D; D; D; D; D; D; D; 2 54. 53; U; U; U; U; U; F; F; F; D; D; D; D; D; D; 2 54. 54; U; U; U; U; U; F; F; F; F; D; D; D; D; D; 2 54. 55; U; U; U; U; U; F; F; F; F; F; D; D; D; D; 2 54. 56; U; U; U; U; U; F; F; F; F; F; F; D; D; D; 2 54. 57; U; U; U; U; U; F; F; F; F; F; F; F; D; D; 2 54. 58; U; U; U; U; U; F; F; F; F; F; F; F; F; D; 2 54. 59; U; U; U; U; U; F; F; F; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15 54. 60; U; U; U; U; U; U; D; D; D; D; D; D; D; D; 2 54. 61; U; U; U; U; U; U; F; D; D; D; D; D; D; D; 2 54. 62; U; U; U; U; U; U; F; F; D; D; D; D; D; D; 2 54. 63; U; U; U; U; U; U; F; F; F; D; D; D; D; D; 2 54. 64; U; U; U; U; U; U; F; F; F; F; D; D; D; D; 2 54. 65; U; U; U; U; U; U; F; F; F; F; F; D; D; D; 2 54. 66; U; U; U; U; U; U; F; F; F; F; F; F; D; D; 2 54. 67; U; U; U; U; U; U; F; F; F; F; F; F; F; D; 2 54. 68; U; U; U; U; U; U; F; F; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15 54. 69; U; U; U; U; U; U; U; D; D; D; D; D; D; D; 2 54. 70; U; U; U; U; U; U; U; F; D; D; D; D; D; D; 2 54. 71; U; U; U; U; U; U; U; F; F; D; D; D; D; D; 2 54. 72; U; U; U; U; U; U; U; F; F; F; D; D; D; D; 2 54. 73; U; U; U; U; U; U; U; F; F; F; F; D; D; D; 2 54. 74; U; U; U; U; U; U; U; F; F; F; F; F; D; D; 2 54. 75; U; U; U; U; U; U; U; F; F; F; F; F; F; D; 2 54. 76; U; U; U; U; U; U; U; F; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15 54. 77; U; U; U; U; U; U; U; U; D; D; D; D; D; D; 2. 78; U; U; U; U; U; U; U; U; F; D; D; D; D; D; 2. 79; U; U; U; U; U; U; U; U; F; F; D; D; D; D; 2. 80; U; U; U; U; U; U; U; U; F; F; F; D; D; D; 2. 81; U; U; U; U; U; U; U; U; F; F; F; F; D; D; 2. 82; U; U; U; U; U; U; U; U; F; F; F; F; F; D; 2. 83; U; U; U; U; U; U; U; U; F; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15. 84; U; U; U; U; U; U; U; U; U; D; D; D; D; D; 2. 85; U; U; U; U; U; U; U; U; U; F; D; D; D; D; 2. 86; U; U; U; U; U; U; U; U; U; F; F; D; D; D; 2. 87; U; U; U; U; U; U; U; U; U; F; F; F; D; D; 2. 88; U; U; U; U; U; U; U; U; U; F; F; F; F; D; 2. 89; U; U; U; U; U; U; U; U; U; F; F; F; F; F; 1 2 8 9 10 11 12 13 14 15. 90; U; U; U; U; U; U; U; U; U; U; D; D; D; D; 2. 91; U; U; U; U; U; U; U; U; U; U; F; D; D; D; 2. 92; U; U; U; U; U; U; U; U; U; U; F; F; D; D; 2. 93; U; U; U; U; U; U; U; U; U; U; F; F; F; D; 2. 94; U; U; U; U; U; U; U; U; U; U; F; F; F; F; 1 2 8 9 10 11 12 13 14 15. 95; U; U; U; U; U; U; U; U; U; U; U; D; D; D; 2. 96; U; U; U; U; U; U; U; U; U; U; U; F; D; D; 2. 97; U; U; U; U; U; U; U; U; U; U; U; F; F; D; 2. 98; U; U; U; U; U; U; U; U; U; U; U; F; F; F; 1 2 8 9 10 11 12 13 14 15. 99; U; U; U; U; U; U; U; U; U; U; U; U; D; D; 2. 100; U; U; U; U; U; U; U; U; U; U; U; U; F; D; 2. 101; U; U; U; U; U; U; U; U; U; U; U; U; F; F; 1 2 8 9 10 11 12 13 14 15. 102; U; U; U; U; U; U; U; U; U; U; U; U; U; D; 2. 103; U; U; U; U; U; U; U; U; U; U; U; U; U; F; 1 2 8 9 10 11 12 13 14 15. 104; U; U; U; U; U; U; U; U; U; U; U; U; U; U; 1 2 8 9 10 11 12 13 14 15. |
| --- | --- |
| LGE | Proposal 1: DU configuration scheme is based on TDD-UL-DL-ConfigCommon configuration, but it needs to be modified considering additional resource direction patterns. Proposal 2: For SDM/FDM between MT and DU, DU and MT resource support new resource direction pattern U-F-D or F-U-F-D-F. |
| Nokia, Nokia Shanghai Bell | Proposal 4: Additional pattern durations than supported in Rel-15 are not defined Proposal 6: Support the following “D/F/U” sequence order options DU - “DL-Flexible-UL” (=Rel-15) - “UL-Flexible-DL” Proposal 7: Rel-15 slot formats and the existing RRC signaling are used for MT part of the IAB node. Proposal 13: Discuss what kind of the slot format are needed for Rel-16 IAB operation. |
| Ericsson | Proposal 4 The DU configuration of DL, UL, and Flexible is based on the Rel-15 RRC-signaled D/U/F pattern provided by TDD-UL-DL-ConfigurationCommon as in TS 38.213, with extension allowing the “downlink-uplink-flexible” sequence order to be configurable and permuted to any other sequence order. Proposal 5 Extension allowing the “downlink-uplink-flexible” sequence order to be configurable and permuted to any other sequence order is also applied to the MT D/U/F configuration. |
| AT&T | Proposal 1: The following new slot formats should be defined for use by IAB-nodes (DUs and MTs) in the semi-static F1-AP and RRC configurations, and as part the SFI in DCI Format 2_0: F; F; F; F; F; F; F; U; U; U; U; U; U; U. U; U; U; U; U; U; U; F; F; F; F; F; F; F. U; U; U; U; U; U; U; D; D; D; D; D; D; D. U; U; U; U; U; U; U; F; U; U; U; U; U; U. U; U; U; U; U; U; U; F; F; U; U; U; U; U. U; U; U; U; U; U; U; F; F; F; U; U; U; U. U; U; U; U; U; U; U; F; F; F; F; U; U; U. U; U; U; U; U; U; U; F; F; F; F; F; U; U. U; U; U; U; U; U; U; F; F; F; F; F; F; U. U; U; U; U; U; U; U; F; U; U; U; U; U; U. U; U; U; U; U; U; U; F; F; U; U; U; U; U. U; U; U; U; U; U; U; F; U; U; U; U; U; U. U; U; U; U; U; U; U; D; F; U; U; U; U; U. U; U; U; U; U; U; U; D; D; F; U; U; U; U. U; U; U; U; U; U; U; D; D; D; F; U; U; U. U; U; U; U; U; U; U; D; F; F; U; U; U; U. U; U; U; U; U; U; U; D; D; F; F; U; U; U. U; U; U; U; U; U; U; D; D; D; F; F; U; U. U; U; U; U; U; U; U; D; F; F; F; U; U; U. U; U; U; U; U; U; U; D; D; F; F; F; U; U. U; U; U; U; U; U; U; D; D; D; F; F; F; U. |

Offline agreement:

In Rel-16, only existing slot pattern durations are supported.

Offline agreement:

Slot formats with the sequence order UL-Flexible-DL are supported for IAB-node DU resource configurations (F1-AP) and IAB-node MT resource configurations provided in dedicated RRC signaling (e.g. enhancements to the existing TDD-UL-DL-SlotConfig in TDD-UL-DL-ConfigDedicated – details up to RAN2).

- FFS: Support of dynamic indication of UL-Flexible-DL

## Remaining details of IAB-node DU resource configuration

Background:

- Indication of H/S/NA for the DU resource configuration is based on one of the following alternatives:

  - Alt. 1: H/S/NA is additionally explicitly indicated per-resource type (D/U/F) in each slot

    - To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU one of the following sub-alternatives need to be supported:

      - 1a: H/S/NA is applied relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing.

        - FFS: definition of additional restrictions on the usage of the semi-static configuration (e.g. guard symbols) based on deployment scenario or DL/UL switching times within an IAB node, etc.

        - FFS: How the CU can get information about the required guard symbols for a given DU configuration if needed

      - 1b: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing.

        - FFS: Whether and/or how the CU will know the actual H/S/NA resources at the child DU

        - FFS whether S is explicitly indicated or not

      - 1c: H is applied relative to the DU resource configuration (D/U/F) slot timing. S is not explicitly indicated, but implicitly determined by the DU based on whether the corresponding MT configuration indicates the MT resources is F (DU-S). The remaining resources are assumed to be NA at the child DU.

        - FFS: Whether and/or how the CU will know the actual S/NA resources at the child DU

  - Alt. 2: NA is explicitly indicated as a resource type in each slot for both the DU and MT configuration. H/S is not explicitly indicated, but implicitly determined by the DU based on the corresponding MT configuration

    - FFS: how to handle the case where there is not a 1-1 mapping of DUs and MTs in the child IAB node

- Details of the F1-AP signaling design is up to RAN3

Company proposals:

| Huawei | Proposal 1: The number of guard symbols in backhaul link should be adaptable according to specific MT/DU TDM switching pattern, IAB TX/RX RF switching interval, subcarrier spacing and backhaul link propagation delay. Proposal 2: Alt.2 should be adopted to achieve the H/S/NA flavor indication for DU resource and H/S flavor should be implicitly derived without explicit F1-AP signaling. Proposal 3: Some mechanism should be supported to let parent node know the resource usage changes at IAB DU. |
| --- | --- |
| ZTE, Sanechips | - Proposal 1：H/S/NA is configured with DU timing. - Proposal 2：Alt 1a is preferred for indication of H/S/NA for the DU resource. |
| Samsung | Proposal 1: For DU resource configuration, H/S/NA is applied relative to the DU resource configuration (D/U/F) slot timing. Proposal 2: Adaptation period for IAB TDM multiplexing pattern between access and backhaul links should be configurable, where within each adaptation period, a bitmap can be used to indicate the active link. Proposal 3: An IAB node should wait until its child IAB nodes finish their own configuration for multiplexing pattern before issuing a new multiplexing pattern configuration command and a confirmation signaling can be used for notification. |
| Intel | Proposal 1: Support additional supplemental per-link DU configuration. Proposal 2: On indication of H/S/NA for DU semi-static resource allocation, support Alt 1b, where H/S/NA resource allocation is applied at an IAB DU relative to the co-located MT’s resource configuration (D/U/F) slot timing. |
| LGE | Proposal 4: H/S/NA indication is applied relative to the DU resource configuration (D/U/F) slot timing. Proposal 5: To CU obtain the actual MT resource availability, it can be considered that MT reports a) timing gap related values in Option A or b) the number of gap symbols in Option B. |
| CMCC | Proposal 1: The overlapped or the conflict time duration should be known to solve the transmitting and receiving conflict issue within the child IAB. Proposal 3: From the perspective of saving the signalling overhead and reducing the configuration complexity, Alt. 1a and Alt. 2 are more preferred than Alt. 1b. |
| Nokia, Nokia Shanghai Bell | Proposal 1: Only flexible resources for MT can be used as soft resources for DU. Proposal 5: Default resource types for IAB node are the following: - MT part: Flexible - DU part: Flexible Soft Proposal 8: Select Alt1 for indication of H/S/NA for the DU resource - Alt1c is our preference Proposal 9: Inter-IAB DU conflicts can be avoided by the following rules, - When both Parent DU and Child DU are configured as Hard, Parent DU may operate according to NA - When the parent DU is configured as NA and Child DU is configured as Soft, Parent DU may configure Child DU implicitly or explicitly as available (IA) - When the parent DU is configured as Soft and Child DU is configured as Hard, Parent DU may operate according to NA. |
| Ericsson | Proposal 3 The DU H/S resource types are configured per DU cell with respect to each MT carrier frequency. Proposal 6 The indication of H/S/NA resources in the semi-static DU configuration is applied relative to the DU resource configuration (D/U/F) slot timing (i.e., alt. 1a) without considering the MT resource configuration or timing. Proposal 7 The parent node is provided with the information about the timing misalignment between any one of the DU UL/DL resource and any one of the MT UL/DL resource of the served IAB nodes as each IAB node measures. |
| AT&T | Proposal 2: H/S/NA is additionally explicitly indicated per resource type (D/U/F) in each slot. Proposal 3: H/S/NA is applied relative to the MT resource configuration (D/U/F) slot timing. |
| Sharp | Proposal 1:The “timing gap” T_delta between child/parent links and child/grandchildren links should be reported using RRC signaling unless there is an objection from RAN4. Proposal 2:RAN1 should specify as needed procedures for Parent/Child behavior in response to synchronization reports. Proposal 3:Switching time capability should be transmitted in IAB node capability information exchange. Proposal 4:Any constraints on resource indication should be configurable. |
| NTT DOCOMO | Proposal 1: The semi-static and dynamic resource configurations shall be considered simultaneously. Proposal 2: Alt 1b is a straightforward configuration for the semi-static resource configuration. In order to reduce signalling overhead, Alt 1c’ is another alternative in which a joint indication for dynamic indication is necessary. |
| Qualcomm | Proposal 2: Partial slot resource conflicts originating from the timing skew between MT slot timing and DU slot timing are treated in the same way as Hard ¦¦ Hard resource conflict, i.e. the decision on whether to give priority to the DU or to the MT for the use of the resource is left to the IAB node implementation. |
| CEWIT | Proposal 1: We support Alt 1a for the indication of H/S/NA for the DU resource configuration. Proposal 2: The IAB node, with knowledge of MT resource type, must internally calculate the guard period and actual slot structure for DU Proposal 3: DU of an IAB node should insert guard symbol when there is a switching from one mode of operation to another, based on the capabilities of the IAB node, the parent IAB node and the child IAB node. |

Offline agreement:

The H/S/NA attributes for the per-cell DU resource configuration should take into account the associated MT carrier frequency(ies).

- Note: RAN1 assumes that this is mainly needed for intra-band cases

Offline majority view:

To handle potential misalignment in time of the configured DU and MT resources when determining the validity of H/S/NA at the DU, the following is supported:

- H/S/NA is applied/determined relative to the DU resource configuration (D/U/F) slot timing without considering the MT resource configuration or timing

- The parent DU may be provided information about the misalignment between the MT and DU DL Tx/Rx / UL Tx/Rx timing(s) in order to avoid resource type conflicts (if needed)

- It is up to the parent IAB node implementation and network configuration to handle any potential resource type conflicts (e.g. MT scheduling restrictions or configuration of guard symbols). The child IAB node should handle any remaining conflicts using the existing conflict resolution rules

Offline proposal:

H/S/NA attributes for the per-cell DU resource configuration are explicitly indicated per-resource type (D/U/F) in each slot.

## Indication of soft resource availability

Background:

Conclusion:

The following alternatives are considered (to be down-selected in RAN1#98) for the explicit indication of the availability of soft resources:

- Alt 1) Indicate which MT resources are “IA” for the child DU (DU-IA)

- Alt 2a) Indicate DU-IA and MT resource type (MT-D/MT-U/MT-F)

- Alt 2b) Indicate DU-IA and DU resource type (DU-D/DU-U/DU-F)

- Alt 3) Jointly or separately indicate DU-IA, the DU resource type, and/or MT resource type

- FFS: monitoring occasions for the explicit indication at the MT

- FFS: whether the processing time for applying an explicit indication at the child DU is defined or left to implementation.

Company proposals:

| Huawei | Proposal 4: Explicit indication based on a new DCI format is introduced to indicate which MT resources are “IA” for the child DU - The time offset between the dynamic indication and the corresponding time resource should be configured to MT |
| --- | --- |
| Vivo | Proposal 1: For IAB-node, the processing times between a downlink indication for DU child link availability from parent node and PDSCH/PUSCH transmission on DU child link should be defined. |
| ZTE, Sanechips | - Proposal 5：Explicit indication of the availability of soft resources should be decoupled from MT configuration. Whether explicit indication of availability can further indicate or override DU resource type should be studied. |
| Intel | Proposal 3: To explicitly indicate soft resource availability, use two SFI indications for parent link and child links respectively. |
| LGE | Proposal 6: Flexibility of IA/INA resource allocation needs to be restricted. Proposal 7: It is considerable that an IAB node request its DU soft resource availability to its parent node. |
| CMCC | Proposal 2: There is no need to indicate flexible/soft resources through a DCI signalling. |
| Nokia, Nokia Shanghai Bell | Proposal 10: When operating according to implicit indication in the TDM scenario, MT determines resource usage for flexible time resources according to Rel-15 rules defined in TS 38.213 (Section 11.1) and based on received DCI and/or higher layer configuration. - If Flexible MT resource is not assigned for Parent BH link it is considered as “IA” – i.e. DU resource is implicitly indicated as available - otherwise, if Flexible MT resource is assigned for Parent BH DL or Parent BH UL link it is considered as “INA” - i.e. DU resource is implicitly indicated as not available. Proposal 11: Slot-based monitoring for explicit indication is sufficient for Rel-16 Proposal 12: Support Alt3 (jointly indicating DU-IA, The DU resource type and MT resource type.) - Explicit indication is based on existing DCI Format 2_0. - Explicit indication needs to support also interference coordination for child links. - Introduce three additional resource types to support dynamic capacity allocation between parent and child links - Child DL (CDL) - Child UL (CUL) - Child Flexible (CF) Proposal 14: For the usage of soft resources at a child DU, use processing times defined for flexible resources in Rel-15 as the basis to determine when certain soft resource at the latest be made available for the DU. |
| Ericsson | Proposal 8 The explicit indication of the availability of soft resource is to indicate certain DU resources are “IA” for the child DU (i.e., according to the DU slot timing) by explicitly informing the IAB node that the corresponding MT resources will not be used. Proposal 9 Explicit indication is not constrained to the current DCI Format 2_0. Proposal 10 The processing time for applying an explicit indication at the IAB DU impacts the IAB node’s utilization rate of soft resources, which should be left to implementation. |
| AT&T | Proposal 4: IAB should support explicit indication of soft resource availability by using GC-PDCCH signalling via DCI Format 2-0 from the parent IAB node to the child IAB node to jointly indicate DU-IA and MT resource type (MT-D/MT-U/MT-F). Proposal 5: The explicit indication of soft resource availability can be sent on any symbol within a slot. |
| Sharp | Proposal 5: For the explicit indication of soft resources’ availability, down-select to include either Alt 2a or Alt 2b. |
| NTT DOCOMO | Proposal 3: DU-IA and MT-D/U/F should be dynamically indicated to align with Rel-15 operation and to save signalling overhead. Proposal 4: For explicit indication of soft resource availability, SFI-like indication via DCI format 2-0 for joint indication of DU soft resource availability and MT’s link direction is supported. The following sub-options should be considered to indicate the new slot format consisting of MT-D/MT-U/MT-F/DU-A. - Alt.1-1: reuse DCI format 2-0 and use reserved entries (56-254) in current slot format table to indicate the availability of DU soft resource together with link direction for MT resource - Alt.1-2: reuse DCI format 2-0 and enhance slotFormats in slotFormatCombination with 2-bit per symbol to directly indicate one out of MT-D/MT-U/MT-F/DU-A |
| CAICT | Proposal 1: For dynamic signaling configuration, alt 2a) and alt 3) are more preferred. |
| CEWIT | Proposal 4: The signalling of IA/INA is based on MT slot structure, based on which the IAB node should derive the slots of DU which is available for Tx/Rx in child/access link Proposal 5: The signalling of IA/INA for the soft resource at DU of an IAB node should be associated with a validity duration, which is a fixed value or value configured semi-statically or dynamically Proposal 6: IAB node signals IA to the child node irrespective of the DU configuration of the child node Proposal 7: The parent node signals explicit IA for the soft resource at DU of the IAB node through slot format indicator (SFI) in the downlink control information (DCI) Proposal 8: An IAB node derives IA to use the soft resource in a slot when the slot is configured as F in both semi-static and dynamic configuration Proposal 9: The slots where child DU is configured as soft-UL should be known to parent DU apriori, in order to provide the explicit signaling of IA in advance. |

Offline agreement: For the explicit indication of DU-IA:

- At least DU-IA can be indicated independently of the MT resource configuration: The DCI indicates DU-IA per-resource type (D/U/F)

- FFS: Whether this indication is via a new DCI format or DCI Format 2_0 using existing or reserved entries of Table 11.1.1-1 in 38.213

- FFS: Whether for the purpose of signaling optimization a DU/MT resource type can additionally be jointly indicated with DU-IA which overrides the flexible resources in the semi-static DU/MT resource configuration