3GPP TSG-RAN WG1 Meeting #98	R1- 1909534

August 26th – 30th, 2019

Prague, Czech Republic

Agenda item:	7.1.3

Source: 	Qualcomm Incorporated

Title: 	Summary of CRs on UCI multiplexing on PUSCH

Document for:	Discussion/Decision

# Introduction

In this contribution, a summary of CRs on UCI multiplexing on PUSCH is provided in Section 2.

# Summary of CRs

There are in total 4 CRs submitted to 7.1.3 which are included in this summary and listed below.

R1-1908327	Draft CR on UCI multiplexing in PUSCH		ZTE

Recommendation: The CR is not needed. No Spec update is needed.

Reason: The CR proposes to remove a paragraph in Subclause 9 of TS38.213, which appears to be duplication of another paragraph. However, a few companies commented on RAN1 email reflector that both paragraphs are needed. One paragraph describes UE behavior when PUCCH(s) overlaps with single PUSCH. The other paragraph describes UE behavior when PUCCH(s) overlaps with multiple PUSCHs.

R1-1908455	Draft CR on PUCCH resource determination with consideration of processing time	 Samsung

Recommendation: The CR is not needed. No Spec update is needed.

Reason: The CR proposes to add N3 timeline check from last symbol of the last PDCCH to first symbol of A/N transmission on determined PUCCH resource. The change is not needed, because current specification already has N1 timeline defined for this purpose.

R1-1909236	Draft TS 38.213 CR on UE procedure for reporting multiple UCI types		Qualcomm

Recommendation: Endorse the CR.

Reason: The purpose of the CR is to implement an agreed TP in an email discussion [97-NR-02] after RAN1 #97 meeting.

R1-1908566	Miscellaneous correction in 38.214		CATT

Recommendation: Further discussion is needed.

Reason: Many miscellaneous corrections are proposed in this CR. Each correction needs further discussion.