3GPP TSG RAN WG1 Meeting #98	R1- 1909564

Prague, Czech Republic, August 26th – 30th, 2019

Title:	Draft response to LS on Combination 2 of Uu QoS characteristics values for V2X services

Response to:	S2-1813386

Release:	Rel-16

Work Item:	NR_L1enh_URLLC

Source:	Huawei [RAN WG1]

To:	SA WG2, RAN WG2

Cc:	-

Contact Person:

Name:	                 Yongxia Lyu

E-mail Address:	  lvyongxia@huawei.com

1. Overall Description:

RAN1 has received the LS S2-1813386 from SA2. There are two new combinations of QoS characteristics values in the LS: 1) PDB = 5ms, PER = 10-4 and MDBV = 1354 bytes for Collision Avoidance and Platooning with high LoA, and 2) PDB ~1.5 ms, PER=10-5 and MDBV ~1300 bytes for Emergency Trajectory Alignment and Sensors information Sharing with high LoA. SA2 proposed the following question to RAN1.

SA WG2 would like to ask RAN WG2 and RAN WG1 whether, for Uu over E-UTRA and NR, the two new combinations of QoS characteristics values indicated above are feasible or not.

RAN1 has sent an LS R1-1905904 as reply to the feasibility of the first combination of QoS values. As to the second combination of QoS values, based on the evaluation results assuming air interface latency as 1 ms or 0.5 ms, RAN1 expects that it is also feasible with the configuration of large bandwidth (e.g., 200 MHz for DL and 100 MHz for UL) and advanced antenna configuration (e.g., 8Tx/8Rx for DL and 32Tx/32Rx or more for UL) at BS, at least for the single UE case.

2. Actions:

To SA WG2

RAN1 expects Combination 2) assuming air interface latency as 1ms to be feasible. RAN1 respectfully asks SA2 to take this into consideration.

4. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #98b		14 – 18 Oct 2019, Chongqing, China

TSG RAN WG1 Meeting #99		18 – 11 Nov 2019, Reno, US