3GPP TSG RAN WG1 Meeting #98	R1-1909581

Prague, Czech Republic, August 26th – 30th, 2019

Title:	Draft LS to RAN4 on EM interference in IIoT environments

Response to:

Release:	Release 16

Work Item:	Study on Channel Modeling for Indoor Industrial Scenarios

Source:	RAN1

To:	RAN4

Cc:

Contact Person:

#### Name:	Guo Bolun

#### E-mail Address:	guobolun@huawei.com

Attachments:	None.

1. Background:

In 3GPP RAN#81 meeting, a new study item (SI) was approved to study on channel model for indoor industrial scenarios. It was recognized that the current 3GPP channel model in TR 38.901 contains a common channel model with scenario-specific model parameters and settings for scenarios such as Urban Macro, Urban Micro, Rural Macro, and Indoor Hotspot (InH). However, it is noted that InH is based on indoor office / shopping mall environment. To address industrial scenarios that exhibit more diverse and unique environmental features, it is needed that the InH in TR 38.901 should be extended to cover additional characteristics of industrial scenarios. RAN1 is now investigating the IIoT channel model in a relative wide frequency range (from 0.5 GHz up to mmW).

In 3GPP RAN1 #96bis meeting, it has been agreed to further discuss how to specify the source of potential electromagnetic (EM) interference in the IIoT channel model. There is an initial agreements as follows: Model the channel between the EM interferer(s) and BS or UT using the same channel model(s) as for the BS-UT links

- Note: The interfering equipment may not necessarily be a point source

- Note: Characterization of the EM interference source is out of the scope of the present SI, and other groups (e.g. RAN4) may be more competent to handle such work

2. Questions:

There are transportation and electric machinery (such as electric motors, welding processes, rectifiers, power lines and switching devices) in indoor industrial scenarios. This machinery may be a potential source of electromagnetic interference, of which RAN1 therefore seeks a more detailed understanding. For that purpose, a signal model for the interference source(s) and a channel model from the interference source(s) to the Rx are needed.

RAN1 would like to ask RAN4 the following questions:

Q1: Is there any study in RAN4 about EM interference, particularly in indoor factory and similar scenarios?

Q2: Which frequency band(s) are likely to experience strong EM interference?

Q3: Can RAN4 provide other useful consideration or information about EM interference?

3. Actions:

To RAN4:

RAN1 respectfully asks RAN4 to answer the following questions:

- Is there any study in RAN4 about EM interference, particularly in indoor factory and other similar scenarios?

- Can RAN4 provide guidance which frequency bands are likely to have strong EM interference?

- Can RAN4 provide other useful consideration or information about EM interference?

3. Date of Next RAN1 Meetings:

RAN1#98bis	14 - 20 Oct 2019	in Chongqing, China

RAN1#99 	18 – 22 Nov 2019	in Reno, NV, USA