3GPP TSG RAN WG1 98			R1-1909582

Prague, Czech Republic, 26th – 30th August, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Feature lead summary for Tuesday offline session

Document for:	Discussion and Decision

# Introduction

In this contribution, the outcome of Tuesday offline sessions on the following issues is summarized:

- UCI omission

- CBSR

- Port selection codebook

Summary

## UCI omission

Offline agreement:

The selected UCI omission scheme should meet the following criteria when CSI omission occurs:

- CSI calculation is identical to that for without omission – otherwise the UE may end up recalculating the CSI if UCI omission occurs.

  - When UCI omission occurs, the associated CQI may not be calculated conditioned on the PMI after omission

- The occurrence of UCI omission can be inferred from the associated CSI report without any extra signaling.

- The resulting UCI payload after omission should not be ambiguous (payload ambiguity would require the gNB to perform blind decoding of UCI Part 2).

- When CSI omission occurs, dropping all NZCs associated with any particular layer should not be done.

Note: CSI omission occurs when the allocated UL resource for UCI is not sufficient for full CSI reporting.

Offline agreement:

Denote the non-zero LC coefficient (NZC) associated with layer , beam , and FD-basis  as . The associated bitmap component (including zero(s)) is.

For the purpose of UCI omission, the parameters in UCI Part 2 is divided into 3 groups where Group n is of a higher priority than Group (n+1), n=0, 1.

For further discussion, to be finalized this week (RAN1#98):

- Group 0 includes at least: SD rotation factors, SD indicator, FD indicator, SCI(s) for all the NRep reports

- Group 1 includes at least: reference amplitude(s) for weaker polarization,

- Group 2 includes at least:

- Note: G1 and G2 exclude the indices associated with the strongest coefficient(s)

In RAN1#98bis, decide the following aspects (list alternatives during offline):

- Priority rule for determining G1 and G2: based on weak-strong, based on lm indexing, …

- Which group(s) βl,m(λ)   belong to: depending on whether bitmap dropping is used or not

- SD/FD indicator: same as full report (both retained and dropped bases) vs. only indicating retained bases

- “Rate matching” scheme: …

- …

## CBSR

# Offline agreement:

# On CBSR for Rel.16 Type II codebook, the three agreed alternatives for down selection are further clarified as follows. No other alternatives or sub-alternatives will be considered for down selection.

- Alt1. Analogous to Rel.15 Type I

  - Hard restriction (0 or 1) can be applied to any of the spatial beams (the restriction is applied for both polarizations of the beam) and is higher-layer configured with one size-N1N2O1O2 bitmap B

- Alt2. Analogous to Rel.15 Type II (SD beam group restriction + per coefficient amplitude restriction)

  - Four beam groups are selected via higher-layer configured bitmap B1

  - For each spatial beam in each of the four beam groups, soft restriction (maximum amplitude of 0, ½, , or 1) is applied to any of the coefficients associated with the beam (the restriction is applied for both polarizations of the beam). This maximum amplitude restriction is higher-layer configured with four bitmaps

- Alt3. Rel. 15 Type II SD beam group restriction + joint per SD beam restriction

  - Four beam groups are selected via higher-layer configured bitmap B1

  - Amplitude restriction:

    - Alt 3A (Sum power ratio): For each beam l0 in each of the four beam groups, power ratio threshold γl0 (definition and values FFS) is configured, the following criterion should be satisfied: λ=0RI-1m=0M-1k=01pres,l02λ,m,k≤γl0

    - Alt 3B (Restriction on W2=W2WfH): For each beam l0 in each of the four beam groups and FD index k0, 0≤k0<N3, wideband gain threshold Γl0 (maximum threshold of 0, 22, 12, or 1) is configured, the following criterion should be satisfied: w2(l0,k0)maxl,kw2(l,k)≤Γl0

      - i.e. the “wideband gain” in the frequency domain of the precoder is restricted similarly to Rel. 15

  - This maximum amplitude restriction is higher-layer configured with four bitmaps

## Port selection codebook

# Offline agreement:

On the support of Rel.16 Type II port selection codebook for RI=3-4, evaluate the need for supporting the following scheme in RAN1#98bis:

- Reuse the Rel-15 Type II W1 matrix and the Rel-16 Type II  and Wf

- Note: if there is no consensus on the need for this feature, such extension to RI=3-4 is not supported in Rel.16