3GPP TSG RAN WG1 Meeting #98	R1-1909588

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	6.2.2.5

Source:	Huawei, HiSilicon

Title:	Feature lead summary for support of quality report in Msg3 and connected mode

Document for:	Discussion and decision

Introduction

At RAN#83, the Rel-16 work item on additional enhancements for NB-IoT was revised and approved in [1]. One of the objectives in this work item is to support Msg3 quality reporting for non-anchor access. A new objective is added to support quality reporting in connected mode for anchor and non-anchor carriers.

Improved multi-carrier operation:

- Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2, RAN4]

- Specify support for quality reporting in connected mode for anchor and non-anchor carriers. The quality report is not carried in the physical layer. [RAN1, RAN2, RAN4].

In RAN1#94, the following agreements and working assumptions are reached:

Agreement

For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%

- FFS: Whether the details on the hypothetical NPDCCH are specified or not

For non-anchor access, RAN1 further studies how UEs report the measured channel quality

Working Assumption

For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR)

- FFS: Whether the UE performs measurement on other carriers

In RAN1#94bis, the following agreements are reached:

Agreement

- RAN1 does not define search space for hypothetical NPDCCH for channel quality report in Msg3 on non-anchor access.

- From RAN1 point of view, specification support for measurement period for non-anchor access in RAN1 specifications is not needed

- RAN1 does not define measurement reference resource for non-anchor access.

For further study:

The following scenarios with regards to downlink channel quality reporting in msg3 for non-anchor carrier access.

- For EDT/non-EDT, msg3 associated with PDCCH order PRACH, IDLE

- PUR

In RAN1#95, the following agreements are reached:

Agreement

- In case 4 bits is used for a non-anchor carrier, all repetition i.e. 12 candidate values, {1,2,4,8,16,32,64,128,256,512,1024,2048} can be reported in Msg3.

- In case of 2 bits is used for a non-anchor carrier, 3 candidate values can be reported in Msg3. Select one of the following alternatives for determining the 3 values:

  - Depending on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS.

  - Depending on R, "DCI subframe repetition number" indicated in DCI format N1 for Msg2 scheduling.

  - Depending on Rdecoded, based on the number of repetitions for NPDCCH scheduling Msg2 where UE decodes successfully.

In RAN1#96, the following agreements are reached:

For further study in future meetings:

For channel quality report in Msg3 on non-anchor access, RAN1 identified the following scenarios in which UE may monitor a carrier that is different from the carrier in which the UE receives Msg2 (i.e. RAR):

- Idle mode

  - Non-EDT random access procedure

  - EDT procedure

- Connected mode

  - NPDCCH ordered NPRACH procedure

  - Random access procedure to apply for UL-SCH resource

Agreement

For the measurement on carrier(s) other than the one UE is receiving RAR for non-anchor access, if supported, RAN1 to select one or more among the following candidates:

- Paging carrier

- Anchor carrier

- Carrier(s) configured by CarrierConfigDedicated-NB in connected mode

- Other carriers configured by network with implicit or explicit signaling

The following issues are identified for RAN1 further study

- Whether amount of time or gap is needed for the measurement

- Which carrier(s) UE reported if more than one carrier is measured

- What kind of NRS UE can use for the measurement and whether needs indication

Agreement

In case of 2 bits are used for a non-anchor carrier, one of 3 candidate values can be reported in Msg3

- Which depends on Rmax, the maximum number of repetitions for NPDCCH Type 2 CSS.

For further study in future meetings:

If supported, RAN1 further discuss how to derive the candidate values reported in Msg3 if no NPDCCH Type2 CSS on the carrier.

In RAN1#96bis, the following agreements are reached:

Agreement

For channel quality report in connected mode, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%.

Agreement

For channel quality report in connected mode other than Msg3, UE performs measurement on the carrier it is assigned to monitor in USS for NPDCCH and the associated NPDSCH.

- FFS other carrier(s)

Agreement

For channel quality report in Msg3 for non-anchor access, RAN1 send LS to RAN2 requesting the following information:

- Number(s) of available bits for the quality reporting in Msg3

- Whether the quality report in Msg3 is transmitted in RRC or MAC CE

Send LS to RAN2 and CC RAN4. Ask, from RAN2, perspective, whether the above might be different for different cases. The LS is endorsed in R1-1905593.

Agreement

For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is no larger than 2.

- FFS detail of the maximum number (1 or 2)

Agreement

eNB can enable/disable channel quality report in Msg3 for non-anchor access via SIB

- FFS details

- FFS whether the field in Rel-14 for anchor access is reused

In RAN1#97, the following agreements are reached:

Agreement

For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is 1.

Agreement

For channel quality report in Msg3 on non-anchor access in idle mode, channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported.

- FFS: Support of network indication of which carrier to use to report Msg3 (e.g. explicit signaling in SI)

Agreement

For channel quality report in connected mode other than Msg3, RAN1 assumes the following aspects are decided by RAN2 and RAN4.

- number of candidate values for channel quality reports

- mapping value of reported channel quality

- report in RRC message or MAC CE

LS to RAN2 and RAN4 to convey this agreement is endorsed in R1-1907624 (Zhe, Huawei)

For further discussion

For channel quality report in connected mode other than Msg3, RAN1 further discuss whether physical layer triggering mechanism is needed.

For further discussion

RAN1 further discuss the followings for channel quality report in Msg3 on non-anchor access in connected mode

- The support of channel quality report in Msg3 in connected mode

  - If supported, which carrier can be measured for the channel quality report in Msg3

    - RAR carrier

    - unicast carrier

  - If supported, how to trigger the report in Msg3

In this meeting, there are 7 contributions [2]-[8] submitted to this agenda. This document summarizes the views and proposals from different companies. As the feature lead, several issues are identified and listed hereafter for discussion.

Summary of contributions

A list of views from all contributions is summarized as follows.

R1-1908031 Quality report in Msg3 and connected mode in NB-IoT Ericsson Observation 1 On a non-anchor carrier where RAR is sent, there are enough NRS subframes for the UE to perform the measurements. Proposal 1 Only one DL non-anchor carrier should be used for channel quality reporting in Msg3. Proposal 2 When an NB-IoT UE is capable of DL channel quality reporting in Msg3 and a non-anchor is configured for RAR, the default operation should be that the UE should report the DL quality of the carrier where the UE monitors RAR. Proposal 3 For the carrier indicated by the network (if other carrier than the carrier where the UE monitors RAR), specify rules for NRS presence for the UE to perform channel quality measurements. The NRS presence rules can be similar to the current NRS presence rules during RAR on non-anchor carrier, with an NRS window (analogue to the RAR window) and Rmax (analogue to Rmax for Type-2 CSS). Proposal 4 Conclude that from RAN1 point of view, the RAN2 working assumption that the same code points are used for DL quality reporting in connected mode as for Msg3 can be confirmed. Proposal 5 For quality reporting in connected mode, the measurement takes place on the unicast carrier (i.e. the carrier with the UE-specific NPDCCH search space). Proposal 6 Quality reporting in Msg3 in connected mode is either not supported or treated with low priority in Rel-16 (meaning that it can be supported if the additional effort to support it is minimal). Proposal 7 Consider supporting quality reporting in connected mode also after Msg5. Proposal 8 Conclude that from RAN1 point of view, support of DL channel quality reporting in dedicated PUR transmission is feasible and beneficial but leave to RAN2 whether to introduce support for it or not.

R1-1908090 Support of quality report in Msg3 and connected mode Huawei, HiSilicon Proposal 1: For channel quality report in Msg3 in idle mode, UE reports channel quality of the carrier UE monitors to receive Msg2 (i.e. RAR). Proposal 2: Channel quality report in Msg3 in connected mode is not supported. Proposal 3: DL quality report in connected mode is enabled via Msg4 or RRC reconfiguration message.

R1-1908268 Support of quality report in Msg3 and Connected Mode for NB-IoT ZTE Observation 1: In RRC_CONNECTED mode, if a UE loses uplink synchronization, the DL channel quality of the UE may also have changed. In this case, channel quality measurement and report are also necessary, and ”PDCCH order” based triggering mechanism is beneficial for reducing the triggering signaling overhead and the latency of reporting the channel quality compared with “ MAC CE” based triggering mechanism. Observation 2: There are enough reserved bits in “PDCCH order” that can be used as triggering indicator for channel quality report of UE. Observation 3: In RRC_CONNECTED mode, Msg3 is transmitted during contention based random access procedure. Proposal 1: For channel quality report in connected mode other than Msg3, UE does not perform measurement on other carriers. Proposal 2: For channel quality report in connected mode other than Msg3, the following triggering mechanism should be supported. - PDCCH order; - MAC CE; Proposal 3: In RRC_CONNECTED mode, channel quality is measured only on carrier(s) on which NPDCCH of USS is monitored. Proposal 4: For channel quality report in Msg3 on non-anchor carrier in RRC_CONNECTED mode, the following triggering mechanism should be supported. - PDCCH order based triggering - Reusing SIB2 based enable/triggering mechanism in RRC_IDLE mode

R1-1908304 Support of Quality report in Msg3 on non-anchor carrier Nokia, Nokia Shanghai Bell Proposal 1: For the Msg3 channel quality report on an access non-anchor carrier in idle mode, the network can configure the UE to perform a channel quality measurement on a carrier it signals in SI. Observation: No new physical layer triggering mechanism is needed for the channel quality report in connected mode other than Msg3. Proposal 2: For the Msg3 channel quality report on non-anchor access in connected mode, the measurement is made on the unicast carrier. Proposal 3: An enhanced PDCCH order can be used to trigger a Msg3 quality report. FFS: If the PDCCH order needs to be modified (e.g. an indicator bit)

R1-1908450 Discussion on Msg3 report for non-anchor access Samsung Observation 1: DL quality of non-anchor carrier derived by NRS measurement on anchor carrier can hardly reflect actual interference condition on non-anchor carrier. Proposal 1: The following carriers are used for DL quality measurement to support Msg3 report for non-anchor access: - If UE is capable of generating Msg3 after transmission of PRACH: - Non-anchor carrier on which RAR is monitored, UE measure subframes containing NRS corresponding to Type-2 CSS for RAR - Otherwise - Non-anchor carrier on which paging is monitored, UE measure subframes containing NRS corresponding to NPDCCH candidate of paging - Non-anchor carrier on which the position of NRS is configured by eNodeB Proposal 2: For DL quality report in connected mode, the triggering signal is down-selected between: - DCI - MAC CE - RAR - Proposal 3: NRS presence for DL quality measurement should be studied to support DL quality measurement on non-anchor in connected mode.

R1-1908748 DL Quality Report on non-anchor carrier in NB-IoT MediaTek Inc. Proposal 1: DL quality report is transmitted in Msg3 in RAR carrier without explicit signalling in SI Proposal 2: Channel quality report in Msg3 on non-anchor access is measured by connected UE in unicast carrier and reported in Msg3 in Contention-Free Random Access (CFRA) during UL timing alignment procedure. Proposal 3: Channel quality report in Msg3 on non-anchor access by connected UE is triggered by NPDCCH order in Contention-Free Random Access (CFRA) during UL timing alignment procedure. Proposal 4: Whether NPDCCH trigger for DL channel quality report in Msg3 on non-anchor access in connected mode can be supported without Msg1 and Msg2 can be further studied.

R1-1908838 Support of quality report in msg3 and connected mode Qualcomm Incorporated Proposal 1: For DL quality reporting in msg3 in connected mode, the UE measures the carrier in which it receives RAR. Proposal 2: For DL quality reporting in connected mode, do not introduce PHY triggering mechanism.

Issues and discussion

Quality report for non-anchor access in Msg3 (IDLE mode)

An LS from RAN2#105bis about non-anchor carrier CQI reporting in MSG3 [9] is received. The key points are extracted as follows.

RAN2 has discussed the options to report CQI for non-anchor carrier in MSG3 for NB-IoT and has made the following agreements for non-EDT and EDT: - Existing Msg3 signalling is used for reporting downlink channel quality when UE makes access on non-anchor carrier. - Existing SIB2 signalling is used to enable UE to report downlink channel quality on non-anchor carrier. Note that this implies the code points specified in Release 14 for anchor carrier CQI reporting in MSG3 are used for non-anchor carrier CQI reporting in MSG3.

RAN1 has agreed that channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported and the maximum number of carriers reported in Msg3 is 1.

In this meeting, 7 contributions submitted. Ericsson propose that the carrier reported in Msg3 is indicated by the network using explicit signaling in SI and without the presence of such indication the UE reports RAR carrier. Nokia propose that the carrier reported in Msg3 is indicated by the network using explicit signaling in SI. Samsung propose that if UE is capable of generating Msg3 after NPRACH transmission then UE reports RAR carrier, otherwise paging and other carriers also can be measured in some cases with NRS presence. Huawei, HiSilicon and MTK propose that only RAR carrier is used. Qualcomm and ZTE do not further discuss this.

  - Network indicate carrier for the measurement: Ericsson, Nokia

  - RAR or paging carrier depends UE capable: Samsung

  - Not support/not mention: Huawei/HiSilicon, MTK, Qualcomm, ZTE

There seems slight majority view that only RAR carrier is measured in idle mode. Based on the document inputs and feature lead’s understanding of the intention of above RAN2 LS, we can consider,

Recommended Proposal 1: For channel quality report in Msg3 on non-anchor access in idle mode, RAN1 does not further discuss any other carrier than that UE monitors to receive Msg2 (i.e. RAR) in Rel-16.

Quality report for non-anchor access in Msg3 (CONNECTED mode)

In this meeting, some companies further discuss channel quality report in Msg3 on non-anchor access in connected mode. Ericsson propose that quality reporting in Msg3 in connected mode is either not supported or treated with low priority in Rel-16. Huawei propose that Channel quality report in Msg3 in connected mode is not supported. ZTE propose that only USS carrier is report in case there is Msg3 in connected mode and it is enabled by NPDCCH order or reusing SIB2 based enable mechanism in Idle mode. Nokia propose that the measurement is made on unicast carrier and triggered by NPDCCH order. MTK propose that the measurement is made on unicast carrier and triggered by NPDCCH order in Contention-Free Random Access (CFRA) during UL timing alignment procedure. Qualcomm propose that UE measures RAR carrier.

  - Yes: ZTE, Nokia, MTK, Qulacomm?

  - No: Ericsson, Huawei/HiSilicon

Feature lead understand this was not specified in Rel-14 and RAN2’s agreement says reuse existing Msg3 signaling, thus it would be necessary RAN1 further discuss around this to achieve common understanding.

Recommended Proposal 2: For channel quality report in Msg3 on non-anchor access in connected mode, RAN1 further discuss whether it is supported in Rel-16.

- If supported, the triggering mechanism is down-selected between:

  - PDCCH order based triggering

  - Reusing SIB2 based enable/triggering mechanism in RRC_IDLE mode

- If supported, the measurement carrier used in Msg3 report is down-selected between:

  - RAR carrier

  - Unicast carrier

Quality report in connected mode (other than Msg3)

Feature lead noticed that for connected mode, RAN2 has made the following agreements for NB-IoT during RAN2 105bis [10].

Agreements: - The DL channel quality reported in connected mode corresponds to the carrier used for the unicast transmission (i.e. configured by MSG4 or by a subsequent reconfiguration procedure). - eNB enables the reporting of the DL Channel quality in connected mode. - FFS whether dedicated or broadcast signaling is used for enabling - Periodic reporting or on-demand reporting are not supported. - DL channel quality reported in connected mode is optional for the UE - Working assumption that the same code points as Msg3 reporting is used. - FFS whether MAC or RRC is used for reporting

In this RAN1 meeting, 6 of 7 contributions discuss the channel quality report in connected mode other than Msg3.

Ericsson proposes to consider quality reporting in connected mode also after Msg5.

Huawei proposes that DL quality report in connected mode is enabled via Msg4 or RRC reconfiguration message.

ZTE propose that channel quality report in connected mode is triggered by PDCCH order or MAC CE.

Nokia propose that USS carrier is used and mentioned no new physical layer triggering mechanism is needed.

Samsung propose channel quality report in connected mode is triggered by DCI or MAC CE or RAR.

Qualcomm propose to not introduce PHY triggering mechanism.

  - No/not mention PHY trigger: Nokia, Qualcomm, Ericsson, Huawei/HiSilicon

  - Yes/mention PHY trigger: ZTE, Samsung

It seems at this stage, the possible conclusion should refer to physical layer triggering mechanism discussion for channel quality report in connected mode. Based on the inputs above,

Recommended Proposal 3: For channel quality report in connected mode (other than Msg3), no physical layer triggering scheme is introduced in Rel-16.

Others

Ericsson propose that channel quality report in dedicate PUR has benefit, feature lead understand that is may not proper to dicuss PUR related issue in this agenda item.

- References

- RP-190757	WID revision: Additional enhancements for NB-IoT, March 2019.

- R1-1908031	Quality report in Msg3 and connected mode in NB-IoT	Ericsson

- R1-1908090	Support of quality report in Msg3 and connected mode	Huawei, HiSilicon

- R1-1908268	Support of quality report in Msg3 and Connected Mode for NB-IoT	ZTE

- R1-1908304	Support of Quality report in Msg3 on non-anchor carrier	Nokia, Nokia Shanghai Bell

- R1-1908450	Discussion on Msg3 report for non-anchor access	Samsung

- R1-1908748	DL Quality Report on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1908838	Support of quality report in msg3 and connected mode	Qualcomm Incorporated

- R2-1905263, “LS on non-anchor carrier CQI reporting in MSG3”, RAN2, Qualcomm, RAN2#105bis, Xian, China, 8 – 12 April 2019.

- Draft Report of 3GPP TSG RAN2#105bis meeting, Xi'an, China