3GPP TSG RAN WG1 Meeting #98    	            R1-1909601

Prague, Czech, August 26th – 30th, 2019

Source:	Qualcomm Incorporated

Title:	Offline summary on Rel.15 PDCCH maintenance

Agenda Item:	7.1.3

Document for: 	Discussion and Decision

R1-1909385	Correction on CSI-RS based RA procedure triggered by higher layers	ASUSTEK COMPUTER (SHANGHAI)

Discuss further offline

Monday offline discussion:

Comment 1: The reference should be 331 (not 212).

Comment 2: Make sure that duplication with MAC spec is avoided.

Conclusion: ASUS to continue working on the draft CR.

R1-1908568	Correction on Type0-PDCCH CSS sets	CATT

Check further offline

Monday offline discussion:

Comment 1: current spec is already clear.

Comment 2: if change is anyway necessary, why not to make it clear

Conclusion: CATT will still work on the CR.

R1-1909136	Correction on NR-DC PDCCH blind decoding capability reporting	Ericsson

Check further offline

Monday offline discussion:

Comment 1: It is unclear which part the “Otherwise” belongs to. This can be resolved by doing “enter (line change)” after “For NR-DC operation, the UE may indicate, through pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, respective maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG.”

Comment 2: It is fine to include the italicize changes in the alignment CR. However, the change on “When a UE is configured” should be stand-alone CR.

Comment 3: Even deleting the sentence may work.

Conclusion:

- Opt.1: Prepare a CR for the following changes. Including the italicize changes in the alignment CR.

- Opt.2: All following changes are in the alignment CR.

For NR-DC operation, the UE may indicate, through pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, respective maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG. If the UE reports pdcch-BlindDetectionCA, - the value range of pdcch-BlindDetectionMCG-UE or of pdcch-BlindDetectionSCG-UE is [1, …, pdcch-BlindDetectionCA-1], and - pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA.

R1-1909405	Draft  CR on monitoring occasion of SS#0	ZTE

Check further offline

Monday offline discussion:

Conclusion: following is suggested wording from the offline session.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in Subclause 13, and if the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of - a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or - a random access procedure that is not initiated by a PDCCH order that triggers a non-contention based random access procedure

R1-1909436	Clarifications of overlapping PDCCH monitoring occasions in 38.213	Huawei, HiSilicon

Check further offline

Monday offline discussion:

Comment 1: 213 editor interprets 1.

Comment 2: the issue should be avoidable by gNB implementation.

Comment 3: better to clarify that this case will no longer be clarified.

Conclusion: continue discussion.

R1-1909444	Correction on searchspaceSIB1 in TS38.213	Huawei, HiSilicon

Check further offline

Monday offline discussion:

Comment 1: delete all “maximum”.

Comment 2: Table 10.1-1 is not for specific CSS set.

Comment 3: no different UE behaviour by the change. Not necessary.

Conclusion: continue discussion.