3GPP TSG RAN WG1 Meeting #98	R1-1908122

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.6.2

Source:	Ericsson

Title:	UCI Enhancements for NR URLLC

Document for:	Discussion, Decision

# 1	Introduction

In RAN1#97 the following was agreed regarding multiple HARQ ACK transmission in a slot:

Agreements: For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH. - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association. - FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth. - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot. Agreements: For sub-slot-based HARQ-ACK feedback procedure, the starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot - For a given sub-slot configuration, a UE can be configured with PUCCH resource set(s) - FFS same or different PUCCH resource sets can be configured for different sub-slots within a slot. Agreements: - When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for following: - FFS: For PUCCH-SpatialRelationInfo - Note: SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are not related to HARQ-ACK feedback. - FFS: For other UCI types, e.g. SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList. - FFS: At least one HARQ-ACK codebook follows R15 PUCCH configuration.

Also, after RAN1#97 the email discussion [97-NR-05] was initiated to discuss the view for resolving intra-UE collision of PUCCH/PUSCH resources corresponding to different traffics.

In this contribution we discuss our view on solutions for enabling multiple PUCCHs transmissions with HARQ-ACK feedback in a slot, as well as handling collisions for intra-UE UL transmissions.

# 2	Discussion

## 2.1	Design principals for multiple HARQ-ACK per slot

Our view on the feature of multiple HARQ-ACK transmission per slot is to enable the functionality based on a design that reuses Rel-15 features as much as possible with simplified modifications when necessary. The design should be easily extendable for use cases that address more services with different delay and reliability requirements.

Few agreements have been made that help to have a simplified approach to resolve the remaining issues for supporting multiple HARQ-ACK codebooks per slot based on the few principals that we propose at the following:

A UE is configured with a set of RRC parameters for HARQ-ACK transmissions in PUCCH-Config applicable to slot based (as in Rel-15) and an additional set of RRC parameters for HARQ-ACK transmissions for PUCCH-Config applicable to sub-slot based.

- The set of RRC parameters include at least PUCCH resource sets (already agreed) and dl-DataTo-UL-ACK (a.k.a. K1).

When a UE receives a PDSCH, a physical layer indication determines whether the sub-slot based RRC parameters for HARQ-ACK transmissions are applicable or slot based ones.

- FFS on the physical layer identification

PDSCHs with sub-slot based procedures are processed separately from the PDSCHs with slot-based procedures to generate the corresponding HARQ codebook.

The above proposals result in a design that the UE can determine for each PDSCH whether the obtained K1 value is with a granularity of sub-slot or slot. Furthermore, separate processing of PDSCHs corresponding to K1 with sub-slot and slot based granularity, simplifies the procedures where the Rel-15 procedures can be used for each of the tracks (slot-based or sub-slot based). This is illustrated in example Figure 1.

Figure 1: An illustrative example for separate processing of URLLC and eMBB PDSCHs.

## 2.1.1	Remaining aspects of PUCCH configurations

- K1 value range

It is claimed that indication based on sub-slot would mean that for a certain feedback delay in absolute time unit, there is a larger range for K1 is needed since the units are based on sub-slot rather than slots.

However, when a gNB schedules a transmission based on sub-sot, it intends to shorten the delay between the DL transmission and the feedback to meet a certain delay requirement.  Hence, the required absolute time of delay would be shorter which implies that the value range of K1 would be large enough to meet such delay requirements even with granularity of sub-slots. It is also argued that the K1 range value should be increased for the purpose of TDD configurations. It is important to note that in such cases, the delay between the DL transmission and the feedback does not meet the delay requirements and it would be irrelevant if the K1 value range is increased or not, since it would not be used by the gNB, as explained above. Hence from our perspective, the current value range of K1 is sufficient even for sub-slot based K1 granularity for delay critical services.

The value range for K1 indication should not be extended.

- Spatial relations and PUCCH resources

It was agreed that all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for following:

  - FFS: For PUCCH-SpatialRelationInfo

  - Note: SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are not related to HARQ-ACK feedback.

- FFS: For other UCI types, e.g. SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList.

- FFS: At least one HARQ-ACK codebook follows R15 PUCCH configuration.

With respect to open issues on other parameters in PUCCH configuration for Rel-16, our view is as the following:

- The spatial relation between SSB/CSI-RS/SRS and PUCCH is not affected whether sub-slot based configuration is used or slot-based. This is a property that is controlled by MAC-CE, and can be even the same for different UL BWPs. Therefore, the same spatial relation can be used for eMBB and URLLC.

- The configuration of PUCCH SR resources are coupled to corresponding logical channels via scheduling. Hence, association with different traffics such as URLLC and eMBB based on LCHs where they can be prioritized.

- The configuration of PUCCH resources for CSI as in Rel-15 can ensure the required reliability. If CSI reporting needs to be updated, triggered based CSI reporting is a more suitable approach.

Therefore, we propose the following:

PUCCH-SpatialRelationInfo is not configured separately for multiple PUCCH HARQ-ACK transmission per slot.

SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are configured as in Rel-15.

Regarding the number of PUCCHs with HARQ ACK transmission in a slot, different services have different requirements in terms of latency and hence HARQ ACK timeline and specifying a too high number for the HARQ ACK can complicate the scheduling unnecessarily. In our view, maximum 3-4 would be a reasonable choice.

Based on the above discussion, we propose the following:

Maximum 4 PUCCH transmissions with HARQ-ACK can be supported per slot.

## 2.1.2	Remaining aspects of HARQ codebooks

- Type I HARQ codebook construction

As discussed earlier, for PDSCHs with sub-slot based indications, Rel-15 procedures can be directly used for generating a HARQ codebook of Type 1 or Type 2 based on its configuration. Type 1 HARQ codebook, is subject to an in-built overhead since the configured values for K1 impact the size of type 1 HARQ codebook while the UE may not receive PDSCH for all the configured K1 values. In this case, the UE is expected to generate NACK for those entries in the Type 1 codebook.  This overhead can be additionally increased when used for URLLC traffic since DL transmissions are targeted to be scheduled such that the corresponding HARQ feedbacks is provided soon enough to meet the URLLC delay requirements. This in turn implies that depending on the range of configured values for K1, there are scenarios that more configured candidate K1 values would not correspond to a PDSCH transmission, but a NACK entry in the corresponding HARQ codebook. An example illustrated in Figure 2. Hence, it is beneficial to reduce overhead on Type 1 HARQ codebook, especially for sub-slot based configurations. It is important to note that solutions that reduce the overhead of Type I HARQ codebook should not result in increasing the overhead in another dynamic signalling.

Figure 2: Illustration of PDSCH groups associated to HARQ-code book for a corresponding sub-slot.

Therefore, we propose the following:

Further study to design and support a mechanism that reduces Type I HARQ codebook overhead for PDSCH with K1 sub-slot based indication without increasing overhead in other related dynamic signaling.

- HARQ codebook types for eMBB and URLLC

Another aspect that needs to be clarified is the HARQ-procedures when a UE receives mix of eMBB and URLLC traffic. As discussed earlier, from our perspective, it is preferred to separate the HARQ codebook for eMBB and URLLC traffic and transmit on different PUCCH resources. That simplifies the design while providing the required quality for these two services. To realize this operation, HARQ codebook type is separately configured for slot-based K1 and sub-slot based K1.

Regarding to the codebook type, adopting the same codebook type, i.e. dynamic or semi-static, is a simpler approach but it is preferable to investigate a bit further before precluding the option of different codebook types for slot-based and sub-slot based K1.

A HARQ codebook can correspond to PDSCHs with slot-based K1 indication or PDSCHs with sub-slot based K1 indication.

The same HARQ codebook type is configured for PDSCHs with slot-based and sub-slot based K1 indication as baseline.

- FFS whether and how configurations of codebooks with different types for slot-based and sub-slot based K1 indications are beneficial

## 2.2	UL control/control and data/control resource collision

In our view, the overlapping between PUCCH/PUSCH resources corresponding to different services (URLLC and eMBB here) should be resolved such that the reliability and delay requirements of the URLLC services are met. Moreover, we prefer approaches that strive to reuse the Rel-15 procedures as much as possible with potentially additional simplifications, if feasible. Hence, the approach that we have considered here for handling the collision is based on the simplified Rel-15 procedures by performing parallel processing and eMBB UCI dropping, if necessary, with the constraint of minimizing the eMBB HARQ-ACK dropping as much as possible.

Based on the above, we propose the following principles to resolve overlapping between PUCCH/PUCCH and PUCCH/PUSCH between services with different priorities.

To resolve collision between PUCCH and/or PUSCH resources for different services, a UE performs the following steps:

- In the first step, follow Rel-15 procedures to resolve overlapping between PUCCH and/or PUSCH resources corresponding to the same service type (a.k.a. same priority), if any.

- In the second step, if a low priority (e.g. eMBB) resource (PUCCH or PUSCH) overlaps with more than one higher priority (e.g. URLLC) PUCCH or PUSCH resource, the low priority (e.g. eMBB) resource is dropped with the corresponding UCI/data.

- In the third step, to resolve overlapping for a pair of PUCCH/PUSCH resources with different priorities, if any, follow the rules e.g. in  Table 1.

It is important to note that after step one, none of the URLLC PUCCH/PUSCH resource overlaps with another PUCCH/PUSCH URLLC resource in a slot. The same applies to eMBB PUCCH/PUSCH resource. Moreover, when step two of the procedure is executed, there is not a resource that overlaps with more than one resource where these two resources correspond to different services (i.e. one corresponds to URLLC and the other one to eMBB, if they overlap). Some examples are shown in Figure 1.

Note that the rules in Table 1 aim to multiplex eMBB HARQ-ACK on PUCCH on the overlapping URLLC resource. However, the multiplexing is performed if the delay and reliability requirements for the URLLC channel is not compromised. To achieve these, some simplified conditions are proposed to be fulfilled as described in detailed description of the procedures in the table.

Moreover, in order to simplify the procedure and avoid an iterative approach, that the rules below ensure that the multiplexing for a pair of overlapping resources should not result in a resource that overlaps with any other PUCCH/PUSCH resource in the slot.

Table 1: Collision handling rules for a pair of PUCCH/PUSCH resources with different services (e.g. URLLC and eMBB). For simplifying description, CSI is counted as eMBB UCI.

|  | eMBB wAN | eMBB CSI/CSI+SR | eMBB SR only | eMBB PUSCH |
| --- | --- | --- | --- | --- |
| URLLC PUCCH (AN) | Rule 1 | Rule 1 | Drop eMBB | Rule 2 |
| URLLC PUCCH (AN+SR) | Rule 1 | Drop eMBB | Drop eMBB | Drop eMBB |
| URLLC PUCCH (SR) | Drop eMBB | Drop eMBB | Drop eMBB | Drop eMBB |
| URLLC PUSCH | Rule 3 | Rule 3 | Drop eMBB | Pending rule (due the ongoing discussions in RAN2). |
| Rule 1: Rel-15 is used as baseline for multiplexing with following simplifications. If multiplexing timeline is not met, drop eMBB UCI and transmit only URLLC UCI. Otherwise, a PUCCH resource set configured for URLLC AN for the corresponding sub-slot URLLC that supports the total number of URLLC UCI and eMBB UCI is used for multiplexing. - If the determined PUCCH resource based on the last DCI for URLLC PDSCH, overlaps with any other PUCCH/PUSCH resource in the slot, multiplexing is not performed and eMBB UCI is dropped and URLLC UCI is transmitted in the URLLC PUCCH. - Otherwise, total UCI (i.e. URLLC UCI eMBB UCI excluding SR) is multiplexed on the new determined URLLC PUCCH resource |  |  |  |  |
| Rule 2: If multiplexing timeline is not met, the UE drops eMBB PUSCH and transmit only URLLC AN. Otherwise, the UE multiplexes URLLC AN on eMBB PUSCH based on Rel-15. Note that Since PUSCH is scheduled after DL grants corresponding to URLLC AN, the DAI, beta and time and frequency resources for PUSCH should be scheduled properly to meet the delay and reliability required for URLLC. |  |  |  |  |
| Rule 3: UCI can be piggy-back on PUSCH if the reliability requirement of the PUSCH is met. For the case that the URLLC PUSCH is scheduled by UL grant, the gNB when schedules the PUSCH is aware of the upcoming overlapping with the UCI PUCCH resource. Hence, if gNB allocates resources to PUSCH such that with eMBB UCI, the reliability of PUSCH for URLLC can be met, multiplexing can be done. Otherwise, it should be avoided. Hence, gNB should indicate to the UE whether to multiplex or not. The indication can be done by using the beta factor, where a zero and non-zero value, indicates to drop or multiplex the eMBB UCI, respectively. |  |  |  |  |

Figure 3: Illustrations of two examples of Step1 and Step 2 collision handling procedures.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE is configured with a set of RRC parameters for HARQ-ACK transmissions in PUCCH-Config applicable to slot based (as in Rel-15) and an additional set of RRC parameters for HARQ-ACK transmissions for PUCCH-Config applicable to sub-slot based.

The set of RRC parameters include at least PUCCH resource sets (already agreed) and dl-DataTo-UL-ACK (a.k.a. K1).

Proposal 2	When a UE receives a PDSCH, a physical layer indication determines whether the sub-slot based RRC parameters for HARQ-ACK transmissions are applicable or slot based ones.

FFS on the physical layer identification

Proposal 3	PDSCHs with sub-slot based procedures are processed separately from the PDSCHs with slot-based procedures to generate the corresponding HARQ codebook.

Proposal 4	The value range for K1 indication should not be extended.

Proposal 5	PUCCH-SpatialRelationInfo is not configured separately for multiple PUCCH HARQ-ACK transmission per slot.

Proposal 6	SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are configured as in Rel-15.

Proposal 7	Maximum 4 PUCCH transmissions with HARQ-ACK can be supported per slot.

Proposal 8	Further study to design and support a mechanism that reduces Type I HARQ codebook overhead for PDSCH with K1 sub-slot based indication without increasing overhead in other related dynamic signaling.

Proposal 9	A  HARQ codebook can correspond to PDSCHs  with slot-based K1 indication or PDSCHs with sub-slot based K1 indication.

Proposal 10	The same HARQ codebook type is configured for PDSCHs with slot-based and sub-slot based K1 indication as baseline.

FFS whether and how  configurations of codebooks with different types for slot-based and sub-slot based K1 indications are beneficial

Proposal 11	To resolve collision between PUCCH and/or PUSCH resources for different services, a UE performs the following steps:

In the first step, follow Rel-15 procedures to resolve overlapping between PUCCH and/or PUSCH resources corresponding to the same service type (a.k.a. same priority), if any.

In the second step, if a low priority (e.g. eMBB) resource (PUCCH or PUSCH) overlaps with more than one higher priority (e.g. URLLC) PUCCH or PUSCH resource, the low priority (e.g. eMBB) resource is dropped with the corresponding UCI/data.

In the third step, to resolve overlapping for a pair of PUCCH/PUSCH resources with different priorities, if any, follow the rules in e.g. Table 1.

# References

RP-190726	New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC)

RP-190728	New WID: Support of NR Industrial Internet of Things (IoT)