3GPP TSG-RAN WG1 Meeting #98	R1-1909617

# Prague, CZ, August 26th – 30th 2019

Agenda item:	6.1.1

Source: 	Qualcomm Incorporated

Title: 	Summary of offline discussion on eMTC CRs

Document for:	Discussion and Decision

# RV cycling in eMTC

The discussion is related to this tdoc:

R1-1908820	RV cycling in eMTC	Qualcomm Incorporated

The following was discussed:

TS 36.211 contains the following text for frequency hopping:

<TS 36.211>

For BL/CE UEs, the PRB resources within the narrowband  for PUSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.10 and 5.3.3.1.11 in [3]. The PUSCH is transmitted with repetitions. The PUSCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUSCH transmission if . For BL/CE UE in CEModeA, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set and the frequency hopping flag in DCI format 6-0A indicates frequency hopping, otherwise frequency hopping is disabled. For BL/CE UE in CEModeB, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set, otherwise frequency hopping is disabled. If frequency hopping is not enabled for PUSCH, the PUSCH repetitions are located at the same PRB resources at the same narrowband .

</TS 36.211>

The proposed CR has the following change:

< change for 36.213>

For BL/CE UEs, the same redundancy version is applied to PUSCH transmitted in a given block of  consecutive subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH. The PUSCH transmission spans  consecutive subframes including non-BL/CE uplink subframes where the PUSCH transmission is postponed. For the  block of consecutive subframes within the set of  subframes, the redundancy version (rvidx) is determined according to Table 7.1.7.1-2 using , where  , and . The  blocks of subframes are

</Change>

The quantity  has the same definition in both, so the “non-BL/CE uplink” should be present in both, or none.

We propose to adopt one of the following:

Proposal 1: Align 36.211 and 36.213 on the definition of :

- Alternative 1: Adopt the CR in R1-1908820.

- Alternative 2: Adopt the following wording (similar for PUSCH):

  - The PDSCH transmission spans  consecutive subframes including non-BL/CE downlink subframes where the PDSCH transmission is postponed. Non-BL/CE downlink subframes include downlink subframes that are signaled to be non-BL/CE subframes, and uplink subframes.

# Larger TBS without configuraton

R1-1908822         Support of larger UL TBS without explicit configuration   Qualcomm Incorporated, Ericsson

The category M1 definition in 36.306 is as follows:

So, the support is based on the UE capability. 36.331 has the following:

<36.331>

ce-PUSCH-NB-MaxTBS

Activation of 2984 bits maximum PUSCH TBS in 1.4 MHz in CE mode A, see TS 36.212 [22] and TS 36.213 [23].

</36.331>

But in none of 36.211 or 36.213 is clarified that the UE does not expect more than 1000 bit TBS when not configured with ce-PUSCH-NB-MaxTBS. The proponent would like to clarify that this CR is a result of interoperability testing (eNB scheduling more than 1000 bits without configuration). We make the following proposal:

Proposal 2: For the issue of scheduling larger TBS without configuration:

- Alternative 1: Adopt the CR in R1-1908822

- Alternative 2: Capture the following in Chairman’s notes:

  - A BL/CE UE of UL category M1 not configured with ce-pusch-nb-maxTbs is not expected to receive a DCI indicating a TBS of more than 1000 bits.