3GPP TSG RAN WG1 Meeting #98 R1-1909632 Prague, CZ, August 26th - 30th, 2019 CR-Form-v15.0. CHANGE REQUEST. 38.213; CR; DRAFT; rev; -; Current version:; 15.6.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on monitoring occasion of SS#0. Source to WG:; ZTE. Source to TSG:; R1. Work item code:; NR_newRAT-Core; Date:; 2019-8-16. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; There was an agreement in RAN1#95 as follows, but the blue part on monitoring occasion of Type0/0A/2-CSS is not captured in the current specification. Agreements: When the CORESET ID field is 0, the TCI state ID field in the MAC-CE indicates a TCI-state in the PDSCH-Config. - For CORESET#0, - A UE follows the indicated TCI state or the QCL-D of the SSB of the active BWP which is selected through the random access procedure with a PRACH transmission not initiated by a PDCCH order that triggers a non-contention based random access procedure, whichever occurs more recently - MAC CE based TCI indication delay for CORESET 0 is the same as MAC CE based TCI indication delay for other CORESETs - A UE is expected to be configured only with TCI state of CSI-RS/TRS which is QCLed with an SSB based on the corresponding QCL type defined in section 5.1.5 in 38.214 for the active BWP. - If different reference signals are configured in the TCI state, they should be QCLed with the same SSB with corresponding QCL type. - Note: this does not require new way of handling QCL chain between the CSI-RS/TRS in the TCI-state and the QCLed SSB. - It is up to NW whether to indicate the TCI-state for the CORESET#0 by the MAC-CE. - If the TCI-state for the CORESET#0 is not indicated by the MAC-CE, the UE shall follow the SSB selected in the most recent random access procedure with a PRACH transmission not initiated by a PDCCH order that triggers a non-contention based random access procedure to receive any PDCCH with any RNTI on the CORESET#0. - For CONNECTED state, if Type0/0A/2-CSS is SS#0, the UE monitors the common search space on the PDCCH monitoring occasions determined based on the association between an SSB index and PDCCH monitoring occasions according to Section 13 of TS38.213 (which is called default association), where; - the SSB is the one QCLed to the CSI-RS/TRS in the TCI-state indicated for the CORESET#0 or is the one selected through the random access procedure with a PRACH transmission not initiated by a PDCCH order that triggers a non-contention based random access procedure, whichever comes the recent. - RAN1 will ask RAN2 about handling of commonControlResourceSet if Option 1 is agreed. - TP for 213 section 5 will be discussed in RLM session. - For component 1 of FG2-4, if a UE reports X active TCI state(s), the UE is expected to be configured/activated with X active QCL assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell. - TCI states applicable to CORESET#0 are (1) up to the first 64 sorted by TCI-state IDs, and (2) which contains CSI-RS sourced by SSB. - QCL assumption for CORESET#0 is updated by q_new after RACH procedure for BFR/RLM. - Note: In RRC connected mode, for CORESET other than #0, MAC-CE indicates a TCI-state in the PDSCH-Config RRC IE with the restriction of the TCI-state indexes configured for the CORESET RRC IE - Note: it is gNB responsibility whether/how to ensure the performance of broadcast PDCCH whose QCL-TypeD is TRS - Note: The active TCI state for the CORESET#0 is counted as 1 in the UE capability signalling Based on the blue part, for CONNECTED state, if Type0/0A/2-CSS is SS#0, UE only needs to monitor on PDCCH occasions which are associated with the selected SSB instead of all SSBs. The selected SSB is either by activated TCI state or through the random access procedure. However, there is no clear description to reflect the blue part agreement in the current specification.. -. Summary of change:; Clarify the monitoring occasion of Type0/0A/2-CSS in SS#0 based on the above agreements.. Consequences if not approved:; It is confusing whether the UE needs to monitor the occasions of SS#0 corresponding to all SSBs or just selected SSB.. Clauses affected:; 10.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: It is expected that gNB/UE vendors implement based on the agreement. So no implementation change is expected. Otherwise, in RRC CONNECTED state, UE may need to monitor PDCCH occasions of Type0/0A/2-CSS in SS#0 associated with all SSBs, then the available PDCCH candidates in USS will be significantly reduced since the priority of decoding PDCCH candidates in CSS is higher than USS if the total number of PDCCH candidates are limited. In addition, high power consumption will be caused.. This CR's revision history:; This is the second version for this CR. The first version is in R1-1909405..

10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, or TPC-SRS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, or CS-RNTI(s).

For a DL BWP, if a UE is not provided searchSpace-SIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1. If the active DL BWP and the initial DL BWP have same SCS and same CP length and the active DL BWP includes all RBs of the CORESET with index 0, or the active DL BWP is the initial DL BWP, the CORESET configured for Type0-PDCCH CSS set has CORESET index 0 and the Type0-PDCCH CSS set has search space set index 0.

For a DL BWP, if the UE is not provided searchSpaceOtherSystemInformation for Type0A-PDCCH CSS set, the UE does not monitor PDCCH for Type0A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH CSS set are given in Table 10.1-1.

For a DL BWP and a Type1-PDCCH CSS set, a UE is provided a configuration for a search space by ra-SearchSpace. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set.

If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in Subclause 13, and if the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of

-	a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or

-	a random access procedure that is not initiated by a PDCCH order that triggers a non-contention based random access procedure

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set based on the search space set associated with the value of searchSpaceID.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB and for corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, QCL-TypeA, and QCL-TypeD properties, when applicable [6, TS 38.214], and if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. A SCS is provided by subCarrierSpacingCommon in MIB.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set does not have same QCL-TypeD properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, or a CS-RNTI,

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpace-SIB1

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | 1 |

< Unchanged parts are omitted >