3GPP TSG-RAN WG1 Meeting # 98	R1-1909633

# Prague, Czech Republic,  August 26th – 30th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR |  | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the resource mapping of PTRS and PDSCH in TS38.211 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In subclause 7.4.1.5.1 of TS 38.211, it is specified that: “for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in subclause 7.4.1.5.3 are not used for PDSCH transmission”. In subclause 7.4.1.5.3, the RE for CSI-RS mapping is defined with the following condition: “the resource element k,lp,μ is within the resource blocks occupied by the CSI-RS resource for which the UE is configured” This restricts PDSCH from mapping on any resource of configured aperiodic or semi-persistent zero-power CSI-RS irrespective of the corresponding triggering state or activation state. However, that is inconsistent with subclause 5.1.4.2 of 38.214 where untriggered aperiodic and de-activated semi-persistent zero-power CSI-RS resources are available for PDSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction on zero-power CSI-RS mapping with respect to resources declared as ‘not available’ for PDSCH in subclause 7.4.1.5.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent resource mapping rule for PTRS on REs of zero-power CSI-RS. Inconsistent resource mapping rule for zero-power CSI-RS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on CSI-RS mapping when the ZP-CSI-RS resources are overlapping with resources declared as ‘not available’ for PDSCH. - If the UE implements this CR and the gNB does not, or the gNB implements this CR and the UE does not, the UE may not be in line with the gNB on determination of ZP-CSI-RS and PDSCH resource mapping. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

##### 7.4.1.5.1	General

Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined

-	for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or by the CSI-RS-Resource-Mobility field in the CSI-RS-ResourceConfigMobility IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3

-	for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission subject to subclause 5.1.4.2 of [6, TS 38.214]. The UE performs the same measurement/reception on channels/signals except PDSCH regardless of whether they collide with ZP CSI-RS or not.

<Unchanged parts are omitted>