3GPP TSG-RAN WG1 Meeting #98	R1-1909648

Prague, Czech Republic, August 26-30, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | xxxx | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DMRS for NPUSCH Format 2 in TDD NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In DMRS sequence generation for NPUSCH Format 2, the sequence index of wm is chosen according to i=07c8ns+i2imod 3 with cinit=NIDNcell, so that wm can be changed in different slots in a cell. However in TDD NB-IoT, in some cases the sequence index of wm cannot be changed since only one slot in a radio frame can be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | ns in the equation i=07c8ns+i2imod 3 is changed in TDD NB-IoT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | wm is fixed in a cell in TDD NB-IoT for 3.75kHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 10.1.4.1.1	Reference signal sequence for

<Unchanged parts are omitted>

The reference signal sequence for NPUSCH format 2 is given by

where is defined in Table 5.5.2.2.1-2 with the sequence index chosen according to i=07c8n'+i∙2imod 3 with . For frame structure type 1, n'=ns . For frame structure type 2,  n'=5nf+nsmod 5 forand n'=20nf+nsmod 5 for .

<Unchanged parts are omitted>