3GPP TSG-RAN WG1 Meeting #97	Tdoc R1-1909654

Reno, USA, May 13th –17th, 2019

Agenda Item:	7.2.11

Source:	Ericsson

Title:	Indoor industrial channel model calibration results

Document for:	Information

# 1	Introduction

The purpose of the present tdoc is to summarize the calibration results for the indoor industrial channel model in the email discussion [97-NR-10] Channel model and calibration.

[97-NR-10]

Email discussion/approval using the tables in R1-1907758 as a starting point for additional modeling of IIIoT and to converge on calibration assumptions until 7/12, followed by calibration results till next meeting – Henrik (Ericsson)

# 2	Calibration assumptions and metrics

The assumptions and metrics for the calibration campaign have been agreed in [2]. In the email discussion following this agreement, some further issues and need for clarifications were identified as follows:

- The factory hall heights were not clearly specified. In the email discussion, it was proposed to use 25 m for sub-scenarios 1 & 3, and 15 m for sub-scenarios 2 & 4.

- It isn’t clear whether metric 3) “CDF of delay and angle spread (ASD, ZSD, ASA, ZSA) according to definition in Annex A.1 of TR 38.901” should be determined for all links or only for serving links. In the email discussion, it was proposed to use only serving links, similar to previous calibration campaigns involving UMa/UMi/InH.

- Metric 4) “CDF of first path excess delay for serving cell” isn’t considered since the statistical model for the absolute delay in NLOS has not yet been agreed

- Cluster scaling factors to be applied in Step 7 are not available for the agreed N=25 clusters. In the email discussion, it was proposed to use the scaling factors for N=20.

# 3	Calibration results

Calibration results have been contributed from three companies: Ericsson, Nokia, and ZTE. These results are in the attached excel file.

A set of comparative CDF plots of the calibration metrics are shown in the Appendix.

# References

RP-182138, SID on Channel Modeling for Indoor Industrial Scenarios, Ericsson, 3GPP TSG-RAN Meeting #81, Gold Coast, Australia, September 10th – 13th 2018.

R1-1907968, Summary of email discussion on indoor industrial channel model and calibration, Ericsson, RAN1#97, Reno, USA, May 13-17, 2019.

# Appendix: CDF plots of calibration metrics