3GPP TSG-RAN WG1 Meeting #98	Tdoc R1-1909655

Prague, Czech Republic, Aug 26th –30th, 2019

Agenda Item:	7.2.11

Source:	Ericsson, NTT DOCOMO, Huawei, CEA-LETI, Fraunhofer IIS, Fraunhofer HHI, ZTE, Sanechips

Title:	Way forward on the Indoor industrial channel model

Document for:	Decision

# 1	Introduction

The present tdoc contains proposals for finalizing the Indoor industrial channel model in RAN1#98.

# 2	Remaining issues

In the email discussion [97-NR-11] [2], additional proposals for addressing remaining issues and model finalization were identified. Based on these proposals and additional suggestions before and during the offline session, a set of revised proposals were treated in the offline session [3].

## 3.1	Path loss

Proposal: Use the following path loss model for the Indoor industrial scenario

- Sub-scenario 1: AB model

- Sub-scenario 2:

  - Option 1: AB model

  - Option 2: CI model

  - Option 3: AB model lower bounded by the path loss in sub-scenario 1

- Sub-scenario 3: CI model

- Sub-scenario 4: AB model

- NLOS path loss lower bounded by LOS path loss in all cases

- The parameters for each scenario are given by the table below

- The applicability range of the path loss models is 1<d3D<600 m

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| Indoor Industrial | LOS | PLLOS=31.84+21.50log10d3D+19.00log10fc | σSF=4.3 | 1≤d3D≤600 m |
|  | NLOS | Sub-scenario 1: PL=33+25.5log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS) | σSF=5.7 |  |
|  |  | Sub-scenario 2 (option 1): PL=18.6+35.7log10(d3D)+20log10(fc) PLNLOS=max⁡(PL,PLLOS) | σSF=7.2 |  |
|  |  | Sub-scenario 2 (option 2): PL=32.4+28log10(d3D)+20log10(fc) PLNLOS=max⁡(PL,PLLOS) | σSF=8.3 |  |
|  |  | Sub-scenario 2 (option 3): PL=18.6+35.7log10(d3D)+20log10(fc) PLNLOS=max⁡(PL,PLLOS,PLsub-scenario 1) | σSF=TBD |  |
|  |  | Sub-scenario 3: PL=32.4+23.0log10(d3D)+20log10(fc) PLNLOS=max⁡(PL,PLLOS) | σSF=5.9 |  |
|  |  | Sub-scenario 4: PL=33.63+21.9log10(d3D)+20log10(fc) PLNLOS=max⁡(PL,PLLOS) | σSF=4.0 |  |

## 3.2		LOS probability

Proposal: fix the following parameters and values:

- dsubsce= 0 for all sub-scenarios

- psubsce = 1 for all sub-scenarios

- correlation distance= dclutter/2 = 5 m (sub-scenarios 1 and 3), 1 m (sub-scenarios 2 and 4).

## 3.3	Fast fading parameters

Proposal: Cross-correlations involving K are set to “N/A” in NLOS

Proposal: The autocorrelation distance of K is set to “N/A” in NLOS

Proposal: To ensure better statistical significance of simulations in the indoor industrial scenario, reduce the LSP correlation distances to 10 m. Also add a note that it is recommended to consider methods to improve the statistical confidence, such as using multiple random seeds in simulations, when the simulation scenario is small.

Proposal: For the case with the number of clusters N = 25, use the following scaling factors for cluster angle generation in step 7:

- CϕNLOS=1.358

- CθNLOS=1.282

Proposal: Use the following DS model:

| LOS | NLOS |
| --- | --- |
| μlgDS=log10(26(V/S)+14)-9.35 | μlgDS=log10(30(V/S)+32)-9.44 |

V = hall volume in m3

S = total surface area of hall in m2 (walls+floor+ceiling)

Proposal: Use the mean value 0.15 and 0.19 as the σlgDS for the LOS and NLOS scenario, respectively.

Proposal: FFS on how to capture the dense multipath as observed in some measurements

- Options could include but are not limited to:

  - Increasing the number of clusters to emulate the dense multipath but keeping fixed 20 rays per cluster according to the modeling framework

  - Keeping the number of cluster but increasing the number of rays and intra-cluster delay spread

  - Modeling the residual PDP with an exponential decay

  - Dividing clusters into sub-clusters

## 3.4	Spatial consistency

Proposal: When using the spatially consistent UT mobility modeling procedure A in IIOT sub-scenarios, the absolute time of arrival model can be used to derive  by  for UT, where is the parameter determined in absolute time of arrival model in NLOS case (referred in RAN1 agreement as  0 = d3D/c + for NLOS).

Proposal: When using the spatially consistent UT mobility modeling procedure A in IIOT sub-scenarios together with the absolute time of arrival model, the update distance should be the minimum of 1m and d3d/10.

Proposal: 3GPP TR38901 spatially-consistent model Procedure A is extended to support dual mobility feature according to the methodology in R1-1908074.

## 3.5	Absolute time of arrival

Proposal: For random  in NLOS use the following:

- Option 2: Select the lognormal distribution for random in ns for all sub-scenarios, with parameters μlgΔτ=1.5 and σlgΔτ=0.4

  - Note: this proposal is based on the analysis of the raw data shared by different companies in [97-NR-11]

- Option 3: The NLOS additional delay can be modelled as τR= max(α d + C + n, 0),

where τR  is the additional delay, α is the slope, d is the Tx/Rx-distance, C is the intercept, n is the random variable following an Exponential distribution µ exp(-µ n), for n>0. The max() function makes sure that the additional delay value is positive.

  - Note: This proposal is based on measurements by Huawei+Fraunhofer IIS

Proposal: Let the excess delay in NLOS, Δτ, be upper bounded by 2L/c0, where L is the largest dimension of the factory hall, L=max(length,width,height), and c0 is the speed of light

Proposal: Do not model any cross-correlation of Δτ for links between one UT and different BS

Proposal: Model Δτ as spatially correlated with an exponential autocorrelation function with correlation distance 6 m for clutter-embedded BS scenarios and 11 m for elevated BS scenarios

## 3.6	Dual mobility

Proposal: The value of p can be decided by the user. A typical value is 0.2.

## 3.7	Embedded devices

Proposal: For embedded devices, an additional loss parameter Lembedded is added to the path loss in step 12. The additional path loss parameter Lembedded to embedded devices is left as a simulation assumption, where it may be formulated e.g. as a value or range, or derived using a methodology similar to the building penetration loss in clause 7.4.3.1

Note: definition of “embedded device” needed

## 3.8	Map-based hybrid model

Proposal: To add necessary new table references in TR38.901 section 8 for map-based hybrid model, where the new tables are defined in TR38.901 section 7 for stochastic-based IIOT channel model.

Proposal: In the map-based hybrid channel model, the absolute time of arrival is derived as  for lRT-th deterministic cluster and + for n-th random cluster, where all parameters mentioned here are already defined in current map-based hybrid model.

## 3.9	Naming

Proposal: Adopt acronym IIOT or InF or Indust_IOT for the indoor industrial sub-scenario, and the following acronyms for the respective sub-scenarios

- Option 1: IIOT1, IIOT2, IIOT3, IIOT4, IIOT5

- Option 2: IIOT-Lo-Em, IIOT-Hi-Em, IIOT-Lo-El, IIOT-Hi-El, IIOT-Hi-Hi

- Option 3: InF1, InF2, InF3, InF4, InF5

- Option 4: InF-SL, InF -DL, InF -SH, InF -DH, InF-HH

- Option 5: Indust_IOT1, Indust_IOT2, Indust_IOT3, Indust_IOT4, Indust_IOT5

## 4.0	Others

Proposal: RAN1 need to send a liaison statement to RAN4 for further clarifying EM interference issue (R1-1909581).

Note: Check with Patrick and Wanshi for views on sending an LS at the end of the SI.

# References

R1-1909334, Addition of indoor industrial channel model, Ericsson, Prague, Czech Republic, Aug 26-30, 2019.

R1-1909528, Summary of email discussion on remaining aspects of the indoor industrial channel model, Ericsson, Prague, Czech Republic, Aug 26-30, 2019.

R1-1909580, Summary of offline discussion on the Indoor industrial channel model, Ericsson, Prague, Czech Republic, Aug 26-30, 2019.