3GPP TSG RAN WG1 Meeting #98																R1-1908344

Prague, Czech Republic, August 26 – 30, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | DRAFT | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Intra-slot frequency hopping for short PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#89, it was agreed to support intra-slot frequency hopping for short PUCCH with 2-symbols: Agreements: - For 2-symbol NR-PUCCH, frequency hopping is supported at least for localized (contiguous) PRB allocation in each symbol - FFS for distributed (non-contiguous) PRB allocation Furthermore, in the UE feature list, for PUCCH format 0 with 2 symbols, frequency hopping is the default mode of operation. This is from UE feature 4-1 “basic UL control channel”: 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” UE feature 4-3 is “PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled”” However, in TS 38.211, it only mentions long PUCCH formats for intra-slot frequency hopping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that intra-slot frequency hopping is supported for short PUCCH formats (PUCCH format 0 and PUCCH format 2). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This CR clarifies the intended behavior for intra-slot frequency hopping for PUCCH formats 0 and 2 with 2 symbols in TS 38.211. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: - This CR clarifies the intended behavior for intra-slot frequency hopping for PUCCH formats 0 and 2 with 2 symbols. - No impact on gNB and UE behaviour |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.3.2.1	General

The physical uplink control channel supports multiple formats as shown in Table 6.3.2.1-1. In case intra-slot frequency hopping is configured for PUCCH formats 1, 3, or 4 according to clause 9.2.1 of [5, TS38.213], the number of symbols in the first hop is given by  where  is the length of the PUCCH transmission in OFDM symbols. In case intra-slot frequency hopping is configured for PUCCH formats 0 or 2 with = 2, according to clause 9.2.1 of [5, TS38.213], the first symbol is in the first hop, and the second symbol is in the second hop.

<Omitted Parts>