3GPP TSG RAN WG1 Meeting #98 R1-1909663 Prague, Czech Republic, August 26th – 30th, 2019 CR-Form-v12.0. CHANGE REQUEST. 38.214; CR; DRAFT; rev; -; Current version:; 15.6.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR for clarification of no PT-RS on PUSCH scheduled by RAR UL grant. Source to WG:; NTT DOCOMO, Inc.. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-08-28. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In the current specification, PT-RS is present on PUSCH if phaseTrackingRS is configured and if RNTI equals MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI. In EN-DC and RRC_CONNECTED mode, UE can be configured phaseTrackingRS in DMRS-UplinkConfig over LTE. In this case, current specification is not clear whether the PT-RS is present or not on PUSCH scheduled by RAR UL grant, because RAR UL grant is not scrambled by any RNTI.. Summary of change:; Clarify that the current specification of RNTI is DCI scrambling RNTI, it means PUSCH scheduled by RAR UL grant has no PTRS because RAR UL grant is not scrambled by any RNTI. Also, CS-RNTI is used for configured grant which is not scheduled by DCI, the case of configured grant is also covered.. Consequences if not approved:; There is ambiguity between UE and gNB whether PT-RS is present on PUSCH scheduled by RAR UL grant.. Clauses affected:; 6.2.3. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR clarifies that the PT-RS is not present on PUSCH scheduled by RAR UL grant. It is expected UEs and networks are implemented in accordance to this CR and therefore no change is required on UE and network implementation.. This CR's revision history:; This is the first version for this CR..

6.2.3	UE PT-RS transmission procedure

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant.