3GPP TSG RAN1 Meeting #98																			R1-1909666

Prague, CZ

August 26th – 30th, 2019

Source: 	Qualcomm

Title:	Additional option for aperiodic CSI-RS triggering latency

Agenda item:	7.2.14

Document for:	Discussion and Decision

# Introduction

In [1], two possible solutions were listed for further discussion for a TEI-16 proposal on the use of larger CSI-RS triggering latency values:

- Solution1: UEs reporting 224 or 336 follow the behavior of UEs reporting one of the values of {14, 28, 48};

- Solution2: 48 is used as the beam switching threshold for UEs reporting 224 or 336;

In this contribution, we describe an alternative preferred solution, Solution 3.

# Proposal

Solution 3:

- UE reports a value from the set: {“14”, “28”, “48”, “224”, “336”, “14-336”, “28-336”, “48-336”}

  - For the existing Rel-15 values, no change to the Rel-15 procedures and requirements

  - For the new Rel-16 values:

    - The second value in the indicated pair of values is the supported minimum CSI-RS triggering offset to be used in the case of aperiodic CSI-RS with configured higher layer parameter repetition set to ‘ON’

    - The first value in the indicated pair of values is the threshold to be used for all other cases and requirements for CSI-RS triggering offset and beam switching delay

# References

[1]	 R1-1909228, “Rel-16 TEI Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336”, vivo, ZTE