3GPP TSG RAN WG1 Meeting #98    	            R1-1909677

Prague, CZ, August 26th – 30th, 2019

Source:	NTT DOCOMO, INC.

Title:	Rel-15 CRs on RNTI scrambling PDSCH/PUSCH

Agenda Item:	7.1.3

Document for: 	Discussion and Decision

Proposal: capture following as conclusion in chairman’s notes:

For a PUSCH scheduled by a DCI format 0_0 or format 0_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, SP-CSI-RNTI, or CS-RNTI, the scrambling initialization of the PUSCH corresponding to these PDCCHs is by C-RNTI, MCS-C-RNTI, TC-RNTI, SP-CSI-RNTI, or CS-RNTI, respectively.

For a PUSCH corresponding to a configured grant, the scrambling initialization of the PUSCH is by CS-RNTI.

For a PDSCH scheduled by a PDCCH with DCI format 1_0 or format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI, the scrambling initialization of the PDSCH corresponding to these PDCCHs is by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI, respectively.

For the SPS PDSCH without receiving a corresponding PDCCH, the scrambling initialization of the PDSCH is by CS-RNTI.