3GPP TSG-RAN WG1 Meeting #98	R1-1909678

# Prague, CZ, August 26th – 30th 2019

Agenda item:	6.1.1

Source: 	Qualcomm Incorporated

Title: 	Summary of offline discussion on eMTC CRs

Document for:	Discussion and Decision

# RV cycling in eMTC

The discussion is related to this tdoc:

R1-1908820	RV cycling in eMTC	Qualcomm Incorporated

The following was discussed:

TS 36.211 contains the following text for frequency hopping:

<TS 36.211>

For BL/CE UEs, the PRB resources within the narrowband  for PUSCH transmission in the first subframe are obtained from the DCI as described in clauses 5.3.3.1.10 and 5.3.3.1.11 in [3]. The PUSCH is transmitted with repetitions. The PUSCH transmission spans  consecutive subframes, including non-BL/CE UL subframes where the UE postpones the PUSCH transmission if . For BL/CE UE in CEModeA, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set and the frequency hopping flag in DCI format 6-0A indicates frequency hopping, otherwise frequency hopping is disabled. For BL/CE UE in CEModeB, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set, otherwise frequency hopping is disabled. If frequency hopping is not enabled for PUSCH, the PUSCH repetitions are located at the same PRB resources at the same narrowband .

</TS 36.211>

The proposed CR has the following change:

< change for 36.213>

For BL/CE UEs, the same redundancy version is applied to PUSCH transmitted in a given block of  consecutive subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH. The PUSCH transmission spans  consecutive subframes including non-BL/CE uplink subframes where the PUSCH transmission is postponed. For the  block of consecutive subframes within the set of  subframes, the redundancy version (rvidx) is determined according to Table 7.1.7.1-2 using , where  , and . The  blocks of subframes are

</Change>

The quantity  has the same definition in both, so the “non-BL/CE uplink” should be present in both, or none.

We propose to adopt one of the following:

Proposal 1: Align 36.211 and 36.213 on the definition of , adopt the CR in R1-1908820.

Discuss whether the following change is needed to TS 36.213:

For BL/CE UEs, the higher layers indicate the set of BL/CE DL subframes according to fdd-DownlinkOrTddSubframeBitmapBR [11]. Subframes that are not BL/CE DL subframes are referred to as non-BL/CE DL subframes.

[…]

For BL/CE UEs, the higher layers indicate the set of BL/CE UL subframes according to fdd-DownlinkOrTddSubframeBitmapBR and fdd-UplinkSubframeBitmapBR [11]. Subframes that are not BL/CE UL subframes are referred to as non-BL/CE UL subframes.