3GPP TSG-RAN WG1 Meeting #98	R1-1909684

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.5.1

Source:	Thales

Title:	Summary#2 on simulation assumptions for NTN

Document for:	Discussion

# Introduction

This document aims to summarize the companies’ views on :

- SLS parameters and assumptions for calibration

- SLS parameters and assumptions for performance evaluation

- LLS parameters and assumptions

- Link budget analysis

The top priority for this meeting is to reach consensus on all the proposal of section 2.1 so the calibration results can be finalized for the next meeting.

The remaining LLS assumptions flagged as FFS must be completed before the end of the meeting so the companies can share LLS results for the next meeting.

The last considerations concerning the link budget analysis should also be addressed during this meeting so it can be finalized for the next meeting.

# Discussion on System Level Simulation assumptions.

## Simulations assumptions for calibration

### Clarifications on wrap-around methodology for NTN

During last meeting, the following achievements were made :

Agreement:
A wrap around mechanism should be considered as a baseline for single satellite simulation for intra-satellite interference modeling based on additional bore-sight beam directions which should be computed based on the methodology captured in Table X.4.

•	FFS: Details of the wrap around mechanism

Agreement:
Details on the wrap around methodology used should be provided by the companies together with their simulation results.

Note: The FFS in the previous agreement on wrap around does not need to be addressed

Agreement:

For single satellite simulation : Baseline: 19-beam layout considering wrap-around mechanism (i.e. 18 beams surrounding the central beam and allocated on 2 distinct “tiers”)

The need to find a common understanding for the wrap around mechanism has been highlighted by several companies.

In [15] (ZTE), three options for the wrap around mechanism has been identified :

- Option-1: Only 19-beams are considered for simulation. For UE associated to each beam, the remaining 18-beams are treated as interfere in case of sharing same frequency band/polarization if frequency reuse factor is larger than 1.

- Option-2 : Additional two tiers beams are considered in the simulation for wrapping around. For UE associated to each beam (green-marked), the remaining 60-beams (grey-marked) are treated as interfere in case of sharing same frequency band/polarization if frequency reuse factor is larger than 1. For the statistic, only the UEs allocated in the inner-19 beams are considered.

- Option-3: Same number of beams as the Option-2 are considered in the simulation. But for the interference calculation, for UE associated to each beam (green-marked as one of inner-19 beams), only the adjacent beams within the two closest tiers (grey-marked) are treated as interfere in case of sharing same frequency band/polarization if frequency reuse factor is larger than 1. For the statistic, only the UE allocated in the inner-19 beams are considered.

| Option-1 | Option-2 | Option-3 |
| --- | --- | --- |

It appears that most companies have adopted one of these options :

| Wrap around approach | Companies |
| --- | --- |
| Option 1 | [3] (Thales) |
| Option 2 | [13] (Ericsson), [14] (ESA), [15] (ZTE), |
| Option 3 | [9] (Panasonic), [4] (Nokia), |
| Others | [2] (Huawei), [10] (Nomor), [8] (Sony), Nokia (19-beam layout considering wrap-around mechanism (i.e. 60 beams surrounding the central beam and allocated on 4 distinct “tiers”)) |

As a consequence, it is proposed to adopt the following option-2 method as wrap-around mechanism.

Offline Proposal : The following wrap around mechanism should be adopted for calibration and captured in TR 38.821:

For FRF = 1, two additional tiers beams are considered in the simulation for wrapping around the 19-beam layout.

For FRF > 1, four additional tiers beams are considered in the simulation for wrapping around the 19-beam layout.

For UE associated to each beam, the remaining beams are treated as interfere in case of sharing same frequency band/polarization.
For the statistic, only the UEs allocated in the inner-19 beams are considered.

### Clarifications on beam layout parameters

It has been identified by several companies [3] (Thales) [4] (Nokia) that the ABS values for Set 2 satellite parameters were not provided in Table 6.1.1-6 of TR 38.821.

It appears the central beam elevation angle options can be simplified for single satellite simulations [2]. Moreover, the number of calibration scenarios is already significant and should not be increased.

Offline Proposal : Update the Table 6.1.1-6 of TR 38.821 V0.7.0 as follow :

Table 6.1.1-6 : Beam layout parameters for single satellite simulation

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | S-band : 2 GHz Ka-band : 20 GHz for DL | S-band : 2 GHz Ka-band : 20 GHz for DL |
| Adjacent beam spacing (ABS) on UV plane | S-band : - Set 1 : ABS = 0.0061 - Set 2 :ABS = 0.0111 Ka-band : - Set 1 : ABS = 0.0027 - Set 2 : ABS = 0.0067 | S-band : - Set 1 : ABS = 0.0668 - Set 2 : ABS = 0.1334 Ka-band : - Set 1 : ABS = 0.0267 - Set 2 : ABS = 0.0667 |
| Satellite location | Any position on the geostationary orbit | Any position on the LEO orbit |
| Central beam center elevation angle target | Baseline : 45 deg | Baseline : 90 deg |
| Central beam bore sight direction coordinates in UV plane | Baseline : (0.107,0) | Baseline : (0,0) |
| Gateway direction coordinates in UV plane | Baseline : Same as central beam bore sight direction coordinates in UV plane Note : Not needed for calibration |  |

### Considerations on multi-satellite simulations :

During the last meeting, the following agreement has been made :

Agreement:

Both multi-satellite and single satellite simulations should be considered for calibration and performance evaluation.

However, only two companies shared their views on the multi-satellite simulation framework since most of the companies have focused their effort on the first stage of calibration for single satellite simulations. Based on [2] inputs (Huawei), defining the multi-satellite simulation framework will not be straightforward and several issues need to be addressed first. As a consequence, finding a common ground on additional parameters and methodology will require a lot of effort.

Given that the calibration for single satellite simulation has not yet converged, the channel model for evaluation has not yet been finalized and that it remains only 2 meetings, it is proposed to reconsider performing calibration for multi-satellite simulations.

As suggested in [7] (Intel), performance evaluation for multi-satellite simulations can still be provided by the companies with the description of the multi-satellite framework they used.

Finally, multi-satellite simulations have been considered necessary by RAN1 since the inter-satellite interference levels may exceed the intra-satellite interference levels. However, one can also consider that it is up to implementation to design a constellation where the inter-satellite interference level remains within an acceptable range w.r.t the intra-satellite interference level. Many solutions exist to address this issue and proposal 13 of [2] (Huawei) already presents some of them. At the end, one can argue that elaborate a suitable constellation design is not within RAN1 scope.

Offline Proposal : RAN1 should renounce performing calibration for multi satellite simulations.

Offline Proposal : Performance evaluation for multi-satellite simulations can be provided by the companies with the description of the multi-satellite framework they used. Considerations on calibration results

Rx antenna gain computation with multiple element terminals

It should be clarified how the received antenna gain calculation with multiple element terminals should be done since some discrepancy on this part has been observed among the first calibration results.

From our understanding, the formula given in section 6.1.3.1 should be used. As a consequence; for S-Band handheld terminal (1, 1, 2) with omni-directional antenna elements, the received antenna gain should be computed as follow :

GR dbi=GR,edBi+10log10NR,a-LpdB=0+3-3=0[dBi]

However, some companies interpret the formula as follow :

GR dbi=GR,edBi+10log10NR,a-LpdB=0+0-3=-3[dBi]

Offline Proposal : For handheld terminal, RAN1 should adopt the following association between antenna port and antenna element :

1 TX with each antenna element associated to one antenna port.

2 RX with each antenna element associated to one antenna port.

Calibration results collection

It seems the results shared by the companies (at least for Coupling Loss) are not far from being aligned.

Proposal : It is proposed to capture the calibration in the attached excel file.

## Simulation assumptions for performance evaluations.

### Considerations on NTN channel model

Several issues have already been identified in the current fast fading channel models in TR 38.811 V15.1.0 [13] [17]. The corresponding draft CR should be shared for review via the RAN1 reflector. Then, the CR should be directly submitted to RAN plenary as the SI was handheld by the plenary.

Proposal : RAN1 to carefully check the correctness of the channel models in TR 38.811 V15.1.0 and identify the necessary updates. The corresponding draft CR should be shared and discussed via the RAN1 reflector before submitting the final CR directly to RAN plenary.

# Discussion on Link Level Simulation assumptions

## Clarification on Link Level calibration ?

It is up to RAN1 to identify which are the NTN specific considerations making LLS calibration needed and to decide whether LLS calibration is needed.

Proposal : RAN1 should decide whether LLS calibration is needed :

Option 1 : LLS calibration is needed due to the following NTN specific considerations :

Option 2 : LLS calibration is not needed

## Considerations on power amplifier nonlinearity :

It has been proposed [11] (Fraunhofer) several times to introduce HPA non-linearity modeling in the LLS assumptions. Some models have already been proposed in [18] (ESA) and extended in [11] (Fraunhofer).

On the other side, some companies have been reluctant to introduce such considerations since it seems to be beyond the scope of RAN1. In RAN1, it is usually assumed that the HPA is operated with sufficient back-off in such a way the HPA modeling is not needed. However, it was not clear whether this assumption is consistent with the satellite parameter sets provided in Table 6.1.1-1 and Table 6.1.2-2 of TR 38.821. Based on the offline discussions, it seems that the EIRP density values provided in these tables include already a back-off from 4 to 5 dB (classical back-off values used in satellite). However, this is still far from the 8 to 10 dB back-off values considered usually for cellular base station. Based on this knowledge, RAN1 should decide.

In these conditions, the consistency between the SLS and LLS performance evaluations would be preserved.

Proposal : RAN1 should decide which option should be adopted concerning the HPA non-linearity modeling:

Option 1 : HPA non-linearity modeling is not considered as baseline for link level simulations since it can be assumed that the satellite HPA is operated with sufficient back-off. The EIRP density values provided in Table 6.1.1-1 and Table 6.1.2-2 of TR 38.821 are assumed consistent with this assumption.

Option 2 : HPA non-linearity modeling is discarded for link level simulations since it can be assumed that the satellite HPA is operated with sufficient back-off. However, the EIRP density values provided in Table 6.1.1-1 and Table 6.1.2-2 of TR 38.821 are not consistent with this assumption and should be revisited .

Option 3 : Link Level simulations neglecting HPA non-linearity under the assumption of sufficient back-off must model the resulting degradation due to signal power loss since the EIRP density values provided in Table 6.1.1-1 and Table 6.1.2-2 of TR 38.821 are given assuming the HPA is operated with only [5] dB of back-off .

Proposal : It is proposed to add the following notes in the last rows of Table 6.1.1-1 and Table 6.1.2-2 of TR 38.821 :

Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite Note 3 : All these satellite parameters must be applied per beam. Note 4 : The EIRP density values must be considered identical for all frequency re-use factor options. Note 5 : The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB.

## Considerations on phase noise modeling :

During the last meeting, all the LLS assumptions concerning phase noise modeling have been flagged as FFS.

The phase noise model discussed here should be representative of the aggregate phase noise generated by the following contributors :

- The transmitter,

- The satellite payload for the transparent scenarios,

- The receiver.

RAN1 shall decide whether S-Band phase noise modeling is optional or not.

Some companies [12] (Fraunhofer) [18] (ESA) have proposed to consider the phase noise profile available in DVB-S2X standard for Ka-Band. This model is already representative of the aggregation of the three contributors. S-band phase noise profile has also been proposed. The model is extracted from the DVB-RCS2 Guidelines. This profile is only representative of the satellite terminal contributions (which is usually quite preponderant w.r.t to the other contributors).

The phase noise models available in TR 38.803 have also been proposed for Ka-band [13] (Ericsson). However, these models are representative of only one contributor. As consequence, the aggregation procedure remains unclear.

Proposal : S-Band phase noise modeling is optional.

Proposal : RAN1 should adopt the phase noise profile provided in Table 6 of R1-1905216 [18] for S-band phase noise modeling.

Proposal : RAN1 should decide which options should be adopted as baseline for Ka-band phase noise modeling.

Option 1 : The phase noise profile provided in Table 7 of R1-1905216 [18].

Option 2 : The example phase noise models in TR 38.803. The aggregation procedure of the phase noise profiles is FFS.

Proposal : It is proposed to update the phase noise raw of Table 6.1.2-1, Table 6.1.2-2 and  Table 6.1.2-3 of TR 38.821 as follow :

| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |
| --- | --- |

## Clarification on LLS parameters for DL synchronization

Concerning the frequency offset assumptions, the Note 1 does not apply for the satellite oscillator contribution. The final frequency offset should be computed as follow :

FO=AUE+DSsat+DSUE×fservice, DL

Where AUE denotes the UE crystal accuracy, DSsat denotes the Doppler shift due to satellite movement, DSUE denotes the Doppler shift due to UE movement, fservice,DL denotes the central frequency used on the DL service link.

It is also proposed to clarify the metrics requirement for DL synchronization evaluation.

Proposal : It is proposed to update Table 6.1.2-1 of TR38.821 as follow :

Table 6.1.2-1 : LLS parameters for DL synchronization evaluation

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| DL RS | SSB |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| UE elevation angle | For GEO (optional) : 10°, For LEO : 30° |  |
| Frequency Offset | - UE crystal accuracy: 10 ppm - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle Note 1 : The final frequency offset is computed as follow : where : denotes the final frequency offset in Hz denotes the UE crystal accuracy in ppm denotes Doppler shift due to satellite movement in ppm denotes the Doppler shift due to UE movement in ppm denotes the carrier frequency used on the service Down Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed. Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1-8] |  |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |
| Metrics | One-shot initial cell detection accuracy of PCID; CDF of timing and frequency residual offset at SNR point corresponding to 90% likelihood for one-shot detection accuracy of cell ID.. Note 4: FAR of PCID detection requirement = 1% |  |

Note: The SNR range to be evaluated should be based on the link budget analysis for each channel

## Clarification on LLS parameters for PRACH performance evaluation

Frequency offset :

During the last meeting, the frequency offset assumptions have been flagged as FFS. RAN1 shall achieve some progress on the subject before the end of this meeting so PRACH performance evaluation can be performed for the next meeting.

It should be clarified whether the frequency offsets resulting from UE oscillator drift should be fully taken into account for PRACH performance evaluation or whether they should be considered (at least partially) compensated.

It should be clarified which values should be considered for the frequency offsets resulting from Doppler shifts for PRACH performance evaluation. It has been proposed in [7] (Intel) to consider the double of the values proposed for DL synchronization evaluation in Table 6.1.1-8 for TR38.821.

Finally, the initial timing offset assumption is still flagged as FFS for PRACH performance evaluation. It should be clarified based on DL synchronization results.

Proposal : It should be clarified whether the frequency offsets resulting from UE oscillator drift should be fully taken into account for PRACH performance evaluation or whether they should be considered (at least partially) compensated :

Option 1 : The frequency offsets resulting from UE oscillator drift should be fully considered for PRACH performance evaluation.

Option 2 : The frequency offsets resulting from UE oscillator drift should be considered partially compensated for PRACH performance evaluation.

Option 3 : The frequency offsets resulting from UE oscillator drift should be considered perfectly compensated for PRACH performance evaluation.

Proposal : It should be clarified which values should be considered for the frequency offsets resulting from Doppler shifts for PRACH performance evaluation.

Option 1 :

Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8

Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle

Option 2 :

Doppler shift in channel due to satellite movement : twice the max. Doppler shift values provided in Table 6.1.1-8

Doppler shift in channel due to UE movement : twice the max. value to be computed based on the UE speed and the elevation angle

Proposal : It is proposed to update the Table 6.1.2-2 for TR38.821 as follow :

Table 6.1.2-2 : LLS parameters for PRACH performance evaluation

| Configurations | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx 2 Rx optional | 1 Rx 2 Rx optional |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle - Residual frequency offset after synchronization: [0.1] ppm Note 1 : The final frequency offset is computed as follow : where : denotes the final frequency offset in Hz denotes the residual frequency offset after synchronization in ppm denotes Doppler shift due to satellite movement in ppm denotes the Doppler shift due to UE movement in ppm denotes the central frequency used on the service Up Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| Initial timing Offset (Note 1) | TBC |  |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate, FAR |  |
| Note 1: Ideal common delay compensation is assumed. |  |  |

## Clarification on LLS parameters for data transmission performance evaluation

The frequency offset and frequency tracking assumptions for data transmission performance evaluation should be completed before the end of the meeting.

Proposal : It is proposed to update the Table 6.1.2-3 for TR38.821 as follow :

Table 6.1.2-3 : LLS parameters for data transmission performance evaluation

| Parameters | S-band |  | Ka-band |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz |  | DL 20GHz |
| Channel coding scheme | NR channel coding |  |  |
| Subcarrier spacing | 15kHz, 30kHz |  | 60kHz, 120kHz |
| Modulation order, Coding rate | To be reported by the companies |  |  |
| Channel estimation | Realistic estimation |  |  |
| Frequency offset | Residual Frequency error after DL synchronization: [0.1] ppm assuming UL pre-compensation |  |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1-8] |  |  |
| Frequency tracking | FFS |  |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |  |
| Channel model | For GEO (optional): Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO: Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |  |
| TRP antenna configuration | 1Tx |  | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |  | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise Model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |  |
| Metrics | BLER, Throughput |  |  |

# Discussion on Link budgets analysis

## Clarification on the link budget analysis purpose

| Companies | Comments / Proposals |
| --- | --- |

RAN1 shall clarify the link budget analysis purpose. From our understanding, the initial intent was to evaluate the lowest CNIR values that can be experienced in NTN to help adjusting the CINR range that should be considered when running link level simulations. If it is still the case, the following clarification should be considered :

- The 1st or 10th percentile CIR values coming from calibration results should be considered.

- The worst case scintillation loss should be considered :

  - Ionospheric loss : S_4 = 0.6 (strong regime)

  - Tropopheric loss : values coming from Table 6.6.6.2.1-1 of TR38.811

- The EIRP density and G/T values provided in Table 6.1.1-1 and Table 6.1.1-2 should be down scaled of 3 dB to be representative of the case where the UE is located at the edges of the beam.

However, if the final intent of budget analysis is to provide mean achievable CINR values in NTN, the configuration parameters should be revisited (e.g. the elevation angles).

## Considerations on uplink transmission bandwidths

It has been proposed [13] (Ericsson) to assume that handheld UE transmits with 180 kHz bandwidth in S band (i.e. 1 PRB with 15 kHz subcarrier spacing).

However, for PRACH transmission the handheld device shall transmit over 6 resource blocks.

Proposal : It is proposed to update the Table 6.1.3.2-1 of TR38.821 as follow :

Table 6.1.3.2-1 Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| System bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Channel bandwidth | DL : system bandwidth/ frequency reuse factor UL: - UL in S band (handheld UE): - 1,08 MHz for PRACH transmission - 180 kHz otherwise - Others: system bandwidth/ frequency reuse factor Note: The UL bandwidth may be challenge. |
| Satellite altitude | 600km, 1200km, 35786km |
| Elevation angle | 30 (LEO), 10 (GEO) |
| Atmospheric loss | Equation (6.6-8) in TR 38.811 |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Scintillation loss | Section 6.6.6 in TR 38.811 - Ionospheric loss : S_4 = TBC - Tropospheric loss : Table 6.6.6.2.1-1 of TR38.811 |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1, 2, 3 |
| [10th] percentileCIR within a satellite beam | Based on system-level calibration results |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable if frequency reuse factor = 2 is considered. |
| Terminal type | - Ka band: VSAT, (M, N, P) = (4, 8, 2) - S band: (M, N, P) = (1,1,2) |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Terminal RF parameters | Table 6.1.1-3 |
| Satellite RF parameters | Set-1 in Table 6.1.1-1 and Set-2 in Table 6.1.1-2 Note 1 : The EIRP density and G/T values must be downscaled of 3 dB |
| Polarization loss | - 3dB per linear polarization receive antenna - 0dB for circular polarization antenna receiver |
| Outcome | CNIR |

Proposal : It is proposed to update section 6.1.3.1 of TR38.821 as follow :

======================== Start of TP for TR 38.821 =============================

============================== Text Omitted ================================

#### 6.1.3.1		Link Budget Calculation

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as follows

|  | CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB] | (6.1.3.1-1) |
| --- | --- | --- |

The formula for CNR calculation is

|  | CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ] | (6.1.3.1-2) |
| --- | --- | --- |

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz,  PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLSM is shadowing margin, PLSL[dB] is scintillation loss, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth.

============================== Text Omitted ================================

========================= End of TP for TR 38.821 =============================

## Considerations on CIR values:

The CIR values that shall be used for the link budget analysis should be extracted from the calibration results. However, the target elevation angles considered for the link budget analysis and the calibration evaluation are not aligned.

Proposal : RAN1 shall decide which options should be adopted to obtain the CIR values to be used for link budget analysis.

Option 1 : Use the CIR values extracted from the calibration results since the CIR values can be considered unchanged by the elevation angle assumptions.

Option 2 : Compute additional calibration results with the elevation angle assumptions considered for link budget analysis.

If option 2 is adopted, the procedure to carry out calibration for GEO scenarios with a 10 degree elevation angle should be clarified since in these conditions a fair amount of the 61 beams will be pointed directly to outer space.

Traditionally, for calibration, the results obtained for DL are sufficient to align the results. However, since UL CIR values are needed for the link budget analysis, the calibration should be extended to UL transmissions. To do so, the channel bandwidth assumptions provided in Table 6.1.3.2-1 should be used.

Proposal : The calibration should be extended to UL transmissions so the CIR results can be used for link budget analysis. The channel bandwidth assumptions provided in Table 6.1.3.2-1 should be used.

## Link budget results

Proposal : The following table should be integrated in TR38.821 to capture the link budget results.

Table 6.1.3.3-1 Link budget results

| Study case | Case 1 |  | Case 2 |  | Case … |  | Case N |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission mode | DL | UL | DL | UL | DL | UL | DL | UL |
| TX: EIRP/spot/BW [dBm] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| RX: G/T [dB/T] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Bandwidth [Hz] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Free space path loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Atmospheric loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Shadow fading margin [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Scintillation Loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Polarization loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Additional losses [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| CNR [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| CIR [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| CINR [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |

Proposal : It is proposed to capture the link budget results in the attached excel file.

# Conclusion

The proposal addressed during offline discussions will be reported below :

Offline Proposal : The following wrap around mechanism should be adopted for calibration and captured in TR 38.821:

For FRF = 1, two additional tiers beams are considered in the simulation for wrapping around the 19-beam layout.
For FRF > 1, four additional tiers beams are considered in the simulation for wrapping around the 19-beam layout.
For UE associated to each beam, the remaining beams are treated as interfere in case of sharing same frequency band/polarization.

For the statistic, only the UEs allocated in the inner-19 beams are considered.

Offline Proposal : It is proposed to update the Table 6.1.1-6 of TR 38.821 V0.7.0 as follow :

Table 6.1.1-6 : Beam layout parameters for single satellite simulation

| Scenario | Scenario A | Scenario C2/D2 |
| --- | --- | --- |
| Carrier frequency | S-band : 2 GHz Ka-band : 20 GHz for DL | S-band : 2 GHz Ka-band : 20 GHz for DL |
| Adjacent beam spacing (ABS) on UV plane | S-band : - Set 1 : ABS = 0.0061 - Set 2 :ABS = 0.0111 Ka-band : - Set 1 : ABS = 0.0027 - Set 2 : ABS = 0.0067 | S-band : - Set 1 : ABS = 0.0668 - Set 2 : ABS = 0.1334 Ka-band : - Set 1 : ABS = 0.0267 - Set 2 : ABS = 0.0667 |
| Satellite location | Any position on the geostationary orbit | Any position on the LEO orbit |
| Central beam center elevation angle target | Baseline : 45 deg | Baseline : 90 deg |
| Central beam bore sight direction coordinates in UV plane | Baseline : (0.107,0) | Baseline : (0,0) |
| Gateway direction coordinates in UV plane | Baseline : Same as central beam bore sight direction coordinates in UV plane Note : Not needed for calibration |  |

Offline Proposal : RAN1 should renounce performing calibration for multi satellite simulations.

Offline Proposal : Performance evaluation for multi-satellite simulations can be provided by the companies with the description of the multi-satellite framework they used. Considerations on calibration results

Offline Proposal : For handheld terminal, RAN1 should adopt the following association between antenna port and antenna element :

- 1 TX with each antenna element associated to one antenna port.

- 2 RX with each antenna element associated to one antenna port.

Offline Proposal : LLS calibration is not needed

Offline Proposal : HPA non-linearity modeling is not considered as baseline for link level simulations since it can be assumed that the satellite HPA is operated with sufficient back-off. The EIRP density values provided in Table 6.1.1-1 and Table 6.1.2-2 of TR 38.821 are assumed consistent with this assumption.

Offline Proposal : It is proposed to complete the notes in the last rows of Table 6.1.1-1 and Table 6.1.2-2 of TR 38.821 :

Note 1: This value is equivalent to the antenna diameter to be used in Sec. 6.4.1 of TR 38.811. Note 2: This beam size refers to the Nadir pointing of the satellite Note 3 : All these satellite parameters must be applied per beam. Note 4 : The EIRP density values must be considered identical for all frequency re-use factor options. Note 5 : The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB.

Offline Proposal : It is proposed to update Table 6.1.2-1 of TR38.821 as follow :

Table 6.1.2-1 : LLS parameters for DL synchronization evaluation

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| DL RS | SSB |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| UE elevation angle | For GEO (optional) : 10°, For LEO : 30° |  |
| Frequency Offset | - UE crystal accuracy: 10 ppm - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle Note 1 : The final frequency offset is computed as follow : where : denotes the final frequency offset in Hz denotes the UE crystal accuracy in ppm denotes Doppler shift due to satellite movement in ppm denotes the Doppler shift due to UE movement in ppm denotes the carrier frequency used on the service Down Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed. Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1-8] |  |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |
| Metrics | One-shot initial cell detection accuracy of PCID; CDF of timing and frequency residual offset at SNR point corresponding to 90% likelihood for one-shot detection accuracy of cell ID.. Note 4: FAR of PCID detection requirement = 1% |  |

Note: The SNR range to be evaluated should be based on the link budget analysis for each channel

Offline Proposal : It is proposed to update the Table 6.1.2-2 for TR38.821 as follow :

Table 6.1.2-2 : LLS parameters for PRACH performance evaluation

| Configurations | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx 2 Rx optional | 1 Rx 2 Rx optional |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Frequency Offset | - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle - Residual frequency offset after synchronization: [0.1] ppm Note 1 : The final frequency offset is computed as follow : where : denotes the final frequency offset in Hz denotes the residual frequency offset after synchronization in ppm denotes Doppler shift due to satellite movement in ppm denotes the Doppler shift due to UE movement in ppm denotes the central frequency used on the service Up Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| Initial timing Offset (Note 1) | TBC |  |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate, FAR |  |
| Note 1: Ideal common delay compensation is assumed. |  |  |

Offline Proposal : It is proposed to update the Table 6.1.2-3 for TR38.821 as follow :

Table 6.1.2-3 : LLS parameters for data transmission performance evaluation

| Parameters | S-band |  | Ka-band |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz |  | DL 20GHz |
| Channel coding scheme | NR channel coding |  |  |
| Subcarrier spacing | 15kHz, 30kHz |  | 60kHz, 120kHz |
| Modulation order, Coding rate | To be reported by the companies |  |  |
| Channel estimation | Realistic estimation |  |  |
| Frequency offset | Residual Frequency error after DL synchronization: [0.1] ppm assuming UL pre-compensation |  |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1-8] |  |  |
| Frequency tracking | FFS |  |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |  |
| Channel model | For GEO (optional): Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO: Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |  |
| TRP antenna configuration | 1Tx |  | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |  | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise Model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |  |
| Metrics | BLER, Throughput |  |  |

# Reference

| [1] | R1-1908013 | On beam layout definition and system level performance for NR-NTN, MediaTek Inc |
| --- | --- | --- |
| [2] | R1-1908047 | Discussion on performance evaluation for NTN, Huawei, HiSilicon, CAICT |
| [3] | R1-1908227 | Calibration considerations and results for system level simulations, THALES |
| [4] | R1-1908249 | Considerations on Performance Evaluation in NTN, Nokia, Nokia Shanghai Bell |
| [5] | R1-1908486 | Preliminary Link-Level and System-Level Evaluations, Samsung |
| [6] | R1-1908589 | Initial link level results for NTN, CATT, CAICT |
| [7] | R1-1908641 | Considerations on performance evaluation for NTN, Intel Corporation |
| [8] | R1-1908773 | Calibration results of system level simulation, Sony |
| [9] | R1-1908816 | System level evaluation and link budget for NTN, Panasonic Corporation |
| [10] | R1-1908993 | Preliminary Calibration Results for NTN System-Level Evaluations, Nomor Research GmbH |
| [11] | R1-1908996 | On Downlink Performance in NTN, Fraunhofer IIS, Fraunhofer HHI |
| [12] | R1-1908999 | NTN LLS Calibration Assumptions, Fraunhofer IIS, Fraunhofer HHI |
| [13] | R1-1909110 | On NTN link level and system level evaluations, Ericsson |
| [14] | R1-1909372 | NTN link budget calibration: downlink numerical results, ESA |
| [15] | R1-1909398 | Discussion on the simulation for NTN, ZTE |
| [16] | R1-1908228 | Link budget analysis, THALES |
| [17] | R1-1908995 | Corrections for TR38.811 Section 6.7.2 Non-Terrestrial Networks channel models, Nomor Research GmbH |
| [18] | R1-1905216 | Key satellite parameters and simulation assumptions for NTN, ESA |

# APPENDIX

List of scenarios :

| Companies | Comments / Proposals |
| --- | --- |
| Huawei [2] | Proposal 4: Adopt simulation configurations in Table 2.3 as the start point for single satellite SLS. |
| ZTE [15] | In order to evaluate different satellite configuration for commercial usage, two sets of beam configuration are agreed in last meeting. |

Satellite parameters definition ambiguity

| Companies | Comments / Proposals |
| --- | --- |
| Ericsson [13] | Proposal 4 RAN1 to discuss and agree on the numbers of transmit and receive beams simultaneously generated per satellite. Proposal 5 RAN1 to clarify whether the two sets of satellite parameters are applied per satellite or per beam. Proposal 6 For a given number of transmit/receive beams per satellite, RAN1 to discuss how to apply the satellite parameters to each beam (if they are per-satellite parameters). |
| Huawei [2] | Observation 9: It is unclear whether the value of satellite EIRP density changes or not when different FRF is used. Proposal 5: Adopt the same total beam power method when different FRF is used |

Satellite beam diameter

| Companies | Comments / Proposals |
| --- | --- |
| Nokia [4] | Proposal 3: Update the satellite beam diameter at Nadir point (km) of Table 6.1.1-1 and Table 6.1.1-2 in TR 38.821. Observation 1: If using same θ_3dB for satellite beam width configuration, then the satellite-edge beam diameter is more than three times of its beam diameter at Nadir point, and moreover is larger than the maximal beam diameter given in Table 4.6-1 of TR 38.811. Proposal 4: The satellite-edge beam diameter in NTN should be controlled within the reasonable range to provide a stable communication service. Proposal 12: The maximal beam diameter at the satellite edge should be captured into TR 38.821 and used for the derivation of maximal differential delay supported in this SI. |

UE characteristics :

| Companies | Comments / Proposals |
| --- | --- |
| Panasonic [9] | Proposal 1: Reconsider the transmitter and receiver model for terminal type “other” because G/T and EIRP are too low compared to a commercial antenna system. |
| CATT [6] | Proposal 2: Revisit the parameter setting for phase-array based UE. |

Simulation are and UE distributions

| Companies | Comments / Proposals |
| --- | --- |
| Ericsson [13] | Proposal 10 The simulation area of each beam is the Voronoi cell associated with the corresponding beam center. Proposal 11 In NTN system level simulation, a total of X*N UEs are uniformly distributed in the simulation area, where X is the average number of UEs per beam and N is the number of beams. |
| Thales [3] | Proposal 3 The UE dropping procedure inside beam should consist in randomly draw the UE location such that the UE’s long-lat coordinates are included inside the beam HPBW footprint. |
| ZTE [15] | Proposal 2: Uniform UE distribution on earth surface should be considered in simulation. |

Scintillation loss :

| Companies | Comments / Proposals |
| --- | --- |
| ESA [14] | Proposal 3: For the calibration purpose, the ionospheric scintillations shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |
| Huawei [2] | Note that S_4 = 0.7 is used in all simulation configurations. |
| Thales [3] | Proposal 7 The ionospheric scintillation loss should be discarded for calibration. |
| Panasonic [9] | No scintillation loss is assumed |
| Nomor [10] | Ionospheric scintillation loss : Neglected (assume UE latitude between ±20° and ±60°) |
| Sony [8] | The effect of ionospheric scintillation is PLS = 0 because the middle latitudes (between ±20° and ±60°) is supposed in this contribution. |
| MediaTek [1] | Ionospheric scintillation: Weak, S4 = 0.3 |
| Nokia [4] | The parameter S4 in scintillation pathloss is set to 0.2. |

Wrap-around :

| Companies | Comments / Proposals |
| --- | --- |
| Ericsson [13] | Observation 2 Different wrap-around methods will give quite different statistics in terms of geometry [5], making it difficult (if not impossible) to calibrate system level simulators among companies and compare results from different sources. Proposal 8 RAN1 to assume an NTN system simulation setup without wrap-around: simulate 61 beams and collect statistics from the inner 19 beams. |
| ESA [14] | Proposal 1: The wrap-around methodology shall be agreed to reduce the discrepancy among all calibration results. Proposal 2: It is recommended to generate 61 beams to ensure that all grid-points in the 19 beams have the same number of interferers (e.g., no border effects). |
| Huawei [2] | Proposal 1: For wrap around 4-tier beams (i.e., 108 beams) are added outside the initial 19-beam layout in the UV plane for single satellite case. Observation 7: The proposed NTN-specific wrap around method can guarantee 2-tier interferences for different FRF. Proposal 2: The numbering scheme in NTN-specific wrap around method needs further study. |
| Thales [3] | Proposal 4 RAN WG1 should agree on a simple wrap-around methodology to reduce the disparity between calibration results. The approach described below can considered as a starting point if considered sufficiently explicit : The calibration is performed inside the 19 beams composing the 19-beam layout. When computing geometry metrics, only the 19 beams should be considered. Note that this approach leads to an underestimation of the interference level inside the outer beams. |
| ZTE [15] | Proposal 1: Wrapping around method with following details can be considered: • Additional 2-tier beams for wrapping around • All beams except for the serving as interfere • UE within inner-19 beams are considered for statistic |
| Panasonic [9] | 19-beam layout (i.e. 2 tiers) are used to obtain the geometry distribution. For each observed beam, interference from beams within 2 tiers (i.e. 18 beams) are assumed. (In total 61 beams including interfering beams are deployed in the simulation) |
| Nomor [10] | Wrap around method: Geographical distance based wrapping |
| Sony [8] | We use the geological wrap around method for the simulation in this contribution. In the geological wrap around method, minimum distance between UEs and all beam centre candidates is calculated based on 2D distance on UV plane as shown in Fig.1. |
| Nokia [4] | Proposal 1: The central beam in wrap-around is the beam where the considered UE is geographically located, and the interference statistic of this UE is based on its central beam and 18 beams surrounding this central beam. Based on the calibration results, it is better to use more beams for interference statistic due to stronger inter-beam interference in NTN. If considering 2-tier outer beams of inner 19 beams for interference statistics, then only one beam has uniformly surrounded 60 beams for interference statistics. Therefore, we would propose to consider 4-tier outer beams of central beam for interference statistics, i.e. (i.e. 60 beams surrounding the central beam and allocated on 4 distinct “tiers”). |

Clarifications on beam layout parameters

| Companies | Comments / Proposals |
| --- | --- |
| Thales [3] | Proposal 1 An additional set of ABS values should be captured in Table 6.1.1-6 of [2] corresponding to Set 2 satellite parameters. |
| Nokia [4] | Proposal 2: Add ABS values of Set-2 satellite parameters for GEO and LEO, and update ABS value of Set-1 satellite parameters for LEO, in Table 6.1.1-6 of TR 38.821. |

Multi-satellite simulations :

| Companies | Comments / Proposals |
| --- | --- |
| Huawei [2] | Proposal 7: To simplify the constellation design, the constellations should be walker star constellation and the inclination should be set to 87.5 degree. Proposal 8: Reference constellation defined in Table 3.2 can be considered for Set-1 satellite parameters in multiple-satellite SLS. Proposal 9: Reference constellation defined in Table 3.3 can be considered for Set-2 satellite parameters in multiple-satellite SLS. Proposal 10: A subset of satellites (e.g., 3 or 7 satellites) in the reference constellations are sufficient for multiple-satellite system level simulation. Proposal 11: For multi-satellite case calibration, the target satellite should be located at (earth radius+satellite orbit, 0, 0) in ECEF coordinate. Proposal 12: For a target satellite, the beam layout generation method should be the same to single satellite case. The beam layout of other satellites can be replicated and rotated from the target satellite. Proposal 13: New beam layout generation mechanism should be studied to solve the beam overlapping problem, such as turning off some beams or changing the HPBW of edge beams. Proposal 14: FRF=1 for intra-satellite beams and FRF=3 or 2 for inter-satellite beams should not be considered for multi-satellite cases. Proposal 15: FRF configuration should be the same for each satellite in multi-satellite simulation, e.g., 1, 3 and 2 (polarization re-use is enabled). Proposal 16: Parameters listed in Table A.1 and Table A.2 should be used for simulation platform calibration. |
| Intel [7] | Observation 1: Interference from different satellites is suppressed by directive UE antenna in Ka Band Proposal 1: De- prioritize multi-satellite simulations in Ka-band Proposal 2: Parameters of multi-satellite deployment used for multi-satellite simulations should be provided by the companies together with their simulation results |
| Nokia | As agreed in RAN1#97 meeting, both multi-satellite and single satellite simulations should be considered for calibration and performance evaluation. However, the satellite parameters and beam deployment agreed in RAN1#97 meeting are mainly for single-satellite evaluation, and so that no calibration result for multi-satellite evaluation are submitted to RAN1#98 meeting and consequently less contributions for mutli-satellite evlauation. Therefore, we would like to propose to discuss the simulation assumptions for multi-satellite evaluation. |

NTN channel model :

| Companies | Comments / Proposals |
| --- | --- |
| Ericsson [13] | Observation 3 The NTN fast fading channel models in TR 38.811 V15.1.0 may lead to non-positive definite correlation matrices for large scale parameters. Proposal 9 RAN1 to carefully check the correctness of the channel models in TR 38.811 V15.1.0 before adopting them in Rel-16 NTN evaluations. |
| Nomor [17] | The cross-correlation parameter values of Table 6.7.2-4a, Table 6.7.2-4b, Table 6.7.2-8a, Table 6.7.2-8b are set to zero. |

Link level calibration:

| Companies | Comments / Proposals |
| --- | --- |
| Fraunhofer [12] | Proposal 1: RAN1 shall decide whether LLS calibration is needed or not. Proposal 2: RAN1 shall use the LLS assumptions for calibration given in Table X.13. Proposal 3: RAN1 shall decide whether specific calibration for DL synchronization performance and PRACH performance are needed. |

Power amplifier nonlinearity:

| Companies | Comments / Proposals |
| --- | --- |
| Fraunhofer [11] | Proposal 1: Link-level simulations considering satellite HPA non-linearity should model the ICI between multiple allocations in the same downlink beam, even under AWGN channel or ideal channel knowledge assumptions. Proposal 2: Input value range of the HPA AM/AM and AM/PM characteristics specified for NR NTN downlink simulations shall cover the high dynamic range of the OFDM waveforms: recommended range is from -30 dB to 10 dB. Proposal 3: Option for relaxing the EVM requirements for NR NTN scenarios in order to optimize BLER and throughput performance should be FFS. Proposal 4: Appropriate HPA non-linearity modeling should be included in the link- and system-level simulation assumptions for NR NTN downlink. Proposal 5: Simulations neglecting HPA non-linearity for NR NTN downlink under the assumption of sufficient back-off must model the resulting degradation in BLER and throughput due to signal power loss. |

Phase noise :

| Companies | Comments / Proposals |
| --- | --- |
| Ericsson [13] | Observation 1 PN model heavily depends on the assumptions on the VCO and PLL performance and limitations for mm-wave frequencies. The assumptions can be quite different for different technologies. Proposal 3 The example phase noise models in TR 38.803 are used as baseline models for NTN studies of the Ka band. In the S-band PN modeling is optional. |

LLS parameters for DL synchronization

| Companies | Comments / Proposals |
| --- | --- |
| Samsung [5] | The UE crystal accuracy is one of the elements contributing to the frequency offset in initial cell search, and its value should be random from a detector perspective. It should be clarified that the value should be obeying a uniform distribution with maximum value as specified in current TR, such that there is no confusion on its randomness, and also no confusion on its generation in the evaluation. Moreover, the cell ID detection performance from joint PSS and SSS detection should also be reported, in addition to the metric of CDF of timing and frequency residual offset as in current TR. Actually, the cell ID detection performance is the key metric of concern in NR Rel-15. |

LLS parameters for PRACH performance evaluation

| Companies | Comments / Proposals |
| --- | --- |
| Intel [7] | Observation 2: The maximum frequency shift for UL transmission is equal to double Doppler shift Proposal 3: The following table captures values of maximum frequency shift for UL transmission considered for link level simulations Max frequency shift for UL transmission (Note 1); Scenario A : 0.30 ppm Scenario C2/D2 : - 1200 km: 40 ppm - 600 km: 45 ppm. Max frequency shift for UL transmission if pre/post compensation mechanism is assumed at satellite payload side; Scenario A : n/a Scenario C2/D2 : - Satellite alt. = 1200 km : - beam diameter = 90 km (Set 1 - S-band) : 1.81 ppm - beam diameter = 40 km (Set 1 - Ka-band): 0.81 ppm - beam diameter = 190 km (Set 2 - S-band) : 3.82 ppm - beam diameter = 90 km (Set 2 - Ka-band) : 1.81 ppm - Satellite alt. = 600 km : - beam diameter = 50 km (Set 1 - S-band) : 2.10 ppm - beam diameter = 20 km (Set 1 - Ka-band) : 0.84 ppm - beam diameter = 90 km (Set 2 - S-band) : 3.77 ppm - beam diameter = 50 km (Set 2 - Ka-band) : 2.10 ppm. Note 1 : Min. Elevation angle for both sat- user equipment is equal to 10 degree.. |

Uplink transmission bandwidths:

| Companies | Comments / Proposals |
| --- | --- |
| Ericsson [13] | Proposal 7 For link budget calculation in NTN uplink, it is assumed that handheld UE transmits with 180 kHz bandwidth in S band and VSAT transmits with 400 MHz bandwidth in Ka band. |

CIR values :

| Companies | Comments / Proposals |
| --- | --- |
| Ericsson [13] | Observation 10 In an NTN system with 1 frequency reuse and high load, the operating DL CINR can be limited by interference. Proposal 13 RAN1 to continue link budget analysis by evaluating CIR from system-level simulation to calculate CINR. |
| ZTE [15] | Proposal 4: At least for link budget in GEO, multiple satellite can be considered to obtain the CIR. Proposal 5: 10 UE per beams can be considered to evaluate the CIR for UL channel. |