3GPP TSG-RAN WG1 Meeting #98	R1-1909698

Prague, Czech Republic, 26th – 30th August 2019

Agenda Item:	6.2.1.3

Source:	Ericsson

Title:	Feature lead summary #2 for Scheduling of multiple DL/UL transport blocks for LTE-MTC

Document for:	Discussion and Decision

# 1	Introduction

RAN1 and RAN2 agreements made in earlier meetings for this topic are summarized in [1] and [2]. Proposals from contributions [3] – [15] are summarized in the feature lead summary in [16].

This contribution lists remaining issues for the unicast case, i.e. the multicast case is not treated in this document.

(When referring to contributions, the first name in the source list is written within square brackets. The full source lists are included in the References section.)

# 2	Comebacks from Wednesday online session

From draft session notes after Wednesday online session:

Possible Agreement Key requirement for the design of DCI for multiple DL/UL TB: - At least when a single TB is scheduled, the scheduling flexibility should be same as that of Rel-15 Supported by Sierra Wireless, Ericsson, Intel, Sequans, Huawei/HiSi, Sony, Nokia/NSB Have concerns: ZTE/Sanechips, LGE

During the offline session, the following compromise was identified as a possible way forward.

Possible Agreement

For the design of DCI for multiple DL/UL TB:

- At least when a single TB is scheduled, aim for similar scheduling flexibility as that of legacy DCI

  - Possible exceptions at least for some cases are the frequency hopping flag and RV index field

From draft session notes after Wednesday online session:

Possible Agreement - For unicast, the maximum number of scheduled TBs with one single DCI for CE mode A for either UL or DL is RRC configured within the set {2, [4], 8} in a UE specific manner. - The design methodology for the DCI for different maximum number of TBs is further studied - For the 2 TB case, target for 2~3 bits DCI overhead Possible Agreement - The following working assumption is confirmed. - For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported

During the offline session, the following compromise was discussed but no conclusion was reached.

Possible Agreement:

- For unicast, down select between the following options in the next meeting.

  - Option 1: Scheduling of up to 8 TBs is supported with a single DCI design.

    - Target for up to 6 bits DCI overhead

  - Option 2: For unicast, the maximum number of scheduled TBs with one single DCI for CE mode A for either UL or DL is RRC configured within the set {2, [4], 8} in a UE specific manner.

    - The design methodology for the DCI for different maximum number of TBs is further studied

    - For the 2 TB case, target for up to 3 bits DCI overhead

- The following working assumption is confirmed.

  - For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported

# 3	Scheduling gaps

[Huawei], [Intel] and [Nokia] propose to confirm the RAN1#97 working assumption about introducing scheduling gaps.

The following working assumption is confirmed:
For unicast, scheduling gaps for multiple transport blocks is supported and a scheduling gap can be configured by [RRC and/or DCI].
- The support of scheduling gaps is UE optional feature regardless of the support of multiple TBs.
- FFS: Details on the scheduling gap such as duration, applicability, etc.

[Huawei], [Nokia], [Sierra], [Sony], [Xiaomi] and [ZTE] propose that the scheduling gaps are configured by RRC. [Sony] and [ZTE] propose that the RRC configuration includes e.g. gap duration and gap position. [Sony] and [Xiaomi] propose that this RRC configuration can be dynamically activated/deactivated via DCI, whereas [Nokia] proposes that this can be FFS. [Sierra] proposes that when multi-TB scheduling with gaps is enabled by RRC signalling, a special DCI field can be used to select one of a few different gap configurations.

For unicast, the optional scheduling gaps for multiple transport blocks are configured by RRC. Any additional gap configuration (de)activation/selection by DCI is FFS.

[Huawei] suggests that scheduling gaps may only be needed when the number of scheduled TBs is relatively large. [ZTE] proposes that the gaps are only supported in the interleaving case, that the gap position should be between the interleaved blocks, and that the gap is inserted when the product of the number of repetitions and the number of TBs exceeds an RRC configured threshold.

For unicast, discuss whether the use of scheduling gaps should be restricted to transmissions of a minimum transmission length and/or a minimum number of scheduled TBs.

[Sony] proposes that an MPDCCH transmission received during a scheduling gap can be used to achieve early termination of an ongoing multi-TB uplink transmission.

For unicast, discuss whether the use of scheduling gaps can be used to allow early termination of an ongoing (multi-TB) uplink transmission.

# 6	Timing relationships

For the individual (non-bundled) HARQ feedback case, [Lenovo] and [ZTE] propose that the start of the HARQ feedback is related to the end of the associated DL TB in the FD-FDD case but related to the end of the last DL TB in the HD-FDD case.

For the HARQ-ACK bundling case, [Intel], [Nokia] and [ZTE] propose that the start of the HARQ feedback should be based on the last DL TB.

For unicast, discuss whether the start of the HARQ feedback should be the same or can be different in the FD-FDD and HD-FDD cases. (Note that the answer may be different in the HARQ-ACK bundling and non-bundling cases.)

Furthermore, [Huawei] proposes to reduce the MPDCCH monitoring occasions for multi-TB scheduling.

# 7	New data indication

No schemes for using the same new data indicator for several HARQ priorities is described, and it is considered that the new data indication needs to be individually provided for each allocated HARQ process.

For unicast, the new data indication is individually provided for each allocated HARQ process.

# 7	RV indication

The potential for reduction of the information needed for specifying the initial RV for the allocated HARQ processes is discussed in several contributions.

For downlink, the need for individual RV indication may depend on if HARQ-ACK bundling is configured and how the HARQ-ACK bundling is designed. For uplink, and downlink with individual HARQ-ACK for the transmitted TBs, individual indication of the RV being used for retransmissions has potentially a larger gain.

As discussed by e.g. [Ericsson] [Huawei] [Qualcomm], the RV field only indicates the initial RV index and the RV will be cycled for the subframes in the transmission and with 4 or more repetitions all RVs will be used. Thus, when 4 or more repetitions are used there is less of a need for individually assigning the RV to each HARQ process.

For unicast, a single RV value is used for all HARQ processes at least when 4 or more repetitions are used. Whether the RV is signalled via DCI or predefined is for further study.

It is proposed by [Sierra] that when multiple TBs are scheduled by one DCI, the RV for initial TBs is fixed at 0 and the RV for retransmissions TBs is fixed at 2.

It is proposed by [ZTE] that the RV field is at least 1 bit including RV0 and RV2.

For the case of scheduling multiple TBs with less than 4 repetitions, the following options with respect to the RVs are identified:

- Individual RV for each allocated HARQ process indicated via an RV information of [1 or 2] bits per HARQ process.

- A common RV for both initial transmission and retransmission indicated via a single RV information of [1 or 2] bits.

- Fixed (predefined) RV ([RV0]) for initial transmissions and a common RV for retransmissions indicated via a single RV information of [1 or 2] bits.

- Fixed (predefined) RVs for both initial transmissions ([RV0]) and retransmissions ([RV2]).

For unicast, when less than [2 or 4] repetitions are scheduled, the allocation of RV for the allocated HARQ processes is down selected from the following options:
- Option 1: Individual RV for each allocated HARQ process indicated via an RV information of [1 or 2] bits per HARQ process.
- Option 2: A common RV for both initial transmission and retransmission indicated via a single RV information of [1 or 2] bits.
- Option 3: Fixed (predefined) RV ([RV0]) for initial transmissions and a common RV for retransmissions indicated via a single RV information of [1 or 2] bits.
- Option 4: Fixed (predefined) RVs for both initial transmissions ([RV0]) and retransmissions ([RV2]).

# 7	DCI encoding

Possibilities to reduce the size of the DCI by joint encoding is proposed by several companies. It is considered that the exact realization of joint encoding is best determined after the functionalities discussed in the previous sections are agreed. However, the design of joint encoding needs to take into account if the scheduling flexibility for certain fields may be reduced when performing joint encoding. Thus, the following is proposed:

For unicast, define for which (if any) DCI parameters {NDI; Number of allocated HARQ processes; HARQ process numbers} the scheduling flexibility may be reduced if joint encoding is considered.

For unicast, joint encoding of at least {NDI; HARQ process numbers} is supported.
- If individual RV is supported, it is also included in the joint encoding.
- If configurable maximum number of TBs is supported, the joint encoding is used at least when the configured maximum number of TBs is at least 4.

Similarly, the size of the DCI may be decreased by limiting the scheduling flexibility of other fields when multiple TBs are scheduled. To proceed, the following is proposed:

For unicast, define for which (if any) DCI parameters {Resource assignment; MCS; Number of contiguous HARQ processes} the scheduling flexibility may be reduced when multiple TBs are scheduled.

# 5	Interleaving

Several different views regarding the interleaving granularity are expressed in the contributions.

- [Huawei] proposes that the interleaving granularity is an integral multiple of the frequency hopping interval (Ych).

- [Lenovo] proposes that interleaving is performed every valid subframe for CE mode A and every Nacc absolute subframe for CE mode B.

- [LG] proposes that when interleaving is used, the cyclic (Nacc) repetition pattern should be preserved.

- [Nokia] proposes that interleaving is done every N subframes, where N may be fixed (e.g. the same as Ych) or configurable.

- [Sierra] proposes, for full-PRB transmission, that the TB changes every subframe in CE mode A and every N subframes in CE mode B, where N is configurable in the range {4, 8, 16}.

- [Xiaomi] proposes that the interleaving granularity equals the granularity of the frequency hopping or the RV cycle (or RRC configurable if neither frequency hopping nor RV cycle is configured).

- [ZTE] proposes that the interleaving granularity is the product of the number of frequency hopping narrowbands times the interleaving granularity configured for the non-hopping case. [ZTE] also proposes that the interleaving granularity design should be based on the TB length in the sub-PRB case.

For unicast full-PRB transmission in CE mode A, the interleaving granularity is 1 subframe.

For unicast full-PRB transmission in CE mode B, the interleaving granularity is to be down selected between the following options:
- Option 1: Equal to the frequency hopping interval (Ych) or a multiple of it
- Option 2: Equal to the accumulation interval (Nacc) or a multiple of it
- Option 3: Given by a new cell- or UE-specific configuration parameter

For unicast, the interleaving pattern, at least in the frequency hopping case, is FFS.

For unicast sub-PRB transmission, the interleaving granularity is FFS.

For unicast, the invalid subframe relation to the interleaving is FFS.

Furthermore, [ZTE] proposes that interleaving is triggered if the total transmission length (number of repetitions times number of TBs) is larger than a threshold.

# 3	HARQ feedback

[Intel] proposes that the Rel-14 HARQ-ACK bundling design is reused in CE mode A. This would mean that HARQ-ACK bundling for multi-TB scheduling is only supported without repetition for MPDCCH or PDSCH.

[Nokia] proposes that HARQ-ACK bundling is configured via higher-layer signalling. [ZTE] proposes that it is enabled/disabled by RRC. [Intel] proposes that it is enabled/disabled by RRC and DCI similar as in Rel-14 HARQ-ACK bundling.

For unicast, discuss whether the Rel-14 HARQ-ACK bundling design can be reused as the basis for the HARQ-ACK bundling for multi-TB scheduling in CE mode A.

[Nokia] proposes that the maximum bundle size is 8. [Sony] proposes that it is 4. [Qualcomm] proposes that it is no more than 4, and that it is tailored to the number of TBs and the number of PUCCH repetitions. [Intel] proposes that the HARQ-ACK bundle size equals the number of scheduled TBs. [ZTE] proposes that it equals the number of scheduled TBs in the interleaving case and that HARQ-ACK bundling is not supported in the non-interleaving case. [LG] proposes that the maximum bundle size depends on whether interleaving is enabled or not. [Sequans] proposes to use a table to map the HARQ-ACK feedback from all TBs to N bits that are mapped to N consecutive PUCCH transmissions (where N can e.g. be fixed to 4).

For unicast multi-TB scheduling with HARQ-ACK bundling, down select between the following options:
- Option 1: The maximum bundle size is 4.
- Option 2: The bundle size equals the number of scheduled TBs (max 8).
- Option 3: The TBs to retransmit are given by a table entry pointed out by the PUCCH contents (max 4 PUCCH bits).

Furthermore, [Sony] proposes that when a HARQ-ACK bundled ‘NACK’ is transmitted, it is followed by individual PUCCH transmissions indicating the ACK/NACK status for the individual TBs.

[LG] proposes that for individual (non-bundled) HARQ-ACK feedback, the UE transmits ‘ACK’ for positive acknowledgment of a TB and ‘DTX’ for negative acknowledgment of a TB, whereas ‘NACK’ can represent negative acknowledgment for all scheduled TBs.

PUCCH resource allocation aspects are discussed in the contributions from [Lenovo] and [Samsung].

# References

- R1-1907630, “RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE”

- R2-1908188, “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”

- R1-1908020, “Scheduling of multiple DL/UL transport blocks in LTE-MTC”, Ericsson

- R1-1908080, “Scheduling of multiple transport blocks”, Huawei, HiSilicon

- R1-1908186, “LTE-M Multiple Transport Block Grant Design Considerations”, Sierra Wireless

- R1-1908257, “Consideration on scheduling enhancement for MTC”, ZTE

- R1-1908293, “Scheduling of multiple DL/UL transport blocks”, Nokia, Nokia Shanghai Bell

- R1-1908445, “Scheduling of multiple transport blocks for MTC”, Samsung

- R1-1908521, “Discussion on multiple transport blocks scheduling in MTC”, LG Electronics

- R1-1908615, “Scheduling of multiple DL/UL TBs for eMTC”, Intel Corporation

- R1-1908727, “Design of scheduling of multiple DL/UL transport blocks for MTC”, Lenovo, Motorola Mobility

- R1-1908758, “On scheduling of multiple DL/UL transport blocks	”, Sony

- R1-1908827, “Scheduling of multiple DL/UL transport blocks”, Qualcomm Incorporated

- R1-1909112, “Further discussion on the scheduling of multiple DL/UL TBs”, Xiaomi Communications

- R1-1909466, “Consideration for scheduling multiple UL/DL TBs”, Sequans Communications

- R1-1909497, “Feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC”, Ericsson