3GPP TSG-RAN WG1 Meeting #98	R1-1909718

Prague, CZ, August 26th – 30th, 2019

Agenda Item:	7.2.5.1

Source:	Thales

Title:	Summary#3 on simulation assumptions for NTN

Document for:	Discussion

# Introduction

This document aims to summarize the companies’ views on :

- LLS parameters and assumptions

- Link budget analysis

# Discussion on Link Level Simulation assumptions

## Clarification on LLS parameters for DL synchronization

It is also proposed to clarify the metrics requirement for DL synchronization evaluation.

Proposal : It is proposed to update Table 6.1.2-1 of TR38.821 as follows:

Table 6.1.2-1 : LLS parameters for DL synchronization evaluation

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| DL RS | SSB |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| UE elevation angle | For GEO (optional) : 10°, For LEO : 30° |  |
| Frequency Offset | - UE crystal accuracy: 10 ppm - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle Note 1 : The final frequency offset is computed as follows : where : denotes the final frequency offset in Hz denotes the UE crystal accuracy in ppm denotes Doppler shift due to satellite movement in ppm. Pre/post Doppler shift compensation can be assumed. denotes the Doppler shift due to UE movement in ppm denotes the carrier frequency used on the service Down Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed. Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1-8] |  |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |
| Metrics | One-shot initial cell detection accuracy of PCID; CDF of timing and frequency residual offset at SNR point corresponding to 90% likelihood for one-shot detection accuracy of cell ID. Note 4: FAR of PCID detection requirement = 1% |  |

Note: The SNR range to be evaluated should be based on the link budget analysis for each channel

## Clarification on LLS parameters for PRACH performance evaluation

Frequency offset :

During the last meeting, the frequency offset assumptions have been flagged as FFS. RAN1 shall achieve some progress on the subject before the end of this meeting so PRACH performance evaluation can be performed for the next meeting.

It should be clarified whether the frequency offsets resulting from UE oscillator drift should be fully taken into account for PRACH performance evaluation or whether they should be considered (at least partially) compensated.

It should be clarified which values should be considered for the frequency offsets resulting from Doppler shifts for PRACH performance evaluation. It has been proposed in [7] (Intel) to consider the double of the values proposed for DL synchronization evaluation in Table 6.1.1-8 for TR38.821.

Proposal : It is proposed to update the Table 6.1.2-2 for TR38.821 as follows :

Table 6.1.2-2 : LLS parameters for PRACH performance evaluation

| Configurations | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx 2 Rx optional | 1 Rx 2 Rx optional |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| Frequency Offset | - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle - Residual frequency offset after synchronization: [0.1] ppm Note 1 : The final frequency offset is computed as follow : where : denotes the final frequency offset in Hz denotes the residual frequency offset after synchronization in ppm denotes Doppler shift due to satellite movement in ppm. Pre/post Doppler shift compensation is considered. denotes the Doppler shift due to UE movement in ppm denotes the central frequency used on the service Up Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to TR38.803 section 6.1.9.5. |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate, FAR |  |
| Note 1: Ideal common delay compensation is assumed. |  |  |

## Clarification on LLS parameters for data transmission performance evaluation

The frequency offset and frequency tracking assumptions for data transmission performance evaluation should be completed before the end of the meeting.

Proposal : It is proposed to update the Table 6.1.2-3 for TR38.821 as follows:

This table should be used as a baseline framework for link level data transmission evaluation. Additional parameters should be provided by the companies together with the simulation results.

Table 6.1.2-3 : LLS parameters for data transmission performance evaluation

| Parameters | S-band |  | Ka-band |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz |  | DL 20GHz |
| Channel coding scheme | NR channel coding |  |  |
| Subcarrier spacing | 15kHz, 30kHz |  | 60kHz, 120kHz |
| Channel estimation | Realistic estimation |  |  |
| Frequency offset | Residual Frequency error after DL synchronization: [0.1] ppm assuming UL pre-compensation |  |  |
| Frequency drift | Doppler rate values provided in Table 6.1.1-8 |  |  |
| Frequency tracking | Option 1 : drift pre-compensation is assumed Option 2 : no pre-compensation is assumed |  |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |  |
| Channel model | For GEO (optional): Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO: Baseline TDL/CDL model in TR38.811, with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |  |
| TRP antenna configuration | 1Tx |  | 1Tx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |  | VSAT with 60 cm equivalent aperture diameter (4, 8, 2) with directional antenna element (HPBW=65 °, directivity 8 dBi) |
| Phase noise Model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to TR38.803 section 6.1.9.5. |  |  |
| Metrics | BLER, Throughput |  |  |

# Discussion on Link budgets analysis

## Considerations on link budget analysis assumptions

Proposal : It is proposed to update the Table 6.1.3.2-1 of TR38.821 as follow :

Table 6.1.3.2-1 Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| System bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Channel bandwidth | DL : system bandwidth/ frequency reuse factor UL: - UL in S band (handheld UE): - 360 kHz - Otherwise: system bandwidth/ frequency reuse factor Note: The UL bandwidth may be challenge. |
| Satellite altitude | 600km, 1200km, 35786km |
| Target elevation angle | 30 (LEO), 10 (GEO) |
| Atmospheric loss | Equation (6.6-8) in TR 38.811 |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Scintillation loss | Section 6.6.6 in TR 38.811 - Ionospheric loss: = 2.2 dB (note 1) - Tropospheric loss : Table 6.6.6.2.1-1 of TR38.811 |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1, 2, 3 |
| Average CIR within a satellite beam | Based on single satellite system-level calibration methodology, statistics are only collected for the UEs located in the central beam of the 19-beamlayout. The central beam boresight direction is computed based on the target elevation angle assumption. When the generated beam has a partial or full coverage outside the earth, it is discarded. For UL calibration, - For Handheld device, the channel bandwidth is 360 kHz. - For VSAT, the channel bandwidth equals the system bandwidth allocated to each beam divided by 10. - The devices in one beam are allocated on adjacent frequency resources. The same resource allocation is assumed for all the beams. |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable if frequency reuse factor = 2 is considered. |
| Terminal type | - Ka band: VSAT - S band: (M, N, P) = (1,1,2) |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Terminal RF parameters | Table 6.1.1-3 |
| Satellite RF parameters | Set-1 in Table 6.1.1-1 and Set-2 in Table 6.1.1-2 |
| Polarization loss | - 3dB per linear polarization receive antenna - 0dB for circular polarization antenna receiver |
| Outcome | CNIR |
| Note 1 : Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of TR38.811 after frequency scaling. dB |  |

Proposal : It is proposed to update section 6.1.3.1 of TR38.821 as follow :

======================== Start of TP for TR 38.821 =============================

============================== Text Omitted ================================

#### 6.1.3.1		Link Budget Calculation

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as follows

|  | CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB] | (6.1.3.1-1) |
| --- | --- | --- |

The formula for CNR calculation is

|  | CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ] | (6.1.3.1-2) |
| --- | --- | --- |

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz,  PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLSM is shadowing margin, PLSL[dB] is scintillation loss, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth.

============================== Text Omitted ================================

========================= End of TP for TR 38.821 =============================

## Link budget results

Proposal : The following table should be integrated in TR38.821 to capture the link budget results.

Table Z.1 Link budget results

| Study case | Case 1 |  | Case 2 |  | Case … |  | Case N |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmission mode | DL | UL | DL | UL | DL | UL | DL | UL |
| TX: EIRP/spot/BW [dBm] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| RX: G/T [dB/T] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Bandwidth [Hz] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Free space path loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Atmospheric loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Shadow fading margin [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Scintillation Loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Polarization loss [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| Additional losses [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| CNR [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| CIR [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| CINR [dB] | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |

Proposal : It is proposed to capture the future link budget results in the attached excel template R1-190XXXX - NTN_ NTN_Link_Budget_Analysis_Phase1.xlsx.

# Conclusion

The proposal addressed during offline discussions are reported below :

Offline Proposal : It is proposed to capture the future calibration results in the attached excel template R1-190XXXX - NTN_Calibration_Phase1.xlsx. Companies are encouraged to submit their calibration results for the next meeting.

Offline Proposal : It is proposed to update Table 6.1.2-1 of TR38.821 as follows:

Table 6.1.2-1 : LLS parameters for DL synchronization evaluation

|  | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 20 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811[v15.1.0], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811[v15.1.0], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Subcarrier Spacing(s) | 15kHz, 30kHz | 120kHz, 240kHz |
| DL RS | SSB |  |
| Antenna Configuration at the TRP (satellite) | 1Tx | 1Tx |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| UE elevation angle | For GEO (optional) : 10°, For LEO : 30° |  |
| Frequency Offset | - UE crystal accuracy: 10 ppm - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle Note 1 : The final frequency offset is computed as follows : where : denotes the final frequency offset in Hz denotes the UE crystal accuracy in ppm denotes Doppler shift due to satellite movement in ppm. Pre/post common Doppler shift compensation can be assumed. denotes the Doppler shift due to UE movement in ppm denotes the carrier frequency used on the service Down Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed. Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| Frequency drift | [Doppler rate values provided in Table 6.1.1-8] |  |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to in TR38.803 section 6.1.9.5. |  |
| Metrics | One-shot initial cell detection accuracy of PCID; CDF of timing and frequency residual offset at SNR point corresponding to 90% likelihood for one-shot detection accuracy of cell ID. Note 4: FAR of PCID detection requirement = 1% |  |

Note: The SNR range to be evaluated should be based on the link budget analysis for each channel

Offline Proposal : It is proposed to update Table 6.1.2-2 for TR38.821 as follows :

Table 6.1.2-2 : LLS parameters for PRACH performance evaluation

| Configurations | S-band | Ka-band |
| --- | --- | --- |
| Carrier Frequency | 2 GHz | 30 GHz |
| Channel Model | For GEO (optional) : Baseline TDL/CDL model in TR38.811[v15.1.0], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle 10 deg For LEO : Baseline TDL/CDL model in TR38.811[v15.1.0], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor for suburban LOS elevation angle [30] deg |  |
| Antenna Configuration at the TRP (satellite) | 1 Rx 2 Rx optional | 1 Rx 2 Rx optional |
| Antenna Configuration at the UE | (1, 1, 2) with omni-directional antenna element | VSAT with 60 cm equivalent aperture diameter |
| Frequency Offset | - Doppler shift in channel due to satellite movement : max. Doppler shift values provided in Table 6.1.1-8 - Doppler shift in channel due to UE movement : max. value to be computed based on the UE speed and the elevation angle - Residual frequency offset after synchronization: [0.1] ppm Note 1 : In case the network performs both pre and post common Doppler shift compensation, the final frequency offset is computed as follows : where : denotes the final frequency offset in Hz denotes the residual frequency offset after synchronization in ppm denotes the residual Doppler shift due to satellite movement in ppm after common Doppler compensation. denotes Doppler shift due to UE movement in ppm denotes the central frequency used on the service Up Link in Hz A uniform distribution in [ - FO max value, + FO max value] shall be assumed Note 2 : Doppler spectrum on Rayleigh fading taps based on Jake model should be considered in addition to Doppler shift (see section 6.9.2 in TR 38.811) Note 3 : For a Rayleigh fading tap a minimum Doppler of 1 Hz should be considered. |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |
| Phase noise model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to TR38.803 section 6.1.9.5. |  |
| PRACH design | Each company should provide details on configuration (i.e. format, SCS, N_CS, …). New formats are not precluded. |  |
| Metric | PRACH detection rate, FAR |  |
| Note 1: Ideal common delay compensation is assumed. |  |  |

Offline Proposal : It is proposed to update Table 6.1.2-3 for TR38.821 as follows:

This table should be used as a baseline framework for link level data transmission evaluation. Additional parameters should be provided by the companies together with the simulation results.

Table 6.1.2-3 : LLS parameters for data transmission performance evaluation

| Parameters | S-band |  | Ka-band |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz |  | DL 20GHz, UL 30 GHz |
| Channel coding scheme | NR channel coding |  |  |
| Subcarrier spacing | 15kHz, 30kHz |  | 60kHz, 120kHz |
| Channel estimation | Realistic estimation |  |  |
| Frequency offset | Residual Frequency error after DL synchronization: [0.1] ppm assuming UL pre-compensation |  |  |
| Frequency drift | Doppler rate values provided in Table 6.1.1-8 |  |  |
| Frequency tracking | Option 1 : drift pre-compensation is assumed Option 2 : no pre-compensation is assumed |  |  |
| UE speed | 3 km/h | 0 km/h, 1000 km/h |  |
| Channel model | For GEO (optional): Baseline TDL/CDL model in TR38.811[v15.1.0], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions . These parameters should be provided by the companies. For LEO: Baseline TDL/CDL model in TR38.811[v15.1.0], with delay/angular scaling factors equals to the mean delay/angular spread and mean K factor based on the selected channel conditions . These parameters should be provided by the companies. |  |  |
| satellite antenna configuration | 1Tx/Rx |  | 1Tx/Rx |
| UE antenna configuration | (1, 1, 2) with omni-directional antenna element |  | VSAT with 60 cm equivalent aperture diameter |
| Phase noise Model | S-band phase noise modeling (optional) Ka-band phase noise modeling : phase noise profile according to TR38.803 section 6.1.9.5. |  |  |
| Metrics | BLER, Throughput |  |  |

Offline Proposal : It is proposed to update Table 6.1.3.2-1 of TR38.821 as follow :

Table 6.1.3.2-1 Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2GHz for DL and UL (S band), 20 GHz for DL and 30GHz for UL (Ka band) |
| System bandwidth | 30MHz (S band), 400MHz (Ka band) |
| Channel bandwidth | DL : system bandwidth/ frequency reuse factor UL: - UL in S band (handheld UE): - 360 kHz - Otherwise: system bandwidth/ frequency reuse factor Note: The UL bandwidth may be challenge. |
| Satellite altitude | 600km, 1200km, 35786km |
| Target elevation angle | 30 (LEO), 10 (GEO) |
| Atmospheric loss | Equation (6.6-8) in TR 38.811 |
| Shadowing margin | 0 dB for VSAT as terminal and 3 dB for others |
| Scintillation loss | Section 6.6.6 in TR 38.811 - Ionospheric loss: = 2.2 dB (note 1) - Tropospheric loss : Table 6.6.6.2.1-1 of TR38.811 |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1, 2, 3 |
| Average CIR within a satellite beam | Based on single satellite system-level calibration methodology, statistics are only collected for the UEs located in the central beam of the 19-beamlayout. The central beam boresight direction is computed based on the target elevation angle assumption. When the generated beam has a partial or full coverage outside the earth, it is discarded. For UL calibration, - For Handheld device, the channel bandwidth is 360 kHz. - For VSAT, the channel bandwidth equals the system bandwidth allocated to each beam divided by 10. - The devices in one beam are allocated on adjacent frequency resources. The same resource allocation is assumed for all the beams. |
| Satellite antenna polarization | Circular polarization |
| Polarization reuse | Enable if frequency reuse factor = 2 is considered. |
| Terminal type | - Ka band: VSAT - S band: (M, N, P) = (1,1,2) |
| Free space path loss | Equation (6.6-2) in TR 38.811 |
| Terminal RF parameters | Table 6.1.1-3 |
| Satellite RF parameters | Set-1 in Table 6.1.1-1 and Set-2 in Table 6.1.1-2 |
| Polarization loss | - 3dB per linear polarization receive antenna - 0dB for circular polarization antenna receiver |
| Outcome | CNIR |
| Note 1 : Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of TR38.811 after frequency scaling. dB |  |

Offline Proposal : It is proposed to update section 6.1.3.1 of TR38.821 as follow :

======================== Start of TP for TR 38.821 =============================

============================== Text Omitted ================================

#### 6.1.3.1		Link Budget Calculation

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as follows

|  | CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB] | (6.1.3.1-1) |
| --- | --- | --- |

The formula for CNR calculation is

|  | CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ] | (6.1.3.1-2) |
| --- | --- | --- |

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz,  PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLSM is shadowing margin, PLSL[dB] is scintillation loss, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth.

============================== Text Omitted ================================

========================= End of TP for TR 38.821 =============================

Offline Proposal : It is proposed to capture the future link budget results in the attached excel template R1-190XXXX - NTN_ NTN_Link_Budget_Analysis_Phase1.xlsx.

# Reference

| [1] | R1-1908013 | On beam layout definition and system level performance for NR-NTN, MediaTek Inc |
| --- | --- | --- |
| [2] | R1-1908047 | Discussion on performance evaluation for NTN, Huawei, HiSilicon, CAICT |
| [3] | R1-1908227 | Calibration considerations and results for system level simulations, THALES |
| [4] | R1-1908249 | Considerations on Performance Evaluation in NTN, Nokia, Nokia Shanghai Bell |
| [5] | R1-1908486 | Preliminary Link-Level and System-Level Evaluations, Samsung |
| [6] | R1-1908589 | Initial link level results for NTN, CATT, CAICT |
| [7] | R1-1908641 | Considerations on performance evaluation for NTN, Intel Corporation |
| [8] | R1-1908773 | Calibration results of system level simulation, Sony |
| [9] | R1-1908816 | System level evaluation and link budget for NTN, Panasonic Corporation |
| [10] | R1-1908993 | Preliminary Calibration Results for NTN System-Level Evaluations, Nomor Research GmbH |
| [11] | R1-1908996 | On Downlink Performance in NTN, Fraunhofer IIS, Fraunhofer HHI |
| [12] | R1-1908999 | NTN LLS Calibration Assumptions, Fraunhofer IIS, Fraunhofer HHI |
| [13] | R1-1909110 | On NTN link level and system level evaluations, Ericsson |
| [14] | R1-1909372 | NTN link budget calibration: downlink numerical results, ESA |
| [15] | R1-1909398 | Discussion on the simulation for NTN, ZTE |
| [16] | R1-1908228 | Link budget analysis, THALES |
| [17] | R1-1908995 | Corrections for TR38.811 Section 6.7.2 Non-Terrestrial Networks channel models, Nomor Research GmbH |
| [18] | R1-1905216 | Key satellite parameters and simulation assumptions for NTN, ESA |