3GPP TSG-RAN WG1 Meeting #98	R1- 1909730

August 26th – 30th, 2019

Prague, Czech Republic

Agenda item:	7.1.3

Source: 	Qualcomm Incorporated

Title: 	Summary#2 of CRs on UCI multiplexing on PUSCH

Document for:	Discussion/Decision

# Introduction

In this contribution, a summary of CRs on UCI multiplexing on PUSCH is provided in Section 2.

# Summary of CRs

There are in total 4 CRs submitted to 7.1.3 which are included in this summary and listed below.

R1-1908327	Draft CR on UCI multiplexing in PUSCH		ZTE

The CR is updated with the following TP:

If a UE transmits multiple PUSCHs in a slot and the UE would multiplex aperiodic CSI in one of the multiple PUSCHs If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCHs and the timing conditions for overlapping PUCCHs and PUSCHs in Subclause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH with aperiodic CSI and does not transmit the PUCCH.

Offline conclusion: Neither CR R1-1908327 nor the above TP is needed.

R1-1908455	Draft CR on PUCCH resource determination with consideration of processing time	 Samsung

Offline proposal: The CR R1-1908455 is not need. it is clarified the following case is an error case. How to handle this error case is up to UE implementation.

- When multiple HARQ-ACKs are multiplexed in a determined PUCCH resource based on the last received PDCCH, the time gap between the end of the last symbol of the last PDSCH (which is scheduled by the last PDCCH) and the beginning of the CP of the first symbol of the determined PUCCH resource including the effect of timing advance is smaller than T1.

Adopt the following TP for TS38.214 Subclause 5.3

Otherwise the UE may not provide a valid HARQ-ACK corresponding to the scheduled PDSCH is not required to provide valid HARQ-ACK information in the PUCCH as defined by the assigned HARQ-ACK timing K1.

R1-1909236	Draft TS 38.213 CR on UE procedure for reporting multiple UCI types		Qualcomm

Offline conclusion: endorse the CR.

R1-1908566	Miscellaneous correction in 38.214		CATT

The CR is updated to R1-1909614. Further discussion is needed.