3GPP TSG RAN WG1#98		R1-1909761

Prague, Czech Republic, 26th– 30th August 2019

Agenda Item:	7.2.2.1.2

Source:	Motorola Mobility, Lenovo

Title:	Feature lead summary #2 for NR-U DL Signals and Channels

Document for:	Discussion, Decision

# Introduction

This document summarizes documents submitted to RAN1#98 on AI 7.2.2.1.2.

The following is an excerpt from the work item description (RP-182878), highlighting added by the feature leader:

- Physical layer aspects including [RAN1]: - Frame structure including single and multiple DL to UL and UL to DL switching points within a shared COT with associated identified LBT requirements (TR Section 7.2.1.3.1). - NR-U supports a mode of operation where for a carrier and at least for intra-band CA on serving cells on unlicensed bands, all DL channels / signals can be operated with the same numerology, and all UL channels / signals can be operated with the same numerology. - Subcarrier spacing for control and data channels supporting 15kHz, 30kHz, and 60kHz (air-interface perspective; optionality to be discussed separately). - Wide band operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with multiple serving cells, and wideband operation (in integer multiples of 20MHz) for DL and UL for NR-U supported with one serving cell with bandwidth > 20MHz with potential scheduling constraint subject to input from RAN2 and RAN4 on feasibility of operating the wideband carrier when LBT is unsuccessful in one or more LBT subbands within the wideband carrier. For all wide-band operation cases, CCA is performed in units of 20MHz (at least for 5GHz). - NR-U Discovery Reference Signal (DRS) containing at least SS/PBCH block burst set transmission and possibly CSI-RS, RMSI-CORESET(s)+PDSCH(s), OSI and paging with properties and extensions from NR Rel-15 in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2). 60kHz based SSB/PBCH block is outside the scope of the WI. - PRACH including possible extension of PRACH format(s) in line with agreements during the SI phase (TR 38.889, Section 7.2.1.2) to support minimum bandwidth requirement given by regulation. Determine the applicability of Rel-15 NR formats to NR-U operation.RAN1 should decide whether 60 kHz subcarrier spacing for PRACH is supported, based on a unified design with 15 kHz and 30 kHz PRACH for meeting occupied channel bandwidth (OCB) requirements. - UL control including extension of PUCCH format(s) to support PRB-based frequency block-interlaced transmission and use of Rel-15 NR PUCCH formats 2 and 3 for NR-U operation. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1. - UL data channel including extension of PUSCH to support PRB-based frequency block-interlaced transmission; support of multiple PUSCH(s) starting positions in one or multiple slot(s) depending on the LBT outcome with the understanding that the ending position is indicated by the UL grant; design not requiring the UE to change a granted TBS for a PUSCH transmission depending on the LBT outcome. The necessary PUSCH enhancements based on CP-OFDM. Applicability of sub-PRB frequency block-interlaced transmission for 60kHz to be decided by RAN1. - SRS including the introduction of additional flexibility in configuring/triggering SRS in line with agreements during the study phase. - For DL data channel, support of multiple PDSCH starting positions. - Mechanism to detect a gNB’s transmission burst in line with the TR 38.889, Section 7.2.1.2 related to UE power consumption. - DL control in line with the agreements during the study phase (TR 38.889, Section 7.2.1.2) including extensions allowing dynamic change of the time domain instances in which the UE is expected to receive PDCCH, modifications enabling DRS transmissions without gaps in the time-domain, and indication of time domain COT structure;

According to the guidance RP-191581 provided by RAN plenary #84, the following list of items is given:

Note that this may not be an exhaustive list of essential/optimization items. If something is neither mentioned as essential nor as optimization, no guidance from RAN is provided. Note that the completion of essential items in Proposal 1 means completion of the discussion of the item, and not necessarily mean the item will be supported in NR-U. Note that further discussion on completing the design to support features already agreed that are not listed as optimisations should be considered as high priority as well. 7.2.2.1.2 DL signals and channels Essential - Channel occupancy time and frequency domain structure indication for LBE - UE COT detection for FBE - gNB control for the Cat 4 UL transmission (including CG) switching to cat 2 when fall in gNB COT - Behavior when P/SP-CSI-RS fail to transmit due to LBT Optimizations - CSI-RS enhancement outside of DRS - More opportunities for CSI-RS to compensate the possible LBT failure - A-TRS directly QCL with SSB

Please note that there are statements in individual contributions that make the feature lead’s job unnecessarily complex, such as “our proposals can be found in [x].”

In the future please take care to capture proposals that are relevant for the (sub-)agenda item explicitly, not by reference! For other proposals, of course references are preferred.

Agreement(s) from RAN1 AH1901 (Taipeh):

- The UE may assume the presence of a signal, such as the DMRS in any [PDCCH or GC-PDCCH] transmission, to detect transmission bursts by the serving gNB, to enable power saving by not necessitating performing blind decodes to detect the transmission burst (Note: The power saving possibility by not necessitating blind decodes assumes performance relaxation for PDCCH decoding is not needed. Also, this does not mandate a two-step PDCCH decoding process for the UE with respect to DMRS detection). - If a preamble transmitted at the start of a burst is agreed to be specified (this does not preclude usage of preambles by implementation), it may be used in addition to the DMRS of PDCCH/GC-PDCCH or any other signals in the gNB transmission to detect the start of transmission bursts by the serving gNB and potentially for power saving - Note: Whether a preamble, if defined, can be used for power saving in all cases depends on the details of the design. - Note: Other signals present in the transmission burst may also be used for the purpose of detection of the transmission burst - FFS: Potential enhancements to DMRS design to address issues with detection probability - The payload of a PDCCH and/or GC-PDCCH transmission can contain information regarding COT structure that may be used by the UE for power saving

Agreement(s) from RAN1 96bis (Xian):

Only further discuss/consider Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) as potential enhancements to PDSCH transmissions.

Agreement(s) from RAN1 97 (Reno):

- Additional durations from 2 to 13 (other than 2, 4 and 7 which are already supported in Rel-15) for PDSCH mapping type B are supported. - FFS: A duration of 14 - FFS: DMRS aspects for the additional durations (Reuse Rel-15 patterns or define new patterns for these additional durations) - Capability signalling will be defined for UEs to indicate which specific subset of durations are supported by the UE. - FFS: Some duration(s) are mandatory for the UE to support for initial access - FFS: PDSCH processing times for the new durations (reuse the values for durations of 2/4/7 or define new ones) - Note: PDCCH monitoring occasions will be discussed separately as part of the NR-U WI - Note: This agreement does not by itself imply an increased requirement in PDSCH processing and decoding capabilities compared to Rel-15

# PDSCH Transmission

## Support for new PDSCH type B duration 14

Support: NTT DOCOMO, AT&T, Nokia

Not support: ZTE, Sanechips, Qualcomm

FL Suggestion: Postpone the discussion (keep FFS)

## Processing time for new PDSCH type B durations

Option 1: Same as for 2/4/7:

ZTE, Sanechips, Samsung, CAICT, AT&T (use mapping type A rules for 8-14 symbols)

Option 2: For UE processing capability 1

if L > 7, d1,1 = 0

if L = 5 or 6, d1,1 = 7-L

if L = 3, d1,1 = 2+d

For UE processing capability 2

if L > 7, d1,1 = 0

if L = 5 or 6,

Alt. 1: d1,1 = 0

Alt. 2: d1,1 = d

Alt. 3: d1,1 = max{d-(L-4),0}

if L = 3,

Alt. 1: d1,1 = d

Alt. 2: d1,1 = max{d-(L-2),0}

Alt. 3: d1,1 = 1+d

LG

Option 3: For UE processing capability 1

if L > 7, d1,1 = 0

if L = 6, d1,1 = 1

if L = 5, d1,1 = 2

if L = 3, d1,1 = 3+min(d,1)

For UE processing capability 2

if L > 7, d1,1 = 0

if L = 6, d1,1 = min(d,2)

if L = 5, d1,1 = min(d,1)

if L = 3, d1,1 = d

Intel

Option 4: For UE processing capability 1

if L > 7, d1,1 = 0

if L = 6, d1,1 = 1

if L = 5, d1,1 = 2

if L = 3, d1,1 = 3+d

For UE processing capability 2

if L > 7, d1,1 = 0

if L >2 and L <7, d1,1 = d

Nokia, Nokia Shanghai Bell, NTT DOCOMO

FL Suggestion: Clarify Option 1, and check if this is agreeable

For UE processing capability 1

if L>=7 then d1,1 = 0

if L>=4 and L<=6, then d1,1 = 7-L

if L>=2 and L<=3, then d1,1 = 3+d, where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

For UE processing capability 2

if L>=7 then d1,1 = 0

if L>=3 and L<=6, then d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH

if the number of PDSCH symbols allocated is 2

if the scheduling PDCCH was in a 3-symbol CORESET and the CORESET and the PDSCH had the same starting symbol, then d1,1 = 3

otherwise d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

## DM-RS positions for new PDSCH type B durations 9, 10 (DSS)

Several documents suggest tables and designs including pos2/pos3 locations for additional DMRS locations, however Ericsson suggests that defining pos0/pos1 is sufficient for the WI.

FL Suggestion: Discuss position tables and if we need to specify DM-RS positions for pos2/pos3

Single symbol

Option 1:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | ,7 | ,4,7 | - |
| 10 |  | ,7 | ,4,7 | - |

Supported by: Huawei, HiSilicon

Option 2:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | ,7 | ,4,7 | ,4,7 |
| 10 |  | ,8 | ,4,8 | ,4,8 |

Supported by: vivo

Option 3:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | ,7 | ,5,7 | ,5,7 |
| 10 |  | ,8 | ,3,8 | ,3,6,9 |

Supported by: Nokia, Nokia Shanghai Bell

Option 4:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 | l0’ | l0’, l6’ | l0’, l3’, l6’ | l0’, l3’, l6’ |
| 10 | l0’ | , l8’ | l0’, l4’, l8’ | l0’, l3’, l6’, l9’ |

In the table above, ln’ is set to be n. If LTE CRS exists on the symbol l=n, ln’ is incremented until when LTE CRS doesn’t exist on symbol l=ln’.

Supported by: Panasonic, Intel

Option 5:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | ,7 | ,4,7 |  |
| 10 |  | ,7 | ,4,7 |  |

Use time-domain shifting mechanism to avoid collisions with LTE CRS when dynamic spectrum sharing is enabled.

Supported by: Mediatek

Option 6:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | ,8 | ,4,8 | ,3,6,9 |
| 10 |  | ,8 | ,4,8 | ,3,6,9 |

If a DM-RS symbol for PDSCH mapping type B of length 9 and 10 coincides with any symbol containing LTE CRS as indicated by higher layer, the DM-RS can be shifted by one symbol.

Supported by: Samsung

Option 7:

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | , 7 |  |  |
| 10 |  | , 7 |  |  |

Supported by: Ericsson

Double symbol DM-RS:

Option 1: Not supported for 9,10: Huawei, HiSi.

Option 2:

| ld in symbols | DM-RS positions |  |  |
| --- | --- | --- | --- |
|  | PDSCH mapping type B dmrs-AdditionalPosition |  |  |
|  | 0 | 1 | 2 |
| 9 |  | ,5 |  |
| 10 |  | ,7 |  |

Supported by: vivo

Option 3:

| ld in symbols | DM-RS positions |  |  |
| --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |
|  | dmrs-AdditionalPosition |  |  |
|  | 0 | 1 | 2 |
| 9 | l0’ | l0’, l5’ |  |
| 10 | l0’ | l0’, l7’ |  |

In the table above, ln’ is set to be n. If LTE CRS exists on the symbol l=n, ln’ is incremented until when LTE CRS doesn’t exist on symbol l=ln’.

Supported by: Panasonic, Intel

Option 3:

| ld in symbols | DM-RS positions |  |  |
| --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |
|  | dmrs-AdditionalPosition |  |  |
|  | 0 | 1 | 2 |
| 9 |  | ,7 |  |
| 10 |  | ,7 |  |

Use time-domain shifting mechanism to avoid collisions with LTE CRS when dynamic spectrum sharing is enabled.

Supported by: Mediatek

Option 4:

| ld in symbols | DM-RS positions |  |  |
| --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |
|  | dmrs-AdditionalPosition |  |  |
|  | 0 | 1 | 2 |
| 9 |  | ,7 |  |
| 10 |  | ,7 |  |

Supported by: Ericsson

Proposals for the online session:

Single symbol

1.) Choose one of the following locations for duration 9, or consider not supporting two additional DM-RS positions (where the proposals collapse to a single option).

| ld in symbols | DM-RS positions |  |  |  |
| --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |
|  | 0 | 1 | 2 | 3 |
| 9 |  | , 7 | ,4,7 | ,4,7 |
| 9 |  | , 7 | ,5,7 | ,5,7 |

2.) Choose one of the following locations (and corresponding shifting option) for duration 10, and/or consider not supporting two additional DM-RS positions (where the proposals collapse to a single option).

| ld in symbols | DM-RS positions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type B |  |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  |  |
|  | 0 | 1 | 2 | 3 | Shifting (see below) |
| 10 |  | , 7 | ,4,7 | ,4,7 | Option B |
| 10 |  | , 8 | ,6,8 | ,6,8 | Option A |
| 10 |  | , 8 | ,4,8 | ,4,8 | Option B |

Shifting options for above proposals to avoid collisions with LTE-CRS

-	Option A: if the PDSCH duration ld is 10 OFDM symbols for normal cyclic prefix, and the PDSCH allocation collides with resources reserved for a LTE CRS transmission,  shall be incremented by 1

-	Option B: if the PDSCH duration ld is 10 OFDM symbols for normal cyclic prefix, and the PDSCH allocation collides with resources reserved for a LTE CRS transmission, shall be incremented by 1

Double symbol DM-RS:

3.) Proposed conclusion: PDSCH type B durations 9,10 will not be optimised for double symbol DM-RS for avoiding LTE-CRS collisions.

## DM-RS positions design for new PDSCH type B durations

Option 1) Design one table where positions are applicable irrespective of DSS.

Supported by: Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson

Option 2) DM-RS positions are shifted for durations 9, 10 if DSS is indicated (i.e., different positions as for DSS and non-DSS cases for those durations).

Supported by: vivo, Panasonic, Mediatek, Samsung, Intel

FL Suggestion: Discuss option 1 vs. option 2.

# Channel occupancy time and frequency domain structure indication for LBE

Frequency domain occupancy information

Supported by field (e.g. bitmap) in GC-PDCCH: Qualcomm, ETRI, Lenovo, vivo (also possible through UE-specific PDCCH), Mediatek, Panasonic, Samsung, Intel, Nokia (from preconfigured set), Huawei, HiSi, LG, ETRI, Google (for 5 GHz band)

Supported by detection of PDCCH/DM-RS on each LBT bandwidth: NTT DOCOMO, ETRI, Charter, Lenovo

Rel-15 already support indication of SFI for multiple carriers. Simple modification of Rel-15 RRC configuration to indicate PRB range for LBT subband: Ericsson

FL Suggestion: Discuss frequency domain indication in GC-PDCCH, but also UE behaviour in case that information is missing.

Option 1) Support bitmap of available LBT bandwidths in GC-PDCCH (keep Rel-15 SFI configuration flexibility untouched, add a bitmap in the GC-PDCCH DCI)

Supported by: LG, Nokia, Intel, Samsung, Huawei, HiSi, Sharp, Panasonic, Lenovo, MotM, vivo, CAICT, Mediatek, Charter, Sony, NTT DOCOMO, ETRI, Futurewei, Qualcomm

Option 2) SFI configuration includes availability of LBT bandwidths re-using Rel-15 approach (modified RRC configuration of SFI entries)

Supported by: Ericsson

FL Suggestion: Adoption of Option 1.

UE behaviour when GC-PDCCH is received correctly:

UE behaviour when GC-PDCCH is not available at the UE (not configured, missed GC-PDCCH, partial slot without GC-PDCCH):

| UE behaviour when… | GC-PDCCH is received correctly | GC-PDCCH is not available at the UE |
| --- | --- | --- |
| Cat2/Cat4 Switching COT Sharing |  | UE follows the grant/indication |
| Freq. domain indication (WB) |  |  |
| PDCCH Monitoring Switching |  |  |
| CSI Measurement |  |  |
| End-of-COT Indication |  |  |

End of channel occupancy

Include “out-of-COT” state in SFI: Huawei, vivo (new “E” state), NTT DOCOMO (specific SFI index, e.g. 254), Spreadtrum (re-use “F” not followed by other D/U)

Include COT duration field e.g. in GC-PDCCH, DCI format 2_0: ZTE, vivo (ending symbol position), Nokia (slot/symbol where legacy SFI ends), Charter, Mediatek, Samsung, LG, Intel, Sony, ETRI, Qualcomm, CAICT

Different RNTI to indicate end: NTT DOCOMO

FL Suggestion: Discuss above proposals to converge on a single supported solution.

# PDCCH Monitoring

Proposal from offline email discussion (follow-up from RAN1#97 offline):

- Support a mechanism for the UE to change its PDCCH (incl. GC-PDCCH) monitoring behaviour based on one or more of the following, to be further discussed:

  - Indication, such as by a DCI content or DCI CRC pass, for example

    - Indication of flexible symbol in GC-PDCCH to drop monitoring

  - Relation to the detection of a DL burst, for example

    - [A->B] before and after the start of the DL burst

    - [B->C] before and after the end of the first full slot of a detected burst

    - [C->B] before and after the end of the DL burst

    - [B->A] before and after the end of the last slot of a detected burst

  - Timer after DCI detection

  - Configuration

    - Note spec change may not be needed

- Note: monitoring behaviour may include CORESET and search space set parameter(s)

Support (indicated by email as by 10:00) by: Nokia, NSB, MTK, Charter, ETRI, Ericsson

# References

The documents listed below were used for the summary, plus documents submitted to AI 7.2.2.2.5 for frequency domain indication where applicable.

R1-1908107	DL channels and signals in NR unlicensed band	Huawei, HiSilicon

R1-1908138	Discussion on physical DL channel design in unlicensed spectrum	vivo

R1-1908203	Considerations on DL reference signals and channels design for NR-U	ZTE, Sanechips

R1-1908336	On NR-U DL Signals and Channels	Charter Communications

R1-1908383	DL Signals and Channels for NR-U operation	MediaTek Inc.

R1-1908415	DL signals and channels for NR-U	OPPO

R1-1908423	DL signals and channels for NR-U	Panasonic Corporation

R1-1908463	DL signals and channels for NR-U	Samsung

R1-1908533	Physical layer design of DL signals and channels for NR-U	LG Electronics

R1-1908622	DL signals and channels for NR-unlicensed	Intel Corporation

R1-1908687	On DL signals and channels	Nokia, Nokia Shanghai Bell

R1-1908764	DL Signals and Channels for NR-U	Sony

R1-1908807	DL signals and channels for NR-U	ETRI

R1-1908962	Discussion on DL signals and channels for NR-U	Spreadtrum Communications

R1-1909009	DL signals and channels for gNB initiated COT	InterDigital, Inc.

R1-1909042	On COT detection and structure indication for NR-U	Apple Inc.

R1-1909079	Remaining issues for downlink signals and channels	AT&T

R1-1909086	Downlink structure and procedure for NR-U operation	Sharp

R1-1909162	Discussion on PDCCH monitoring in Phase A	ITRI

R1-1909175	DL signals and channels for NR-U	NTT DOCOMO, INC.

R1-1909215	Discussion on the DL signals and channels for NR-U	Beijing Xiaomi Mobile Software

R1-1909243	DL signals and channels for NR-U	Qualcomm Incorporated

R1-1909296	DL signals and channels for NR-U	Ericsson

R1-1909346	Discussions on DL signals and channels design in NR-U	CAICT

R1-1909384	Discussion on DL signals and channels	ASUSTEK COMPUTER (SHANGHAI)

R1-1909389	DL Frame Structure and COT Structure Aspects for NR-U	Motorola Mobility, Lenovo

R1-1909392	Discussion on DL signals and channels in NR unlicensed	Google Inc.