3GPP TSG RAN WG1 Meeting #98	R1-1909786

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.8.2

Source:	Huawei, HiSilicon

Title:	Proposals for Offline Discussion over Multi-TRP/Panel Transmission on Thursday

Document for:	Discussion and Decision

# M-DCI

# [Offline Proposal 4-2]: For multi-DCI based multi-TRP transmission with separate ACK/NACK feedback

- UE is allowed to transmit two TDMed long PUCCHs within a slot

- UE is allowed to transmit TDMed short PUCCH and long PUCCH within a slot

- UE is allowed to transmit TDMed short PUCCH and short PUCCH within a slot

[FFS whether/how to use PRI indication with the granularity of sub-slot for eMBB with M-TRP]

# S-DCI

# [Offline Agreement 10] When 2 TCI states are indicated by a TCI code point, at least for DMRS type 1 and type 2 for eMBB, if indicated DMRS ports are from two CDM groups, the first TCI state is applied to the first indicated CDM group and the second TCI state is applied to the second CDM group(s).

    - FFS the definition of the first or second indicated CDM group

# URLLC

# [Offline Proposal 15]: For single-DCI based M-TRP URLLC schemes 2a/2b/3/4, indicated DMRS ports are from one CDM group.

# Note: Further offline discussion was conducted to briefly exchange the understanding for other M-TRP proposals and issues including switching eMBB/URLLC schemes, DMRS entry/table design, combination of URLLC schemes.