3GPP TSG RAN WG1 Meeting #98	R1-1909811

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.4.7

Source:	Futurewei

Title:	Feature lead summary#3 of AI 7.2.4.7: Support of NR Uu controlling LTE sidelink

Document for:	Discussion and decision

# Introduction

A new WID on NR-V2X was approved at RAN#83, with one of the objectives as follows:

2. Specify support for NR Uu to provide control for LTE sidelink

- Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and

- Sidelink mode 3-like RRC-configured SPS scheduling with either RRC-based activation/deactivation as per the study outcome or DCI-based activation/deactivation [RAN1, RAN2].

  - RAN1 to make a decision on which option is supported until RAN#84.

- 16 companies submitted contributions for this AI ([1]-[16]). This summary aims to collect views and suggest proposals based on the submitted contributions.

# Decisions at previous meetings

## RAN1#96

Agreements:

- Scheduling by gNB using RRC for LTE sidelink scheduled mode is supported from RAN1 perspective under the premise that there is sufficient time for coordination between the NR and LTE modules. No DCI to activate/release

  - RRC message delivers the SPS grant configuration and releases the SPS configuration.

  - Support of this scheduling mode is subject to UE capability (may or may not have capability for both LTE & NR)

  - Note: some specification LTE change is needed to support the reception of a grant through RRC

    - RRC message contains mode 3 grant content and timing

    - Up to the Editor to capture it as mode 3 or new LTE sidelink mode

  - No intention to have additional NR & LTE specification change (other than those described above) for this function in Rel-16

- RAN1 studied the feasibility of SPS scheduling by gNB for LTE sidelink with DCI activation/release, but there is no consensus to support it

## RAN1#96bis

Agreements:

Regarding RRC-based versus DCI-based activation/release of LTE sidelink SPS, RAN1 agrees to make the choice on the basis of at least:

- Spec impact

- Flexibility

- Performance, including latency

- Implementation complexity

- Timing of the activation/deactivation

## RAN2#105bis

Agreements:

For scheduling LTE SL UEs, the gNB uses RRC messages to deliver the SPS grant configuration.

Separate system information block should be designed to support LTE resource pool configuration via NR Uu. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

gNB should be able to configure the LTE V2X mode 4 sidelink resource pool via dedicated signalling. In addition, gNB should be able to configure mode3 SL resources via dedicated signaling. It will be defined as a container (OCTET STRING) and actual information follows what defined in LTE RRC.

## RAN1#97

Agreements:

- DCI-based activation/deactivation is supported

  - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability

  - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

  - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

    - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

# Mode-3 operation

Support for mode-3 was agreed at RAN1#96, with additional details at RAN1#97, and in particular, activation/deactivation by DCI. Most companies discuss DCI design, and DCI timing.

## DCI design

The general guidelines of the DCI for activation/deactivation of a LTE sidelink resource allocation were agreed at RAN1#97. In particular:

- NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling

- The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V

In order to fulfil these constraints, two options are discussed by most companies:

- Option 1: one NR DCI format is designed for scheduling both NR sidelink and V2X sidelink, with one bit to indicate if the DCI is scheduling NR or LTE. Considered by 5 companies ([1], [3], [4], [9], [15])

- Option 2: two distinct NR DCI formats are designed for scheduling LTE sidelink and NR sidelink. One RNTI is used for scheduling LTE and another RNTI is used for scheduling NR. Considered by 10 companies ([1], [2], [3], [4], [5], [6], [8], [10], [14], [15])

It seems that there is a clear majority in favor of option 2.

Proposal for offline discussion:

- A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

In addition, the aspect of DCI size is discussed by most companies. From companies expressing views, there is consensus that the DCI for scheduling LTE sidelink is padded to an existing NR DCI size. Which DCI size to pad it to is not widely discussed, although at least one concrete proposal was listed in [2]. This aspect should continue to be discussed by RAN1.

Proposal for offline discussion:

- The DCI for activation/deactivation is padded to match another NR DCI size

  - FFS what the DCI for activation/deactivation size is

Note that other proposals are also discussed in the contributions. [2] proposes to have the UE sending a confirmation message after receiving a DCI for activation/deactivation. [11] proposes that the DCI format for LTE V2X SPS activation/deactivation via NR Uu should share a common DCI format with NR Uu configured-grant Type 2. [12] proposes to consider piggybacking the DCI content.

## DCI timing

An important aspect that was not concluded at the last meeting was the time at which the DCI grant applies:

  - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI

    - Z is the same timing offset in current LTE V2X specs

    - X>0. FFS value(s) of X, and if one or multiple values of X are possible

12 companies discuss this aspect. Two options are discussed:

- Option 1: a single X value is supported. 4 companies consider this option ([8], [9], [11], [14])

- Option 2: multiple X values are supported. 10 companies consider this option ([1], [2], [3], [4], [5], [6], [7], [8], [9], [13])

For companies supporting option 2, further details are discussed: whether X is linked to a capability, or reported by UE.

- Option 2-1: the X values is linked to a UE capability: considered by [1], [2], [4]

- Option 2-2: X is reported by the UE: considered by [3], [5], [6]

Option 2 has a large majority.

Proposal for offline discussion:

- X=X1+X2, with X1 being the coordination time between LTE and NR module and X2 the k2 equivalent

  - FFS whether X2 is needed

- Multiple X1 values are supported. How the UE reports the supported X1 value will be determined in the UE capability discussion

  - A UE reports a single X1 value

Further discussion after the online discussion.

The proposal in Chairman’s notes is as follows:

Proposal:

- X is dynamically indicated, where the minimum value of X is subject to UE capability

  - Multiple mininmum values are supported. How the UE reports the supported the minimum value will be determined in the UE capability discussion

    - A UE reports a single minimum value

After email discussion, and further offline discussion, the proposal was modified as follows:

Proposal for offline discussion:

- X is dynamically indicated using a field in the DCI

  - FFS whether the DCI field provides an index to a table or the value of X

  - The minimum value of X is subject to UE capability

  - From the system perspective, multiple minimum values are supported.

  - How the UE reports the supported the minimum value will be determined in the UE capability discussion

    - A UE reports a single minimum value

Other companies discuss whether X is signaled in the DCI (e.g., [13]), or RRC-configured ([9]). Given that few companies have expressed views on these points, the suggestion is to discuss further at RAN1#98b.

## Others:

One company [2] discusses which synchronization applies for the grant, and proposes that in the case of NR Uu controlling LTE sidelink, gNB should be considered as the synchronization source when determining the timing of SPS configuration.

Three companies ([2], [3], [6]) discuss how the TDD configuration and bandwidth are communicated to the LTE UE.

Two companies ([4], [16]) discuss  UE capability signaling for the support of LTE PC5 scheduling by NR Uu.

One company ([8]) discussed power control on the LTE sidelink.

Two companies ([8], [16]) discuss scheduling assistance information (e.g., UE location reporting, BSR signaling)

No proposal is made here for these aspects. Companies are encouraged to further consider them for contributions at RAN1#98b.

# Mode 4

Within the context of this AI, mode-4 configuration is also discussed by 6 companies ([3], [7], [8], [10], [12], 16]). 5 companies indicate that no work is needed in RAN1 (([3], [7], [8], [10], [12]). One company [16] indicates that NR Uu control of LTE mode-4 operation should be a UE capability. The FL suggestion is to let RAN2 work on mode-4 configuration, and to discuss support of this UE capability in RAN2 as well.

Proposal for offline discussion:

- No further RAN1 work on configuration for LTE sidelink mode 4 is needed.

- Inform RAN2 of this conclusion and that the introduction of a capability to support this feature is left to RAN2.

# References

| No. | Tdoc | Type | Source | Title | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | R1-1908046 | decision | Huawei, HiSilicon | NR Uu control for LTE sidelink | present |
| 2 | R1-1908156 | discussion | vivo | Support of NR Uu controlling LTE sidelink | present |
| 3 | R1-1908289 | decision | Nokia, Nokia Shanghai Bell | Discussion of NR Uu controlling LTE sidelink | present |
| 4 | R1-1908370 | discussion | OPPO | On DCI signalling to control LTE sidelink and UE capability reporting | present |
| 5 | R1-1908402 | discussion | MediaTek | Support of NR-Uu controlling LTE sidelink | present |
| 6 | R1-1908483 | discussion | Samsung | On NR Uu controlling LTE sidelink | present |
| 7 | R1-1908586 | discussion | CATT | Disucssion on NR Uu controlling LTE sidelink | present |
| 8 | R1-1908640 | discussion | Intel | LTE sidelink communication under gNB control | present |
| 9 | R1-1908734 | discussion | Lenovo, Motorola Mobility | Discussion on support of NR Uu controlling LTE sidelink | present |
| 10 | R1-1908742 | decision | Futurewei | Support of NR Uu controlling LTE sidelink | present |
| 11 | R1-1908796 | discussion | ZTE, sanechips | Discussion on NR Uu controlling LTE sidelink | present |
| 12 | R1-1908892 | discussion | ITRI | The discussions on NR-Uu controlling LTE sidelink | present |
| 13 | R1-1908908 | discussion | LGE | Discussion on NR Uu controlling LTE sidelink | present |
| 14 | R1-1908919 | decision | Ericsson | NR Uu controlling LTE sidelink transmissions | present |
| 15 | R1-1909034 | discussion | InterDigital | Support of NR Uu Controlling LTE Sidelink | present |
| 16 | R1-1909259 | discussion | Qualcomm | NR Uu control of LTE sidelink | present |