3GPP TSG RAN WG1 Meeting #98	R1-1909814

Prague, CZ

August 26th – 30th, 2019

Agenda item:    7.2.2.1.1

Source:             Qualcomm Incorporated

Title:                  Feature lead summary on initial access signals and channels for NR-U

Document for:  Discussion and Decision

# Introduction

This paper provides a summary of the papers submitted to 7.2.2.1.1. It also serves as summary for offline discussion on the agenda item.

From RAN #84, the plenary provided guidance on the features to be prioritized as listed below. We will take that into consideration during the offline discussion.

Essential

- Wideband PRACH design (long sequence vs repetition)

- Supported PRACH formats (legacy PRACH and new PRACH)

- RMSI PDSCH to SSB rate matching (impacted by RAN4 sync raster decision, if the decision does not guarantee SSB placement at the edge of the initial DL BWP)

  - Also impact default PDSCH SLIV table configuration

- RMSI (PLMN) transmission in Scell

Optimizations

- CSI-RS FDM with SSB (impacted by RAN4 sync raster decision)

- Additional PRACH numerology

- Multiplexing of PRACH and other channels

- Whether to introduce LBT gap between ROs

# Agreements this meeting

Agreement:

No new PRACH formats are specified for operation in unlicensed spectrum.

# Discussions

## DRS

### SS/PBCH block transmission pattern within a slot

The following agreement was made in RAN1#96bis:

Agreement: Select one of the following options in RAN1 #97: - Alt 1: Legacy SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot - Support Type0-PDCCH in symbol (#6, #7) for length-2 coreset 0 and symbol #6 or symbol #7 for length-1 coreset 0 for the second SSB in a slot - FFS: configurable between symbols #6 and #7 for the length-1 coreset 0 for the second SSB in a slot - Alt2: New SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot - Alt 3: Legacy SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7) for coreset 0 for the second SSB in a slot - For SSB positions and type-0 PDCCH monitoring in a slot, only Alternatives 1 and 2 (from the previous agreement in RAN1#96bis) are considered further. - Note: As per the previous agreement one of Alternative 1 and 2 is to be selected at RAN1#97 Furthermore, the following was concluded in RAN1#97: - Conclusion: There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item.

Amongst the relevant contributions, the following options are explicitly proposed:

- Rel15 (1 company): Spreadrrum

- Alt-1 (1 company): ZTE

- Alt-2 (5 companies): Huawei, ZTE, Mediatek, Samsung, Nokia

- Alt-3 (1 company): Panasonic

| Company Name | Position |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: Alternative 2 shall be adopted in NR-U as SSB position and Type-0 PDCCH monitoring in a slot. - Support Type0-PDCCH in symbol (#0, #1) for length-2 CORESET 0 and symbol (#0) for length-1 CORESET 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7, #8) for length-2 CORESET and symbol (#7) for length-1 CORESET 0 for the second SSB in a slot |
| ZTE, Sanechips | Proposal 1: Either of Alt 1 and Alt 2 for SS/PBCH block patterns can be adopted for NR-U. |
| MediaTek Inc. | Proposal 1: Support SS/PBCH blocks to be located symbols (2, 3, 4, 5) and (9, 10, 11, 12) in a slot for NR-U. In other words, support Alt2 for SSB positions in a slot. |
| Samsung | Proposal 1: For NR-U DRS, Alt 2 shall be supported. |
| Nokia, Nokia Shanghai Bell | Proposal 3: Support Alt2: New SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot. |
| ETRI | Proposal 1: It is proposed to revisit the discussion on the SSB pattern for DRS design. |
| Panasonic Corporation | Proposal 2: Support Alt 3: Legacy SSB positions in a slot - Support Type0-PDCCH in symbol (#0, #1) for length-2 CORESET 0 and symbol (#0) for length-1 CORESET 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7) for CORESET 0 for the second SSB in a slot |
| Spreadtrum Communications | Proposal 1: The SSB pattern of Case A (15kHz SCS) and Case C (30kHz SCS) as defined in R15 FR1 is supported. |

Discussion:

Most companies noted the conclusion from RAN1#97 and – not being aware of any consensus, assumed that the Rel-15 pattern applies and thus made no explicit proposals.

Following the RAN1 #97 conclusion, we do not plan to spend any offline time to discuss this topic.

### SSB Transmission Candidates Opportunities, Repetition, and Timing Derivation

For NR, the initial access UE’s assume the SSB transmission period is 20ms. There are proposals to slow it down in NR-U to at least 40ms, matching the behaviour of DRS transmission in LTE-LAA.

Agreement: For SSB transmissions as part of DRS: - It is considered beneficial to expand the maximum number of candidate SSB positions within DRS transmission window to [Y], for e.g., Y = [64] - FFS: How to derive frame timing from detected SS/PBCH block - Transmitted SSBs do not overlap - FFS: Shift granularity between candidate SSBs positions/candidate groups of SSBs - Maximum number of transmitted SSBs is [X] within DRS transmission window. X <= 8 - FFS: Duration of DRS transmission window - FFS: Duration of the transmitted DRS within the window, including SSBs and other multiplexed signals/channels - FFS: relationship between transmitted SSB index and QCL assumption at UE - FFS: If and how to support beam repetition for soft combining of SSBs within the same DRS transmission Agreement: The maximum DRS transmission window duration is 5 ms. - The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS. - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions. - FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values. Agreement: For a given cell, the UE may assume that the PBCH DMRS sequence index is the same for SS/PBCH blocks that are transmitted at the same candidate positions across DRS transmission windows. Agreement: UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

DRS transmission window(s): this is the (set of) interval(s) where the pattern of SSB candidates is realized.

- Periodicity:

  - Vivo: 40 80, 160 ms

- Duration:

  - Vivo: 1-5 ms

- Configuration:

  - Depending on DRS LBT type (cat-2 vs cat-4): Nokia

| Company Name | Position |
| --- | --- |
| Vivo | Proposal 1: NR-U supports flexible configuration of DRS period (e.g. {40ms, 80ms, 160ms}) and DRS window duration (e.g. {1ms, 2ms, 3ms, 4ms, 5ms}). |
| Nokia, Nokia Shanghai Bell | Proposal 10: To support DRS transmissions not meeting the criteria for SCS (5% duty cycle, 1 ms duration), consider splitting DRS into two configurations, where the first DRS configuration uses Cat 2 LBT, and the second DRS configuration uses Cat 4 LBT. Exact interaction between the two configurations is FFS. |

Procedural aspects related to this topic are to be further discussed in 7.2.2.2.2.

### How to satisfy OCB requirement during SSB transmission

SS/PBCH block transmission itself will not satisfy the OCB requirement. Currently we agreed to allow other signals/channels to be multiplexed with SSB transmission, including RMSI PDSCH, CSI-RS, other PDSCH, etc.

Agreement: - Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for - Meeting OCB requirement - Compacting signals in time domain to limit the required number of channel access and for short channel occupancy - Support of stand-alone NR-U deployments - Support of automatic neighbour relations (ANR) functionality in an NR-U deployment - Resolution of PCI confusion in an NR-U deployment - Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals - The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial - Note: This does not imply that RMSI-CORESET+PDSCH and CSI-RS can only be transmitted as part of the NR-U DRS, and does not imply that these are necessarily part of all NR-U DRS transmissions. Conclusion: No changes are required to the time and frequency position of the PSS/SSS/PBCH relative to each other in one PSS/SSS/PBCH block. Agreement: - The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum. - CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS.

On FD SSB repetition for OCB:

- Not needed: Intel

On RMSI broadcast for NSA DRS:

- Should not support: Huawei

- Should support: Vivo, AT&T, ZTE, NTT DOCOMO

Offline discussion output:

The problem is identified as how to support RMSI transmission for ANR purpose for a cell not intended to support initial access.

Two alternative solutions captured so far:

Alt 1: gNB configures the UE to do ANR measurement report the CGI for a PCI on a given at an SSB at a frequency not on a sync raster point, and after the UE detectesd the SSB it will proceed with RMSI decoding with an implicit coreset0 configuration whereby the frequency location of the CORESET #0 scheduling the PDSCH carrying the RMSI is implicitly known

Alt 2: gNB configures the UE to do ANR measurement report the CGI for a PCI on a given at an SSB at a frequency not on a sync raster point and the MIB in SSB will point to the frequency location of the coreset #0

- This requires the SSB to coreset #0 offset design to support the offset between the coreset #0 and this particular SSB position

- FFS: number of CORESET 0 offsets configurable in MIB when SSB is not on sync raster entry

If RAN4 decides on one sync raster point per 20MHz, adopt Alt 1, else adopt Alt 2.

On CSI-RS multiplexing for OCB, a few companies bring up issues to further discuss (see section 2.1.7) regarding FDM/TDM needs, rate matching needs and dependency on the RAN4 sync raster discussion.

| Company Name | Position |
| --- | --- |
| Huawei, HiSilicon | Proposal 5: Considering the required resources and its functionality, RMSI transmission in Scell is unnecessary. |
| Vivo | Proposal 7: RMSI transmission in Scell could be one optional feature to solve PCI confusion problem through SON function and UE is not needed to read RMSI when RRM measurement. |
| ZTE, Sanechips | Proposal 5: RMSI (PLMN) should be allowed to be transmitted in SCells for NR-U in order to support PCI collisions/confusion solved by ANR. Proposal 6: Multiplexing CSI-RS and SS/PBCH block in frequency domain is impacted by sync raster placement. Thus how to FDM CSI-RS with SS/PBCH block can wait for RAN4 sync raster decision. |
| Intel Corporation | Proposal-2: From initial access point of view it is pre-mature to introduce multiple SSB locations in frequency with repetition |
| AT&T | Proposal: RMSI can be transmitted on SCells in Rel. 16 NR-U Proposal 2: Send an LS to RAN2 informing them about the decision |
| NTT DOCOMO, INC. | Proposal 7: RMSI transmission on NR-U SCell is possible and configurable as in Rel-15. |

On CSI-RS multiplexing with SSB in DRS: see section 2.1.7

On OCB when RMSI is not needed:

- there is little support for specifying FD repetition of SSB.

On broadcasting RMSI in NSA case:

- it was clarified in RAN1#96bis that proponents are mainly interested in the PLMN information in the RMSI

- further proposed discussion:

  - is this information mainly for SON ANR?

  - is it necessary to prohibit RMSI on NR-LAA SCells, given that NR-U SA is supported? If so, how would UE know that a cell is LAA-only?

Discussion (high priority):

Further discuss RMSI transmission in sCell.

- Is it mandated to transmit or gNB choice?

- If it is transmitted, what is the UE behaviour?

### Type0-PDCCH monitoring

Agreement: The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties: - TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed) - Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot - FFS start at symbol #6 of #7 or both - FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions) Agreement: - The SCS for all SSBs and Coreset #0 on a carrier is always the same for operation of NR in unlicensed spectrum. CORESET #0 frequency domain resource configuration should be 48 RBs for 30KHz SCS and 96 RBs for 15KHz SCS. Agreement: Only coreset #0 lengths of 1 and 2 symbols are supported for NR-U Furthermore, the following was concluded in RAN1#97: - Conclusion: There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item.

A few companies made proposals addressing changes to PDCCH monitoring:

- Symbol offset of PDCCH monitoring occasion for the second half-slot SSB: Huawei, ZTE, Mediatek, Samsung

- Slots for PDCCH monitoring: Nokia (same slot)

- Search spaces for PDCCH monitoring: Intel (one or two)

- NR_U-specific searchSpaceZero configuration with restriction on M/O values: ZTE, Mediatek, Samsung, NTT DOCOMO

A few companies noted that two-symbol CORESET0 is incompatible with two SSBs/slot for M=1/2 and O=0 or 5, for Rel-15-based configuration:

- LGE: proposes to assume that the second half-slot SSB and corresponding CORESET0 are not present

- Nokia: observed the restriction and concluded that both SSBs would be QCLed, if present in this case.

- Spreadtrum: proposes that M=1 with two SSBs in a slot associated to the same Type0-PDCCH be necessary when 2-symbol CORESET0 is used.

- NTT DOCOMO: UE assumes that the second CORESET0 is not present in the slot.

On QCL-SSB/CORESET0 determination the following views were provided:

- NTT DOCOMO: Q-factor to be broadcast in PBCH: (reuse bits currently used for controlResourceSetZero)

- Intel: Consider defining Type-0 PDCCH monitoring based on SSB beam index-b instead of SSB index-i

| Company Name | Position |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: Alternative 2 shall be adopted in NR-U as SSB position and Type-0 PDCCH monitoring in a slot. - Support Type0-PDCCH in symbol (#0, #1) for length-2 CORESET 0 and symbol (#0) for length-1 CORESET 0 for the first SSB in a slot - Support Type0-PDCCH in symbol (#7, #8) for length-2 CORESET and symbol (#7) for length-1 CORESET 0 for the second SSB in a slot |
| ZTE, Sanechips | Proposal 2: For NR-U, only configuration parameter = 0 or = 5, and = 1/2 in Table 13-11 in 3GPP TS 38.213 should be supported to let SS/PBCH block and its corresponding Type-0 PDCCH in the same slot. Wherein, first symbol index for starting monitoring Type-0 PDCCH should be changed from “{0, if is even}, { ,（=1,2,3） if is odd}” to “{0, if is even}, {6 or 7, if is odd}”. |
| MediaTek Inc. | Proposal 2: Similar to NR, NR-U supports that both CORESET and search space configurations for Type-0 PDCCH to be carried by PBCH. Proposal 4: Support Table 1 for search configuration for NR-U in FR1. Table 1: Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR1 Index; Number of search space sets per slot; First symbol index. 0; 0; 1; 1; 0. 1; 0; 2; 1/2; {0, if is even}, {7, if is odd}. 2; 5; 1; 1; 0. 3; 5; 2; 1/2; {0, if is even}, {7, if is odd}. 4; 0; 2; 1/2; {0, if is even}, {, if is odd}. 5; 5; 2; 1/2; {0, if is even}, {, if is odd}. 6; Reserved. 7; Reserved. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. |
| Samsung | Proposal 2: For NR-U DRS, • a UE assumes the periodicity of NR-U DRS transmission is the same as the periodicity of SS/PBCH block; • a UE monitors PDCCH in the Type0-PDCCH CSS set over one slot with Type0-PDCCH CSS set periodicity equal to the periodicity of the associated SS/PBCH block; • the slot index for monitoring PDCCH in the Type0-PDCCH CSS set is the same as the one containing the associated SS/PBCH block. Proposal 3: NR-U DRS shall support the following configurations of Type0-PDCCH CSS: Configuration index; Number of symbols for CORESET#0; Number of Type0-PDCCH CSS within a slot; Starting symbol for Type0-PDCCH CSS. 1; 1; 1; 0. 2; 1; 2; 0 if SS/PBCH block index is even, and 1 if SS/PBCH block index is odd. 3; 1; 2; 0 if SS/PBCH block index is even, and 7 if SS/PBCH block index is odd. 4; 2; 1; 0. 5; 2; 2; 0 if SS/PBCH block index is even, and 7 if SS/PBCH block index is odd. |
| LG Electronics | Proposal #1: If two SS/PBCH blocks (i.e., SS/PBCH block index n and SS/PBCH block index n+1) and corresponding 2-symbol CORESETs index 0 are configured to be transmitted in the same slot, UE assumes that neither a SS/PBCH block index n+1 nor CORESET index 0 associated with the SS/PBCH block is transmitted. Proposal #3: UE assumes that a SS/PBCH block corresponding to synchronization raster defined for NR-U provides type0-PDCCH common search space configuration. (i.e. “it can be proposed to mandate transmitting RMSI for a SS/PBCH block on synchronization raster for NR-U”) |
| Intel Corporation | Proposal-3: Consider the following design principles for Type-0 PDCCH monitoring slot configuration: - Consider defining Type-0 PDCCH monitoring based on SSB beam index-b instead of SSB index-i - Consider Type-0 PDCCH monitoring slots with the same periodicity and offset as the DRS transmission window - Consider 2 consecutive slot PDCCH monitoring - Consider both single search space set and 2 search spaces sets per slot configurations |
| Nokia, Nokia Shanghai Bell | Proposal 1: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions). Proposal 2: NR-U should also support half-slot-based DRS with transmitted RMSI. If only slot-based DRS is supported, as per current status, SSBs in the same slot have the same QCL assumption. |
| Spreadtrum Communications | Proposal 2: If the number of search space sets of Type0-PDCCH per slot is two, i.e. M=1/2, the two SSBs in a slot are associated with the two Type0-PDCCH respectively at the beginning of the slot, and duration of CORESET of Type0-PDCCH can be one symbol. Proposal 3: If the number of search space sets of Type0-PDCCH per slot is one, i.e. M=1, the two SSBs in a slot is associated with the same Type0-PDCCH at the beginning of the slot, and duration of CORESET of Type0-PDCCH can be one and two symbols. |
| NTT DOCOMO, INC. | Proposal 1: Monitoring of Type0 PDCCH of the 2nd SSB at symbol#7 is supported only for one symbol CORESET#0 case. When two symbols CORESET#0 is configured and Type0-PDCCH monitoring occasions for two search space sets are configured in a slot containing candidate SSB positions, UE assumes that 2nd SSB position in a slot is not used for SSB transmission so that an effective number of search space sets per slot is 1. Proposal 2: Two tables (one for 30kHz SCS case, another for 15kHz SCS case) for controlResourceSetZero configurations are newly defined for NR-U frequency bands. - For NR-U frequency bands, “SS/PBCH block and CORESET multiplexing pattern” is fixed to 1, “Number of RBs: ” is fixed to 96/48 (for 15/30 kHz SCS), and “Offset (RBs)” parameter is fixed to a single specific value according to SS raster design to be decided in RAN4. - For NR-U frequency bands, 4 bits of controlResourceSetZero configuration are used to indicate “Number of Symbols: ” by using 1 bit (i.e., 1 or 2) and “Value of Q” by using remaining 3 bits (e.g., Q={1, 2, …, 8}). Proposal 3: In NR-U frequency band, a UE monitors PDCCH in the Type0-PDCCH CSS set over multiple slots +n∙X within a 20 ms period. - X is determined based on the value of Q and number of search space sets per slot, e.g., X= ⌊Q∙M⌋, at least when SSB and corresponding RMSI PDCCH are to be transmitted in the same slot. - n is 0, 1, …, N-1 where N is the number of candidate SSB transmission positions for a certain SSB (i.e., same QCL properties) within DRS transmission window, at least when SSB and corresponding RMSI PDCCH are to be transmitted in the same slot. Proposal 4: One table for searchSpaceZero configurations is newly defined for NR-U frequency bands. - The configurability for value of “O”, “Number of search space sets per slot” and value of “M” as searchSpaceZero configurations index 0-7 for Rel-15 NR FR1 is reused. - The searchSpaceZero configuration also indicates whether only candidate SSB positions with even index are to be used or all candidate SSB positions are to be used. |

Discussion:

Given the lack of consensus for the Alt-1/2/3 discussion, RAN#97 concluded “No further online or scheduled offline discussions will be conducted on [the topic of SSB positions and type-0 PDCCH monitoring in a slot]”.

We thus propose that a high bar be placed on necessity of changes to the Rel-15 type-0 PDCCH monitoring. In particular, we believe the following topics to be optimizations, rather than essential topics:

- the symbol offset for second half-slot.

- number of slots for monitoring (Rel-15: two consecutive slots)

- number of search spaces in a slot (Rel-15: up to two TDM-ed spaces)

- restrictions on M/O values.

On the interaction of 2-symbol CORESET0 and the second SSB in a slot, the following paragraph in 38.213, section 10 appears relevant:

“If the UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in Subclause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell.”

The handling of Q-factor broadcast in PBCH needs further discusion.

- Type-0 PDCCH monitoring based on SSB beam index-b instead of SSB index-i

  - This can be considered in 7.2.2.2.1

- Q-broadcast in PBCH: this topics merits further discussion and can be taken up in this agenda item

FL Proposal: RAN1 to discuss whether Q-factor broadcast in PBCH is acceptable.

- Alt 1: PBCH includes Q factor indication, and the UE will only monitor for Type0-PDCCH corresponding to the QCL’ed SSBs

- Alt 2: PBCH does not include Q factor indication, and the UE has to monitor for Type0-PDCCH corresponding to all possible SSB positions.

### PDSCH resource allocation in DRS

In Rel-15, PDSCH allocations for Type-0 PDCCH cannot rate-match around SSBs (c.f. 3GPP TS 38.214, section 5.1.4): “When receiving the PDSCH scheduled with SI-RNTI in PDCCH Type0 common search space, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.”

Furthermore, Type-1 allocation (for RMSI) in Rel-15 only allows contiguous PDSCH PRBs.

| Company Name | Position |
| --- | --- |
| Huawei, HiSilicon | Proposal 2: NR-U shall support that RMSI PDSCH scheduled by Type-0 PDCCH is rate-matched around the corresponding SSBs. Proposal 3: DMRS in both RMSI PDSCH and PBCH can be used to decode RMSI PDSCH when RMSI PDSCH rate matching around SSB is allowed. |
| Vivo | Proposal 2: NRU supports RMSI rate matching around SSB if RAN4 decides that SSB can’t be located at or close to the edge of operation carrier. Proposal 3: RMSI are always assumed to be rate matching around SSB. Proposal 4: Bitmap ssb-PositionsInBurst indicates valid QCL index of actually transmitted SSBs and all candidate SSBs of indicated valid QCL index inside configured DRS transmission window is considered as ‘actually transmitted SSB’ for rate matching and other further operation (RO mapping, valid PDCCH &PUCCH determination and etc.) Proposal 5: NR-U UEs shall consider RMSI is rate matched around CSI-RS based on configuration or predefined pattern, i.e. REs scheduled for the RMSI PDSCH reception and overlapping with CSI-RS transmission are not used. Proposal 6: The default time domain resource allocation table for RMSI scheduling should be updated to favour NRU DRS transmission, e.g. including Type B allocation for symbol #8-13. |
| ZTE, Sanechips | Proposal 3: For NR-U, additional combinations of the start symbol S and the allocation length L can be supported in default PDSCH SLIV table as follows, assuming the legacy SSB located in symbols {2, 3, 4, 5} and {8, 9, 10, 11, 12} with Alt 1: ‐ {S=1, L=5}; {S=8, L=5}; {S=8, L=6}; {S=7, L=5}; {S=2, L=11}. Proposal 4: For NR-U, RMSI PDSCH should be able to be rate matched around SS/PBCH blocks in DRS occasion. The assumption for RMSI PDSCH and SS/PBCH blocks in 3GPP TS 38.214 should be revisited. |
| MediaTek Inc. | Proposal 5: Rate matching RMSI around SSBs can be considered if RAN4 decides not to place sync raster around the edges of channels. |
| OPPO | Proposal 1: Multiple FDM fashions between RMSI-PDSCH and SSBs should be further considered in NR-U. Proposal 2: DMRS sharing among channels within DRS should be considered in NR-U. |
| Samsung | Proposal 4: For NR-U DRS, PDSCH of RMSI can rate match around SS/PBCH block(s) within the same slot. Proposal 5: The DCI format for RMSI (e.g. DCI format 1_0 with CRC scrambled by SI-RNTI) indicates the actually transmitted SS/PBCH blocks within the slot. |
| LG Electronics | Proposal #2: Consider following options for a UE to recognize rate-matching pattern for RMSI PDSCH around SS/PBCH block in the same slot and Option 2 is preferred since it does not require any additional signalling overhead. - Option 1: Dynamic indication of rate-matching information for RMSI PDSCH (e.g., via PBCH, RMSI PDCCH) - Option 2: Implicit indication of rate-matching information for RMSI PDSCH - If RMSI PDSCH resource is overlapped with the resource for associated SS/PBCH block, UE assumes that the SS/PBCH block is transmitted. If RMSI PDSCH resource is overlapped with the resource for not associated SS/PBCH block, UE assumes that the SS/PBCH block is not transmitted. |
| Intel Corporation | Proposal-4: Consider RMSI rate-matching around SSBs to allow more flexibility for NR-U channel raster and SS raster design. |
| Nokia, Nokia Shanghai Bell | Proposal 4: Support rate matching around SSB(s) for PDSCH scheduled with SI-RNTI within a DRS signal. Observation 7: Wideband DMRS of CORESET#0 PDCCH could be used for channel estimation to receive RMSI-PDSCH. Observation 8: CORESET#0 PDCCH DMRS could be allocated upon bandwidth of RMSI-PDSCH which may be larger than bandwidth of CORESET#0 (e.g. 51 vs 48 with 30 kHz SCS). Proposal 5: At least within DRS, CORESET#0 PDCCH DMRS span over entire sub-band. Observation 9: One option to support CSI-RS configuration within the DRS is to define time domain rate matching for PDSCH carrying RMSI. Proposal 6: Support dynamic symbol-level rate-matching for PDSCH scrambled with SI-RNTI in DRS. Proposal 8: If also half-slot-based DRS is support, for NR-U introduce new RMSI-PDSCH allocation Table, which supports at least the following new starting symbol S and length L value pairs: • S = 1, L = 5 (one symbol for LBT gap in the end of DRS; new Type B) • S = 1, L = 12 (one symbol for LBT gap in the end of DRS, new Type A) • S = 2, L = 11 (one symbol for LBT gap in the end of DRS; new Type A) • S = 8, L = 6 (new Type B) • S = 8, L = 5 (one symbol LBT gap in the end of DRS; new Type B) • S = 9, L = 5 (new Type B) Proposal 9: If only full-slot-based DRS is support, for NR-U introduce new RMSI-PDSCH allocation Table, which supports at least the following new starting symbol S and length L value pairs: • S = 1, L = 12 (one symbol for LBT gap in the end of DRS, new Type A) • S = 2, L = 11 (one symbol for LBT gap in the end of DRS; new Type A) |
| ETRI | Proposal 2: Add an indication in RMSI PDCCH regarding rate-matching of RMSI PDSCH around SSB(s) within a DRS - FFS: details of indication method (e.g., corresponding SSB or (other) candidate SSB or both) Proposal 3: For RMSI PDSCH resource allocation, the mapping type B can be used. - In order to support half-slot-based DRS, 5 and 6 symbols for PDSCH duration should be mandatory. Proposal 4: DM-RS shifting to an available symbol should be considered for RMSI PDSCH DM-RS when PDSCH mapping type B overlaps with SSB(s). |
| Spreadtrum Communications | Proposal 4: If PDSCH rate matching around SSB(s) within the same slot is supported, it can be considered in the following cases. - The case where the search space set of broadcast PDCCH scheduling the broadcast PDSCH is search space zero. - The case where the search space set of unicast PDCCH scheduling the unicast PDSCH is search space zero. Proposal 5: If RMSI PDSCH rate matching around SSB(s) within the same slot is supported, “frequency-region level rate matching” can be supported, i.e. UE may assume the frequency resource in all symbols in the RBs overlapping with SSB is not used in resource mapping. “whether to support RMSI PDSCH rate matching around SSB still needs RAN4 feedback.” |
| Sharp | Proposal 1: SIB1 PDSCH is not mapped to the same frequency location as the SS/PBCH block when the PDSCH overlaps with the SS/PBCH block in time domain. Proposal 2: The UE doesn’t assume any SS/PBCH block candidates as “reserved resources” for SIB1 PDSCH if RAN4 decision is SS/PBCH block allocated to the 20 MHz bandwidth edge (same as Rel-15 NR). Proposal 3: The UE assumes SS/PBCH block bandwidth as “reserved resources” for SIB1 PDSCH if RAN4 decision is SS/PBCH block can be allocated in the middle of the 20 MHz bandwidth. Proposal 4: Default PDSCH time domain resource allocation A for normal CP supports (S, L) = (2, 6) with type A or type B, (S, L) = (8, 6) with type B, (S, L) = (2, 5) with type A or type B and (S, L) = (8, 5) with type B. |
| NTT DOCOMO, INC. | Proposal 5: For NR-U, the new RMSI PDSCH time domain resource allocation table is defined with at least following configurations. - Existing configurations of Row index {1, 2, 12, 13, 14, 16} - New configurations with {S,L}={8,6}, {1,12}, {2,11} Proposal 6: For NR-U, the UE rate matching behaviour around candidate SS/PBCH block transmission resources for receiving RMSI PDSCH is defined. - If it is indicated that only candidate SSB positions with even index are to be used, UE assumes PDSCH is transmitted on the resource which collides with candidate SSB resource (except for the SSB resource associated with the scheduled RMSI). - If it is indicated that all candidate SSB positions are to be used, UE assumes SSB is transmitted on the candidate SSB resource. |
| Beijing Xiaomi Mobile Software | Proposal 2: Rate-Match of RMSI-PDSCH over SSB should be supported in NR-U at least for some certain case. |
| Qualcomm Incorporated | Proposal 2. RMSI PDSCH rate matching around SSB is not supported in NR-U. |
| Ericsson | Proposal 2 In line with NR Rel-15, mapping of SIB1 around an SS/PBCH block is not supported. |
| WILUS Inc. | Proposal 1: As a default PDSCH SLIV table configuration for NR-U operation, symbol #2 at the 1st half slot and symbol #8 at the 2nd half slot in a slot can be used as the starting symbol of RMSI-PDSCH with 4 or 5-symbol length regardless of CORESET#0 symbol length. Proposal 2: - If RMSI PDSCH resources collides with associated candidate SS/PBCH block transmission on symbols in a same slot, then the UE assumes that SS/PBCH block is transmitted on the collision resource. - If RMSI PDSCH resource does not collide with associated candidate SS/PBCH block transmission and occurs collision with other candidate SS/PBCH block transmission, then the UE assume that RMSI PDSCH is transmitted on the resource |

Multiplexing of RMSI PDSCH and SSB in DRS: Companies have expressed the following positions:

a) Allow or evaluate RM around SSB - 10 or 11 companies:

- Huawei, ZTE, OPPO, Samsung, LG Electronics, Intel (?), Nokia, ETRI, NTT DOCOMO, Xiaomi, WILUS

b) Allow or evaluate RM around SSB, if RAN4 places SSB in the centre of the band; these companies propose or seem open to FD-RM (i.e. all frequencies used by SSBs are considered reserved, not just the SSB REs) – 4 or 5 companies

- Vivo, Mediatek, Spreadtrum, Sharp, Intel (?)

c) No RM around SSB – 2 companies

- Qualcomm, Ericsson

d) Allow/consider RM around CSI-RS – 2 companies

- Vivo, Nokia

Discussion:

Though we have the following conclusions from RAN1 #97 “There is no consensus between Alternative 1 and Alternative 2 on the topic of SSB positions and type-0 PDCCH monitoring in a slot. No further online or scheduled offline discussions will be conducted on this topic. If there is consensus on such enhancements, the topic can be revisited in the Rel-16 NR-U work item”, there are proposals to change type0-PDCCH monitoring configuration

- Alt 1: Legacy type0-PDCCH monitoring pattern supported and it is up to gNB choice which to deploy to avoid collisions between SSBs and RMSI

- Alt 2: Some of the type0-PDCCH monitoring patterns are invalid for NR-U, and only keep O=0 and M=1/2 entries

- Alt 3: Reuse legacy pattern 2 and pattern 3 defined for FR2 for NR-U FR1.

FL Discussion (high priority):

- Further discuss whether Rate-Match support of RMSI-PDSCH over SSB is warranted for NR-U, RAN1 shall consider the following aspects regarding rate-matching of RMSI PDSCH around SSB in DRS:

  - Which SSBs to rate-match around, and how that information is conveyed to the UE

  - Handling of DMRS, in particular for DMRS symbols shared with SSB; two options are proposed:

    - DMRS shifting

    - DMRS sharing with other channels.

  - Whether the feature provides sufficient benefits, to justify the extra complexity and spec impact, in particular absent decisions on Alt-1/2/3 for SSB position

This discussion will be delayed after RAN4 has a conclusion on sync raster

New PDSCH allocations for RMSI: a few companies express openness to consider additional PDSCH allocation SLIV entries, for optimal utilization of DRS resources:

- Optimizing half-slot RMSI allocation: Vivo, ZTE, Nokia, NTT DOCOMO, WILUS

- Optimizing full-slot RMSI allocation: ZTE, Nokia, NTT DOCOMO

- Allowing LBT at the end of DRS: ZTE, Nokia, NTT DOCOMO

Discussion:

- Further discuss if new Type-B PDSCH length (other than 2/4/7) is mandated for NR-U for initial access purpose

- Do we need to change the Default A PDSCH table?

  - Half-slot SSB optimisations. Note that the placement of CORESET0 is at the beginning of the slot without consensus to change the Type0-PDCCH monitoring pattern

  - Full-slot SSB optimisations: lengths 10, 11 and 12 are not optimized for single-symbol LBT at the end of the slot: is it essential to add such an optimization to Default A?

This discussion will be delayed after there is a conclusion on PDSCH rate matching around SSBs

### Paging/OSI and DRS COT

In 38.889, the following was captured:

The enhancements identified for physical layer signal and channels are captured in Section 7.2.1.2. For SS/PBCH block transmission in NR-U DRS, it has been identified as beneficial to include CSI-RS and RMSI-CORESET(s) and RMSI-PDSCH(s) in the same contiguous burst when transmission of CSI-RS/RMSI are configured. Optionally OSI and paging can be transmitted in the same DRS if there are available resources.

| Company Name | Position |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: Inclusion of RMSI, OSI and paging in DRS is indicated by PBCH. |
| Samsung | Proposal 7: At least one of the following set of configurations on Type0A/2-PDCCH CSS should be supported for NR-U DRS, and PDCCH/PDSCH of OSI/paging are wrapped-around together with the transmission of the QCLed SS/PBCH blocks: • Option 1: A burst of OSI/Paging is transmitted right after the burst of SS/PBCH blocks; • Option 2: OSI/Paging shared the same burst with SS/PBCH blocks by muting some SS/PBCH block transmissions. |

Two companies discussed the inclusion of OSI and Paging in DRS (Huawei, Samsung).

- Huawei considers that the multiplexing of Paging and OSI in DRS can be signalled in PBCH.

- Samsung indicates two TDM multiplexing option for paging: inside DRS and immediately following DRS

Discussion:

- Further discuss what modifications are needed to ensure that paging can, when sufficient resources exist, be transmitted in the same COT as DRS:

  - Whether Type-2 PDCCH monitoring can be configured:

    - inside SSBs slots in the SS Burst

    - immediately after the SS Burst, in the same COT

  - Impact on PDSCH scheduled by Type-2 PDCCH, and potential SLIV table changes

  - How/whether UE knows whether paging and DRS are located in the same COT.

### CSI-RS multiplexing in DRS

| Company Name | Position |
| --- | --- |
| Vivo | Proposal 5: NR-U UEs shall consider RMSI is rate matched around CSI-RS based on configuration or predefined pattern, i.e. REs scheduled for the RMSI PDSCH reception and overlapping with CSI-RS transmission are not used. |
| ZTE, Sanechips | Proposal 6: Multiplexing CSI-RS and SS/PBCH block in frequency domain is impacted by sync raster placement. Thus how to FDM CSI-RS with SS/PBCH block can wait for RAN4 sync raster decision. |
| Samsung | Proposal 6: CSI-RS can be configured in the symbols not overlapping SS/PBCH block in NR-U DRS. |
| Intel Corporation | Proposal-5: Consider periodic CSI-RS with slot offset defined with respect to an associated SSB |
| Nokia, Nokia Shanghai Bell | Observation 9: One option to support CSI-RS configuration within the DRS is to define time domain rate matching for PDSCH carrying RMSI. Proposal 6: Support dynamic symbol-level rate-matching for PDSCH scrambled with SI-RNTI in DRS. Observation 10: TRS could be mapped around second SSB in the slot l ϵ{7,12} or l ϵ{8,13} depending on SSB location in the slot. Proposal 7: Consider new time domain locations for TRS symbols within a slot carrying DRS, e.g. around first and/or second SSB. |
| Qualcomm Incorporated | Observation 3: SSB transmission PRBs is restricted by the following requirements: - when CSI-RS (at least 24 PRBs) is used to fulfil the OCB requirement, it should neither puncture nor be punctured by SSB. - if RMSI PDSCH is restricted to type-1 Rel-15 frequency allocation, FDM-ed SSB should be transmitted in the lowest or highest PRBs of the 20 MHz channel. Proposal 4: RAN4 should account for the considerations of the above observation, when considering the sync raster point(s) in each 20 MHz channel. |

Companies consider the inclusion of CSI-RS in DRS for two purposes:

- RRM / Tracking: proponents are mostly interested in full-band CSI-RS configurations.

  - CSI-RS configured on symbols outside SSB/RMSI: Samsung, Nokia

    - Nokia proposes “new time domain locations for TRS symbols within” DRS slot

  - RM RMSI around CSI-RS: Vivo, Nokia,

- OCB fulfilment: proponents tend to focus on FDM.

  - FDM with RMSI is not possible if RMSI uses all frequencies unoccupied by SSB (to maximize capacity in the SSB slot): Nokia => consider RM-ing RMSI PDSCH around CSI-RS.

  - FDM with SSB is already possible, if RAN4 agrees to SSB at the edge of the channel band: Qualcomm, ZTE

Configuration/trigger of CSI-RS multiplexing in DRS slot:

- Intel proposes to use P-CSI-RS instead of A-CSI-RS, triggered by the presence of SSB

Discussion: Further discuss enhancements needed to multiplex CSI-RS in DRS:

- Wait for the RAN4 outcome before further discussions on FDM of CSI-RS with SSB.

- For full-band CSI-RS allocations discuss:

  - Whether CSI-RS can be configured in separate symbols, or RM is necessary

    - If RM is needed, how does UE become aware of the need to RM, and of the CSI configuration to RM around?

  - the need for new time domain locations for TRS symbols

### Sync raster

| Company Name | Position |
| --- | --- |
| ZTE, Sanechips | Proposal 6: Multiplexing CSI-RS and SS/PBCH block in frequency domain is impacted by sync raster placement. Thus how to FDM CSI-RS with SS/PBCH block can wait for RAN4 sync raster decision. |
| MediaTek Inc. | Proposal 3: For Type0-PDCCH CORESET configuration, down-select one of the following values for the offset between CORESET#0 and SSB by considering RAN4’s decision on sync raster. - For SCS=15kHz, {38, 0, 76} PRBs - For SCS=30kHz, {14, 0, 28} PRBs Proposal 5: Rate matching RMSI around SSBs can be considered if RAN4 decides not to place sync raster around the edges of channels. |
| Samsung | Proposal 8: Single synchronization raster entry for each 20 MHz nominal channel, and fixed frequency offset between the lowest RB of CORESET#0 and the lowest RB of SS/PBCH block, wherein the offset is to be down-selected in RAN4: - Option 1: Fixed frequency offset is 0 RB (edge aligned); - Option 2: Fixed frequency offset is 14 RBs (center aligned). |
| Intel Corporation | Proposal-4: Consider RMSI rate-matching around SSBs to allow more flexibility for NR-U channel raster and SS raster design. |
| Spreadtrum Communications | “whether to support RMSI PDSCH rate matching around SSB still needs RAN4 feedback.” |
| Sharp | Proposal 2: The UE doesn’t assume any SS/PBCH block candidates as “reserved resources” for SIB1 PDSCH if RAN4 decision is SS/PBCH block allocated to the 20 MHz bandwidth edge (same as Rel-15 NR). Proposal 3: The UE assumes SS/PBCH block bandwidth as “reserved resources” for SIB1 PDSCH if RAN4 decision is SS/PBCH block can be allocated in the middle of the 20 MHz bandwidth. |
| NTT DOCOMO, INC. | Proposal 8: Although SSB frequency position for NR-U is up to RAN4 decision, it should be close to the edge in order to keep Rel-15 CSI-RS design. |
| Qualcomm Incorporated | Observation 3: SSB transmission PRBs is restricted by the following requirements: - when CSI-RS (at least 24 PRBs) is used to fulfil the OCB requirement, it should neither puncture nor be punctured by SSB. - if RMSI PDSCH is restricted to type-1 Rel-15 frequency allocation, FDM-ed SSB should be transmitted in the lowest or highest PRBs of the 20 MHz channel. Proposal 4: RAN4 should account for the considerations of the above observation, when considering the sync raster point(s) in each 20 MHz channel. |
| Ericsson | Proposal 1 For CORESET0 bandwidth of 48 PRBs and {SS/PBCH block, PDCCH} SCS of {30, 30} kHz support [0 and 1] PRB offset between CORESET0 and SS/PBCH blocks. |

This topic will be discussed further in RAN4.

From RAN1 perspective:

- Consequences if SSB is not close to the edge (within [0/1/2/3/4] RBs) of initial DL BWP

  - RMSI PDSCH rate matching around the RB or resources occupied by SSB is needed, or introducing type 0 frequency domain resource allocation for RMSI PDSCH scheduling, or limiting the SIB1 payload size

- Consequences if SSB is close to the edge (within [0/1/2/3/4] RBs) of initial DL BWP

  - None identified so far, though some enhancements within RAN1 are being considered

Note: RAN1 is discussing some of the above solutions for other benefits.

### Other (LBT, DRX Indicator)

For Connected DRX, MediaTek points out that a DRX Indicator in DRS would help UE save power and avoid UE missing PDSCH. We also note that at least one similar proposal was made in the agenda item for section 7.2.2.2.2.

Other companies discuss the LBT category to use for DRS.

| Company Name | Position |
| --- | --- |
| MediaTek Inc. | Proposal 6: NR-U supports DL transmission indicator for UE power saving and to avoid missing PDSCH due to DRX. The DL transmission indicator could be transmitted within NR-U DRS to avoid LBT overhead. |
| Nokia, Nokia Shanghai Bell | Proposal 10: To support DRS transmissions not meeting the criteria for SCS (5% duty cycle, 1 ms duration), consider splitting DRS into two configurations, where the first DRS configuration uses Cat 2 LBT, and the second DRS configuration uses Cat 4 LBT. Exact interaction between the two configurations is FFS. |
| Beijing Xiaomi Mobile Software | Proposal 1: Cat2 LBT for DRS transmission in one COT should be supported with some specification modification. |

The LBT topic is further discussed in the channel access agenda item.

Time permitting, RAN1 should consider the proposal of a DRX indicator in DRS.

## PRACH

In NR release 15, the short PRACH formats occupy 12 contiguous RBs which correspond to 2.16/4.32MHz for SCS of 15/30 kHz. The operation in unlicensed spectrum have various regulatory requirements like minimum occupied channel bandwidth (OCB), transmit power limits (EIRP, PSD), channel access (LBT, MCOT), etc. Based on these requirements the following guidance was provided in RAN#84 on the prioritization of work items related to PRACH for NRU:

Essential

- Wideband PRACH design (long sequence vs repetition)

- Supported PRACH formats (legacy PRACH and new PRACH)

Optimizations

- Additional PRACH numerology

- Multiplexing of PRACH and other channels

- Whether to introduce LBT gap between ROs

### PRACH Numerology

Description: The SCS to support for PRACH is yet to be determined. The positions of different companies are provided in the table below:

| Company | Position |
| --- | --- |
| ZTE | Proposal 12: 15 kHz SCS and 30 kHz SCS should be supported in below 7 GHz for NR-U. |
| Samsung | Proposal 12: NR-U shall support 30 kHz SCS only for PRACH. |
| Intel | Proposal-13: Do not support 60 kHz SCS for NR-U PRACH in FR. |
| Xiomi | Proposal 3: SCS of 60 kHz is not considered for NR-U PRACH |
| Qualcomm | Proposal 6: Only 15 and 30 kHz SCS will be supported for NR-U PRACH. |
| Huawei | 15, 30 and 60 kHz SCS are supported. Proposal 6: For SCS 60 kHz, L_RA=139. |

Feature lead summary: A summary of the company positions on SCS to support for PRACH in provided below:

- 15 kHz: Qualcomm, ZTE, Intel, Xiomi, Huawei

- 30 kHz: Qualcomm, Samsung, ZTE, Xiomi, Intel, Huawei

- 60 kHz: Huawei

FL Proposal:

- Only SCS of 15 and 30 kHz is supported for NR-U PRACH.

### PRACH frequency mapping options and evaluation results

The following was agreed in RAN97 ([1]):

Agreement

For a new enhanced design of NR-U PRACH in addition to the Rel-15 design (sequence length of 139) further discussion is limited to the following options

- ZC sequence of the following lengths

- 15 kHz: Choose one of L_RA= [571, 1151]

- 30 kHz: Choose one of L_RA= [283, 571]

- Repetition of Rel-15 PRACH sequences in frequency domain with potentially some mechanisms to improve the cubic metric

  - Consider one of 2 and 4 repetitions for 30 kHz and one of 4 and 8 repetitions for 15 kHz

- Note: Decision will be based on previously agreed evaluation metrics, capacity per cell (i.e., number of preambles per RACH occasion and number of RACH occasions) for the same time and frequency resources, specification impact and implementation complexity.

- Note: Companies should state any deviations in assumptions from the agreed evaluation assumptions.

Following table summarizes the different company proposals:

| Company | Position |
| --- | --- |
| Huawei | Observation 7: A repeated length-139 ZC sequence: - performs worse than a single ZC sequence of similar length in terms of CM, MCL and PRACH capacity - results in worse PRACH capacity than what NR Rel-15 supports - needs deployment and specification of sequence manipulations for CM reduction - needs a new method for specifying the cyclic shifts - needs a new method for specifying the root index ordering Proposal 6: The NR-U PRACH is based on a single long ZC sequence with the following lengths: - For 15 kHz SCS: LRA=1151 - For 30 kHz SCS: LRA=571 - For 60 kHz SCS: LRA=139 Proposal 7: The following cyclic shifts are used for ZC sequences of length NZC: - If NZC=283: NCS∈0,4,5,6,7,8,9,10,11,12,14,16,20,23,28,35 - If NZC=571: NCS∈0,8,10,11,12,14,16,19,21,24,28,33,40,47,57,71 - If NZC=1151: NCS∈0,17,20,23,26,29,33, 38,44,50,57,67,82,95,115,143 Proposal 8: The logical root indices 0,1,2,3,4,5,… are mapped to the sequence root indices as 1, NZC-1,2, NZC-2, 3, NZC-3,…. |
| VIVO | Observation 1: Obvious CM reduction can be achieved if phase rotation is applied to preamble repetitions when preamble repeated 4 and 8 times in frequency. Observation 2: Long sequence and preamble repetition can achieve similar MCL performance. Observation 3: More number of preamble repetitions in frequency domain does not necessary bring about higher MCL. Observation 4: The phase rotation applied on preamble is transparent to the receiver since gNB can detect the preambles without knowing the phase rotations on the preamble repetitions. Observation 5: NRU PRACH occupying about half of the resource in sub band, i.e. 4 repetitions for 15 kHz and 2 repetitions for 30 kHz, have advantages over preamble repeated across the whole sub band in multiplexing with other channels. Observation 6: Rel-15 length 139 preamble scheme have less spec impact compared with long preamble sequence with or without repetition. Proposal 9: Length-139 preamble based repetition should be adopted for NRU PRACH. Proposal 10: Phase rotation should be applied to the preamble repetitions to limit the cubic metric of NRU PRACH with multiple preamble repetitions where the phase rotation combination for multiple ROs is defined (e.g. Table 1) Proposal 11: The number for preamble repetitions is preferred to be 4 and 2 for SCS = 15 kHz and 30 kHz, respectively. |
| ZTE | Proposal 7: New Long PRACH sequences should not be introduced in Rel-16 NR-U. Observation 1：For SCS=30KHz, in order to obtain the optimal CM by the repetition scheme, the following rules exist between the repetitions. - The four repetitions have two different phase rotation values, the phase relationship of the two phase values is , the first repetition has the same phase rotation value as the last one, and the middle two repetitions have the same phase rotation value. Observation 2：For SCS=15KHz，in order to obtain the optimal CM by the repetition scheme, the following rules exist between the repetitions. The eight repetitions have three different phase rotation values, the first repetition has the same phase rotation value as the last one, and the most middle two repetitions have the same phase rotation value of the repetition, the rest of the repetitions have the same phase rotation value of the repetition. There is a phase relationship between the first two repetitions, while a phase relationship between the first repetition and the forth repetition. Proposal 7: New Long PRACH sequences should not be introduced in Rel-16 NR-U. Proposal 8: Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers with repetitions in frequency can be supported for NR-U. Proposal 9: For 4 repetitions for 30KHz SCS and 8 repetitions for 15KHz SCS with the phase rotation can be further studied and discussed. Proposal 10: The phase rotation method1 can significantly reduce the CM value of the repeating schemes. |
| Fujitsu | Proposal 1: In order to meet OCB requirement for PRACH transmissions on unlicensed band, repetition of Rel-15 NR PRACH short sequences in frequency domain should be supported in NR_U. |
| MTK | Proposal 7: Long-sequence (ZC=839) based NR PRACH preamble formats are not supported in NRU |
| Samsung | Proposal 9: NR-U shall support reusing legacy Rel-15 NR PRACH sequence with repetitions in frequency domain. Proposal 10: Repetitions of PRACH sequence can be mapped to contiguous RBs for Alt1. |
| LG | Observation #1: In 30 kHz subcarrier spacing, four PRACH sequence repetition and the length-571 contiguous mapping PRACH sequence could obtain almost same MCL. Observation #2: In 15 kHz subcarrier spacing, eight PRACH sequence repetition and the length-1151 contiguous mapping PRACH sequence could obtain almost same MCL. Observation #3: Single long PRACH sequence with contiguous mapping is not preferred (thus, not supported) for NR-U. Proposal #4: Support frequency domain repetition of Rel-15 PRACH sequences with cyclic shift and phase shift as PRACH preamble structure for NR-U. - For 15 kHz subcarrier spacing, - Number of repeated sequences: 8 - Cyclic shift pattern: [a, a+1, a, a+1, a, a+1, a, a+1] - Phase shift pattern: [1, 1i, 1i, -1, -1, 1i, 1i, 1] - For 30 kHz subcarrier spacing, - Number of repeated sequences: 4 - Cyclic shift pattern: [a, a+1, a, a+1] - Phase shift pattern: [1, 1i, 1i, 1] |
| Intel | Proposal-9: In addition to the Rel-15 design for NR short PRACH (sequence length of 139), support enhanced design of NR-U PRACH by adopting single long ZC sequence of the following lengths: - 15 KHz: LRA = 1151 - 30 KHz: LRA = 571 Proposal-10: Support similar mapping rule as for Rel-15 NR short PRACH (sequence length of 139) to map logical index to sequence number (or root index) for NR-U PRACH with longer sequence lengths. - Use a subset of logical indices to restrict high cubic metric root indices |
| NOKIA | Observation 11: Based on WID, new enhanced design of PRACH for NR-U should support minimum bandwidth requirement (16 MHz) by regulation to be applicable in UE initiated COT consisting of PRACH transmission. Proposal 11: The following options are in scope (to meet the OCB requirement as required by the WID) - 15 kHz SCS - Single sequence with length of 1151 - 8 repetitions of length 139 - 30 kHz SCS - Single sequence with length of 571 - 4 repetitions of length 139 |
| NEC | Proposal 1: For a new enhanced design of NR-U PRACH, ZC sequence of length 1151 for 15 kHz and ZC sequence of length 571 for 30 kHz are proposed. Proposal 2: For NCS design of new length, it’s beneficial that the NCS of time domain window is similar as that of legacy length, i.e. 139. |
| NTT | Proposal 9: Longer ZC sequence with L_RA = 1151/571 for 15/30kHz is specified for wideband PRACH design. |
| Qualcomm | Observation 8: Long ZC sequence with contiguous allocation has the maximum capacity for both SCS =15 and 30 kHz. Observation 9: Long ZC sequence with contiguous allocation have lower CM than repeated 139 length ZC sequences even with the application of CM reduction methods. Observation 10: Long ZC sequence with full band contiguous allocation with L_RA=1151 and 571 have better MCL than long ZC sequence with half band contiguous allocation L_RA=571 and 283, respectively for 15/30 kHz SCS. Observation 11: Long ZC sequence with contiguous allocation provides the best MCL value for both SCS =15 kHz and 30 kHz. Note that, the P_TX computation is pessimistic. If the P_TX formula is updated as min(P_max, 23+X-Backoff)), X =2, then Alt4 (long continuous allocation) has even better MCL gains. Proposal 12: Long ZC sequence with contiguous frequency allocation should be used for NRU PRACH. Following ZC sequence lengths are supported: - For 15 kHz: L_RA=1151 - For 30 kHz: L_RA=571. |
| Ericsson | Proposal 3: For the enhanced PRACH sequence design, support repetition of the Rel-15 Zadoff-Chu sequence of length LRA = 139. For 30 kHz SCS, support 2 repetitions with sequence values mapped to 278 contiguous subcarriers. For 15 kHz SCS, support 4 repetitions with sequence values mapped to 556 contiguous subcarriers. In both cases, support a CM reduction mechanism using a phase ramp + common phase shift per repetition. The following items are FFS: - Phase ramp and common phase shift per repetition, e.g., values in Table 5 and Table 6 in [8]. - Supported values of cyclic shift spacing NCS as well as mapping of logical sequence index i to sequence number u with minimal change to Rel-15. Proposal 4: Support a parameter in the cell-specific RACH configuration (i.e., RACH-ConfigCommon) which indicates to the UE which PRACH sequence mapping shall be used. The configuration choices are (1) legacy Rel-15 sequence mapping, or (2) enhanced Rel-16 mapping. The enhanced Rel-16 mapping details are to be agreed in RAN1 separately. As in Rel-15, the cell-specific RACH configuration is provided to the UE either through SIB1 or dedicated signalling depending on if the cell is a SpCell or SCell, respectively. |

Feature lead summary:

12 companies have given their positions for frequency mapping options. Out of these, 7 companies have provided evaluation results in the current submission. The positions of different companies on PRACH sequence to frequency mapping are summarized below:

- ZC sequence of following length:

  - 15 kHz: 1151 and 30 kHz: 571

  - Companies in support: Qualcomm, NTT, NEC, Intel, Nokia, Huawei

- Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers, with repetitions in frequency:

  - 15 kHz 4 repetitions, 30 kHz 2 repetitions: Ericsson, VIVO

  - 15 kHz 8 repetitions, 30 kHz 4 repetitions:  Nokia,,

  - Repetition in general: Fujitsu, Samsung, Vivo, ZTE, E///, LG

The complete set of results from all companies in this submission are given in the appendix. We summarize below a subset of results which includes alternatives proposed by the companies for comparison:

- SCS=30 kHz, 15 kHz

- L_RA=139 with repetitions, number of reps = 2, 4 (30 kHz) and 2, 8 (15 kHz)

- Long ZC sequence L_RA = 283, 571 (30 kHz), 571, 1151 (15 kHz)

SCS=30 kHz evaluation results:

| Company | ZC-139 with 2 repetitions |  |  |  | ZC-139 with 4 repetitions |  |  |  | ZC-L_RA (L_RA=283) |  |  |  | ZC-L_RA (L_RA=571) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW occupied (MHz) | MCL | Network Capacity | Cell Capacity (#FD ROs*64) | BW occupied (MHz) | MCL | Capacity | Cell Capacity (#FD ROs*64) | BW occupied (MHz) | MCL | Network Capacity | Cell Capacity (#FD ROs*64) | BW occupied (MHz) | MCL | Network Capacity | Cell Capacity (#FD ROs*64) |
| ZTE | 8.34 |  |  | 128 | 16.68 | 130.27 | 1380(1)/64(2) | 64 | 8.490 |  |  | 128 | 17.13 | 129.17 | 11970(1)/64(2) | 64 |
| Huawei |  | 126 | 3036 |  |  | 128.1 | 1518 |  |  |  |  |  |  | 128.5 | 7980 |  |
| VIVO |  | 127.2 | 1518 |  |  | 128 | 1518 |  |  |  |  |  |  | 128.6 | 8970 |  |
| Intel |  |  |  |  |  | 126.9 |  |  |  |  |  |  |  | 128.2 |  |  |
| LG |  | 128.4 |  |  |  | 129.9 |  |  |  |  |  |  |  | 129.9 |  |  |
| Qualcomm |  | 127.7 | 1104 |  |  | 128 | 1104 |  |  |  |  |  |  | 129.2 | 6840 |  |
| Ericsson |  | 127.6 | 3588 |  |  |  |  |  |  | 127.7 | 7896 |  |  |  |  |  |

SCS=15 kHz evaluation results:

| Company | ZC-139 with 4 repetitions |  |  |  | ZC-139 with 8 repetitions |  |  |  | ZC-L_RA (L_RA=571) |  |  |  | ZC-L_RA (L_RA=1151) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW occupied (MHz) | MCL | Network Capacity | Cell Capacity (#FD ROs*64) | BW occupied (MHz) | MCL | Network Capacity | Cell Capacity (#FD ROs*64) | BW occupied (MHz) | MCL | Capacity | Cell Capacity (#FD ROs*64) | BW occupied (MHz) | MCL | Network Capacity | Cell Capacity (#FD ROs*64) |
| Intel | 8.34 |  |  | 128 | 16.68 | 126.9 |  | 64 |  |  |  | 128 | 17.265 | 128.7 |  | 64 |
| LG |  | 130.9 |  |  |  | 132.6 |  |  |  |  |  |  |  | 132.4 |  |  |
| VIVO |  | 130.5 | 4692 |  |  | 129.5 | 2346 |  |  |  |  |  |  | 131.5 | 34500 |  |
| Qualcomm |  | 130 | 1104 |  |  | 130.4 | 1104 |  |  |  |  |  |  | 132.3 | 23000 |  |
| Ericsson |  | 129.1 | 7452 |  |  |  |  |  |  | 130.8 | 31920 |  |  |  |  |  |
| Huawei |  | 129.1 | 4692 |  |  | 131.0 | 2346 |  |  |  |  |  |  | 131.7 | 34500 |  |
| ZTE |  |  |  |  |  | 133.16 | 1380(1)/64(2) |  |  |  |  |  |  | 133.15 | 33350(1)/64(2) |  |

Reference case ZC-139

| Company | 15 kHz SCS ZC-139 |  |  |  | 30 kHz SCS ZC-139 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BW occupied (MHz) | MCL | Capacity | Cell Capacity (#FD ROs*64) | BW occupied (MHz) | MCL | Capacity | Cell Capacity (#FD ROs*64) |
| Huawei | 2.09 | 122.2 | 18768 |  | 4.17 | 122.6 | 6072 |  |
| Ericsson |  | 123.5 | 25392 | 512 |  | 124.0 | 7176 | 256 |
| ZTE |  |  |  |  |  | 124.60 | 5520(1)/192(2) |  |

(1) Obtained as # of freq domain ROs * # of roots * # of cyclic shifts, for the agreed simulation assumptions (300 m ISD).

(2) Obtained as # of freq domain ROs * 64

Observation:

- There is nearly equal support for long ZC sequence and for legacy length 139 ZC sequence with repetitions.

- Most companies’ results indicate long ZC sequence provides more capacity and better MCL than Rel. 15 with repetition for both 30 kHz and 15 kHz SCS.

Discussion (high priority):

- For long ZC sequence alternative, consider L_RA=1151 for 15KHz SCS and L_RA=571 for 30KHz SCS.

- Further discuss which alternative to choose.

- Discuss N_cs table to be generated for long ZC sequence

- Discuss Gaps in frequency domain in case of repetitions

Outcome of offline discussion:

- There is nearly equal support for long ZC sequence and for legacy length 139 ZC sequence with repetitions.

- For long ZC sequence, L_RA=1151 for 15KHz SCS and L_RA=571 for 30KHz SCS

- For legacy 139 ZC sequence with repetition design, we have

  - Alt 1: number of repetitions is 4 for 30 kHz and 8 for 15 kHz, continuous mapping the repetitions (no RE level gap between different repetitions)

  - Alt 2: Number of repetitons is 2 for 30Hz and 4 for 15KHz

    - Alt 2.1: Interlaced repetitions with one RO occupies repetitions (0, 2)/(0,2,4,6) and another RO occupies repetitions (1,3)/(1,3,5,7) for 30KHz/15KHz

    - Alt 2.2: Using the ROs at the two ends of 20MHz with one RO occupies (0,3)/(0,1,6,7) for 30KHz/15KHz

    - Alt 2.3: Continuous repetitions with one RO occupies repetitions (0, 1)/(0,1,2,3) and another RO occupies repetitions (2,3)/(4,5,6,7) for 30KHz/15KHz

    - Note: For continuous repetitions within one RO, the repetitions are continuously mapped (no RE level gaps between repetitions).

Outcome from the first online session

Proposal:

Define two long ZC sequences for PRACH for 15 and 30 kHz SCS as follows in addition to the Rel-15 PRACH sequences:

- 15 kHz: 1151 and 571

- 30 kHz: 571 and 283

### PRACH Formats

Description: NR supports multiple PRACH formats (Format A, B, C) to cater to different use cases such as different cell sizes. Since NR-U is likely to cater to relatively small cell sizes, applicability of the different formats to NR-U needs to be investigated. Note that the PRACH format design should also consider LBT related aspects that are described later in this document.

The company positions on the PRACH formats to support are provided below.

| Company | Position |
| --- | --- |
| Huawei | Proposal 9: NR-U PRACH formats are based on NR PRACH formats A and B. |
| VIVO | Proposal 8: Only PRACH format A1, A2 and A3 are supported in NR unlicensed spectrum. |
| ZTE | Proposal 11: The PRACH formats based on NR PRACH formats A and B can be supported in NR-U. |
| Fujitsu | Proposal 2: NR-U only supports short preamble format A and format B defined in Rel-15 NR. |
| Samsung | Proposal 11: NR-U shall only support short PRACH preamble format A defined in Rel-15 NR. |
| LG | Proposal #7: Rel-15 NR PRACH preamble formats A1, A2, and A3 in terms of CP length, symbol length, and the number of symbols are reused for NR-U. |
| Intel | Proposal-11: Support at least preamble formats A and B for PRACH in NR-unlicensed. - Keep the number of rows in Rel-15 NR configuration index tables unchanged. Observation-12: Exclusion of certain preamble format(s) can be enabled by proper configuration of configurationIndex by the scheduler, without changing the existing Rel-15 NR PRACH configuration index table. |
| Nokia | Proposal 12: Support only PRACH formats A and B in NR-U. |
| NTT | Proposal 10: Only preamble format A is supported for NR-U. |
| Xiomi | Proposal 5: NR-U PRACH need not support the PRACH format 1-4. |
| Ericsson | Proposal 5: Existing Rel-15 PRACH configuration tables which include all PRACH formats may be reused for NR-U operations. |

Feature Lead Summary

Following are company positions:

- Short PRACH Formats:

  - Alt 1: Support Format A only:  VIVO, Samsung, LG, NTT

- Alt 2: Support format A and B only: Huawei, ZTE, Fujitsu, Intel, Nokia

- Alt 3: Support all three formats A, B and C: Ericsson, Qualcomm, Xiaomi

The current NR supports all three formats A, B, and C. Although there are proposals for supporting only a subset of these, due to limited amount of time, all three formats can be retained to reduce the discussion time.

Offline consensus:

When legacy PRACH formats are used, Rel.16 NR-U supports legacy formats A, B and C.

### LBT Gaps between RACH occasions

Description: In NR-Rel15 back to back RACH Occasions in time can be configured. In NR-U, a UE has to pass LBT before transmitting RACH preamble. The UE may get blocked by a RACH transmission of another UE in the previous RACH occasion.

The company positions on this topic are summarized in the table below:

| Company | Position |
| --- | --- |
| VIVO | Proposal 12: To reduce the PRACH delay caused by the blockage issue, LBT Gap between RACH occasions should be supported. Following alternatives can be considered. - Alt.1: Truncated PRACH format in a RO. - Alt.2: Only even or odd numbered time domain RACH occasions in a RACH slot are used based on existing PRACH configuration. |
| ZTE | Proposal 13: The enhancement to PRACH preamble formats to introduce LBT gaps in time domain between PRACH occasions is not necessary and should not be supported in Rel-16. |
| Fujitsu | Proposal 3: For NR-U, NR Rel-15 RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs. |
| Samsung | Proposal 14: NR-U shall support non-consecutive ROs within the same RACH slot, with a gap duration introduced between two neighboring ROs for the PRACH LBT resource overhead. |
| LG | Proposal #5: Consider the followings for PRACH preamble format in NR-U. - Insertion of CCA gap between adjacent RACH occasions (ROs) in time domain - FFS on duration of the CCA gap (e.g. X usec or Y symbol) - Addition of GP (Guard Period) after preamble part in PRACH format - Length of the GP is the same with CP length Proposal #6: Determine CCA gap between adjacent ROs in time domain by considering LBT type applied for PRACH in terms of the channel access priority class and/or variable CWS (in case of Cat-4 LBT based PRACH transmission). |
| Intel | Proposal-16: Introduce LBT gap between neighbouring ROs within a RACH slot to avoid blocking of one UE by RACH transmission of another UE in the previous RO. |
| Panasonic | Observation: When multiple RACH occasions are continuously configured symbols, the previous PRACH resource is used or not is not uncertain as it depends on the other UE's PRACH transmission usage. Proposal 1: What LBT procedure is required should be clarified first when multiple RACH occasions are continuously configured except within gNB initiated COT. |
| Xiomi | Proposal 4: NR-U shall support RO gap configuration on current PRACH formats. |
| NTT | Proposal 11: 2 symbols for SCS = 15 kHz, and 3 symbols for SCS = 30 kHz shall be considered for LBT gap, and LBT gap shall be allocated within a preamble slot, thus, 2 or 3 symbols for LBT gaps should be allocated before ROs. |
| Qualcomm | Proposal 5: Introduce gaps between consecutive TDM-ed RACH occasions for UL LBT operation. • The size of gaps shall match the maximum expected UL LBT duration (e.g. at least larger than that obtained by assuming the largest contention window and no other interferers) • Gaps should be in units of symbols for easier alignment • Minimum gap required is 1 or 2 symbols for 15 or 30 kHz SCS respectively |

Feature lead summary

Back to back RACH occasions with small gaps could lead to blocking of a later RACH occasion by RACH transmissions in an earlier RACH occasion. NR RACH formats may need to be enhanced to provide larger gaps to mitigate the blocking issue.

Summary of company positions:

- NR Rel.15 RO configuration should be enhanced to avoid potential blocking between neighbouring ROs:

  - Provide explicit gap: QCOM, NTT, Xiomi, Intel, Samsung, LG, Fujitsu(?)

  - Truncated PRACH format: VIVO

  - Addition of GP after preamble: LG

  - FFS: Configure the exact LBT gap

- NR-Rel.15 is sufficient to configure non-blocking occasions, gaps not supported for Rel-16:

  - Use non-contiguous time domain RACH occasions: VIVO

  - Solve by implementation: ZTE

- Benefits to be clarified: Panasonic

FL proposal

- NR Rel-15 RACH occasion configuration should be enhanced to avoid potential blocking between neighbouring time-domain RACH occasions.

  - The gap between ROs are integer multiple of OFDM symbols

  - FFS: The exact length of gap between ROs

  - FFS: How to configure the gaps symbols

  - FFS: Placement of the ROs within a slot

### Multiplexing PRACH and other channels

Description: In NR-Rel15, it is possible to multiplex RACH resource with PUSCH/PUCCH of another UE in frequency. In NR-U there may be the following issues which may need to be considered while multiplexing PRACH with other channels in frequency.

- PUSCH/PUCCH frequency domain allocation may be interlaced

- PUSCH/PUCCH transmissions blocking RACH transmission due to LBT

- Interference from neighbouring tones.

The positions of different companies on these multiplexing issues are summarized in the table below:

| Company | Position |
| --- | --- |
| Huawei | Proposal 10: No solution with specification impact is adopted for handling multiplexing between PRACH and PUSCH/PUCCH. |
| ZTE | Proposal 14: Only TDM of PRACH and other channels is supported. |
| Samsung | Proposal 13: NR-U shall support multiplexing between PRACH and PUSCH/PUCCH in a TDM manner only, at least for initial access purpose. |
| Intel | Proposal-14: Support only TDM between PRACH and other UL signals/channels at least for PRACH transmission within UE acquired COT. Observation-15: If PRACH is transmitted within gNB shared COT, where 2 MHZ temporal OCB is allowed by regulation, FDM between short PRACH (L=139) and other UL signals/channels can be supported. - The UEs scheduled with interlaced transmission would puncture their transmission on the REs that overlap with resources allocated to PRACH of other UEs. |
| VIVO | O Other UL channel could rate matched around the RBs of RACH occasions, if this option is adopted. |

Feature lead summary:

Summary of company positions:

- Support TDM only: ZTE, Samsung, Intel

- Rate matched around PRACH: VIVO

- No specification change required: Huawei (FDM if no overlap)

Discuss further after concluding on the PRACH waveform.

### Virtual PRACH occasions

Description: The SSB to RO mappings in NR-U have to be updated if long PRACH formats are selected. Virtual PRACH occasions address this issue to reduce specification changes in NR-U from NR.

The company positions on this topic are summarized in the table below:

| Company | Position |
| --- | --- |
| Qualcomm | Proposal 13: SSB to RO mappings with frequency domain multiplexing are replaced by SSB to VRO mappings, where VROs are a set of 64 long sequences. - Each VRO will have different set of root indices configured by gNB |

Discussion:

Discuss further if methods to reduce RO overhead need to be considered.

# Reference

- R1-1907846, “Chairman's notes of AI 7.2.2 Study on NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1#97, Reno, USA, 13th – 17th, May 2019

- R1-1905801, “Chairman's notes of AI 7.2.2 NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1-96bis, Xi’an, China, 8th – 12th April 2019

- R1-1903629, “Chairman's notes of AI 7.2.2 NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1-96, Athens, Greece, February 25th – March 1st, 2019

- R1-1901419, “Chairman's notes of AI 7.2.2 NR-based Access to Unlicensed Spectrum”, Ericsson, AH-1901, Taipei, Taiwan Jan. 21st – Jan. 25th, 2019

- R1-1813847, “Chairman's notes of AI 7.2.2 Study on NR-based Access to Unlicensed Spectrum”, Ericsson, RAN1-95, Spokane, USA, November 12th – 16th, 2018

- R1-1814317, “Feature lead summary 2 for UL Signals and Channels”, Ericsson, RAN1#95, Spokane, USA, 12th – 16th November 2018

- R1-1903404, “Feature lead summary for initial access signals and channels for NR-U”, Qualcomm, RAN1#96, Athens, Greece, February 25th – March 1st, 2019

- R1-1905785, “Feature lead summary for initial access signals and channels for NR-U”, Qualcomm, RAN1#96b, Xian, China, Apr. 8th – Apr. 12th, 2019

- R1-1907883, “Feature lead summary for initial access signals and channels for NR-U”, Qualcomm, RAN1#97, Reno, USA, May 13 – May 17, 2019

- RP-191581, “Guidance on essential functionality for NR-U”, Qualcomm, RAN1#84, Newport Beach, USA, June 3-6, 2019

- R1-1908106, "Initial access signal and channels in NR unlicensed band", Huawei, HiSilicon, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908137, "Discussion on initial access signals and channles", vivo, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908202, "Considerations on initial access signals and channels for NR-U", ZTE, Sanechips, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908213, "Considerations on PRACH in NR-U", Fujitsu, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908382, "On Initial Access Signals and Channels for NR-U", MediaTek Inc., RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908414, "Initial access signals/channels for NR-U", OPPO, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908462, "Initial access signals and channels for NR-U", Samsung, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908532, "Physical layer design of initial access signals and channels for NR-U", LG Electronics, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908621, "Initial access signals/channels for NR-unlicensed", Intel Corporation, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908686, "Initial Access Signals and Channels for NR-U", Nokia, Nokia Shanghai Bell, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908806, "Discussion on NR-U DRS design", ETRI, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908847, "Discussion on initial access signals and channels", NEC, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908880, "Initial access signals/channels for NR-U", Panasonic Corporation, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1908963, "Discussion on DRS in NR-U", Spreadtrum Communications, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1909078, "Remaining issues for initial access signals and channels", AT&T, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1909085, "Initial access signals/channels for NR-U", Sharp, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1909174, "Initial access signals and channels for NR-U", NTT DOCOMO, INC., RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1909214, "On NR-U initial access signals/channels", Beijing Xiaomi Mobile Software, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1909242, "Initial access signals and channels for NR-U", Qualcomm Incorporated, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1909295, "Initial access signals and channels", Ericsson, RAN1#98, Prague, CZ, August 26th - August 30th, 2019

- R1-1909361, "Initial access channels for NR-unlicensed", WILUS Inc., RAN1#98, Prague, CZ, August 26th - August 30th, 2019

# Appendix: Previous agreements

## DRS

Agreement:

- UE assumes 30KHz SCS for SS/PBCH block for 5GHz band and 6GHz band if the SCS is not indicated by higher layers.

- Support configuration by higher layers of 15KHz or 30KHz SCS for SS/PBCH block

- Include this agreement in a LS to RAN4 for inclusion in specs managed by RAN4

Conclusion:

No changes are required to the time and frequency position of the PSS/SSS/PBCH relative to each other in one PSS/SSS/PBCH block.

Agreement:

The Type0-PDCCH monitoring configuration for NR-U should satisfy at least the following properties:

- TDM of Type0-PDCCH and SSB similar to existing pattern 1 (already agreed)

- Support the monitoring of Type0 PDCCH of the 2nd SSB position in a slot in the gap between 1st and 2nd SSB within the slot

  - FFS start at symbol #6 of #7 or both

- FFS: The Type0-PDCCH candidates associated with an SSB are confined within a slot carrying the associated SSB (with the same QCL assumptions)

Agreement:

Only coreset #0 lengths of 1 and 2 symbols are supported for NR-U

Agreement:

Select one of the following options in RAN1 #97:

- Alt 1: Legacy SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 at least for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#6, #7) for length-2 coreset 0 and symbol #6 or symbol #7 for length-1 coreset 0 for the second SSB in a slot

    - FFS: configurable between symbols #6 and #7 for the length-1 coreset 0 for the second SSB in a slot

- Alt2: New SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#7, #8) for length-2 coreset 0 and symbol (#7) for length-1 coreset 0 for the second SSB in a slot

- Alt 3: Legacy SSB positions in a slot

  - Support Type0-PDCCH in symbol (#0, #1) for length-2 coreset 0 and symbol (#0) for length-1 coreset 0 for the first SSB in a slot

  - Support Type0-PDCCH in symbol (#7) for coreset 0 for the second SSB in a slot

## PRACH

RAN1#93

Agreement:

- An interlaced waveform can have benefits in some scenarios including

  - Link budget limited cases with given PSD constraint

  - As one option to efficiently meet the occupied channel bandwidth requirement.

- A waveform contiguous in frequency may be adequate in some scenarios

  - To inherit legacy contiguous allocation designs.

Note: It is RAN1’s understanding that the temporal allowance of not meeting occupied channel bandwidth by regulation can be exploited if the minimum bandwidth requirement, e.g., 2 MHz, is satisfied.

RAN1#94

UL Signals

Agreement:

- For scenarios in which a block-interlaced waveform is used for PUCCH/PUSCH, it has been identified that from FDM-based user-multiplexing standpoint it can be beneficial to have UL channels on a common interlace structure, at least for PUSCH, PUCCH, associated DMRS, and potentially PRACH

  - Note: This is only from a user-multiplexing perspective. Other aspects of PRACH design need to be considered, i.e., timing estimation accuracy, miss detection rate, PAPR, RACH capacity, transmission power

- For scenarios in which a contiguous allocation for PUSCH and PUCCH is used, it is beneficial to use contiguous resource allocation for PRACH

- FFS: Potential LBT blocking due to TA difference between FDM’d PUSCH, PUCCH, and PRACH

initial access and mobility section

Agreement:

If preamble transmissions are dropped due to LBT failure, then

- From a RAN1 perspective, it is recommended that preamble power ramping is not performed and that the preamble transmission counter is not incremented

RAN1#95

Frame structure

Agreement:

Adopt the following text proposal for section 7.2.1.2 of the TR

------------------------------------------ Start of Text Proposal ----------------------------------------------

It has been identified that support of different numerology candidates at least has the following specification impacts:

- For PRB-based block-interlace design for 15, 30, and 60 kHz SCS, the following spec impacts have been identified: Number of interlaces and number of PRBs per interlace need to be defined; the resource allocation mechanism needs to be defined; channel estimation aspects need to be considered, such as impact on PRG. In addition to the above impact, for sub-PRB-based block-interlace design for 60 kHz SCS, reference signal design (such as DMRS) needs to be revisited and alternative resource allocation mechanism is needed.

- For NR-U DRS design for 15 and 30 kHz SCS, the SS/PBCH block time domain pattern is already supported in Rel-15. For 60 kHz SCS, there is no SS/PBCH block time domain pattern defined in Rel-15. SS/PBCH block to CORESET configuration tables (38.213 Section 13) need to be defined as well.

- For PRACH design for 15, 30, and 60 kHz SCS, signalling mechanism of RACH configuration indicating PRACH numerology may need modification to support more than two numerologies for PRACH for NR-U.

------------------------------------------- End of Text Proposal ---------------------------------------------

The text proposals in Sections 7.1 and 7.2 of R1-1814137 are endorsed for the TR.

Section 7.2

It has been identified that enhancement of one or more legacy PRACH formats is feasible for NR-U. Four potential design alternatives, including no interlacing, have been identified for the frequency mapping of PRACH sequences for NR-U, where consensus on which one(s) to support for NR-U has not yet been achieved:

- Alt-1: Uniform PRB-level interlace mapping

  - In this approach a PRACH sequence for a particular PRACH occasion is mapped to all of  the PRBs of one or more of the interlaces in the PRB-based block interlace structure. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces.

  - It has been identified that a uniform mapping (equal spacing of PRBs) in the frequency domain produces a zero-autocorrelation zone, of which the duration is inversely proportional to the frequency spacing between the PRBs.

- Alt-2: Non-uniform PRB-level interlace mapping

  - In this approach a PRACH sequence for a particular PRACH occasion is mapped to some or all of  the PRBs of one or more of the interlaces in the same PRB-based block interlace structure used for PUSCH/PUCCH. Within a PRB, either all or a subset of REs are used. Different PRACH occasions are defined using an orthogonal set of PRBs, or an orthogonal set of REs within the PRBs, from one or more same/different interlaces.

  - It has been identified that an irregular mapping (non-equal spacing of PRBs/REs) in the frequency domain reduces the false peaks in the PRACH preamble auto-correlation function.

- Alt-3: Uniform RE-level interlace mapping

  - In this approach, a PRACH sequence for a particular PRACH occasion consists of a “comb-like” mapping in the frequency domain with equal spacing between all used REs. Different PRACH occasions are defined by way of different comb offsets.

  - Since this approach does not fit with the common PUSCH/PUCCH interlace structure, one source suggests that only TDM multiplexing of PUSCH/PUCCH and PRACH should be supported. Another source suggests that puncturing/rate matching PUSCH/PUCCH around the used PRACH REs may be used.

- Alt-4: Non-interlaced mapping

  - In this approach, a PRACH sequence for a particular PRACH occasion is mapped to a number of contiguous PRBs, same or similar to NR Rel-15.

  - Some sources propose that to fulfill the minimum OCB requirement, that the PRACH sequence is mapped to a set of contiguous PRBs, and the PRACH sequence mapping is repeated across the frequency domain, potentially with guard RE(s)/PRB(s) between repetitions. For each repetition, a different cyclic shift or different base sequence may or may not be applied.

It has been identified that the long PRACH sequence length defined in NR Rel-15 (L = 839) is not beneficial for NR-U, since PRACH formats based on this length are tailored toward large cells not expected in an NR-U deployment. However, when it comes to shorter sequence lengths, some sources propose reusing the short sequence length (L = 139) defined in NR-Rel-15, whereas other sources propose defining new sequence lengths depending on which of the 4 alternatives above is supported.

It has been identified that the following common design attributes need to be considered in the detailed design of an interlaced PRACH waveform for 4-step random access for NR-U when specifications are developed:

  - Multiplexing of PRACH and PUSCH/PUCCH, considering block interlaced structure used for PUSCH/PUCCH, e.g.,

    - FDM

    - TDM

  - Supported PRACH sequence and PRACH sequence length(s)

  - PRACH capacity

    - Number of PRACH preambles per cell

    - Number of root sequences

    - Number of cyclic shifts

    - Number of PRACH occasions

  - Maximum supported Tx power

  - PAPR/CM

  - Number of PRACH formats

  - Simulation assumptions for evaluation of performance, e.g.,

    - Single vs. multi-cell assumptions

  - Performance metrics

    - Timing estimation error

    - Miss-detection probability

    - False-detection probability

    - False-alarm probability

RAN1#AH-1901

Agreement:

Companies are encouraged to provide results comparing the different alternatives using the following simulation assumptions to select between alternative PRACH designs.

- The Rel-15 PRACH design should be simulated as a baseline

| Property | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| Channel model | TDL-C |
| Delay scaling | 10ns, 100 ns |
| Antenna configuration at BS(1) | (M,N,P) = (1,1,2) with omni-directional antenna element |
| Antenna configuration at UE | Single omni-directional antenna element |
| Antenna port virtualization | No beamforming and no beam selection |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| UE speed | 3 km/h |
| Initial timing offset | Uniformly distributed in [0, 1.2 µs (corresponding to 300 m ISD)] Optional: Uniformly distributed in [0, 2 µs (corresponding to 500 m ISD)] |
| PRACH format | A1 with other formats optional |
| Subcarrier spacing | 15/30 kHz. (with other SCS optional) |
| PRACH sequence and frequency resource allocation | For evaluation purpose, the Rel-15 PRACH ZC sequence (with possible length change) should be simulated. Additional/new sequences can be simulated. Each company should provide details on the sequence (type and length) and the resource allocation (e.g., Alt1~Alt4 and detailed mapping). |
| Total number of preambles per cell | 64, each company should provide details on how these 64 preambles are generated |
| Preamble detector | Each company should provide details on used algorithm |
| Interference assumption | No interference. Optional: -3/0/3dB interference power compared with target PRACH |
| Detection Criteria | 1% maximum mis-detection probability(2) |
|  | 0.1% maximum false alarm probability(3) |
|  | maximum timing estimation error being 50% of the normal CP length |
| Formatting of results (please also reference Section 8 of R1-1704144 for reporting formats) | Mis-detection probability vs. SNR |
|  | False alarm probability vs. SNR(4) |
|  | CDF of timing estimation error |
|  | PRACH capacity (maximum number of preambles) |
|  | Peak-to-average power ratio and cubic metric |
|  | MCL(5) |
| (1) See Table 7-1 of R1-1704144 (2) The missed detection probability is defined as the ratio between the total number of transmitted preambles that are either not detected, or detected as a different preamble, or detected but with timing error greater than the maximum value (i.e., 50% of normal CP length), and the total number of transmitted preambles within an observation interval. (3) Maximum false alarm probability refers to the case when input at receiver is noise only (considering 64 preamble detectors as in 3GPP TS 36.104, section 8.4.1). (4) False alarm probability is defined as the ratio of total number detected but not transmitted preambles, and the total number of possible detection occurrences, where each occurrence (occurrence refers to 64 detections, one for each of the 64 preambles in a cell) is one potential preamble transmission in a RO. (5) In the MCL calculation, needs to consider the maximum transmit power supported by the PRACH design under PSD limitation and PAPR/EVM characteristic of the design. Note: Assumptions on the following should be stated - use of a guard band (if any) - definition of SNR - signal bandwidth used |  |

RAN1#96

Agreement:

For PRACH proposal comparison, the following metrics are to be provided

- Noise power Np = -174+10*log10(L_RA*SCS)+NF  dBm, with NF=5dB and SCS is in Hz.

- SNR actual corresponds to 1% miss detection probability read from the simulation curve

- P_TX is computed as follows

  - Assume 23dBm max power when transmitting legacy PRACH waveform with ZC139

  - P_max is the allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme

  - P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff

    - Backoff is computed as 95% percentile of CCDF of cubic metric over the preambles in the RO

- MCL = P_TX-SNR-Np;

- Number of interlace used for each RO is how many uniform interlaces (M=5 for 30KHz and M=10 for 15KHz) an RO occupies

  - For continuous PRACH design, as the RO occupies some RBs from each interlace, “Partial” is marked in the column.

- N_FDM = Number of FDM RACH occasions within 20MHz

- PRACH capacity, with all RO in 20MHz assigned.

| Parameter | Value | Notes |
| --- | --- | --- |
| Scheme |  | Eg. Alt4-ZC139x2 |
| SCS |  | 15KHz or 30KHz |
| PRACH sequence length (L_RA) |  | Eg. 139, |
| # of repetition (M) |  | If repetition of sequence is used in freq domain |
| N_cs |  | Eg. 11 |
| # of RBs used for one RO (N_RB) |  | # of RBs with PRACH tone assigned. Eg. 12 for ZC139 design |
| # of interlaces used by one RO (N_interlace) |  | # of uniform interlaces (M=5 for 30KHz and M=10 for 15KHz) with RBs used for one PRACH RO |
| RACH frequency span (MHz) |  | The actually used bandwidth with one RO, SCS*L_RA*M |
| Noise level, Np (dBm) |  | Np= -174+10*log10(SCS*L_RA*M)+NF NF=-5dB |
| SNR (dB) |  | SNR needed at 1% misdetection, read from simulation curve |
| P_max (dBm) |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) |  | Backoff is computed as 95% percentile of CCDF of cubic metric over the preambles in the RO |
| P_TX (dBm) |  | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCS (dB) |  | MCL = P_TX-SNR-Np |
| N_FDM |  | # of ROs in 20MHz |
| Capacity |  | Across all ROs in 20MHz |

RAN1#96 Bis

Agreement:

PRB/RE-interlaced PRACH is not considered further. Consider the following alternatives should be studied further as options.

- Alt 1: Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers, with repetitions in frequency

  - FFS: Guard bands between repetitions

  - FFS: Number of repetitions

  - FFS: Whether repetitions are constrained to be contiguous in frequency or not

- Alt 2: A single PRACH sequence mapped to contiguous PRBs according to one of the following alternatives

  - Alt 2.1: ZC sequence with longer length than 139

  - Alt 2.2: New sequence with longer length than 139

RAN1#97

Agreement:

For a new enhanced design of NR-U PRACH in addition to the Rel-15 design (sequence length of 139) further discussion is limited to the following options

- ZC sequence of the following lengths

- 15 kHz: Choose one of L_RA= [571, 1151]

- 30 kHz: Choose one of L_RA= [283, 571]

- Repetition of Rel-15 PRACH sequences in frequency domain with potentially some mechanisms to improve the cubic metric

  - Consider one of 2 and 4 repetitions for 30 kHz and one of 4 and 8 repetitions for 15 kHz

- Note: Decision will be based on previously agreed evaluation metrics, capacity per cell (i.e., number of preambles per RACH occasion and number of RACH occasions) for the same time and frequency resources, specification impact and implementation complexity.

- Note: Companies should state any deviations in assumptions from the agreed evaluation assumptions.

# Appendix: Evaluation results for PRACH proposals

In this section, we present the performance evaluation results for different PRACH proposals from all companies collected from the paper submissions to RAN1 #97. The MCL results are summarized in Table 1 in Section 3.3.2.

## HW

| Scheme | NZC | R | SCS*Nzc*R | Np (dBm) | SNRFD | Pmax | 95% CM (dB) | PTx dBm | MCL (dB) | NFDM | PRACH capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ZC139 | 139 | 1 | 4.17 | -102.8 | -4.05 | 16.12 | 2.3328 | 16.12 | 122.59 | 4 | 6072 (92 cells) |
| ZC139x2 | 139 | 2 | 8.34 | -99.8 | -7.58 | 19.13 | 2.6561 | 19.13 | 126.07 | 2 | 3036 (46 cells) |
| ZC283 | 283 | 1 | 8.49 | -99.7 | -7.64 | 19.2 | 2.3294 | 19.2 | 126.18 | 2 | 7896 (112 cells) |
| ZC139x4 | 139 | 4 | 16.68 | -96.8 | -10.9 | 22.14 | 2.5305 | 20.87 | 128.14 | 1 | 1518 (23 cells) |
| ZC571 | 571 | 1 | 17.13 | -96.7 | -11.18 | 22.25 | 2.333 | 21.03 | 128.51 | 1 | 7980 (114 cells) |

Observation 1: Compared to a single ZC sequence of similar length, a repeated length-139 ZC sequence performs worse, and the degradation increases with more repetitions. For 30 kHz SCS:

- The mis-detection probability is ~0.06-0.28 dB worse

- The 95-percentile CM is ~0.20-0.33 dB larger

- The MCL is ~0.11-0.37 dB lower

- The PRACH capacity is ~61-81% smaller

Observation 2: Compared to the NR Rel-15 length-139 ZC sequence without repetition, a repeated length-139 ZC sequence results in ~50-75% smaller PRACH capacity for 30 kHz SCS.

Observation 3: For 15 kHz SCS, a length-139 ZC sequence with 8 repetitions results in:

- 0.62 dB lower MCL and 93.2% lower PRACH capacity than a length-1151 ZC sequence

- 87.5% lower PRACH capacity than a length-139 ZC sequence

Observation 4: The MCL of a repeated length-139 ZC sequence will be worse than that of a single long ZC sequence of similar length, even if some method is used to reduce its CM.

Observation 5: New methods would be required for specifying the cyclic shifts and the root index ordering of repeated ZC sequences.

Observation 6: Repeated ZC sequences are a new waveform while a Rel-15 NR UE is already capable of transmitting a single ZC sequence with one of 99 different sequence lengths ranging from 31 to 1259 in a 20 MHz bandwidth.

### Observation 7: A repeated length-139 ZC sequence:

- performs worse than a single ZC sequence of similar length in terms of CM, MCL and PRACH capacity

- results in worse PRACH capacity than what NR Rel-15 supports

- needs deployment and specification of sequence manipulations for CM reduction

- needs a new method for specifying the cyclic shifts

- needs a new method for specifying the root index ordering

Proposal 6: The NR-U PRACH is based on a single long ZC sequence with the following lengths:

- For 15 kHz SCS: LRA=1151

- For 30 kHz SCS: LRA=571

- For 60 kHz SCS: LRA=139

Proposal 7: The following cyclic shifts are used for ZC sequences of length NZC:

- If NZC=283:  NCS∈0,4,5,6,7,8,9,10,11,12,14,16,20,23,28,35

- If NZC=571:  NCS∈0,8,10,11,12,14,16,19,21,24,28,33,40,47,57,71

- If NZC=1151:  NCS∈0,17,20,23,26,29,33, 38,44,50,57,67,82,95,115,143

Proposal 8: The logical root indices i=0,1,2,3,4,5,… are mapped to the sequence root indices as 1, NZC-1,2, NZC-2, 3, NZC-3,….

Proposal 9: NR-U PRACH formats are based on NR PRACH formats A and B.

Proposal 10: No solution with specification impact is adopted for handling multiplexing between PRACH and PUSCH/PUCCH.

## VIVO

| Scheme | L=139 | L=1151 | L=139 x 4 | L=139 x 8 | L = 571 x 2 | L=139 | L=571 | L=139 x 2 | L=139 x 4 | L=283 x 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (Rel-15) |  |  |  |  | (Rel-15) |  |  |  |  |
| SCS(kHz) | 15 | 15 | 15 | 15 | 15 | 30 | 30 | 30 | 30 | 30 |
| PRACH sequence length (L_RA) | 139 | 1151 | 139 | 139 | 571 | 139 | 599 | 139 | 139 | 283 |
| # of repetition (R) | 1 | 1 | 4 | 8 | 2 | 1 | 1 | 2 | 4 | 2 |
| N_cs | 8 | 38 | 8 | 8 | 20 | 12 | 38 | 12 | 12 | 20 |
| # of RBs used for one RO (N_RB) | 12 | 96 | 12 | 12 | 48 | 12 | 48 | 12 | 12 | 24 |
| RACH frequency occupancy (MHz) | 2.085 | 17.265 | 8.34 | 16.68 | 17.13 | 4.17 | 17.97 | 8.34 | 16.68 | 16.98 |
| Noise level, Np (dBm) | -105.81 | -96.63 | -99.79 | -96.78 | -96.66 | -102.8 | -96.45 | -99.79 | -96.78 | -96.7 |
| SNR (dB) | -3.77 | -13.86 | -11.3 | -13.37 | -14.19 | -3.58 | -11.11 | -8.48 | -11.39 | -11.47 |
| P_max (dBm) | 13.34 | 22.38 | 19.37 | 22.38 | 22.38 | 16.35 | 22.38 | 19.37 | 22.38 | 22.38 |
| Backoff (dB) | 1.96 | 1.97 | 3.26 | 3.61 | 4.12 | 1.96 | 1.97 | 4.11 | 3.1 | 4.12 |
| P_TX (dBm) | 13.34 | 21.03 | 19.37 | 19.39 | 18.88 | 16.35 | 21.03 | 18.89 | 19.9 | 18.88 |
| MCL (dB) | 122.92 | 131.52 | 130.45 | 129.54 | 129.73 | 122.73 | 128.59 | 127.16 | 128.07 | 127.05 |
| N_FDM | 8 | 1 | 2 | 1 | 1 | 4 | 1 | 1 | 1 | 1 |
| Capacity | 18768 | 34500 | 4692 | 2346 | 15960 | 6072 | 8970 | 1518 | 1518 | 3948 |

Observation 1: Obvious CM reduction can be achieved if phase rotation is applied to preamble repetitions when preamble repeated 4 and 8 times in frequency.

Observation 2: Long sequence and preamble repetition can achieve similar MCL performance.

Observation 3: More number of preamble repetitions in frequency domain does not necessary bring about higher MCL.

Observation 4: The phase rotation applied on preamble is transparent to the receiver since gNB can detect the preambles without knowing the phase rotations on the preamble repetitions.

Observation 5: NRU PRACH occupying about half of the resource in sub band, i.e. 4 repetitions for 15 kHz and 2 repetitions for 30 kHz, have advantages over preamble repeated across the whole sub band in multiplexing with other channels.

Observation 6: Rel-15 length 139 preamble scheme have less spec impact compared with long preamble sequence with or without repetition.

Proposal 9: Length-139 preamble based repetition should be adopted for NRU PRACH.

Proposal 10: Phase rotation should be applied to the preamble repetitions to limit the cubic metric of NRU PRACH with multiple preamble repetitions where the phase rotation combination for multiple ROs is defined (e.g. Table 1)

Proposal 11: The number for preamble repetitions is preferred to be 4 and 2 for SCS = 15 kHz and 30 kHz, respectively.

Proposal 12: To reduce the PRACH delay caused by the blockage issue, LBT Gap between RACH occasions should be supported. Following alternatives can be considered.

- Alt.1: Truncated PRACH format in a RO.

- Alt.2: Only even or odd numbered time domain RACH occasions in a RACH slot are used based on existing PRACH configuration.

## ZTE

| Parameter | Value | Value | Value | Notes |
| --- | --- | --- | --- | --- |
| Scheme | ZC139x1 | ZC139x4 with phase rotation | ZC571x1 | Eg. Alt4-ZC139x2 |
| SCS | 30KHz | 30KHz | 30KHz | 15KHz or 30KHz |
| PRACH sequence length (L_RA) | 139 | 139 | 571 | Eg. 139, |
| # of repetition (R) | 1 | 4 | 1 | If repetition of sequence is used in freq domain |
| N_cs | 13 | 13 | 26 | Eg. 11 |
| # of RBs used for one RO (N_RB) | 12 | 47 | 48 | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| RACH frequency occupancy (MHz) | 4.17 | 16.68 | 17.13 | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) | -102.8 | -96.8 | -96.7 | Np= -174+10*log10(SCS*L_RA*R)+NF |
|  |  |  |  | NF=5dB |
| SNR (dB) | -5.6 | -13 | -11.8 | SNR needed at 1% misdetection, read from simulation curve |
| P_max (dBm) | 16.2 | 22.2 | 22.3 | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) | 2.33 | 2.53 | 2.33 | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. Note: If cubic metric is not used, information on the backoff metric used should be provided. |
| P_TX (dBm) | 16.2 | 20.5 | 20.7 | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) | 124.6 | 130.3 | 129.2 | MCL = P_TX-SNR-Np |
| N_FDM | 4 | 1 | 1 | # of ROs in 20MHz |
| N_TDM | 2 | 2 | 2 | # of ROs in 1ms |

Proposal 7: New Long PRACH sequences should not be introduced in Rel-16 NR-U.

Observation 1：For SCS=30KHz, in order to obtain the optimal CM by the repetition scheme, the following rules exist between the repetitions.

- The four repetitions have two different phase rotation values, the phase relationship of the two phase values is , the first repetition has the same phase rotation value as the last one, and the middle two repetitions have  the same phase rotation value .

Observation 2：For SCS=15KHz，in order to obtain the optimal CM by the repetition scheme, the following rules exist between the repetitions.

The eight repetitions have three different phase rotation values, the first repetition has the same phase rotation value as the last one, and the most middle two repetitions have  the same phase rotation value of the repetition, the rest of the repetitions have the same phase rotation value of the repetition. There is a phase relationship  between the first two repetitions ,while a phase relationship between the first repetition and  the forth repetition .

Proposal 8: Legacy NR PRACH sequence of length 139 mapped to contiguous subcarriers with repetitions in frequency can be supported for NR-U.

Proposal 9: For 4 repetitions for 30KHz SCS and 8 repetitions for 15KHz SCS with the phase rotation can be further studied and discussed.

Proposal 10: The phase rotation method1 can significantly reduce the CM value of the repeating schemes.

Proposal 11: The PRACH formats based on NR PRACH formats A and B can be supported in NR-U.

Proposal 12:  15kHz SCS and 30 kHz SCS should be supported in below 7 GHz for NR-U.

Proposal 13: The enhancement to PRACH preamble formats to introduce LBT gaps in time domain between PRACH occasions is not necessary and should not be supported in Rel-16.

Proposal 14: Only TDM of PRACH and other channels is supported.

## Fujitsu

Proposal 1: In order to meet OCB requirement for PRACH transmissions on unlicensed band, repetition of Rel-15 NR PRACH short sequences in frequency domain should be supported in NR_U.

Proposal 2: NR-U only supports short preamble format A and format B defined in Rel-15 NR.

Proposal 3: For NR-U, NR Rel-15 RO configuration should be enhanced to avoid potential blocking between neighboring time-domain ROs.

## MTK

Proposal 7: Long-sequence (ZC=839) based NR PRACH preamble formats are not supported in NR-U.

## OPPO

Nothing

## Samsung

Proposal 9: NR-U shall support reusing legacy Rel-15 NR PRACH sequence with repetitions in frequency domain.

Proposal 10: Repetitions of PRACH sequence can be mapped to contiguous RBs for Alt1.

Proposal 11: NR-U shall only support short PRACH preamble format A defined in Rel-15 NR.

Proposal 12: NR-U shall support 30 kHz SCS only for PRACH.

Proposal 13: NR-U shall support multiplexing between PRACH and PUSCH/PUCCH in a TDM manner only, at least for initial access purpose.

Proposal 14: NR-U shall support non-consecutive ROs within the same RACH slot, with a gap duration introduced between two neighboring ROs for the PRACH LBT resource overhead.

## LG

15 kHz

| Parameter | Value | Value | Value | Value |
| --- | --- | --- | --- | --- |
| Scheme | Rel-15 (Case 1) | Alt4-ZC139x4 (Case 2) | Alt4-ZC139x8 (Case 3) | Alt4-ZC1151 (Case 4) |
| SCS | 15 kHz | 15 kHz | 15 kHz | 15 kHz |
| PRACH sequence length (L_RA) | 139 | 139 | 139 | 1151 |
| # of repetition (R) | 1 | 4 | 8 | 1 |
| N_cs | 11 | 11 | 11 | 95 |
| # of RBs used for one RO (N_RB) | 12 | 48 | 96 | 96 |
| # of interlaces used by one RO (N_interlace) | - | - | - | - |
| RACH frequency occupancy (MHz) | 15k*139*1 = 2.085M | 15k*139*4 = 8.34M | 15k*139*8 = 16.68M | 15k*1151*1 = 17.265M |
| Noise level, Np (dBm) | -174 + 10log10(2.085M) + 5 = -105.81 | -174 + 10log10(8.34M) + 5 = -99.8 | -174 + 10log10(16.68M) + 5 = -96.8 | -174 + 10log10(17.265M) + 5 = -96.6 |
| SNR (dB) [4] | -4.6 | -11.9 | -14.7 | -14.8 |
| P_max (dBm) | 10+10log10(2.085) = 13.19 | 10+10log10(8.34) = 19.21 | 10+10log10(16.68) = 22.22 | 10+10log10(17.265) = 22.37 |
| Backoff (dB) | 1.975 | 1.805 | 1.899 | 1.976 |
| P_TX (dBm) | min(13.19, 23-1.975) = 13.19 | min(19.21, 23-1.805) = 19.21 | min(22.22, 23-1.899) = 21.101 | min(22.37, 23-1.976) = 21.024 |
| MCL (dB) | 13.19-(-4.6)-(-105.81) = 123.6 | 19.21-(-11.9)-(-99.8) = 130.91 | 21.101-(-14.7)-(-96.8) = 132.601 | 21.024-(-14.8)-(-96.6) = 132.424 |
| #NAME? |  |  |  |  |
| N_FDM | 8 | 2 | 1 | 1 |
| (# of ROs in 20MHz) |  |  |  |  |
| # of preambles per RO | 64 | 64 | 64 | 64 |
| Capacity | 8*64 = 512 | 2*64 = 128 | 1*64 = 64 | 1*64 = 64 |

30 kHz:

| Parameter | Value | Value | Value | Value |
| --- | --- | --- | --- | --- |
| Scheme | Rel-15 (Case 1) | Alt4-ZC139x2 (Case 2) | Alt4-ZC139x4 (Case 3) | Alt4-ZC571 (Case 4) |
| SCS | 30 kHz | 30 kHz | 30 kHz | 30 kHz |
| PRACH sequence length (L_RA) | 139 | 139 | 139 | 571 |
| # of repetition (R) | 1 | 2 | 4 | 1 |
| N_cs | 11 | 11 | 11 | 47 |
| # of RBs used for one RO (N_RB) | 12 | 24 | 48 | 48 |
| # of interlaces used by one RO (N_interlace) | - | - | - | - |
| RACH frequency occupancy (MHz) | 30k*139*1 = 4.17M | 30k*139*2 = 8.34M | 30k*139*4 = 16.68M | 30k*571*1 = 17.13M |
| Noise level, Np (dBm) | -174 + 10log10(4.17M) + 5 = -102.8 | -174 + 10log10(8.34M) + 5 = -99.8 | -174 + 10log10(16.68M) + 5 = -96.8 | -174 + 10log10(17.13M) + 5 = -96.7 |
| SNR (dB) [4] | -5 | -9.4 | -11.9 | -12.2 |
| P_max (dBm) | 10+10log10(4.17) = 16.2 | 10+10log10(8.34) = 19.2 | 10+10log10(16.68) = 22.2 | 10+10log10(17.13) = 22.3 |
| Backoff (dB) | 1.975 | 2.127 | 1.804 | 1.989 |
| P_TX (dBm) | min(16.2, 23-1.9750) = 16.2 | min(19.2, 23-2.127) = 19.2 | min(22.2, 23-1.804) = 21.196 | min(22.3, 23-1.989) = 21.011 |
| MCL (dB) | 16.2-(-5)-(-102.8) = 124.0 | 19.2-(-9.4)-(-99.8) = 128.4 | 21.196-(-11.9)-(-96.8) = 129.896 | 21.011-(-12.2)-(-96.7) = 129.911 |
| #NAME? |  |  |  |  |
| N_FDM | 4 | 1 | 1 | 1 |
| (# of ROs in 20MHz) |  |  |  |  |
| # of preambles per RO | 64 | 64 | 64 | 64 |
| Capacity | 4*64 = 256 | 1*64 = 64 | 1*64 = 64 | 1*64 = 64 |

Observation #1: In 30 kHz subcarrier spacing, four PRACH sequence repetition and the length-571 contiguous mapping PRACH sequence could obtain almost same MCL.

Observation #2: In 15 kHz subcarrier spacing, eight PRACH sequence repetition and the length-1151 contiguous mapping PRACH sequence could obtain almost same MCL.

Observation #3: Single long PRACH sequence with contiguous mapping is not preferred (thus, not supported) for NR-U.

Proposal #4: Support frequency domain repetition of Rel-15 PRACH sequences with cyclic shift and phase shift as PRACH preamble structure for NR-U.

- For 15 kHz subcarrier spacing,

  - Number of repeated sequences: 8

  - Cyclic shift pattern: [a, a+1, a, a+1, a, a+1, a, a+1]

  - Phase shift pattern: [1, 1i, 1i, -1, -1, 1i, 1i, 1]

- For 30 kHz subcarrier spacing,

  - Number of repeated sequences: 4

  - Cyclic shift pattern: [a, a+1, a, a+1]

  - Phase shift pattern: [1, 1i, 1i, 1]

Proposal #5: Consider the followings for PRACH preamble format in NR-U.

- Insertion of CCA gap between adjacent RACH occasions (ROs) in time domain

  - FFS on duration of the CCA gap (e.g. X usec or Y symbol)

- Addition of GP (Guard Period) after preamble part in PRACH format

  - Length of the GP is the same with CP length

Proposal #6: Determine CCA gap between adjacent ROs in time domain by considering LBT type applied for PRACH in terms of the channel access priority class and/or variable CWS (in case of Cat-4 LBT based PRACH transmission).

Proposal #7: Rel-15 NR PRACH preamble formats A1, A2, and A3 in terms of CP length, symbol length, and the number of symbols are reused for NR-U.

## Intel

15 kHz

| Parameter |  |  | Value |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme |  |  | Alt. 1 |  | Alt. 2 |  | Alt. 3 | Eg. Alt2-ZC139x8 |
| SCS |  |  | 15 |  |  |  |  | 15KHz or 30KHz |
| PRACH sequence length (L_RA) |  |  | 139 |  | 139 |  | 1151 | Eg. 139, 1151 |
| # of repetition (R) |  |  | 1 |  | 8 |  | 1 | If repetition of sequence is used in freq domain |
| N_cs |  |  | Depends on environment/network configuration |  |  |  |  | Eg. 11 |
| # of RBs used for one RO (N_RB) |  |  | 1 | 93 |  | 96 |  | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| RACH frequency occupancy (MHz) |  |  | 2.08 | 16.68 |  | 17.26 |  | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) |  |  | -105.8 | -96.8 |  | -96.6 |  | Np= -174+10*log10(SCS*L_RA*R)+NF |
|  |  |  |  |  |  |  |  | NF= 5dB |
| SNR (dB) | DS=10ns |  | -3.51 | -6.48 |  | -9.17 |  | SNR needed at 1% misdetection, read from simulation curve |
|  | DS=100ns |  | -3.41 | -9.68 |  | -11.43 |  |  |
| P_max (dBm) |  |  | 13.18 | 22.22 |  | 22.37 |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) |  |  | 2.3 | 2.6 |  | 2.3 |  | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. |
| P_TX (dBm) |  |  | 13.18 | 20.4 |  | 20.7 |  | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | DS=10ns | 122.49 | 123.68 |  | 126.47 |  | MCL = P_TX-SNR-Np |
|  |  | DS=100ns | 122.39 | 126.88 |  | 128.73 |  |  |
| N_FDM |  |  | 8 | 1 |  | 1 |  | # of ROs in 20MHz |
| Capacity |  |  | Depends on NCS |  |  |  |  | Across all ROs in 20MHz. Should report any constraints on ISD for the scheme evaluated. |

30 kHz:

| Parameter |  |  | Value |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme |  |  | Alt. 1 |  | Alt. 2 |  | Alt. 3 | Eg. Alt2-ZC139x4 |
| SCS |  |  | 30 |  |  |  |  | 15KHz or 30KHz |
| PRACH sequence length (L_RA) |  |  | 139 |  | 139 |  | 571 | Eg. 139, 571 |
| # of repetition (R) |  |  | 1 |  | 4 |  | 1 | If repetition of sequence is used in freq domain |
| N_cs |  |  | Depends on environment/network configuration |  |  |  |  | Eg. 11 |
| # of RBs used for one RO (N_RB) |  |  | 1 | 47 |  | 48 |  | # of RBs occupied by PRACH. Eg. 12 for ZC139 design |
| RACH frequency occupancy (MHz) |  |  | 4.17 | 16.68 |  | 17.13 |  | The actually used bandwidth with one RO, SCS*L_RA*R |
| Noise level, Np (dBm) |  |  | -102.8 | -96.8 |  | -96.7 |  | Np= -174+10*log10(SCS*L_RA*R)+NF |
|  |  |  |  |  |  |  |  | NF= 5dB |
| SNR (dB) | DS=10ns |  | -2.08 | -6.61 |  | -8.73 |  | SNR needed at 1% misdetection, read from simulation curve |
|  | DS=100ns |  | -3.13 | -9.56 |  | -10.77 |  |  |
| P_max (dBm) |  |  | 16.2 | 22.22 |  | 22.33 |  | Maximum allowed transmit power under PSD limit of 10dBm/MHz measured in any 1MHz chunk and considers the RBs used by the proposed scheme |
| Backoff (dB) |  |  | 2.3 | 2.5 |  | 2.3 |  | Backoff is computed as 95% percentile of CCDF of [cubic metric] over the preambles in the RO. |
| P_TX (dBm) |  |  | 16.2 | 20.5 |  | 20.7 |  | P_TX=min(P_max, 23- Backoff) is maximum allowed transmit power for the waveform considering backoff |
| MCL (dB) |  | DS=10ns | 121.08 | 123.91 |  | 126.13 |  | MCL = P_TX-SNR-Np |
|  |  | DS=100ns | 122.13 | 126.86 |  | 128.17 |  |  |
| N_FDM |  |  | 4 | 1 |  | 1 |  | # of ROs in 20MHz |
| Capacity |  |  | Depends on NCS |  |  |  |  | Across all ROs in 20MHz. Should report any constraints on ISD for the scheme evaluated. |

Proposal-8: Support only NR short PRACH formats (L=139) when temporal allowance of 2 MHz OCB is allowed by regulation.

Proposal-9: In addition to the Rel-15 design for NR short PRACH (sequence length of 139), support enhanced design of NR-U PRACH by adopting single long ZC sequence of the following lengths:

- 15 KHz: LRA = 1151

- 30 KHz: LRA = 571

Proposal-10: Support similar mapping rule as for Rel-15 NR short PRACH (sequence length of 139) to map logical index to sequence number (or root index) for NR-U PRACH with longer sequence lengths.

- Use a subset of logical indices to restrict high cubic metric root indices

Proposal-11: Support at least preamble formats A and B for PRACH in NR-unlicensed.

- Keep the number of rows in Rel-15 NR configuration index tables unchanged.

Observation-12: Exclusion of certain preamble format(s) can be enabled by proper configuration of configurationIndex by the scheduler, without changing the existing Rel-15 NR PRACH configuration index table.

Proposal-13: Do not support 60 KHz SCS for NR-U PRACH in FR.

Proposal-14: Support only TDM between PRACH and other UL signals/channels at least for PRACH transmission within UE acquired COT.

Observation-15: If PRACH is transmitted within gNB shared COT, where 2 MHZ temporal OCB is allowed by regulation, FDM between short PRACH (L=139) and other UL signals/channels can be supported.

- The UEs scheduled with interlaced transmission would puncture their transmission on the REs that overlap with resources allocated to PRACH of other UEs.

Proposal-16: Introduce LBT gap between neighbouring ROs within a RACH slot to avoid blocking of one UE by RACH transmission of another UE in the previous RO.

## Nokia

Observation 11: Based on WID, new enhanced design of PRACH for NR-U should support minimum bandwidth requirement (16 MHz) by regulation to be applicable in UE initiated COT consisting of PRACH transmission.

Proposal 11: The following options are in scope (to meet the OCB requirement as required by the WID)

- 15 kHz SCS

  - Single sequence with length of 1151

  - 8 repetitions of length 139

- 30 kHz SCS

  - Single sequence with length of 571

  - 4 repetitions of length 139

Proposal 12: Support only PRACH formats A and B in NR-U.

## NTT

## NEC

## Qualcomm

15 kHz table:

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scheme | Rel 15, ZC-139 | ZC-139x4 | ZC-139x8 | ZC-1151 | ZC-571 |
| SCS | 15 | 15 | 15 | 15 | 15 |
| PRACH sequence length (L_RA) | 139 | 139 | 139 | 1151 | 571 |
| # of repetition (R) | 1 | 4 | 4 | 1 | 1 |
| N_cs | 17 | 17 | 17 | 55 | 46 |
| # of RBs used for one RO (N_RB) | 12 | 24 | 96 | 96 | 48 |
| # of interlaces used by one RO (N_interlace) | N/A | N/A | N/A | N/A | N/A |
| RACH frequency occupancy (MHz) | 2.085 | 8.34 | 16.68 | 17.265 | 8.565 |
| Noise level, Np (dBm) | -105.81 | -99.8 | -96.79 | -96.63 | -99.67 |
| SNR (dB) | -4.63 | -11.48 | -14.86 | -14.97 | -11.65 |
| P_max (dBm) | 13.19 | 19.2 | 22.22 | 22.37 | 19.32 |
| Backoff (dB) | 2.34 | 4.29 | 4.29 | 2.34 | 2.34 |
| P_TX (dBm) | 13.19 | 18.71 | 18.71 | 20.66 | 19.32 |
| P_TX with updated formula | 13.19 | 20.71 | 20.71 | 22.37 | 19.32 |
| MCL (dB) | 123.63 | 129.99 | 130.36 | 132.26 | 130.64 |
| MCL with updated formula | 123.63 | 131.99 | 132.36 | 133.97 | 130.64 |
| N_FDM | 8 | 1 | 1 | 1 | 2 |
| Capacity | 8832 | 1104 | 1104 | 23000 | 13680 |

30 kHz table:

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Scheme | Rel 15, ZC-139 | ZC-139x2 | ZC-139x4 | ZC-571 | ZC-283 |
| SCS | 30 | 30 | 30 | 30 | 30 |
| PRACH sequence length (L_RA) | 139 | 139 | 139 | 571 | 283 |
| # of repetition (R) | 1 | 2 | 4 | 1 | 1 |
| N_cs | 17 | 17 | 17 | 46 | 34 |
| # of RBs used for one RO (N_RB) | 12 | 24 | 48 | 48 | 24 |
| # of interlaces used by one RO (N_interlace) | N/A | N/A | N/A | N/A | N/A |
| RACH frequency occupancy (MHz) | 4.17 | 8.34 | 16.68 | 17.13 | 8.49 |
| Noise level, Np (dBm) | -102.8 | -99.8 | -96.8 | -96.66 | -99.7 |
| SNR (dB) | -4.81 | -8.69 | -12.52 | -11.89 | -8.68 |
| P_max (dBm) | 16.2 | 19.2 | 22.2 | 22.33 | 19.28 |
| Backoff (dB) | 2.34 | 3.04 | 4.27 | 2.34 | 2.34 |
| P_TX (dBm) | 16.2 | 19.2 | 18.73 | 20.66 | 19.28 |
| P_TX with updated formula | 16.2 | 19.2 | 20.73 | 22.33 | 19.28 |
| MCL (dB) | 123.8 | 127.69 | 128.05 | 129.21 | 127.66 |
| MCL with updated formula | 123.8 | 127.69 | 130.05 | 130.88 | 127.66 |
| N_FDM | 4 | 1 | 1 | 1 | 2 |
| Capacity | 4416 | 1104 | 1104 | 6840 | 4512 |

## Ericsson

15 kHz table:

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Scheme | ZC139 (Rel-15) | ZC571 | ZC139x4 | ZC139x4, CM reduction |
| SCS (kHz) | 15 | 15 | 15 | 15 |
| PRACH sequence length (L_RA) | 139 | 571 | 139 | 139 |
| # of repetition (R) | 1 | 1 | 4 | 4 |
| N_cs | 6 (i.e. 23 shifts) | 20 (i.e. 28 shifts) | 5 (i.e. 27 shifts) | 5 (i.e. 27 shifts) |
| # of RBs used for one RO (N_RB) | 12 | 48 | 48 | 48 |
| # of interlaces used by one RO (N_interlace) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) |
| RACH frequency occupancy (MHz) (1) | 2.085 | 8.565 | 8.340 | 8.340 |
| Noise level, Np (dBm) (1) | -105.8 | -99.7 | -99.8 | -99.8 |
| SNR (dB) corresponding to 1% missed detection probability | -4.5 | -11.8 | -11.7 | -11.6 |
| P_max (dBm) (2) | 13.2 | 19.3 | 19.2 | 19.2 |
| Backoff (dB) (3) | 2.3 | 2.3 | 5.4 | 0.5 |
| P_TX (dBm) | 13.2 | 19.3 | 17.6 | 19.2 |
| MCL (dB) | 123.5 | 130.8 | 129.1 | 130.6 |
| N_FDM | 8 | 2 | 2 | 2 |
| Capacity for full network (per time-domain PRACH occasion) (4) | 813823 = 25392 | 257028 = 31920 | 213827 = 7452 | 213827 = 7452 |
| Capacity for one cell (per time-domain PRACH occasion) (5) | 864 = 512 | 264 = 128 | 264 = 128 | 264 = 128 |
| (1) Only includes used subcarriers, for consistency with the SNR definition used in simulations. (2) Based on the assumption of a max PSD of 10 dBm per 1 MHz interval. All four designs have a maximum of 66.7 subcarriers within 1 MHz. (3) Backoff based on 95th percentile of the CCDF of cubic metric. (4) Obtained as # of freq domain ROs * # of roots * # of cyclic shifts, for the agreed simulation assumptions (300 m ISD). (5) Obtained as # of freq domain ROs * 64. |  |  |  |  |

30 kHz table:

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Scheme | ZC139 (Rel-15) | ZC283 | ZC139x2 | ZC139x2, CM reduction |
| SCS (kHz) | 30 | 30 | 30 | 30 |
| PRACH sequence length (L_RA) | 139 | 283 | 139 | 139 |
| # of repetition (R) | 1 | 1 | 2 | 2 |
| N_cs | 10 (i.e. 13 shifts) | 20 (i.e. 14 shifts) | 10 (i.e. 13 shifts) | 10 (i.e. 13 shifts) |
| # of RBs used for one RO (N_RB) | 12 | 24 | 24 | 24 |
| # of interlaces used by one RO (N_interlace) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) | N/A (Contiguous PRB design) |
| RACH frequency occupancy (MHz) (1) | 4.170 | 8.490 | 8.340 | 8.340 |
| Noise level, Np (dBm) (1) | -102.8 | -99.7 | -99.8 | -99.8 |
| SNR (dB) corresponding to 1% missed detection probability | -5.0 | -8.7 | -8.6 | -8.6 |
| P_max (dBm) (2) | 16.2 | 19.3 | 19.2 | 19.2 |
| Backoff (dB) (3) | 2.3 | 2.3 | 2.7 | 1.8 |
| P_TX (dBm) | 16.2 | 19.3 | 19.2 | 19.2 |
| MCL (dB) | 124.0 | 127.7 | 127.6 | 127.6 |
| N_FDM | 4 | 2 | 2 | 2 |
| Capacity for full network (per time-domain PRACH occasion) (4) | 413813 = 7176 | 228214 = 7896 | 213813 = 3588 | 213813 = 3588 |
| Capacity for one cell (per time-domain PRACH occasion) (5) | 464 = 256 | 264 = 128 | 264 = 128 | 264 = 128 |
| (1) Only includes used subcarriers, for consistency with the SNR definition used in simulations. (2) Based on the assumption of a max PSD of 10 dBm per 1 MHz interval. All four designs have a maximum of 33.3 subcarriers within 1 MHz. (3) Backoff based on 95th percentile of the CCDF of cubic metric. (4) Obtained as # of freq domain ROs * # of roots * # of cyclic shifts, for the agreed simulation assumptions (300 m ISD). (5) Obtained as # of freq domain ROs * 64. |  |  |  |  |

Contributions are labelled by the first author