3GPP TSG-RAN WG1 Meeting #98	R1-1909822

Prague, Czech Republic, 26th – 30th August 2019

Source:	WI rapporteur (Ericsson)

Title:	RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE

Agenda Item:	6.2.1

Document for:	Information

- Introduction

This contribution lists RAN1 agreements made for the Rel-16 WI on ‘Additional MTC Enhancements for LTE’ (WI code LTE_eMTC5-Core, WID in RP-191356) until and including RAN1#98.

Notes:

- The WI objective from the WID is inserted below for convenience.

- Beside agreements, the summary also lists submitted feature lead summaries for information.

- The most recent version of the WI work plan is available in R3-191746.

- RAN2 agreements are summarised in R2-1911575.

- Table of contents

1	Introduction	1

2	Table of contents	1

3	WI objective	1

4	UE-group wake-up signal	3

5	Transmission in preconfigured UL resources	7

6	Scheduling of multiple DL/UL transport blocks	15

7	Coexistence of LTE-MTC with NR	21

8	Support of quality report in Msg3	25

9	MPDCCH performance improvement	29

10	CE mode A and B improvements for non-BL UEs	32

11	Use of LTE control region for DL transmission	35

12	Use of RSS for measurement improvements	36

- WI objective

Core part WI objective:

The objective is to specify the following set of improvements for machine-type communications for BL/CE UEs. Improved DL transmission efficiency and/or UE power consumption: - Specify support for mobile-terminated (MT) early data transmission (EDT) [RAN2, RAN3] - Specify quality report in MSG3 at least for EDT [RAN1, RAN2, RAN4] - Specify aperiodic quality report in connected mode using same quality definition as in Msg3 [RAN1, RAN2, RAN4] - Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4] - Specify support for UE-group wake-up signal (WUS) [RAN1, RAN2, RAN4] Improved UL transmission efficiency and/or UE power consumption: - Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4] - Both shared resources and dedicated resources can be discussed - Note: This is limited to orthogonal (multi) access schemes Scheduling enhancement: - Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast [RAN1, RAN2] - Enhancement of SPS can be discussed. Extreme coverage for non-BL UEs: - Specify CE mode A and B improvements for non-BL UEs from among the following list [RAN1, RAN2, RAN4] - Enhancements to idle mode mobility - UE demodulation performance requirements for 2 RX antennas and full duplex FDD - Dual layer DL reception - Feedback based on CSI-RS - ETWS/CMAS in connected mode Stand-alone deployment: - Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs [RAN1, RAN2, RAN4] - This deployment mode should support legacy operation for legacy BL/CE UEs. Mobility Enhancement: - Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS [RAN1, RAN4, RAN2] - Specify relaxation of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs [RAN4, RAN2] Coexistence with NR: - Specify the following performance improvements for LTE-MTC coexistence with NR [RAN1, RAN2, RAN4] - LTE-MTC resource reservation at least in the time domain with at least one of slot-level and symbol-level granularity at least in DL to avoid resource overlap between NR and LTE-MTC when LTE-MTC is deployed within an NR carrier - LTE-MTC subcarrier puncturing for a small number of LTE-MTC DL subcarriers (excluding CRS) to reduce the number of NR resource blocks that need to be reserved for LTE-MTC when LTE-MTC is deployed within an NR carrier - For LTE-MTC in-band operation co-existence with NR, RAN4 will investigate the following: - 15 kHz, 30 kHz, and 60 kHz numerologies for NR FR1 bands, with higher priority given first to 15 kHz and then to 30 kHz - Study feasible LTE-MTC placement allocation without RF backward compatibility impact and compatible with Rel-13 LTE-MTC and Rel-15 NR, to operate simultaneously within various NR channel bandwidths - Channel raster, PRB and subcarrier grid alignment between LTE-MTC and NR - Synchronization issue between LTE-MTC and NR, including timing advance - Frequency band support in LTE-MTC and NR - Testability applicability - Compatibility for Rel-15 NR and Rel-13/14/15 LTE-MTC - The case of NR configured with 15 kHz SS block SCS and the case of 30 kHz SS block SCS as specified in 38.101-1 are included in the study. Note: After RAN1 concludes the objective on R16 LTE-MTC coexistence aspects, evaluate coexistence between R15 NR and R16 LTE-MTC. Connection to 5GC: - Specify support for the following features [RAN2, RAN3] - Support of extended DRX in CM-IDLE - Support of extended DRX in CM-CONNECTED with RRC_INACTIVE (support of sleep cycles up to the NAS and SMS retransmission timers) - Support of User Plane CIoT optimisation - Support of EDT for Data over NAS and User Plane CIoT optimization (see Note) - Support of restriction of use of Enhanced Coverage - Delivery of Expected UE Behaviour information to the RAN - Additional information in SIB to indicate supported CIoT features; indication of CIoT features supported by the UE in RRC Note: Based on the outcome of RAN2/SA2 liaison exchanges, EDT support for connection to 5G-CN may be later updated to also include RRC_INACTIVE.

Performance part WI objective:

Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements.

- UE-group wake-up signal

RAN1#94:

R1-1809521 Summary of 6.2.1.1 UE group MWUS Qualcomm Agreement UE-group WUSs are only multiplexed in the same NB as associated PO - FFS TDM/FDM/CDM for UE-group MWUS multiplexing Agreement - UE-group MWUS is supported based on eNB’s and UE’s capability. - Whether the network supports UE-group MWUS is done by higher layer signalling. - FFS: The number of UE groups is configured by SIB. - Note that the UE-group MWUS is UE optional Agreement Rel-16 UE-group MWUS sequence should consider at least - Fallback to legacy UE behavior - Inter-cell interference randomization - UE group ID for different UE-group MWUS - Reuse of Rel-15 sequences is not precluded - Effect of sequence detection on UE complexity Agreement Study the RAN1 consequence of UE-grouping on the following basis: - UE ID - Coverage - DRX/eDRX - Gap configuration - Services

RAN1#94bis:

R1-1811658 Summary of 6.2.1.1 UE group MWUS Qualcomm Agreement UE grouping is based on at least UE ID or some function of UE ID Agreement The legacy UE should not be prevented from using legacy WUS even in the case of Rel-16 group WUS is enabled - Performance impact on legacy WUS should be carefully considered Agreement Group WUS is based on at least legacy WUS and UE-group ID. Agreement Configuration of group WUS is at least signaled in SI Agreement A Rel-16 group WUS capable UE shall also be capable of Rel-15 legacy WUS

RAN1#95:

R1-1813715 Summary of 6.2.1.1 UE group MWUS Qualcomm Agreement For multiplexing between Rel-16 UE-group WUS and Rel-15 WUS, further evaluate and down select among the following options - TDM - FDM - single-seq CDM - single-seq CDM+TDM - single-seq CDM+FDM - FFS whether legacy WUS is the common WUS for all new UEs or only a part of new UEs. Agreement For multiplexing between different Rel-16 UE-group WUS, further evaluate and down select among the following options - single-seq CDM - FDM - single-seq CDM+TDM - single-seq CDM+FDM Note: At least the maximum number of UE groups should be considered. Agreement The number of UE groups is configurable and broadcasted in SIB. - FFS: Further details on the number of UE groups. For example, whether it is per PO or per gap configuration of a PO Agreement UE group ID is used as a parameter to generate WUS UE group sequence(s). Agreement One group WUS is designed as a single sequence Agreement Further study false detection (cross/auto correlation) performance properties for the following designs: - legacy WUS + cover codes, - legacy WUS + shifted scrambling codes, - legacy WUS + phase shift + cover code + scrambling bits - Including combinations of phase shift, cover code, and/or scrambling bits Other designs are not precluded. Agreement Rel-16 group WUS uses the same gap configurations as for Rel-15 legacy WUS except for differences from possible TDM. - No new gap higher layer signaling will be introduced for TDM

RAN1#96:

R1-1903252 Summary of 6.2.1.1 UE group MWUS Qualcomm R1-1903286 Remaining issues of 6.2.1.1 UE group MWUS Qualcomm Agreement UE-group MWUS sequence is same as UE-group NWUS per RB and the same sequence is repeated in two PRBs. Agreement - A UE is required to monitor WUS(s) in only one WUS (time/frequency) resource location. Agreement - Single-sequence CDM is used in the same WUS resource if more than one UE group is configured to monitor the WUS resource. - Select one among the following: - Up to 2 orthogonal WUS resources may be configured in time domain - Up to 2 orthogonal WUS resources may be configured in frequency domain - Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total) - Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured) - FFS: whether one of the WUS resources is configured to be shared with legacy WUS Agreement If configured, a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource. - FFS: Whether the above is also applicable for Rel-15 WUS UEs - FFS: Whether to additionally support the feature of waking up a subset of all WUS UE groups

RAN1#96bis:

R1-1905530 Summary of 6.2.1.1 UE group MWUS Qualcomm R1-1905588 Remaining issues of 6.2.1.1 UE group MWUS Qualcomm Agreement For evaluation purpose and down-selection on multiplexing schemes in RAN1#97: - UE assumes the same WUS max duration and same transmit power for Rel-15 WUS and Rel-16 WUS Agreement Down-select one of the following options until RAN1#97 based on evaluation results including power saving gain, usage of resources, etc - Up to 2 orthogonal WUS resources may be configured in time domain - Up to 2 orthogonal WUS resources may be configured in frequency domain - Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total) - Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured) Determine in RAN1#97 whether legacy WUS resource is counted as one of the configured WUS resource(s). Agreement Rel-15 WUS and Rel-16 WUS can be configured on the same legacy WUS resource via SI - FFS explicitly or implicitly - Same WUS parameters are assumed for both Rel-15 and Rel-16 WUS in case both are on the same legacy WUS resource Agreement - Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation. - Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap. Agreement If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

RAN1#97:

R1-1907570 Feature lead summary of 6.2.1.1 UE group MWUS Qualcomm Inc R1-1907632 UE-group wake-up signal in LTE-MTC Ericsson Agreement Support the following options for WUS resource configuration - Up to 2 orthogonal resources may be configured in time domain. - Up to 2 orthogonal resources may be configured in frequency domain - FFS: both options can be configured simultaneously to have up to 4 orthogonal WUS resources Maximum total number of WUS resources is no larger than 4 including the legacy WUS resource For further discussion Different WUS sequences associated with the same PO are used for UE groups regardless of whether the UE groups are in the same or different WUS resources. Working Assumption - UE assumes the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence. - Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence Agreement Each UE monitors up to X WUS sequences. - Value of X will be selected between 2 and 3 - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

RAN1#98:

R1-1909394 Feature lead summary of 6.2.1.1 UE group MWUS Qualcomm Inc Agreement Both options can be configured simultaneously to have up to 4 orthogonal WUS resources including legacy WUS resource - Up to 2 orthogonal resources including legacy WUS resource may be configured in time domain - Up to 2 orthogonal resources may be configured in frequency domain Note: The two orthogonal resources do not necessarily include the legacy WUS resource Agreement The maximum number of UE groups per WUS resource is 8. Agreement Different WUS sequences are used in same/different WUS resources. Working Assumption To differentiate WUS sequences in different WUS resources, 2-bit MSB of scrambling initialization c_init is supported Agreement The following working assumption is confirmed with the modification and under the condition that the eNB can set the power offset between Rel-15 and Rel-16 sequences (UE does not need to know the power offset) - UE assumes may assume the transmit power for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence. - Maximum WUS duration for Rel-16 WUS sequence is same as that of Rel-15 WUS sequence Agreement A UE is required to detect 2 sequences, the common WUS and the group WUS of the group to which it belongs. Agreement The specification supports configurability to enable UE group to be changed among WUS resources. - FFS: Details including implicit/explicit signalling

- Transmission in preconfigured UL resources

RAN1#94:

R1-1809528 LTE-M Preconfigured UL Resources Summary Sierra Wireless Agreement Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA - FFS: Validation mechanism for TA - FFS: How the pre-configured UL resources is acquired Agreement For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed Agreement Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types. Agreement HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered: - Whether to support HARQ; - If supported, details of HARQ design including the number of HARQ processes; - Whether ACK/NACK is necessary Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures.

RAN1#94bis:

R1-1810491 LTE-M Preconfigured UL Resources Summary RAN1 #94-bis Sierra Wireless, S.A. Agreement In idle mode, the UE will at least consider one or more of the following attributes when validating TA (combination of multiple attributes is allowed): - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell RSRP changes (serving cell refers the cell that the UE is camping on) - FFS Other attributes: - Neighbour cell RSRP change - TDOA of >=2 eNBs - TA History - Subscription based UE differentiation - Others not precluded (for example, attributes that need to be considered for high mobility UEs) Note that UE power consumption should be taken into account for the FFS attributes Agreement Dedicated preconfigured UL resource is defined as an PUSCH resource used by a single UE - PUSCH resource is time-frequency resource - Dedicated PUR is contention-free Contention-free shared preconfigured UL resource (CFS PUR) is defined as an PUSCH resource simultaneously used by more than one UE - PUSCH resource is at least time-frequency resource - CFS PUR is contention-free Contention-based shared preconfigured UL resource (CBS PUR) is defined as an PUSCH resource simultaneously used by more than one UE - PUSCH resource is at least time-frequency resource - CBS PUR is contention-based (CBS PUR may require contention resolution) Agreement In IDLE mode, HARQ is supported for transmission in dedicated PUR - A single HARQ process is supported, - FFS whether more than one HARQ processes are supported - FFS: The design of the corresponding MPDCCH search space Agreement In idle mode, dedicated PUR is supported. - Support for CFS PUR is FFS. - Support for CBS PUR is FFS. Agreement For UL transmission in preconfigured resource, fallback mechanism to RACH/EDT procedures is supported. Agreement For transmission in preconfigured UL resources, an RRC idle UE may use the latest TA that passed the validation criteria Agreement Pre-configured UL resources for transmission of data are indicated by RRC signaling. At least UE-specific RRC signaling is supported.

RAN1#95:

R1-1813725 LTE-M Preconfigured UL Resources Summary RAN1 #95 Sierra Wireless Agreement For dedicated PUR in idle mode, the UE may skip UL transmissions. - FFS: Resource release mechanism - FFS: Whether or not to support mechanism to disable skipping by eNB Agreement The following evaluation parameters for CFS PUR link level simulation are agreed. Parameter; Value/Description. Channel model; ETU 1Hz. Resource Bandwidth(s); 1, 2, 3 or 6 PRBs (contiguous) for eMTC Proponent to provide details on resource allocation Same bandwidth for all UEs. Traffic Pattern; Fixed (N UEs transmit in the same resource) or (Optional) Bursty (N UEs may transmit in the same resource, with activity factor p for each of them) Note: For all traffic models, it is assumed that a UE has a dedicated DMRS Note: Final decision can be based on the results for both fixed and bursty traffic model. Number of repetitions in a resource; To be declared by proponent. Transport Block Size and Modulation Order; To be declared by proponent. Number of eNB Rx antennas; 2 as baseline, other values are not precluded. Metrics; BLER before HARQ Number of resources (PRB x subframes) used by the technique to serve a given number of UEs Baseline for comparison is dedicated PUR. MCL; 164, 144, 124dB MCL. UE grouping; All UEs in the same MCL level.. Processing Receiver at eNB; No SIC/joint decoding of UEs is assumed. MMSE single user decoding. Not NOMA.. DMRS collision; Each company to provide details on their DMRS assumption. Channel Estimation CFO Timing Error Power Imbalance; Realistic Uniform +/-50 Hz, +/-100 Hz (optional) +/-0.5 CP uniform, +/-1 CP uniform (optional) Uniform +/- [5 or 10] dB. The evaluated proposals should be within the scope of the eMTC WI. - Agreement - If multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and single HARQ process - FFS: if multi-TB grant is enabled/supported - Agreement - In idle mode, at least the following TA validation attributes are supported: - Serving cell changes (serving cell refers the cell that the UE is camping on) - Time Alignment Timer for idle mode - Serving cell RSRP changes (serving cell refers the cell that the UE is camping on) - Based on RSRP measurement definition in existing Rel-15 TS36.214 Include in LS to RAN2, RAN4 to consider in their work. - Agreement - The UE can be configured to use at least these TA validation attributes: - Time Alignment Timer for idle mode - Serving cell RSRP changes - Note: the configuration shall support disabling of the TA validation attributes Include in LS to RAN2, RAN4 For further study: TA validation attributes: - Subscription based UE differentiation (or Stationary indication in held in subscription) - Cell specific indication where TA is valid within that cell Agreement Include in LS to RAN2, RAN4: RAN1 assumes that a UE transitioning from EDT/connected to idle mode can use the valid TA that was used while in EDT/connected mode. Agreement - For dedicated PUR in idle mode, UL grant for HARQ retransmission is transmitted in MPDCCH search space - FFS: Details on the search space (for example USS, CSS) - Agreement - For dedicated PUR in idle mode, upon successful decoding by eNB of a PUR transmission, the UE can expect an explicit ACK FFS: if ACK is sent on MPDCCH (layer 1) and/or PDSCH (layer 2/3) Include in LS to RAN2, RAN4. - Agreement - For dedicated PUR in idle mode, upon unsuccessful decoding by eNB of a PUR transmission, the UE can expect - an UL GRANT for retransmission on the MPDCCH, or - FFS: a NACK, or - FFS: no explicit ACK Include in LS to RAN2, RAN4. - Agreement - Send LS to RAN2, RAN4 to consider in their work. All the agreements with ‘include in LS to RAN2, RAN4’ for NB-IoT and eMTC should be captured in this LS. The LS is endorsed in R1-1813778

RAN1#96:

R1-1903248 LTE-M Preconfigured UL Resources Summary RAN1 #96 Sierra Wireless R1-1903293 LTE-M Preconfigured UL Resources offline conclusion and remain issues Sierra Wireless Agreement In idle mode, the TA validation configuration can include “PUR Time Alignment Timer” - Where the UE considers the TA as invalid if the (current time – time at last TA update) > the PUR Time Alignment Timer - Details on how to specify the “PUR Time Alignment Timer” is up to RAN2 Agreement - In idle mode, when the UE validates TA, the UE considers the TA for the previous serving cell as invalid if the serving cell changes - Above applies for the case where the UE is configured to use the serving cell change attribute Agreement For dedicated PUR in idle mode, the Dedicated PUR ACK is at least sent on MPDCCH - RAN2 can decide if a higher layer PUR ACK is also supported Agreement - For dedicated PUR in idle mode, the PUR search space configuration shall be included in the PUR configuration. - PUR search space is the search space where UE monitors for MPDCCH - FFS: Whether PUR search space is common or UE specific Agreement - When the TA is validated and found to be invalid and the UE has data to send, the UE can obtain a valid TA and may send data via legacy RACH or EDT procedures - FFS whether only TA is acquired and then data sent on PUR is supported - FFS other approaches to obtain a valid TA Agreement - When the UE is configured to use several TA validation criteria, the TA is valid only when all the configured TA validation criteria are satisfied. Agreement For dedicated PUR, in idle mode, the PUR resource configuration includes at least the following - Time domain resources including periodicity(s) - Note: also includes number of repetitions, number of RUs, starting position - Frequency domain resources - TBS(s)/MCS(s) - Power control parameters - Legacy DMRS pattern Agreement In idle mode, at least the following PUR configurations and PUR parameters may be updated after a PUR transmission: - Timing advance adjustment - UE TX power adjustment - FFS: Repetition adjustment for PUSCH FFS: Whether the above update is done in L1 and/or higher layer Agreement In idle mode, the PUR search space configuration includes at least the following: - MPDCCH narrowband location - MPDCCH repetitions and aggregation levels - MPDCCH starting subframe periodicity (variable G) - Starting subframe position (alpha_offset) Agreement For dedicated PUR in idle mode, the PUR resource configuration includes at least the following - A PUSCH frequency hopping indication to enable or disable legacy frequency hopping Agreement In idle mode, a UE can be configured such that TA is always valid within a given cell. - FFS: up to RAN2 how to implement e.g. PUR Time Alignment Timer = infinity

RAN1#96bis:

R1-1903904 LTE-M Preconfigured UL Resources Summary RAN1 #96Bis Sierra Wireless, S.A. R1-1905582 LTE-M Preconfigured UL Resources Summary #2 RAN1 #96Bis Sierra Wireless - Working Assumption #1 - In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters - The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted. - Working Assumption #2 - For dedicated PUR - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived - FFS if the UE monitors any additional RNTI which may be shared with other UEs. - Note: The same RNTI may be used over non-overlapping time and/or frequency resources - Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed. R1-1905563 [DRAFT] LS on PUR Working Assumptions for NB-IoT and LTE-M Sierra Wireless The LS is endorsed in R1-1905570. - Agreement - The UE monitors the MPDCCH for at least a time period after a PUR transmission - FFS: Details of the time period - FFS: UE behaviour if nothing is received in the time period - FFS: If and how often UE monitors MPDCCH after a PUR allocation in which it has not transmitted - Agreement The value(s) of RSRP threshold(s) is UE specific - Agreement - The power control parameters within the PUR configuration, shall at least include: - Target UL power level (P_0) for the PUR transmission Agreement - For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

RAN1#97:

R1-1907583 LTE-M Preconfigured UL Resources Summary RAN1 #97 Sierra Wireless R1-1907629 LTE-M Preconfigured UL Resources Summary #2 RAN1 #97 Sierra Wireless Agreement - For a given UE, for dedicated PUR in idle mode and for a given CE mode, the same size DCI, the same PUR M-PDCCH candidates, and the same RNTI is used for all DCI messages for unicast transmissions. Agreement For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission - FFS: Value of x, and if x is fixed or signaled - FFS: FD-FDD UEs, TDD UEs - FFS: Support for monitoring of PUR SS Window before PUR transmission - Note: The PUR SS Window is the time period where the UE monitors the MPDCCH for at least a time period after a PUR transmission Agreement For dedicated PUR in idle mode, - The maximum mPDCCH repetitions, rmax-mPDCCH-PUR, is included in the PUR configuration Agreement For dedicated PUR in idle mode, the duration of the PUR SS window is configured by eNB - How the duration is configured, and the possible values, will be decided by RAN2. Agreement For dedicated PUR in idle mode, the CE mode is - Option 1: explicitly configured in the PUR configuration. - Option2: based on CE mode of last connection Down select in RAN1#98 Agreement Select one of the following in RAN1#98 - Alt1: In idle mode, the PUR search space PRB pairs is configured between {2, 2+4, 4} PRBs - Alt2: In idle mode, the PUR search space PRB pairs is fixed to 2+4 PRBs Agreement - For dedicated PUR in idle mode, if a UE skips a PUR transmission, it is not mandated to monitor the associated PUR SS window

RAN1#98:

R1-1908189 LTE-M Preconfigured UL Resources Summary RAN1 #98 Sierra Wireless R1-1909688 Summary of offline discussion on PUR for NB-IoT and eMTC Futurewei, Sierra Wireless Agreement The following WA is confirmed: In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported - FFS: Which PUR configurations and PUR parameters will be signaled via L1 - FFS: Definition of PUR configurations and PUR parameters Agreement The following WA is confirmed: For dedicated PUR - During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived - FFS if the UE monitors any additional RNTI which may be shared with other UEs. - Note: The same RNTI may be used over non-overlapping time and/or frequency resources Agreement - In idle mode, the PUR search space PRB pairs is configured between {2, 2+4, 4} PRBs and this configuration is in the PUR configuration. Agreement - The PUR (re)configuration explicitly signals the CE mode which can only be changed via RRC signalling (i.e. not via L1 messages). Agreement For dedicated PUR in idle mode, the PUR MPDCCH candidates are derived using legacy USS rules where rmax= rmax-mPDCCH-PUR - FFS: Initialization of Yp,k Agreement - For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is 4 subframes after the end of the PUR transmission. - For dedicated PUR in idle mode and for FD-FDD/TDD UEs, the start of the PUR SS Window is 4 subframes after the end of the PUR transmission. - FFS whether/when the UE is monitoring for early termination during the PUR transmission Agreement - After data transmission on PUR, if nothing is received by the UE in a time period, the UE shall fallback to legacy RACH/EDT procedure. - FFS: Details on time period Agreement RAN1 will downselect among the following indications for PUR L1 ACK in RAN1#98bis Timing advance adjustment (including TA adjustment of 0) UE TX power adjustment PUSCH repetition adjustment Indication of PUR SS monitoring termination Flag to indicate L1 ACK None of the above (i.e. the legacy L1 ACK will be reused) Note: No new indications will be considered. Agreement For PUR transmissions in a PUR allocation, the TX power is at least based on the estimated path loss and the configured Target UL Power Level (P_0). - FFS: if other power control parameters beyond Target UL Power Level (P_0) are needed (e.g. ramping step) For PUR HARQ re-transmissions in CE mode A, the TPC field in the UL grant is used to adjust the TX power - FFS for CE mode B - FFS whether to reset the TPC accumulation mechanism for every initial PUR transmission - FFS: if other power control parameters beyond Target UL Power Level (P_0) are needed (e.g. ramping step) Agreement - The dedicated PUR ACK DCI at least allows for indicating implicitly or explicitly that the current PUR monitoring is terminated FFS: The dedicated PUR ACK DCI at least allows for indicating whether the UE continues monitoring the PUR search space after a gap (whether the gap is always zero or it can be zero) Agreement - After data transmission on PUR, the UE may expect an explicit indication on MPDCCH for fallback to EDT or RACH Working assumption For eMTC full PRB allocation, for PUR with R>= [64 or 128] repetitions: - Allow for UE-specific cyclic shift ([2 or 4 or 8] cyclic shifts) for DMRS. - The number of PRBs that can be allocated to a UE are the same as in legacy (e.g., max 2PRBs can be allocated to a UE in CE Mode B) - For PUSCH scrambling, the baseline is the current c_init equation by using the configured PUR_RNTI. - Companies are welcome to evaluate baseline c_init, alternative c_init (e.g. c_init for NPDSCH rotation sequence) equations, and other proposals. - Note 1: The Working Assumption is also subject to the potential RAN2 and RAN4 specification impacts. Note 2: It is transparent to a given UE whether the eNB is allocating other UEs in the same resource.

- Scheduling of multiple DL/UL transport blocks

RAN1#94:

R1-1809562 Feature lead summary Scheduling of multiple DL UL transport blocks Ericsson R1-1809615 Updated Feature lead summary Scheduling of multiple DL UL transport blocks Ericsson Agreement Specify scheduling of multiple transport blocks for both CE Mode A and B Agreement The possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD Agreement When scheduling of multiple TBs is enabled, the number of scheduled transport blocks (>= 1) should be dynamically selected via DCI. The maximum number of scheduled transport blocks with one single DCI is [TBD]. - The number of blind decodes for MPDCCH is not increased as a result of scheduling multiple TBs Conclusion When multiple TBs are scheduled by one DCI, study interleaving amongst TBs from different HARQ process in cases of repetitions - Companies are encouraged to submit evaluation results in the next RAN1 meeting Agreement One DCI to schedule multiple TBs for SC-MCCH is not supported Working Assumption For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, each transport block corresponds to a unique HARQ process.

RAN1#94bis:

R1-1811693 Feature lead summary Scheduling of multiple DL UL transport blocks Ericsson R1-1811680 Updated Feature lead summary Scheduling of multiple DL UL transport Ericsson Agreement Confirm the working assumption that - For unicast, when multiple DL/UL transport blocks are assigned by a single DCI, each transport block corresponds to a unique HARQ process. Agreement For CE mode A, the maximum number of scheduled transport blocks with one single DCI is [8] for CE mode A for UL. For CE mode A, the maximum number of scheduled transport blocks with one single DCI is [8] for CE mode A for DL. Agreement The maximum number of scheduled transport blocks with one single DCI for CE mode A for either UL or DL is fixed to [8] Working Assumption For CE mode B, the maximum number of scheduled transport blocks with one single DCI is 4 in the UL, and 4 for the DL. Agreement The maximum number of scheduled transport blocks with one single DCI for CE mode B for either UL or DL is fixed to 4 (working assumption) For further study until next meeting: How to efficiently handle retransmissions when scheduling multiple DL/UL transport blocks. Agreement The UE should only monitor one DCI size in the UE specific search space For further consideration: Optimization of the DCI size and the impact of aspects including number of transport blocks, scheduling pattern (interleaving and scheduling gap), resource assignment, modulation and coding scheme, retransmissions. Agreement Using one DCI to schedule multiple TBs for SC-MTCH is supported, and it is configured and enabled per SC-MTCH via SC-PTM configuration message in SC-MCCH. FFS the maximum number of TBs can be scheduled by one DCI. Agreement For CE mode A, HARQ ACK/NACK feedback bundling or multiplexing on PUCCH can be enabled or disabled by [RRC and/or DCI], when multiple DL transport blocks are assigned by a single DCI. If the network does not enable it, each TB has its own separately encoded HARQ ACK/NACK feedback, i.e., no HARQ ACK/NACK feedback bundling or multiplexing. - RAN1 further compare the performance between HARQ ACK/NACK feedback bundling and multiplexing and down-select between the two options. For CE mode B, further study if there is a benefit for HARQ ACK/NACK feedback bundling or multiplexing on PUCCH. If there is a benefit identified, same configuration principle as CE mode A can be applied, i.e., this feature can be enabled or disabled by [RRC and/or DCI].

RAN1#95:

R1-1813722 Feature lead summary Scheduling of multiple DL UL transport Ericsson R1-1813759 Updated Feature lead summary Scheduling of multiple DL UL transport Ericsson Agreement For CEmodeA, the maximum number of scheduled transport blocks with one single DCI is 8 in the UL, 8 in the DL Agreement The following working assumption is confirmed For CEmodeB, the maximum number of scheduled transport blocks with one single DCI is 4 in the UL, 4 in the DL Agreement For both UL and DL unicast, at least consecutive resource allocation in time is supported when multiple TBs are scheduled by one single DCI. - Above applies only for valid subframes within the consecutive resource allocation in time - FFS: Whether time gaps between two TBs is also supported For future meetings in Rel-16 eMTC: Further consider which Rel-14/15 features that should be possible to configure together with scheduling of multiple DL-UL transport blocks. Agreement For the DL unicast for a UE, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs: - Frequency-hopping flag, PMI confirmation (TM6-specific), Precoding information (TM6-specific), DM-RS scrambling / antenna ports (TM9-specific), Downlink assignment index (TDD-specific), PUCCH power control - FFS: MCS, RV, Resource assignment, Number of PDSCH repetitions Agreement For the UL unicast, when multiple TBs are scheduled by one DCI, the following parameter values are the same across all the TBs: - Frequency-hopping flag, TPC command - FFS: MCS, RV, Resource assignment, Repetition number, Downlink assignment index (TDD-specific) For next meeting For unicast when multi-TBs are scheduled, companies are encouraged to bring in DCI designs which can support - scheduling of initial and retransmission TBs within one DCI - scheduling of initial TBs within one DCI, and retransmissions with one DCI - scheduling of initial TBs within one DCI, and retransmission can only be scheduled by individual DCI RAN1 tries to make a decision on which case is specified in the next meeting based on the trade-off between DCI overhead and scheduling flexibility comparisons of the three cases. Agreement For the case of single DCI scheduling multiple transport blocks with repetitions, scheduling of transport blocks repetitions is down selected between: - Option 1: All the repetitions for one transport block are contiguously scheduled in valid UL/DL subframes - Option 2: The repetitions for one transport block are interleaved with repetitions of all the other transport blocks - Option 3: Option 1 and 2 are supported and eNB configures among them. Agreement The maximum number of TBs for multicast is 8.

RAN1#96:

R1-1903245 Feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC Ericsson Recommendation for future meetings: The use of scheduling gaps for unicast/multicast is further studied. Agreement Support option 3 from below For the case of single DCI scheduling multiple transport blocks with repetitions, scheduling of transport blocks repetitions is down selected between: - Option 1: All the repetitions for one transport block are contiguously scheduled in valid UL/DL subframes - Option 2: The repetitions for one transport block are interleaved with repetitions of all the other transport blocks - Option 3: Option 1 and 2 are supported and eNB configures among them Above applies for unicast only. Agreement For the DL/UL unicast for a UE, when multiple TBs are scheduled by one DCI, the parameter values for {MCS, Resource assignment, Repetitions} are the same across all the TBs scheduled by that DCI. - If specified, in the case where one DCI schedules TB for both initial and retransmission, the above applies for TBs for both initial and retransmission

RAN1#96bis:

R1-1905534 Feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC Ericsson For further discussion Which of the following features that should be possible to configure and/or use in combination with scheduling of multiple TBs - Rel-14 feature for new numbers of repetitions for PUSCH and modulation restrictions for PDSCH/PUSCH in CE mode A - Rel-14 feature for 2984 bits max UL TBS in 1.4 MHz in CE mode A - Rel-14 feature on HARQ-ACK bundling in HD-FDD in CE mode A - Rel-14 features for 5 or 20 MHz max PDSCH/PUSCH channel bandwidths in CE mode A/B - Rel-14 feature for 10 downlink HARQ processes in FDD in CE mode A - Rel-14 feature for dynamic HARQ-ACK delay for HD-FDD in CE mode A - Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B - Rel-15 feature for PUSCH sub-PRB allocation in CE mode A/B - Rel-15 feature for 64QAM for non-repeated unicast PDSCH in CE mode A - Rel-15 feature for uplink HARQ-ACK feedback in DCI in CE mode A/B Companies are encouraged to provide their preference on this issue. Agreement For unicast, the UE is configured for interleaved transmission of subframe repetitions of the respective TBs separately for DL and UL via RRC signaling. Agreement For CE mode A, HARQ ACK/NACK feedback bundling on PUCCH can be enabled or disabled by [RRC and/or DCI], when multiple DL transport blocks are assigned by a single DCI (details FFS). If the network does not enable it, each TB has its own separately encoded HARQ ACK/NACK feedback - FFS: Maximum bundle size Agreement For the DL unicast for a UE, when multiple TBs are scheduled by one DCI, the parameter value for number of PDSCH repetitions is the same across all the TBs scheduled by that DCI, and there is only a single parameter field for number of MPDCCH repetitions and only a single parameter field for SRS request. Agreement For scheduling of multiple TBs with SC-MTCH, the same parameter values for {MCS, resource assignment, number of repetitions} is used for all TBs

RAN1#97:

R1-1907580 Feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC Ericsson R1-1907617 Updated feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC Ericsson Working Assumption - For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported. - Till the RAN1#98 meeting, consider potential simplifications that can help achieve a tradeoff between scheduling flexibility and DCI size. For example: - Configurable maximum number of TBs per grant - Maximum size of DCI - Joint coding of DCI fields - Reduced TBS choices - Reduced resource allocation choices - Reduced/eliminated RV field Working Assumption For unicast, scheduling gaps for multiple transport blocks is supported and a scheduling gap can be configured by [RRC and/or DCI] - The support of scheduling gaps is UE optional feature regardless of the support of multiple TBs - FFS: Details on the scheduling gap such as duration, applicability, etc. Agreement For scheduling of multiple TBs with SC-MTCH, scheduling gaps are supported. Agreement For scheduling of multiple TBs with SC-MTCH, a DCI field indicates the number of scheduled TBs.

RAN1#98:

R1-1909497 Feature lead summary for Scheduling of multiple DL/UL transport blocks for LTE-MTC Ericsson R1-1909698 Feature lead summary #2 for Scheduling of multiple DL/UL transport blocks for LTE-MTC Ericsson Agreement In RAN1#98bis, select one of the following options for unicast in CE mode A For the purpose indicating the number of TBs - Option 1: 1 bit is added to the DCI to indicate 1 TB or multiple TBs - FFS: Details on how to indicate the exact number of TBs in case it is multiple - Option 2: A new or repurposed field(s) in DCI indicates implicitly or explicitly to indicate 1 TB or multiple TBs - FFS: Details on how to indicate the exact number of TBs in case it is multiple - Option 3: 1 bit is added to the DCI to indicate one of the following: - Up to 4 TBs are scheduled - In which case, a bit map is used to indicate details - Up to 8 TBs are scheduled - In which case, a table is used to indicate details Agreement In RAN1#98bis, select one of the following options for unicast in CE mode B For the purpose indicating the number of TBs - Option 1: 1 bit is added to the DCI to indicate 1 TB or multiple TBs - FFS: Details on how to indicate the exact number of TBs in case it is multiple - Option 2: A new or repurposed field(s) in DCI indicates implicitly or explicitly to indicate 1 TB or multiple TBs - FFS: Details on how to indicate the exact number of TBs in case it is multiple - Option 3: 1 bit is added to the DCI to indicate mixed(initial and retransmission) scheduling and non-mixed scheduling - Depending on whether it is mixed or non-mixed scheduling, UE interpretes the field indicating the number of TBs differently - FFS: Details Agreement For unicast, at least when the scheduling with a single DCI includes retransmission(s) in some of the allocated HARQ process(es), the allocation of the HARQ processes is downselected from the following options in RAN1#98bis - Option 1: Contiguous allocation of any starting HARQ process # - FFS: The number of contiguous allocations defined in the DCI - Option 2: Contiguous allocation of a limited set of HARQ processes - FFS: The number of contiguous allocations defined in the DCI - Option 3: Allocation of any set of HARQ processes - Option 4: Allocation of a limited set of HARQ processes - Option 5: Contiguous allocation for mixed(initial+retransmisson) scheduling and non-contiguous allocation for all-initial or all-retx scheduling Down selection to be made separately for CE mode A and B Agreement For multicast, the size of the DCI field indicating the number of scheduled TBs for SC-MTCH is 3 bits Agreement For multicast, optional scheduling gaps can be configured by higher layers. It is left to RAN2 whether to do the configuration in SC-MCCH or SIB. Agreement - For unicast, for an MPDCCH ending in subframe N, the timing relationship for PDSCH is such that no PDSCH associated with the MPDCCH is received before subframe N+2. - For unicast, for an MPDCCH ending in subframe N, the timing relationship for PUSCH is such that no PUSCH associated with the MPDCCH is transmitted before subframe N+4. - For unicast, for a PDSCH transmission ending in subframe N, the corresponding HARQ-ACK is transmitted no earlier than in subframe N+4. Conclusion There is no consensus on the support of HARQ-ACK bundling in CE mode B for unicast multi-TB scheduling Agreement For unicast multi-TB scheduling, HARQ-ACK multiplexing in CE mode B is not supported Agreement For the design of DCI for multiple DL/UL TB: - At least when a single TB is scheduled, aim for similar scheduling flexibility as that of legacy DCI - Possible exceptions at least for some cases are the frequency hopping flag and RV index field Agreement: - For unicast, select option(s) from the following options - Option 1: Scheduling of up to 8 TBs is supported with a single DCI design. - Target for up to 6 bits overhead increase compared to legacy DCI - Option 2: For unicast, the maximum number of scheduled TBs with one single DCI for CE mode A for either UL or DL is RRC configured within the set {1, 2, [4], 8} in a UE specific manner. - The design methodology for the DCI for different maximum number of TBs is further studied - For the 2 TB case, target for up to 3 bits overhead increase compared to legacy DCI - Note: Option 2 will require modification on existing agreement - The following working assumption is confirmed. - For unicast, scheduling of initial and retransmission TB(s) within one DCI is supported - For unicast, the new data indication is individually provided for each allocated HARQ process. Agreement The following working assumption is confirmed: For unicast, scheduling gaps for multiple transport blocks is supported and a scheduling gap can be configured by [RRC and/or DCI]. - The support of scheduling gaps is UE optional feature regardless of the support of multiple TBs. - FFS: Details on the scheduling gap such as duration, applicability, etc. For further discussion in RAN1#98bis - For unicast, discuss whether the use of scheduling gaps can be used to allow early termination of an ongoing (multi-TB) uplink transmission. - For unicast, discuss whether the start of the HARQ feedback on the uplink should be the same or can be different in the FD-FDD and HD-FDD cases. (Note that the answer may be different in the HARQ-ACK bundling and non-bundling cases. - Prioritize HD-FDD case

- Coexistence of LTE-MTC with NR

Coexistence aspects were studied during RAN1#94 through RAN1#97 and identified coexistence performance improvements were communicated in an LS in R1-1907636. RAN#84 revised the WI objective in accordance with the recommendations in the LS. RAN1 started working with the revised WI objective in RAN1#98. The entries from the RAN1 meetings that took place during the study phase are included below but greyed out.

RAN1#94:

R1-1809564 Feature lead summary for Coexistence of LTE-MTC with NR Ericsson Observation From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and eMTC Agreement RAN1 studies additional specification enhancement for improving the performance of coexistence of eMTC with NR.

RAN1#94bis:

R1-1811665 Feature lead summary for Coexistence of LTE-MTC with NR Ericsson For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements of subcarrier and resource block alignment to help determine whether the gains are significant enough to motivate the impacts. For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements of resource configuration (e.g. reservation of LTE-MTC resources) to help determine whether the gains are significant enough to motivate the impacts. Agreement RAN1 clarifies that the enhancements introduced by the WI objective on usage of the LTE DL control channel region for MPDCCH/PDSCH transmissions to LTE-MTC UEs do not only apply to LTE-MTC stand-alone deployments but also to the case when LTE-MTC is deployed within an NR carrier. For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements through CRS reduction to help determine whether the gains are significant enough to motivate the impacts. - Also consider backwards compatibility aspects. For further study: Until the next meeting, invite companies to evaluate the potential performance gains (e.g. in terms of reduced NR resource reservation) from performance improvements of frequency hopping to help determine whether the gains are significant enough to motivate the impacts. - Also consider backwards compatibility aspects.

RAN1#95:

R1-1813723 Feature lead summary for Coexistence of LTE-MTC with NR Ericsson Agreement - RAN1 continues to study the following techniques for performance improvements of resource block alignment until the next meeting: - Puncturing of resource elements at the outlying subcarrier - Rate-matching around the outlying subcarrier - Exploitation of a portion of the NR guard band (this would also require RAN4 study) - RAN1 continues to consider all combinations of LTE-MTC system bandwidths and NR system bandwidths when discussing potential co-existence performance improvements. Agreement - RAN1 continues to study the following techniques for performance improvements of LTE-MTC resource allocation until the next meeting: - Resource reservation at symbol level/slot level/subframe level/subcarrier level - Whether the resource reservation is dynamic or semi-static (if supported) - Whether and how to support LTE-MTC transmission in a portion of the subframe - Impact of resource reservation to legacy UEs - Whether LTE-MTC transmission is postponed or dropped in reserved resources Agreement RAN1 studies LTE-MTC transmission outside the legacy LTE system bandwidth (for reduced NR reserved resource cost for CRS, SIB1-BR, paging, etc.) until the next meeting

RAN1#96:

R1-1903246 Feature lead summary for Coexistence of LTE-MTC with NR Ericsson For further study in future meetings: - RAN1 continues to consider all combinations of LTE-MTC system bandwidths and NR system bandwidths when discussing potential co-existence performance improvements, including the combinations considered particularly challenging (e.g. the cases with 5 MHz NR system bandwidth). - RAN1 continues to study e.g. the following aspects of puncturing and/or rate-matching and/or exploitation of guardband (this would also require RAN4 study) as potential ways to take outlying LTE-MTC subcarriers into account for performance improvement of resource block alignment between LTE-MTC and NR: - How to minimize the number of outlying LTE-MTC subcarriers - To what extent the LTE-MTC UE needs to be aware subcarriers not used for transmission - Performance impacts of the LTE-MTC subcarrier puncturing - Some of the methods may only apply for downlink - RAN1 studies whether to support configurable RB shift for LTE-MTC in some cases (e.g. stand-alone/in-band, FDD/TDD, DL/UL). Agreement RAN1 considers coexistence cases for different NR subcarrier spacing (SCS), with higher priority given first to 15 kHz SCS and then to 30 kHz SCS. For further study in future meetings: - RAN1 continues to study semi-static LTE-MTC resource reservation for improved coexistence with NR SSB, NR CORESET, NR CSI-RS, and NR TRS. - RAN1 continues to study both semi-static and dynamic LTE-MTC resource reservation for improved coexistence with other NR transmissions than NR SSB, NR CORESET, NR CSI-RS, and NR TRS. - RAN1 continues to study how to handle potential collision between LTE-MTC transmissions and NR URLLC related transmissions. For further study in future meetings: - RAN1 continues to study potential support of LTE-MTC transmission outside the legacy LTE system bandwidth (for reduced NR reserved resource cost for CRS, SIB1-BR, paging, etc.) while supporting legacy LTE-MTC transmission for legacy LTE-MTC UEs within the legacy LTE system bandwidth. Conclusion - Non-backwards-compatible approaches for reduced CRS overhead or reduced impact from frequency hopping is not supported in Rel-16.

RAN1#96bis:

R1-1905535 Feature lead summary for Coexistence of LTE-MTC with NR Ericsson For further study until RAN1#97 - RAN1 continues to study puncturing and/or rate-matching of outlying LTE-MTC subcarriers for performance improvement of resource block alignment between LTE-MTC and NR until RAN1#97. - Assume truncation of 1, 4 or 5 LTE-MTC DL subcarriers in case of an even total number of NR RBs and 2, 3 or 6 LTE-MTC DL subcarriers in case of an odd total number of NR RBs. - Consider truncation of outlying LTE-MTC DL subcarriers without the (legacy) UE being aware of the puncturing as a reference case. - Assume that no new transmission scheme (for example, single tone transmission) is introduced after the outlying subcarrier is punctured or rate-matched around. - Assume that CRS is not truncated. - FFS: DMRS - Consider both DL and UL aspects including - PDSCH, PUSCH, MPDCCH, PUCCH Conclusion RAN1 does not further consider placement of LTE-MTC subcarriers into NR guard band since it has no RAN1 specification impact. Conclusion - RAN1 concludes that overlap between NR SSB and LTE-MTC can be avoided using the LTE-MTC DL valid subframe bitmap. For further study until RAN1#97 - RAN1 continues to study finer-granularity LTE-MTC resource reservation until RAN1#97. - Assume that the LTE-MTC resource reservation should accommodate at least part of NR initial CORESET, NR CSI-RS and NR TRS. - The reference case is the existing DL/UL subframe-level LTE-MTC resource reservation. - Assume that the resource reservation is semi-statically configured. It is FFS whether the eNB can furthermore dynamically override or indicate additional reserved resource via DCI. - Companies are encouraged to consider the impact on UE complexity for DCI based indication - Consider whether the time-domain granularity of the LTE-MTC valid subframe configuration should be scaled dependent on the NR subcarrier spacing. - Consider both DL and UL aspects. - PDSCH, PUSCH, PUCCH, MPDCCH - It is FFS whether and when to drop or postpone LTE-MTC transmissions that would fall into LTE-MTC reserved resources (if introduced). Conclusion No further discussion in RAN1 on the consideration of LTE-MTC transmission outside the legacy LTE system bandwidth in Rel-16.

RAN1#97:

R1-1907581 Feature lead summary for Coexistence of LTE-MTC with NR Ericsson Conclusion#1 Truncation of outlying LTE-M subcarriers in DL is beneficial for improving the coexistence with NR in at least some use cases Conclusion#2 RAN1 concludes that finer-granularity LTE-M resource reservation at least in DL is beneficial in terms of performance for improving the coexistence with NR. Conclusion#3 RAN1 recommends at least in the time domain one of slot-level and symbol-level granularity for LTE-M resource reservation R1-1907619 [Draft] LS on LTE-MTC coexistence with NR Ericsson Agreement Include the above conclusions #1, #2, #3 as part of the LS to RAN. LS is endorsed in R1-1907636. Conclusion - The truncation of outlying LTE-M subcarriers in the downlink should be limited to a very small number of subcarriers and the truncation is performed using puncturing. - The truncation of outlying LTE-M subcarriers in the uplink is not specified in Rel-16. Conclusion No further discussion on configurable LTE-M resource block shift in Rel-16.

RAN1#98:

R1-1909498 Feature lead summary for Coexistence of LTE-MTC with NR Ericsson Agreement - FFS: till the next meeting whether the LTE-MTC DL subcarrier puncturing applies per scheduled transmission and/or per narrowband and/or per system bandwidth - DL subcarrier puncturing is supported on both sides of [transmission bandwidth] - The maximum number of LTE-MTC DL subcarriers that can be punctured is 2 - The [maximum] number of punctured subcarriers and their locations are configured by higher layers via [SIB or UE-specific RRC signaling]. It is FFS whether DCI can override or modify the higher-layer configuration. - The same higher-layer configuration of [maximum] number of punctured subcarriers and their locations apply to both MPDCCH and PDSCH. - FFS: till the next meeting whether DMRS, CSI-RS and SFBC RE pairs are punctured or not. Agreement - LTE-MTC DL time-domain resource reservation is supported with a finer granularity than slot level. Transmissions in these reserved resources are dropped. - FFS: How to handle DMRS - Support LTE-MTC DL time-domain resources reservation with slot-level granularity. Transmissions in these reserved resources are dropped. - Whether to support LTE-MTC UL time-domain resource reservation and/or LTE-MTC DL frequency-domain resource reservation is FFS. - The LTE-MTC resource reservation is configured by higher layers via [SIB or UE-specific RRC signaling]. It is FFS whether DCI can override or modify the higher-layer configuration. - It is FFS whether/how the higher layer configuration relates to the existing Rel-13 parameters for LTE-MTC valid subframe configuration and MBSFN configuration.

- Support of quality report in Msg3

RAN1#94:

R1-1809527 Feature summary of 6.2.1.5 Support of quality report in Msg3 Samsung Agreement Prioritize the following alternatives for DL quality report in Msg3 in MTC, for CE Mode A and CE Mode B separately: - CQI (for CE Mode A) - Repetition number related to UE decoding of actual or hypothetical MPDCCH/PDSCH - FFS if aggregation level needs to be reported when repetition number equal to 1 - RSRP/RSRQ Companies are encouraged to provide details and/or performance evaluation results Agreement Whether the DL quality report is included in Msg3 is indicated in SIB and/or RAR. Above applies in case the UE supports DL quality report in Msg3.

RAN1#94bis:

R1-1811691 Feature summary of 6.2.1.5 Support of quality report in Msg3 Samsung Agreement For CE mode A (PRACH CE level 0, 1), the downlink channel quality is down-selected among the following in RAN1#95: - CQI - The repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% - Support both CQI and repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% Agreement For CE Mode B, the downlink channel quality reported in Msg3 is denoted as the repetition number that the UE recommends to achieve a hypothetical MPDCCH decoding BLER of 1% Agreement CRS may be used as the reference signal for measurement of DL quality metric for measurement report in Msg3. Agreement Enabling of DL quality report is indicated in SIB.

RAN1#95:

R1-1813741 Feature summary of 6.2.1.5 Support of quality report in Msg3 Samsung Agreement For CE mode A (PRACH CE level 0, 1), the downlink channel quality is the repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% Agreement DL quality report is transmitted via higher layer signaling, e.g. MAC CE or RRC message

RAN1#96:

R1-1903297 Feature summary of 6.2.1.5 quality report in Msg3 and Connected Mode Samsung R1-1903517 Remaining issues on MTC DL quality report Samsung Agreement For DL quality report in Msg3: - If frequency hopping for MPDCCH is enabled, at least wideband DL quality is reported. - Wideband DL quality is calculated assuming transmission on all narrowband(s) to which the derived repetition number and/or aggregation level relates. - FFS if additionally reports DL quality on a preferred narrowband and/or position of the preferred narrowband. - If frequency hopping for MPDCCH is disabled, DL quality based on one narrowband is reported. Agreement For DL quality report in connected mode - DL quality is with respect to USS for both CE mode A and CE mode B. - DL quality is measured at least on narrowband(s) on which MPDCCH of USS is monitored. Agreement: For DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1): - If the repetition number in DL quality information is larger than 1, only repetition number is reported with pre-defined maximum aggregation level; - If the repetition number in DL quality information equals to 1, FFS between: - Only aggregation level is reported with pre-defined repetition number equals to 1 - Repetition number equals to 1 is reported with pre-defined [maximum] aggregation level

RAN1#96bis:

R1-1905581 Feature summary of DL quality report in MTC Samsung R1-1905589 Remaining issues on DL quality report in MTC Samsung Agreement - DL quality report in Msg3 in IDLE mode and DL quality report in connected mode are configured separately. - DL quality report in Msg3 in IDLE mode is configured per PRACH CE level or per CE mode in the SI. - DL quality report in connected mode is configured by UE-specific RRC signaling. Agreement - For DL quality report in connected mode, DL quality is transmitted via higher layer signaling, e.g. MAC CE or RRC message. Working Assumption - For DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement includes at least the narrowband(s) on which MPDCCH of RAR is monitored. Agreement For channel quality report in Msg3, RAN1 send LS to RAN2 requesting the following information: - Number(s) of available bits for the quality reporting in Msg3 - Whether the quality report in Msg3 is transmitted in RRC or MAC CE Send LS to RAN2 and CC RAN4. Ask, from RAN2, perspective, whether the above might be different for different cases. Include as part of LS to be handled in the NB-IoT agenda item. R1-1905592 [DRAFT] Draft LS on channel quality report in Msg3 for NB-IoT and LTE-M Huawei, HiSilicon The LS is endorsed in R1-1905593 Working Assumption For DL quality report in CE mode A (PRACH CE level 0, 1), the pre-defined maximum aggregation level is fixed to 24.

RAN1#97:

R1-1905939 LS on quality report in Msg3 for LTE-M RAN2, Ericsson R1-1907610 Feature summary of DL quality report in MTC Samsung R1-1907631 Remaining issues on DL quality report in MTC Samsung Agreement - Specify DL quality information with up to 8 bits for DL quality report in at least - Msg3 report for EDT case - Msg3 report for connected mode - Fixed range of DL quality information is defined for DL quality report with up to 8 bits Agreement For both DL quality report in Msg3 in IDLE mode and DL quality report in connected mode: - Confirm the following working assumption: - For DL quality report in CE mode A (PRACH CE level 0, 1), the pre-defined maximum aggregation level is fixed to 24. - For DL quality report in CE mode B (PRACH CE level 2, 3), the pre-defined maximum aggregation level is fixed to 24. Agreement Confirm the following working assumption: - For DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement includes at least the narrowband(s) on which MPDCCH of RAR is monitored. Agreement For DL quality report in connected mode, the mechanism for triggering is based on one or more of the following: - DCI (e.g. UL grant, PDCCH order) - MAC CE For further discussion - Specify DL quality information with the following size for DL quality report in Msg3 for non-EDT case - Up to 8 bits if there is enough room in the scheduled TBS, otherwise 2 bits - Fixed range of DL quality information is defined for DL quality report with up to 8 bits - Adaptive range of DL quality information is defined for DL quality report with up to 2 bits - FFS details Agreement For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if the repetition number in DL quality information equals to 1, select one between the following alternatives in RAN1#98 meeting: - Alternative 1: Repetition number =1 is reported with aggregation level - Alternative 2: Repetition number =1 is reported assuming aggregation level of 24 Agreement For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if frequency hopping for MPDCCH is enabled, select one between the following alternatives in RAN1#98 meeting: - Alternative 1: Only wideband DL quality is reported. - Alternative 2: DL quality on a preferred narrowband and position of the preferred narrowband are reported in addition to wideband DL quality. - The preferred narrowband is selected within the set of narrowband(s) on which wideband DL quality is measured.

RAN1#98:

R1-1909584 Feature summary of DL quality report in MTC Samsung R1-1909739 Remaining issues on DL quality report for MTC Samsung Agreement CQI report and DL quality report are separate features and it is up to eNB configuration/triggering (UE can be configured to report both CQI and DL quality) Agreement - Specify DL quality information with up to 8 bits for DL quality report in Msg3 report non-EDT case. - In addition to the 8-bit DL quality report, DL quality report with 2 bits is supported for non-EDT case - FFS: Details on the range of DL quality information for the 2-bit case - If the quality reporting in Msg-3 is configured in SIB - If the scheduled TBS is large enough for inclusion of the 8-bit DL quality report, the 8-bit DL quality report is used - Otherwise, the 2-bit DL quality report is used. Agreement For IDLE mode DL quality report in Msg3 in CE mode A with up to 8 bits, if the repetition number in DL quality information equals to 1 - Repetition number =1 is reported with aggregation level - A common table is used for CE mode A and B Agreement For DL quality report in Msg3 for IDLE mode UEs, downlink quality is measured on the narrowband(s) on which MPDCCH of RAR is monitored. No additional downlink quality measurement on other narrowband(s) is introduced. R1-1909731 [Draft] Reply LS on quality report in Msg3 for LTE-M Ericsson Agreement The draft LS is endorsed in R1-1909793. Agreement For IDLE mode DL quality report in Msg3 in CE mode A with up to 8 bits, if frequency hopping for MPDCCH is enabled, - Only wideband DL quality is reported. Agreement: - For DL quality report not in Msg3 in connected mode, DL quality report is configured by UE-specific RRC signaling. - For DL quality report in Msg3 in connected mode, the configuration of DL quality report is selected between in RAN1#98bis: - SIB - Reuse the configuration of DL quality report in Msg3 in IDLE mode - UE-specific RRC signaling - Reuse the configuration of DL quality report not in Msg3 in connected mode Working Assumption: If DL quality report in Msg3 in connected mode is enabled by higher layer configuration, UE always report DL quality in Msg3 in connected mode

- MPDCCH performance improvement

RAN1#94:

R1-1809570 Summary of MPDCCH performance improvement Huawei, HiSilicon Agreement CRS for improving channel estimation on USS MPDCCH is supported Agreement The relation between the MPDCCH DMRS and CRS is defined for enabling improved channel estimation. FFS the details on specifying the relation (e.g., whether the ‘definition’ means predefined in specification or done by signalling, relation between the CRS ports and DMRS ports, power offset between DMRS-CRS). Agreement eNB enables/disables “CRS for improving channel estimation on MPDCCH” via dedicated or broadcasted RRC signaling. FFS the details on the signaling. Agreement CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH is supported.

RAN1#94bis:

R1-1811682 Feature lead summary of MPDCCH performance improvement Huawei, HiSilicon Agreement CRS for improving channel estimation on MPDCCH in idle mode is supported. FFS the details. Agreement For localized MPDCCH, the following mappings are supported between CRS and DMRS: - Predefined mapping - Mapping based on CSI report FFS the details. Agreement For distributed MPDCCH, mapping between CRS and DMRS is defined in specifications. - FFS the details (e.g. number of PRBs over which mapping applies, fixed or configurable) Agreement Power offset between CRS and DMRS antenna ports of MPDCCH is signaled by eNB. FFS the signaling details (e.g. provisioning for default power offset value).

RAN1#95:

R1-1813739 Feature lead summary of MPDCCH performance improvement Huawei, HiSilicon Agreement For distributed MPDCCH, predefined mapping between CRS ports and MPDCCH DMRS ports is supported. Agreement For localized/distributed MPDCCH, the predefined mapping between CRS ports and MPDCCH ports is based on precoder cycling across time and frequency domain. FFS the details. Agreement Reuse existing LTE Rel-8 codebooks for precoding of MPDCCH is the starting point. - Unless performance gains are identified further optimization on top of Rel-8 codebook will not be introduced Agreement For UE in Connected/Idle mode, the power offset between CRS and DMRS antenna ports of MPDCCH is indicated by SIB. Agreement - For UE in Idle mode, the configuration of CRS for MPDCCH performance improvement is indicated via SIB/SI. FFS the details of configuration. - For UE in Connected mode, the configuration of CRS for MPDCCH performance improvement is indicated via UE-specific RRC signalling. FFS the details of configuration.

RAN1#96:

R1-1903279 Feature lead summary of MPDCCH performance improvement Huawei, HiSilicon Agreement Precoding cycling in time domain with a granularity of YCH is supported. FFS whether further values are supported in which case the granularity is configurable. Agreement For precoding cycling, the precoder set consists of [2 or 4] existing precoders for 2Tx antennas and 4 existing precoders for 4Tx antennas. FFS: The precoder set is configured or not. For further study - The order and pattern of the used precoder in time and frequency domain for precoder cycling

RAN1#96bis:

R1-1905537 Feature lead summary of MPDCCH performance improvement Huawei, HiSilicon Agreement When CSI-based precoding is configured: - The UE monitors a set of MPDCCH candidates assuming the mapping between DMRS and CRS ports is based on a previously reported PMI. - FFS details, including delay for applicability of PMI. - A fallback mechanism is supported. The details of the fallback mechanism are FFS. Agreement For precoding cycling for 2Tx antennas, predefine one precoder set which consists of 2 existing precoders. FFS: Which of the two precoder among the existing 2TX precoders are used. Agreement For precoding cycling for 4Tx antennas, predefine one precoder set which consists of 4 existing precoders. FFS: Which of the four precoder among the existing 4TX precoders are used. Agreement The precoding cycling in time domain has a granularity of YCH (granularity of frequency hopping) only. - Precoder cycling can be used even in case frequency hopping is not used as long as YCH is configured

RAN1#97:

R1-1907572 Feature lead summary of MPDCCH performance improvement Huawei, HiSilicon Agreement When CSI-based precoding is configured, use predefined precoding cycling as the fallback mechanisms. - FFS: Details on fallback mechanism Agreement For precoder cycling for distributed MPDCCH, select one options for precoding in RAN1#98 - Option 1: Based on rank-1 precoders - Option 2: Based on rank-2 precoders Companies are encouraged to submit simulation results. Agreement Precoders with indices 0, 1 consist of the precoder set for 2Tx Agreement If n is the last subframe in which PMI is reported, the reported precoder is applied to a set of MPDCCH candidates for mapping between CRS port and MPDCCH DMRS port in subframe n+4 Agreement For precoder cycling, at least the granularity of 1 PRB in frequency domain is supported. - FFS: whether other granularities are supported or not.

RAN1#98:

R1-1909621 Feature lead summary of MPDCCH performance improvement Huawei, HiSilicon Agreement A fixed subset of the candidates in the MPDCCH search space(s) are reserved for the precoder cycling fallback. The subset of candidates is designed in such a way that the UE complexity is taken into account for the following aspects: - Number of channel estimations - LLR storage - Blind decodes are not increased For comparable performance, the alternative with the smaller complexity will be selected. Agreement Rank-1 precoder is used for precoder cycling in distributed MPDCCH. - FFS whether and how predefined pairs of Rank-1 precoders specified. Agreement The precoder set for 4Tx is the set of rank-1 precoders with indices 12, 13, 14, and 15 Agreement For both distributed and localized MPDCCH, the granularity of precoding in the frequency domain is only 1 PRB. Agreement Select one of the following alternatives (to be done in RAN1#98bis): Precoder cycling in time domain is done - Alt1: In a pseudo-random manner - Alt2: Sequentially and in a cyclic manner

- CE mode A and B improvements for non-BL UEs

RAN1#94:

R1-1809534 Summary of CE mode A and B improvements for non-BL UEs LG Electronics Agreement Study on the performance benefit of dual layer DL reception when it is supported for non-BL UEs in CE mode A. Companies are encouraged to submit evaluation results. - UE complexity should be also considered - Prioritize SNR region relevant to CE mode A Agreement Study on the performance benefit of CSI-RS based CSI feedback when it is supported for non-BL UEs in CE mode A. - UE complexity should be also considered - Prioritize SNR region relevant to CE mode A

RAN1#94bis:

R1-1811666 Summary of CE mode A and B improvements for non-BL UEs LG Electronics Agreement Dual-layer transmission is not supported for non-BL UEs in CE mode B Email discussion for evaluation parameters for dual layer transmission simulations - Until 22th of October (Changhwan Park, LGE) Agreement CSI-RS based CSI feedback is not supported for non-BL UEs at least in CE mode B

Outcome of email discussion after RAN1#94bis:

Agreement - T-put performance metric based on Rank 1 and Rank 2 will be shown by Hull-curve against SNR according to the simulation parameters given in Table 1 - The SNR range is to be discussed, aiming to conclude in RAN1#95 - In proposing the SNR range, companies are encouraged to check the WID and perform appropriate analysis considering relevant aspects related to non-BL & CE mode A Table 1: Link-level simulation parameters for Dual-layer transmission for non-BL UEs in CE mode A Parameter; Value. Frequency; 700MHz. Channel; EPA with low correlation according to Table B.5.2-2 in TS 36.104. eNB Antenna configurations; 4 Tx, Cross-polarization: +/-45 degrees. UE configurations; Speed: 1km/h. 2 Rx with X-polarized: 0/+90 degrees. Traffic load; Full Buffer. Transmission scheme; TM9 with fixed rank. PDSCH; 6 RBs with 1, 2, 4, and 8 repetitions. corReceiver; Non-Ideal DMRS channel estimation and interference estimation. LMMSE-IRC receiver. Overhead; 2 symbols for DL CCHs, 2 CRS ports and 2 DM-RS ports. CSI/Precoding; No rank adaptation (Rank 1 and Rank 2) Fixed CQI and feedback wideband PMI with periodicity of 10msec (Channel reciprocity property in TDD system can be used for DL PMI adjustment). Rate control; Target 10% BLER after 1st transmission. Cyclic prefix; Normal CP.

RAN1#95:

R1-1813726 Summary of CE mode A and B improvements for non-BL UEs LG Electronics Agreement Companies are encouraged to evaluate DL performance with CSI-RS based CSI feedback for non-BL UE in CE mode A including by comparing downlink throughput performances based on - CRS-based CSI feedback - The current CSI-RS based CSI feedback (detailed configuration of CSI-RS is up to company, including new CSI-RS design, if any) under the simulation assumption in Table 1 used for the performance comparison between Single- and Dual-layer transmissions with the following updates - Rank 1 - CQI adaptions are enabled - Periodic CSI feedback Mode 1-1 is applied with periodicity of 10msec - Aperiodic CSI feedback

RAN1#96:

R1-1903278 Summary of CE mode A and B improvements for non-BL UEs LG Electronics Conclusion - RAN1 has no consensus on the support for dual layer DL reception for non-BL UEs in CE mode A in Rel-16 Agreement Existing Rel-15 CSI-RS based CSI feedback is supported for non-BL CE UEs operating CE mode A. FFS on: - The number of CSI-RS ports - CSI feedback mechanism - Resource assignment issue due to REs used for CSI-RS

RAN1#96bis:

R1-1905561 Summary of CE mode A and B improvements for non-BL UEs LG Electronics Agreement CSI-RS based CSI feedback is only supported in TM9 Agreement The supported number of CSI-RS ports is only 8 Conclusion No further discussion on the modification on the design or configuration for support of CSI-RS for non-BL CE UEs in CE mode A in Rel-16. The baseline is Rel-15 CSI-RS. Agreement For CSI feedback of non-BL CE UE, RI is fixed to 1 if it is included as part of reporting on PUCCH or PUSCH Agreement MPDCCH and PDSCH are punctured around the REs used for the CSI-RS transmission.

RAN1#97:

R1-1905934 LS on ETWS/CMAS in connected mode narrowband RAN2, Qualcomm R1-1907618 Summary of CE mode A and B improvements for non-BL UEs LG Electronics Agreement Table 7.2.4-1 of TS 36.213 is reused for the support of CSI-RS based CSI feedback for non-BL UEs in CE mode A. Conclusion Aperiodic CSI-RS is not supported for the non-BL UE operating in CE mode A in Rel-16 Agreement Regarding the RAN2 LS in R1-1905934, send the following in reply LS to RAN2 - It is feasible for a non-BL UE in CE in connected mode to monitor MPDCCH to receive ETWS indication and/or CMAS indication using Type 0 CSS in the same narrowband where unicast transmission can be received - It is feasible for a non-BL UE in CE in connected mode to monitor USS and Type 0 CSS simultaneously in the same narrowband - Type 0 CSS is also supported in CE mode B - FFS: Details of DCI format LS is endorsed in R1-1907637 Agreement Type 0 CSS is supported in CE mode B - FFS: Details of DCI format Agreement Periodic CSI report mode 1-1 is supported for non-BL UE in CE mode A - FFS: Details

RAN1#98:

R1-1909514 Summary of CE mode A and B improvements for non-BL UEs LG Electronics Agreement At least submode 1 is supported for PUCCH mode 1-1 for non-BL UEs in CE mode A - Further consider the additional support of submode 2 Agreement Assuming RI=1, Table 7.2.2-1E in TS 36.213 is reused without modification for the support of CSI-RS based CSI feedback for non-BL UEs in CE mode A. Agreement For ETWS/CMAS indication to a non-BL UE in CE in connected mode via DCI using Type 0 CSS, the DCI formats are selected between the following two alternatives in RAN1#98bis - DCI format 3/3A with one of the TPC commands repurposed for ETWS/CMAS notification - DCI format 6-1A/[1B] with a new RNTI for ETWS/CMAS notification

- Use of LTE control region for DL transmission

RAN1#94:

R1-1809530 Use of LTE Control Channel Region for DL Transmission Summary Nokia, Nokia Shanghai Bell Agreement FFS whether and for what broadcast transmission (e.g. SIBs, paging, SC-PTM, Msg2, etc) use of LTE control channel region for MPDCCH and PDSCH are applied Agreement The following options can be considered for transmission of MPDCCH and PDSCH in LTE control channel region: For MPDCCH - Option 1: All or part of the MPDCCH are mapped into the DL control region. - Option 2: MPDCCH are rate-matched to all OFDM symbols. FFS on RE mapping details. For PDSCH - Option A: All or part of the PDSCH are mapped into the DL control region. - Option B: PDSCH are rate-matched to all OFDM symbols. FFS on RE mapping details.

RAN1#94bis:

R1-1811695 Summary of Use of LTE Control Channel Region for DL Transmission Nokia, Nokia Shanghai Bell Agreement Support PDSCH broadcast transmission in LTE control channel region Agreement PDSCH are rate-matched in a backward compatible manner on all available OFDM symbols. FFS on RE mapping details.

RAN1#95:

R1-1813724 Summary of Use of LTE Control Channel Region for DL Transmission Nokia, NSB Agreement LTE control channel can be used for MPDCCH that is used to schedule broadcast PDSCH channel Agreement For MPDCCH transmission in LTE control region, part of the MPDCCH are mapped into the LTE DL control region. Agreement For PDSCH transmission in LTE control region, rate-matching is done using legacy mapping then wrap around starting with OFDM symbol 0.

RAN1#96:

R1-1903276 Summary of Use of LTE Control Channel Region for DL Transmission Nokia, Nokia Shanghai Bell Agreement Use of LTE DL control region for PDSCH and MPDCCH is configured by SIB1-BR. Agreement In case of retuning, the UE is not required to receive all the symbols in LTE downlink control region. - No RAN1 specification impact

RAN1#96bis:

R1-1905533 Summary of Use of LTE Control Channel Region for DL Transmission Nokia, Nokia Shanghai Bell Conclusion CRS modulation symbols in OFDM symbols 0 and 1 of the stand-alone carrier are the same as those in the legacy control channel region. Agreement When performing rate-matching for PDSCH in the LTE control region, all resource elements shall be counted in the PDSCH mapping, except for the ones reserved for cell-specific reference signals on any antenna port. - Same as legacy operation Working Assumption For MPDCCH mapping, a fixed mapping in time is used. The resource elements used for MPDCCH in the first lMPDCCHStart symbols in the second slot are copied into the first lMPDCCHStart symbols in the first slot.

RAN1#97:

R1-1907611 Summary of Use of LTE Control Channel Region for DL Transmission Nokia, Nokia Shanghai Bell Agreement Confirm the following working assumption with the addition of the FFS bullet for TDD special subframes - For MPDCCH mapping, a fixed mapping in time is used. The resource elements used for MPDCCH in the first lMPDCCHStart symbols in the second slot are copied into the first lMPDCCHStart symbols in the first slot. - FFS: further MPDCCH mapping for TDD special subframes Working Assumption For MPDCCH/PDSCH transmission in LTE control region, the special subframe usage is the same as Rel-13.

RAN1#98:

R1-1909593 Summary of Use of LTE Control Channel Region for DL Transmission Nokia, Nokia Shanghai Bell Agreement For MPDCCH mapping for TDD special subframes, - For special subframe configurations other than 9 and 10, the resource elements used for MPDCCH in the first lMPDCCHStart symbols in the second slot are copied into the first lMPDCCHStart symbols in the first slot. - FFS: For subframe configurations 9 and 10

- Use of RSS for measurement improvements

RAN1#94:

R1-1809503 Summary on the use of RSS for measurement improvements Sony Agreement From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility - RAN1 to identify the related parameters in the next RAN1 meeting

RAN1#94bis:

R1-1811641 Summary on the use of RSS for measurement improvements Sony Agreement In using RSS for measuring RSRP of neighbour cells, the following parameters are signalled to the UE for each cell: - ce-rss-periodicity-config: RSS periodicity {160, 320, 640, 1280} ms - ce-rss-duration-config : RSS duration {8, 16, 32, 40} subframes - ce-rss-freqPos-config: RSS frequency location (lowest physical resource block number) - ce-rss-timeOffset-config: RSS time offset in number of radio frames - ce-rss-powerBoost-config : RSS power offset relative to LTE CRS {0, 3, 4.8, 6} dB Agreement The parameters related in the use of RSS for measuring RSRP of cells are signalled in the SI. Agreement Send an LS to RAN4: - Request to evaluate the benefits and feasilibity of using RSS for RSRP measurements compared to using LTE CRS - For measurement purposes, assess the benefits and feasibility of linking RSS to CRS LS is endorsed in R1-1811934

RAN1#96:

R1-1901486 LS on RSS based measurements signalling RAN2, Huawei R1-1903521 Draft Reply LS on RSS based measurements signaling Ericsson Agreement The draft LS is approved in R1-1903659 with the following modified text: - RSS periodicity can be considered as a carrier specific parameter, hence it may be omitted from neighbor cell RSS configuration signaling. - RAN1 expects most cells to have common or similar configurations for most parameters. Parameter commonality or similarity between serving cell and neighbor cell(s) may be exploited to reduce the required number of bits. - RAN1 will study whether and how to reduce the overhead from the two most expensive parameters, time offset and frequency location. Depending on the study, this could be done, e.g., by making one or both of them carrier specific or by reducing their resolution.

RAN1#96bis:

R1-1905459 Summary of the use of RSS for measurement improvements Sony R1-1905584 Follow up summary of the use of RSS for measurement improvements Sony Agreement Prioritize future RAN1 work on the following neighbor cells’ RSS parameters when reducing the number of signaling bits: - ce-rss-freqPos-config: RSS frequency location (lowest physical resource block number) - ce-rss-timeOffset-config: RSS time offset in number of radio frames For further discussion until RAN1#97 Further evaluate which RSS parameters are likely to be common and/or similar between camped cell and neighbor cell(s). For those RSS parameters that are not common and/or similar discuss how they are signaled. - Including whether to send LS to RAN2 on RAN1 conclusion on which RSS parameters are likely to be common and/or similar. Agreement Support the indication of the number of CRS ports for each neighbor cell in signaling the neighbor cells’ RSS parameters. For further study Consider until RAN1#97 the following options for reducing the signaling bits of the neighbor cells’ RSS frequency location and RSS time offset: - Option 1: Reduce the signalling resolution; and then UE searches for exact RSS location - The reduced signalling resolution is FFS - Option 2: Restrict the RSS locations by reducing the number of possible RSS locations - The exact restriction method is FFS Either Option 1 or 2 can be complemented by other methods.

RAN1#97:

R1-1907071 Summary of the use of RSS for measurement improvements Sony Working Assumption Restrict the RSS locations by reducing the number of possible RSS locations Agreement Down select and/or combine from the following in RAN1#98 for overhead reduction of the RSS time offset & RSS frequency location: - Method 1: Reduce the number of possible RSS time offsets and RSS frequency locations - Method 2: The RSS time offset & RSS frequency location is a function of Cell ID, i.e. no signaling bits required - Method 3: The restricted RSS locations are dependent upon the Cell ID - Method 4: The RSS time offset & RSS frequency location is carrier specific - Method 5: The RSS frequency location of neighbor cells are within a frequency block relative to the RSS frequency location of the serving cell, e.g. the frequency block can be a narrowband Other methods are not precluded For further discussion Introduce an indicator to indicate whether the RSS locations are from a restricted smaller set of possible RSS locations or from the full set of possible RSS locations for each neighbour cell. The exact signalling method is up to RAN2. For further discussion Study the impact of the UE assuming that the number of CRS ports is carrier specific.

RAN1#98:

R1-1908936 Summary of the use of RSS for measurement improvements Sony Agreement Confirm the following working assumption: - Restrict the RSS locations by reducing the number of possible RSS locations Agreement Methods to restrict the RSS locations: - The RSS time offset(s) & RSS frequency location(s) are a function of Cell ID - The Cell ID function is FFS For further study: - Consider reducing the overhead in signaling the number of CRS ports of the neighbours in using neighbour cells’ RSS for measurement improvements. - Consider ways to provide RSS parameters for UE detected neighbour cells.