3GPP TSG RAN WG1 Meeting #98	R1-1909912

Prague, CZ, August 26th – 30th 2019

Agenda item:		7.2.13

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN1 agreements for Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR)

Document for:		Discussion

# Introduction

This document collects the RAN1 agremeents taken during the work item.

# Discussion

## Power control for dual connectivity

##### RAN1-AH-1901 (January 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

Agreements:

- For Rel. 16 UEs and asynchronous NN-DC operation, where MCG has serving cells only in FR1 and the SCG has serving cells only in FR2, the uplink power control is performed independently across cell groups

  - This is under the assumption that for NR Rel. 16, no joint power limit across FR1 and FR2 is defined by RAN4.

- RAN1 has not identified any use case to support the case where SCG is fully in FR1 and MCG is fully in FR2 for both synchronous & asynchronous NN-DC operation. At the same time, if supported, RAN1 has not identified other RAN1 specification impact other than the power control aspect listed below and UE capability

  - If supported, power control is performed independently across the two cell groups.

Sent an LS to RAN4 (cc RAN2) capturing the above in R1-1901402.

##### RAN1#96 (Feb/Mar 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

  - No agreements, but following proposal in email discussion:

Proposals:

- For Rel. 16 NR-DC with FR1+FR1 band combinations both the semi-static power sharing and dynamic power sharing operations are supported.

  - FFS whether each power sharing operation is associated with the UE capability.

Email discussion till 3/15 – Kianoush (QC) – email discussion did not provide an agreement, but collected PC schemes for further discussion in the next meeting.

##### RAN1#96bis (April 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

  - No agreements. Companies are encouraged to check the alternatives listed in R1-1905839, and use them as the reference to provide further analysis. Revisit in RAN1#97

##### RAN1#97 (May 2019)

- Support of asynchronous and synchronous NR-NR Dual Connectivity

  - No agreements. Feature lead summary on Tdocs submitted in R1-1907891

##### RAN1-98 (August 2019)

##### Agreements:

  - Aim to reuse the existing CA power determination for uplink transmissions on CC(s) in a same CG.

##### Agreements: Slide 3 of R1-1909864 is agreed:

- [Offline consensus] Considering the following two alternatives for semi-static power sharing with PMCG+ PSCG≤PTotalNN- DC

    - Alt.1: For the uplink transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa.

      - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCGin MCG such that pMCG≤PMCG;

      - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and  pSCG can be up to  PSCG'  .

        - Alt.1-1:  PMCG' and PSCG'  are configured by RRC signaling.

        - Alt.1-2:  PMCG' and PSCG'  are determined by RAN4 requirement.

    - Alt.2: For the uplink transmission in MCG and in SCG, UE limits its actual transmission power pMCG to be up to PMCGand  pSCG to be up to PSCG

## Enhancements to single Tx switched uplink solution for EN-DC

##### RAN1-AH-1901 (January 2019)

Agreements:

- It is concluded that it is beneficial to specify enhancements to single Tx switched uplink solution for EN-DC over the existing single Tx switched uplink solution, e.g. via introducing a new reference HARQ configuration using NR/LTE Rel-15 or LTE eIMTA solution, etc.

##### RAN1#96 (Feb/Mar 2019)

Agreements:

For single tx switched UL in EN-DC with TDD PCell, the LTE PCell can be configured with DL-reference UL/DL config

  - For DL HARQ timing, the DL-reference UL/DL configuration is applied

    - Only LTE TDD Pattern 2, 4, 5 can be used as DL-reference

  - For UL HARQ timing is the same as without reference configuration for PCell’s UL/DL configuration other than TDD pattern 0/6,

    - FFS: whether/how to support TDD pattern 0/6 for LTE PCell

  - UE is not expected to transmit on the MCG and SCG simultaneously

  - For type 2 UE (i.e., UE without dynamic power sharing capability):

    - UE is allowed to transmit LTE PUSCH only in the UL subframes designated as UL in the DL-reference configuration

  - For type 1 UE (i.e., UE with dynamic power sharing capability):

    - The UE should not assume that LTE PUSCH is only scheduled in the UL subframes associated configured by the DL-reference configuration

    - The UE should not assume that NR PUSCH is only scheduled in the remaining UL subframes other than those configured by the DL-reference configuration

    - If there is a collision,

      - In the UL subframes designated as UL in the DL-reference configuration, UE is expected to drop NR PUSCH

      - In other UL subframes, UE behaviour to be expected to specified with details FFS

        - E.g., drop LTE PUSCH, drop NR PUSCH, etc.

      - FFS for the case of NR SRS & NR PRACH

    - Note: the impact of switching time (if non-zero) will be further studied.

    - Note: the above does not assume any restriction between gNB and eNB (e.g., tight coordination)

  - FFS: whether/how to support HARQ-offset (similar as in SUO case 1 in EN-DC with LTE FDD PCell)

##### RAN1#96bis (April 2019)

Agreements:

For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following clarifications are agreed:

- For the LTE TDD PCell: use the PDSCH ACK timeline as in Table 10.1.3.1-1 in 36.213)

- For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell

  - For the LTE FDD SCell: use the PDSCH ACK timeline defined for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

  - FFS

    - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

    - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

Agreements:

For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration,

- Support PUCCH formats 3/4/5, PUCCH fallback operation based on implicit resource indication is not supported.

  - FFS PUCCH format and resource determination when the UE only receives DAI=1

  - FFS whether or not there are any additional issues related to SPS

##### RAN1#97 (May 2019)

  - No agreements.

##### RAN1-98 (August 2019)

Agreements:

- Regarding the FFS part (in change marks) in the agreement from RAN1 #96bis:

For DL HARQ timing corresponding to the DL-reference UL/DL configuration used in single Tx in EN-DC with TDD Pcell, the following is agreed:

- For LTE DL CA, the SCell uses the same DL-reference UL/DL configuration as the PCell (already agreed in RAN1 #96bis)

  - For the LTE TDD SCell with different UL/DL configuration (as in SIB1) as the TDD PCell: use the PDSCH ACK timeline for SCell as in case of LTE FDD-TDD CA with LTE TDD PCell (i.e. Table 10.1.3A-1 in 36.213)

  - For the LTE TDD SCell with the same UL/DL configuration (as in SIB1) as the TDD PCell: use the same PDSCH ACK timeline as the LTE TDD PCell (i.e. Table 10.1.3.1-1 in 36.213)

- Support HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell

- Note: from UE perspective, it is expected that HARQ-offset value doesn’t violate the DL/UL configuration (in SIB1).

- For type 1 UE, the feature is optional. FFS for type 2 UE.

Agreements:

R15 specification on “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” (i.e., case1 HARQ timing in single UL), is applied to EN-DC UE capable of dual UL Tx in EN-DC with LTE FDD PCell to mitigate DL de-sensing due to Harmonics, at least including:

- UE behavior specified in 36.213 and 36.212

- FFS: all uplink subframes can be scheduled for LTE

Sent an LS to RAN2 notifying RAN2 on the above agreement in R1-1909880

Agreements:

When UE is configured with DL-reference config on the LTE PCell to support single UL in EN-DC with LTE TDD PCell, regarding the case of LTE PCell’s TDD pattern 0 & 6 (as configured in SIB1)

- Not supported in R16.

Proposal below under email approval until September 5th:

For single UL operation in EN-DC with LTE TDD PCell, and UE is configured with DL-reference UL/DL configuration, if an explicit PUCCH format 1a/1b resource is configured, option 2; otherwise, option 1

- Option 1: Support only PUCCH formats 3/4/5 when DAI=1, i.e., PUCCH fallback operation is not supported.

  - Default PUCCH resource corresponding ARI=00 is used when the UE only receives DAI=1 or SPS PDSCH.

- Option 2: Allocate an explicit PUCCH format 1a/1b resource for the fallback HARQ-ACK transmission corresponding to the PDCCH with DAI = 1 or SPS PDSCH.

Email discussion was inconclusive, to be continued in the next meeting

## Efficient and low latency serving cell configuration/activation/setup

##### RAN1-AH-1901 (January 2019)

  - No agreements.

##### RAN1#96 (Feb/Mar 2019)

  - No agreements. R1-1903812 summarizes all the proposals and collects some companies views to them

##### RAN1#96bis (April 2019)

  - LS to RAN4 in R1-1905912 on the maximum allowed SCell activation delay for Rel16 CA agreed, asking RAN4 feedback on the components of the Rel-15 activation delay.

##### RAN1#97 (May 2019)

  - No agreements.

##### RAN1-98 (August 2019)

Conclusion:

Study further the following two options and aim to conclude in RAN1#98bis:

- Option 1:

  - NW can send activation command MAC CE and also independently use existing signalling to trigger aperiodic/semi-persistent CSI-RS when sending the activation command

  - Request RAN4 to consider specifying additional (tighter) maximum allowed activation delay requirements for following case

    - UE receives activation command MAC CE for Scell activation and also aperiodic/semi-persistent CSI-RS for the Scell ’at the same time’ as the activation command

      - FFS: ’at the same time’, i.e., exact timing between activation command and RS trigger

      - FFS: if separate requirement based on specific RS configuration

- Option 2 - Support enhancement for triggering a specific CSI measurement/reporting configuration during MAC-CE based Scell activation

  - CSI measurement/reporting is based on Rel-15/16 RS(s)

  - FFS RS will be A-TRS, SP CSI-RS, aperiodic CSI-RS, SP TRS, etc.

  - FFS between following candidates for enhanced triggering

    - Implicit (i.e., Scell activation command reception implies specific RS is triggered like in LTE)

    - Separate MAC CE linked to activation MAC CE

    - Enhanced activation command MAC CE

    - Other candidates not precluded

Note: companies (especially those interested in option 2) should bring ‘full proposals’ covering all design aspects for the next meeting.

Agreements:

- From RAN1 perspective, L1 based mechanism for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells can be supported

  - ‘dormancy-like’ => sparse/no PDCCH monitoring on activated Scell while maintaining CSI measurements/reporting

Email discussion/approval of the proposal below until September 5th

- Study further following candidates for L1 based mechanism and aim to conclude at RAN1#98bis

  - Option 1

    - Introduce dedicated DCI on Pcell (or cell x) for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scell(s) (or Scell(s) other than cell x)

    - For case when Scell is configured with multiple BWPs

      - Alt1: DCI indicates the BWP to be used on the Scell (FFS 1 or 2 bits per Scell)

      - Alt2: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of the Scell (1 bit per Scell)

    - For case when Scell is configured with 1 BWP

      - DCI indicates whether to monitor/not monitor PDCCH on the Scell (1 bit per Scell)

    - DCI is applicable for case when UE is not configured with CIF (i.e., no cross-carrier scheduling)

      - FFS for cross-carrier scheduling case

    - DCI details

      - At least 15 DCI bits (i.e., 1 bit per Scell)

      - Monitored in UESS along with other scheduling DCI

      - No extra BDs for monitoring this DCI (so same size or size matched to one of DCI 0-0/0-1/1-0/1-1)

        - Alt 1: DCI CRC scrambled with C-RNTI but some fields reserved in one of DCI 0-0/0-1/1-0/1-1, and other fields used for triggering the dormancy behavior

        - Alt 2: DCI CRC scrambled by another RNTI (i.e., similar to SPS PDCCH release)

    - Application delay

      - <= BWP switching delay;

    - The dedicated DCI is not expected to be used for data scheduling

  - Option 2

    - Introduce new RRC signaling based on which the Bandwidth part indicator field in DCI of cell x is re-interpreted by the UE to indicate the BWP used for cell x and also cell(s) other than cell x

    - For case when UE is configured with 1 BWP on cell x or on cell(s) other than cell x

      - Rel15 behaviour is applied (i.e., no CA enhancement for this case)

    - For case when UE is configured with multiple BWPs on cell x or on cell(s) other than cell x

      - RRC signalling for case when UE is not configured with cross-carrier scheduling

        - Details FFS

      - RRC signalling for case when UE is configured with cross-carrier scheduling

        - Details FFS

    - Grouping of multiple Scells/BWPs

      - Details FFS

    - Application delay

      - <= BWP switching delay;

  - Option 3

    - Introduce new RRC signaling for per-BWP configuration of cross-carrier vs. self scheduling and use the CIF/BWP indicator fields for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells.

      - For case when UE not configured with cross-carrier scheduling/CIF

        - Rel15 behaviour is applied (i.e., no CA enhancement for this case)

      - For case when UE is configured with 1 BWP on scheduled cells other than Pcell

        - Rel15 behaviour is applied (i.e., no CA enhancement for this case)

    - Application delay

      - <= BWP switching delay;

The email discussion agreed the following

For L1 based mechanism for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells, study further the candidates including the following and aim to conclude at RAN1#98bis

o   Option 1

  - Introduce dedicated DCI on Pcell (or cell x) for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scell(s) (or Scell(s) other than cell x)

  - For case when Scell is configured with multiple BWPs

i.      Alt1: DCI indicates the BWP to be used on the Scell (FFS 1 or 2 bits per Scell)

ii.      Alt2: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of the Scell (1 bit per Scell)

Note: It is also possible that the dedicated DCI is used to support both Alt. 1 and Alt. 2

  - For case when Scell is configured with 1 BWP

i.      Alt1: DCI indicates whether to monitor/not monitor PDCCH on the Scell (1 bit per Scell)

ii.      Alt2: DCI indicates switching between sparse PDCCH monitoring and frequent PDCCH monitoring (1bit per Scell)

  - DCI is applicable for case when UE is not configured with CIF (i.e., no cross-carrier scheduling)

i.      FFS for cross-carrier scheduling case

  - DCI details

i.      At least [15] DCI bits (i.e., 1 bit per Scell)

ii.      Monitored along with other scheduling DCI

iii.      No extra BDs for monitoring this DCI

      - Alt 1: DCI CRC scrambled with C-RNTI but some fields reserved in one of DCI 0-0/0-1/1-0/1-1, and other fields used for triggering the dormancy behavior

      - Alt 2: DCI CRC scrambled by another RNTI (i.e., similar to SPS PDCCH release)

      - Other alternatives not precluded (e.g. monitored in CSS with size similar to DCI 2-x)

  - Application delay

i.      <= BWP switching delay; BWP switching delay is starting point

ii.      FFS: if RF switching is applied for inter-band CA, and impact on application delay

  - The dedicated DCI is not expected to be used for data scheduling

  - FFS: If ACK should be transmitted in response to receiving the DCI (e.g. like SPS release PDCCH)

o    Option 2

  - Introduce new RRC signaling which allows to use the existing Bandwidth part indicator field in DCI of cell x/BWP timer expiration to indicate the BWP used for cell x and also cell(s) other than cell x

  - For case when UE is configured with 1 BWP on cell x or on cell(s) other than cell x

i.      Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: other alternatives not precluded

  - For case when UE is configured with multiple BWPs on cell x or on cell(s) other than cell x

i.      RRC signalling for case when UE is not configured with cross-carrier scheduling

      - Details FFS

ii.      RRC signalling for case when UE is configured with cross-carrier scheduling

      - Details FFS

  - Grouping of multiple Scells/BWPs

i.      Details FFS

  - Application delay

i.      <= BWP switching delay; BWP switching delay is starting point

ii.      FFS: if RF switching is applied for inter-band CA, and impact on application delay

iii.      FFS details: e.g. For UE indicating Type 2 capability, whether Type 1 BWP switching delay is used for some BWP switch cases (e.g. for switching between BWPs that differ only in SS configuration)

o    Option 3

  - Introduce new RRC signaling for per-BWP configuration of cross-carrier vs. self scheduling and use the CIF/BWP indicator fields for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells.

i.      For case when UE not configured with cross-carrier scheduling/CIF

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

ii.      For case when UE is configured with 1 BWP on scheduled cells other than Pcell

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

  - Application delay

i.      <= BWP switching delay BWP switching delay is starting point

ii.      FFS: if RF switching is applied for inter-band CA, and impact on application delay

o   Option 1a

- Introduce new RRC signalling to create a mapping of Scell/Scell group(s) to the indication field (x-bit) in the new and new dedicated DCI on Pcell (or cell x) for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on groups of activated Scell(s) (or groups of Scell(s) other than cell x)

i.      FFS details of RRC signaling to group the Scells

- For case when Scell is configured with multiple BWPs

i.      Alt1: DCI indicates the BWP to be used on the group of Scell(s)

ii.      Alt2: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of the group of Scell(s)

Note: It is also possible that the dedicated DCI is used to support both Alt. 1 and Alt. 2

- For case when Scell is configured with one or multiple BWPs

i.      Alt1: DCI indicates whether to monitor/not monitor PDCCH on current active BWP of a the group of Scells (1 bit per group of Scells)

ii.      Alt2: DCI indicates switching between sparse PDCCH monitoring and frequent PDCCH monitoring for the group of Scell(s)

- DCI is applicable for case when UE is not configured with CIF (i.e., no cross-carrier scheduling)

- FFS for cross-carrier scheduling case

- DCI details FFS

- Application delay

- <= BWP switching delay; BWP switching delay is starting point

- FFS: if RF switching is applied for inter-band CA, and impact on application delay

- FFS between below alternatives

  - Alt1: The new DCI is dedicated DCI and is not expected to be used for data scheduling

  - Alt2: The new DCI is included along with scheduling DCI (i.e., enhanced field in scheduling DCI)

- FFS: If ACK should be transmitted in response to receiving the DCI (e.g. like SPS release PDCCH)

o    Option 3a

- When UE is configured with cross-carrier scheduling, use CIF/BWP indicator fields for transitioning between ‘dormancy-like’ and ‘non dormancy-like’ behavior on activated Scells

- Use per serving cell configuration of cross-carrier vs. self scheduling as in Rel-15

- For case when UE not configured with cross-carrier scheduling/CIF

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

- For case when UE is configured with 1 BWP on scheduled cells other than Pcell

      - Alt1: Rel15 behaviour is applied (i.e., no CA enhancement for this case)

Note: Other alternatives not precluded

- Application delay

- <= BWP switching delay BWP switching delay is starting point

- FFS: if RF switching is applied for inter-band CA, and impact on application delay

o    Option 3b

- Configure CIF to be present in DCI format in Pcell search-space even if cross-carrier scheduling is not configured

- Details FFS

- Application delay

- <= BWP switching delay BWP switching delay is starting point

- FFS: if RF switching is applied for inter-band CA, and impact on application delay

Note 1: FFS: If any additional UE behavior is needed when UE is also configured for receiving PDCCH based power saving signal/channel outside active time

Note 2: “BWP” => UE specific RRC configured BWP

Note 3: ‘dormancy like behavior’ may also include CSI measurement/reporting relaxation for some cases

Note 4: It is not precluded to further combine and modify above options

## Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers

##### RAN1-AH-1901 (January 2019)

  - No agreements

##### RAN1#96 (Feb/Mar 2019)

Agreements:

- At least for the case of lower SCS PDCCH scheduling a higher SCS PDSCH the earliest possible starting point for the PDSCH is defined by the end of the PDCCH +

  - >0. Detailed value(s) FFS

  - FFS other factor(s) impacting

- The limit of BDs/CCEs (per slot in the scheduling CC) for the scheduled CC is determined based on the numerology of the scheduling CC.

  - Change the definition of NcellsDL, NcellsDL,μto “the number of configured DL-CCs whose scheduling cell is with active DL BWP having SCS configuration ” as in Section 10.1 of 38.213

##### RAN1#96bis (April 2019)

Agreements:

- For case 1-1 scheduling (PDCCH in the beginning of the slot), when a lower SCS PDCCH schedules a higher SCS PDSCH:

The  is determined a number of symbols based on PDCCH SCS counting from the end of the last symbol of the received PDCCH symbol to the beginning of the first symbol of the corresponding received PDSCH, quantized (using the granularity of PDSCH slot duration) to the next PDSCH slot boundary

Agreements:

Regarding PDSCH-to-HARQ-ACK minimum allowed timing under cross-carrier scheduling with different numerologies:

- Use the Rel-15 specification without changes

Agreements:

Regarding PDCCH-to-PUSCH minimum allowed timing under cross-carrier scheduling with different numerologies:

- Use the Rel-15 specification without changes

Conclusion:

- The PDCCH monitoring occasion determination is based on the numerology of scheduling cell in cross-carrier scheduling with different numerologies

  - No spec change is intended

For the case of cross-carrier scheduling from lower SCS to higher SCS for PDSCH, companies are encouraged to provide sufficient details for the design alternative(s) for scheduling  – to finalize in RAN1#97.

Email discussion for the remaining details for CCS till next meeting – Karri (Nokia)

##### RAN1#97 (May 2019)

Agreements:

- Delta-values for lower SCS PDCCH to higher SCS PDSCH case 1-1 scheduling

  - 15 kHz: 4 symbols

  - 30 kHz: 4 symbols

  - 60 kHz: 8 symbols

- Case 1-2: use the same delta as the case 1-1 scheduling

  - With the quantization step

- Case 2:

  - Use the same delta values as the case 1-1 scheduling INCLUDING the quantization step

Agreements:

- For the case of higher SCS PDCCH scheduling lower SCS PDSCH, for the scheduling timing, use the same definition as with lower SCS PDCCH scheduling higher SCS PDSCH scheduling, without quantization to the next PDSCH slot

Agreements:

- For high-SCS to low-SCS scheduling, the delta for 120 kHz SCS PDCCH: [12] symbols

Agreements:

- For the case of a lower SCS PDCCH scheduling a higher SCS PDSCH (or PUSCH), support using M DCIs in one slot for PDCCH to schedule M respective PDSCH (or PUSCH) transmissions in N slots for PDSCH (or PUSCH), where M<=N (for slot aggregation, M<N) following at least one of the following rules (potential down-selection can be done during Rel-16 UE feature discussion):

Alt 1a: Define max number of unicast DCIs that the UE is expected to decode in each span of PDCCH symbols

Alt 1b: Define an increased number of valid unicast DCIs per PDCCH monitoring occasions

Alt 1c: based on Rel-15 FG 3-5/3-5b

    - Further discussion whether some additional clarification is needed or not  potential clarification can be discussed under UE features

Agreements:

- timeDurationForQCL threshold is determined based on the numerology of the scheduled cells.

Agreements:

When PDSCH and its scheduling PDCCH are in the different CCs, if the PDCCH-to-PDSCH delay < timeDurationForQCL or if the TCI information is absent from the DCI, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell

Conclusion:

If the number of valid unicast DCIs at the same monitoring occasions is increased in the UE feature discussion, further discuss whether/how to update the HARQ-ACK codebook and PUCCH resource determination.

Agreements:

- In Rel-16, support enabling HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group.

  - Note: vs. per cell group in Rel-15

Agreed an LS in R1-1907953 on the RRC parameters for cross-carrier scheduling with different numerologies

##### RAN1-98 (August 2019)

  - No agreements

## Other topics

##### RAN1-98 (August 2019)

Conclusion:

- There is no consensus in RAN1 on how to clarify what synchronous in synchronous NR-DC means in Rel-15, although it is commonly understood in RAN1 that synchronous means at least slot-level synchronization