3GPP TSG-RAN WG1 Meeting #98	R1-1909913

# Prague, CZ, August 26th – 30th 2019

Agenda item:	6.2.4

Source: 	WI rapporteur (Qualcomm Incorporated)

Title: 	Summary of RAN1 agreements for LTE-based 5G terrestrial broadcast

Document for:	Information

# Introduction

This contribution contains the RAN1 agreements and working assumptions for the Rel-16 work item on LTE-based 5G terrestrial broadcast (LTE_terr_bcast, WID in RP-190732) up to and including RAN1#98. The work item objectives are as follows:

- Specify new numerology(ies) for PMCH for support of rooftop reception in MPMT and HPHT-1 scenarios as described in TR 36.776 [RAN1, RAN4]

  - The new numerology(ies) shall have a cyclic prefix length of at least 300us and a core OFDM symbol duration (excluding cyclic prefix) of at least 2.4ms.

  - The definition of the new numerology, apart from link/system level performance, shall consider factors such as UE complexity and frame structure alignment with the cell acquisition subframes (CAS).

  - This objective comprises specifying associated RS pattern(s), MCS, TBS, UE capabilities for the new numerology(ies).

  - RAN1 should strive to define a single numerology to cover both MPMT and HPHT-1 cases.

- Specify, if found necessary, enhancements to the physical channels and signals in the CAS [RAN1, RAN4]

  - This objective includes determining a realistic modelling for the time variation of the desired and interfering signals (e.g. a model between the 50%/50% and 50%/1%), and identifying based on the modelling what channels and signals (if any) need to be enhanced.

  - For the specified enhancements (if any), specify related RRM core requirements (if needed) [RAN4]

- Define a new numerology with 100us CP and 400us core symbol duration for support of mobility of up to 250km/h [RAN1, RAN4]

  - This objective comprises specifying associated RS pattern(s), MCS, TBS, UE capabilities for the new numerology.

# New numerology(ies) for PMCH for rooftop reception

RAN1#96b:

Agreement:

Consider the following candidates for new numerologies. The choice of candidates (either from the table below or other candidates) should be based on the evaluations from aspects of link/system level performance and UE complexity.

New numerologies for support of rooftop reception in MPMT and HPHT-1 scenarios

| Tcp (us) | Tu (ms) | T (ms) | Numerology (kHz) | FFT size | Number of MBSFN subframes per 40ms in MBMS-dedicated carrier (with no gap overhead) | Number of MBSFN subframes per 5ms (with gap overhead in mixed-carrier ) | CP overhead |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 386 | 2.4 | 2.786 | 0.417 | 36846 | 14 | 1 (4.3%) | 13.9% |
| 300 | 2.7 | 3 | 0.370 | 41472 | 13 | 1 (0%) | 10.0% |
| 400 | 2.6 | 3 | 0.385 | 39936 | 13 | 1 (0%) | 13.3% |
| 300 | 2.95 | 3.25 | 0.339 | 45312 | 12 | 1 (15.0%) | 9.2% |
| 400 | 2.85 | 3.25 | 0.351 | 43776 | 12 | 1 (15.0%) | 12.3% |
| 345 | 3.2 | 3.545 | 0.313 | 49152 | 11 | 1 (9.1%) | 9.7% |
| 445 | 3.1 | 3.545 | 0.323 | 47616 | 11 | 1 (9.1%) | 12.6% |
| 300 | 3.6 | 3.9 | 0.278 | 55296 | 10 | 1 (2.0%) | 7.7% |
| 400 | 3.5 | 3.9 | 0.286 | 53760 | 10 | 1 (2.0%) | 10.3% |

Agreement:

In order to limit the UE complexity, the FFT size should only have factors of 2 and/or 3.

Agreement:

For the design of the new numerology for support of rooftop reception, prioritize the case of no additional non-MBSFN subframes after CAS (i.e., one non-MBSFN subframe every 40ms).

Agreement:

The new numerology is designed based on the following principles:

- There is a whole number of symbols in the MBSFN region (39ms MBSFN region is prioritized)

- The new numerology does not need to be aligned with 10ms/1ms radio frame/subframe boundaries

RAN1#97:

Agreement:

Selection of the new numerologies for support of rooftop reception in HPHT-1 and MPMT scenarios is based on the candidates listed in the Table X below.

Table X: New numerologies for support of rooftop reception in MPMT and HPHT-1 scenarios

| Tcp (us) | Tu (ms) | T (ms) | Numerology (kHz) | FFT size | Number of MBSFN subframes per 40ms in MBMS-dedicated carrier (with no gap overhead) | Number of MBSFN subframes per 5ms (with gap overhead in mixed-carrier ) | CP overhead |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 386 | 2.4 | 2.786 | 0.417 | 36864 | 14 | 1 (4.3%) | 13.9% |
| 300 | 2.7 | 3 | 0.370 | 41472 | 13 | 1 (0%) | 10.0% |

RAN1#98:

Agreement:

The numerology of Tcp=300us/Tu=2.7ms should be used for PMCH to support rooftop reception.

Agreement:

An RS pattern with Df=3 is supported

- FFS: Dt and other values for Df

Agreement:

The effect of CFO in PMCH reception should be considered

Agreement:

Companies are encouraged to evaluate the performance of rooftop reception with tone interleaving

Agreement:

TBS mapping of the new numerology for rooftop reception (300/2700) is achieved by applying scaling factors to the legacy TBS determination procedure.

- FFS: Select from the following alternatives:

  - PRB scaling

  - Scaling by using the TBS table for a larger number of layers

  - TBS scaling

# Enhancements to CAS

RAN1#96b:

Agreement:

Any potential solutions relating to the CAS should seek to preserve the existing numerology. The enhancements (if needed) for channels and signals in the CAS are built upon current channels and signals.

Agreement:

Link level evaluation assumptions for baseline evaluations for channels in CAS are as follows:

| Parameter | Values |
| --- | --- |
| System BW | 10 MHz |
| Carrier frequency | 700 MHz |
| CP/Numerology | 15kHz, NCP/ECP: 4.7 us/16.6us |
| Doppler frequency | 1Hz (rooftop antenna), Up to 120km/h and 250km/h for car-mounted |
| Channel estimation | Realistic based on RS design |
| Channel model | To be determined from system level evaluations for different ISDs. TDL-i for NLOS and TDL-v for LOS with the scaled delay spread derived from system level simulations. |
| Number of Rx antennas at the UE | Rooftop: 1 (baseline) /2 Mobile/car receiver: 2 |
| MCS/Number of bits | Based on current LTE specifications |

Agreement:

- The following items are needed for CAS evaluations and companies are encouraged to provide their views on them in RAN1#97:

- FFT window position;

- Receiving antenna pointing for rooftop;

- Whether the transmission of the CAS is single cell or synchronised in an SFN

  - The length of the CP and EI in a synchronised CAS environment.

- For evaluations presented in RAN1#97, companies should provide the options/values used for the above items

Agreement:

For the evaluation of CAS, RAN1 adopts a methodology related to the pathloss model that considers the following:

- Cell reselection procedure (i.e., the UE may select the serving cell depending on the actual pathloss)

- The pathloss may not be constant for a given location

  - The evaluations may consist of multiple drops where the pathloss can be constant for a given drop and be different across drops

RAN1#97:

Agreement:

A Monte Carlo based path loss model is used for evaluations

- The shadow fading, fast fading and the antenna pattern between a given transmitter-receiver pair are assumed to be the same across all time iterations

- For rooftop reception, the receiving antenna pointing is assumed to be stationary in Monte Carlo based pathloss model

- The parameter of α in Monte Carlo based model is set to α=1 for CAS evaluations

Agreement:

For CAS evaluations:

- Baseline is non-SFN transmission across different sites, SFN transmission within sectors of the same site.

- Strongest transmitter based on median pathloss + shadowing is used for antenna pointing for rooftop receivers

  - Results based on using the closest transmitter may also be optionally provided

- Location probability: 99% and/or 95% can be used.

- Handover margin 1 dB

  - Serving cell is selected randomly from the cells that are within 1 dB of the strongest cell (based on instantaneous pathloss and shadowing)

- Note: In the link level simulations, the delay spread should be derived from the system level simulation in the corresponding scenario

RAN1#98:

Agreement:

The 99th percentile should be used for the target location availability of the CAS

Agreement:

PDCCH reception in CAS is enhanced by specifying CFI indicated in MBMS-dedicated MIB and one of the following:

- Aggregation level 16

- Repetition of aggregation level 8, e.g. time/frequency domain repetition

- Note: Only one of the above options can be supported

Agreement:

Enhance PBCH reception with the provision of additional REs for PBCH, e.g., PBCH repetition.

- FFS: Details for PBCH repetition

# New numerology for support of mobility up to 250km/h

RAN1#96b:

Agreement:

The new numerology with T_CP = 100us and Tu = 400us shall be defined as follows:

- Subcarrier spacing Δf = 2.5 kHz

- Subframe duration 1ms

- Slot duration 0.5ms (one symbol per slot, two symbols/slots per subframe)

- No control region

Agreement:

Subframes with Δf = 2.5 kHz are supported in MBSFN subframes on mixed mode carriers

Agreement:

The legacy set of TBS values is the starting point for defining the set of TBS values for subframes with Δf = 2.5 kHz.

- FFS: Additional TBS values

Agreement:

RS patterns for the subframes with Δf = 2.5 kHz shall be chosen based on evaluations.

- FFS: Extensions or modifications to the modulation and TBS index table in 3GPP 36.213.

RAN1#97

Agreement:

Further evaluations should also consider speeds of 150 km/hr in addition to 250 km/hr (which needs to be evaluated as it is required).

- Results for lower ISDs than 15 km are encouraged

Note: Rel-14 legacy RS patterns and numerologies should be evaluated for the above cases

RAN1#98:

Agreement:

The RS pattern of Df=2 and Dt=2 should be specified for the new numerology to support mobility of up to 250km/h