3GPP TSG RAN WG1 Meeting #98	R1-1909916

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.6

Source:	WI Rapporteur (Huawei)

Title:	RAN1 agreements for Rel-16 eURLLC

Document for:	Information

# Introduction

This contribution lists RAN1 agreements made for the Rel-16 WI on ‘Physical Layer Enhancements for NR URLLC’ (NR_L1enh_URLLC, WID in RP-191584) until and including RAN1#98. Note that some key agreements from SI that may have impact on the specifications are included here also. The objectives of the work item is given in Appendix.

# PDCCH enhancements

## DCI format scheduling Rel-16 URLLC

RAN1 #96bis meeting

Agreements: Support configurable number of bits for the following fields for DL DCI format scheduling Rel-16 URLLC. - Carrier indicator (0 bit or at least one non-zero bit) - PRB bundling size indicator (0 or 1 bit) - Rate matching indicator (0, 1 or 2 bits) - ZP CSI-RS trigger (0, 1 or 2 bits) Agreements: The following fields from Rel-15 DCI format 1_1 are not included (in case new DCI format) or can be configured to be absent (0 bit) as in Rel-15 (in case reusing the existing format) in the DL DCI format scheduling Rel-16 URLLC. - Modulation and coding scheme for TB 2 - New data indicator for TB 2 - Redundancy version for TB 2 - CBG transmission information - CBG flushing information Agreements: Keep the following two fields without any change from Rel-15 DCI in DCI format scheduling Rel-16 URLLC: - Identifier for DCI formats (1 bit) (when applicable) - New data indicator (1 bit) Agreements: The following field from Rel-15 DCI format 0_1 are not included (in case new DCI format) or can be configured to be absent (0 bit) as in Rel-15 (in case reusing the existing format) in the UL DCI format scheduling Rel-16 URLLC: - CBG transmission information

RAN1 #97 meeting

Agreements: - Support configurable TDRA table as in Rel-15 DCI format 1_1 (i.e. 0, 1, 2, 3 or 4 bits for time domain resource assignment) for the DL DCI format scheduling Rel-16 URLLC Agreements: Support at least resource allocation type 1 for frequency domain resource assignment for the DCI format scheduling Rel-16 DL URLLC with one of the following modifications compared to Rel-15: - Option 1: a single configurable scheduling granularity applicable for both the starting point and length indication - Alt.1: The scheduling granularity reuses the RBG sizes for RA 0 and can be configured between configuration 1 and 2 as in Rel-15 - Alt. 2: A new RRC parameter to configure the scheduling granularity - Option 2: Separate configurable starting point granularity and length indication granularity

RAN1 #98 meeting

Agreements: - Introduce one new DCI format for DL scheduling and one new DCI format for UL scheduling with configurable sizes for some fields in Rel-16. Agreements: Support separate configurable number of bits (2 or 3 or 4 bits) for “HARQ process number” for new DCI formats for scheduling DL and UL - FFS 0 or 1 bits Note: The following agreement was approved by email discussion post RAN1#98 meeting Agreements: - For resource allocation type 1 for frequency domain resource assignment for the DCI format scheduling Rel-16 URLLC, support the following modification compared to Rel-15: - A single configurable scheduling granularity applicable for both the starting point and length indication. - A new RRC parameter to configure the scheduling granularity

## Increased PDCCH monitoring capability

RAN1 #97 meeting

Agreements: Take the following framework as the working assumption for defining the limit on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span: - PDCCH monitoring span follows the definition in UE feature 3-5b as a starting point - FFS whether any modification needed Agreements: - The per-CC limit on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span for a certain combination (X, Y, ) is C - FFS aspects related to UE capability - FFS the limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is same or different across different spans within a slot - Example of combinations as shown in the following table: - FFS the value of C - Companies are encouraged to report the potential aspects that have impact on the value of C X; Y; C. =0; =1; =2; =3. Combination 1. Combination 2. …. Note: - The table here doesn’t mean increased PDCCH monitoring capability is supported for all SCS. N/A can be filled in the corresponding cell for the SCS not applicable. - FFS interaction with Rel-15-based limitation, e.g., whether to increase the limit for PDCCH monitoring case 1 under the increased PDCCH monitoring capability on the maximum number of non-overlapped CCEs per slot for channel estimation

RAN1 #98 meeting

Agreements: Support (2, 2) (4, 3) (7, 3) defined in UE feature 3-5b as the combination (X, Y) for Rel-16 PDCCH monitoring capability on the per-CC limit on the maximum number of non-overlapping CCEs for URLLC. - Combination (2, 1) (4, 1) (4, 2) (7, 1) (7, 2) are not additionally introduced - FFS (3, 3) or (3,2) - UE reports the supported combinations per SCS - (2, 2)(4, 3)(7, 3) applicable for 15 kHz and 30 kHz - FFS for 60 kHz and 120 kHz Agreements: For a Rel-16 UE supporting enhanced PDCCH monitoring capability, down-select between option 1 and option 2: - Option 1: PDCCH monitoring based on Rel-15 capability for eMBB and PDCCH monitoring based on Rel-16 capability for URLLC can be configured to a UE on the same carrier - UE monitors PDCCH for eMBB following reported Rel-15 capability, and monitors PDCCH for URLLC following reported Rel-16 capability - For Rel-16 PDCCH monitoring capability, the limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot. Each span for Rel-16 PDCCH only cover USS for URLLC (FFS for CSS) - Option 2: PDCCH monitoring for both eMBB and URLLC can be configured based on either Rel-15 capability or Rel-16 capability - gNB configures which capability is used - For Rel-16 PDCCH monitoring capability, - The limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is the same across different spans within a slot, each span can cover CSS and/or USS - Note: the value C is to be separately discussed Agreements: If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied. - A combination C(X, Y) is valid if the span pattern satisfies X and Y of the given combination in every slot, including cross slot boundary - FFS the impact from empty span(s) on the span pattern

# UCI enhancements

## More than one PUCCH for HARQ-ACK transmission within a slot

RAN1 #96bis meeting

Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, support sub-slot-based HARQ-ACK feedback procedure. - A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. - PDSCH transmission is not subject to sub-slot restrictions (if any) - FFS: PDSCH-to-sub-slot association. - FFS: Allowing PUCCH across sub-slot boundary or not. - R15 HARQ-codebook construction is applied in unit of sub-slot at least for Type II HARQ-ACK codebook. - FFS for Type I HARQ-ACK codebook. - R15 PUCCH resource overriding procedures is applied in unit of sub-slot. - Number or length of UL sub-slots in a slot is UE-specifically semi-statically configured. - FFS: Limit of number of PUCCH transmissions carrying HARQ-ACKs in a slot. - FFS: K1 definition. - FFS: Details of PUCCH resource configuration and determination. FFS: Use “Codebook-less HARQ” as a complementary or not. FFS: If HARQ-ACK can be omitted in case latency requirement cannot be met. FFS: PDSCH groupings and PHY identification for separate HARQ-ACK constructions for different service types. Agreements: For supporting multiple PUCCHs for HARQ-ACK within a slot for constructing HARQ-ACK codebook, K1 is defined following R15 approach but in unit of sub-slot.

RAN1 #97 meeting

Agreements: For sub-slot-based HARQ-ACK feedback procedure, K1 is the number of sub-slots from the sub-slot containing the end of PDSCH to the sub-slot containing the start of PUCCH. - Use UL numerology to define the sub-slot grid for PDSCH-to-sub-slot association. - FFS: The configurable value range of K1 needs to be extended, and impact to related DCI field bitwidth. - Note: It has been agreed that K1 is defined following R15 approach but in unit of sub-slot. Agreements: For sub-slot-based HARQ-ACK feedback procedure, the starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot - For a given sub-slot configuration, a UE can be configured with PUCCH resource set(s) - FFS same or different PUCCH resource sets can be configured for different sub-slots within a slot.

RAN1 #98 meeting

Agreements: At least one sub-slot configuration for PUCCH can be UE-specifically configured to a UE. - At least support following two sub-slot configurations for PUCCH: “2-symbol*7” and “7-symbol*2”. - FFS other configurable sub-slot configurations, e.g. 4, 14 sub-slots in a slot. - For the above two sub-slot configurations (“2-symbol*7” and “7-symbol*2”), support a single configuration for PUCCH resource following R15 applicable for all the sub-slots in a slot. - FFS whether or not to additionally support that PUCCH resource configuration can be different for different sub-slots - FFS for other sub-slot configurations, if any. - FFS: If a PUCCH resource across sub-slot boundary is supported.

## At least two HARQ-ACK codebooks simultaneously constructed

RAN1 #96bis meeting

Agreements: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, for both Type I (if supported) and Type II HARQ-ACK codebooks (if supported), and for dynamically-scheduled PDSCH, down-select from below for the PHY identification for identifying a HARQ-ACK codebook: - Opt.1: By DCI format - Opt.2: By RNTI - Opt.3: By explicit indication in DCI (FFS: new field or reuse existing field) - Opt.4: By CORESET/search space - FFS additional option(s) for Type I HARQ-ACK codebook FFS: For SPS PDSCH (including SPS release PDCCH)

RAN1 #97 meeting

Agreements: - When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, all Rel-16 parameters in PUCCH configuration related to HARQ-ACK feedback can be separately configured for different HARQ-ACK codebooks except for following: - FFS: For PUCCH-SpatialRelationInfo - Note: SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList are not related to HARQ-ACK feedback. - FFS: For other UCI types, e.g. SchedulingRequestResourceConfig, multi-CSI-PUCCH-ResourceList. - FFS: At least one HARQ-ACK codebook follows R15 PUCCH configuration.

RAN1 #98 meeting

Agreements: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, following can be separately configured for different HARQ-ACK codebooks: - PUCCH-SpatialRelationInfo - Sub-slot configuration (only applied for the sub-slot-based HARQ-ACK codebook) - FFS whether or not to support the case when there are at least two HARQ-ACK codebooks configured with sub-slots, with the same or different sub-slot configurations Agreements: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, the PHY identification of HARQ-ACK codebook is also used to determine the priority of the HARQ-ACK codebook for collision handling. Agreements: When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, - In case of SPS PDSCH, the following options for identifying a HARQ-ACK codebook (to down-select, combinations are not precluded) - Opt.1: By SPS PDSCH configurations - Opt.2: By the DCI activating the SPS PDSCH - Opt.3: By the CORESET where the activating DCI is received

# PUSCH enhancements

RAN1 #96 meeting

Agreement: - Capture the descriptions of option 1 to 6 (see R1-1903797 and previous agreements) in the TR. Rapporteur note: For better understanding option 4, 5 and 6 in the agreements in the following meetings, details of option 4, 5 and 6 in TR 38.824 are also included here for your information. Note that in the end only option 4 was agreed to be adopted, so for making specifications you could just focus on the agreements related to option 4. ### Option 4 One or more actual PUSCH repetitions in one slot, or two or more actual PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. It further consists of: - The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. - FFS dynamically or semi-statically signalled for dynamic PUSCH and type 2 configured grant PUSCH - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. - The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols. - FFS the detailed interaction with the procedure of UL/DL direction determination - If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is splitted into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. - Handling of the repetitions under some conditions, e.g., when the duration is too small due to splitting, is to be further investigated in the WI phase. - No DMRS sharing across multiple PUSCH repetitions - The maximum TBS size is not increased compared to Rel-15. - FFS: L > 14 - S+L can be larger than 14 - FFS: The bitwidth for TDRA is up to 4 bits. - Note: different repetitions may have the same or different RV. Some examples of option 4 are shown in Figure 6.3.3-1. 4 symbols, 4 repetitions Slot boundary 4 symbols, 2 repetitions Slot boundary 14 symbols, 1 repetition Slot boundary Note: this case requires S+L>14. Examples of option 4 for PUSCH enhancements ### Option 5 One or more actual PUSCH repetitions in one slot, or two or more actual PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. It further consists of: - The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger or smaller than the nominal number. - FFS dynamically or semi-statically signalled for dynamic PUSCH and type 2 configured grant PUSCH - The time domain resource assignment (TDRA) and the number of repetitions K are used to determine the overall resources for all the repetitions (L*K). - If the overall resources go across the slot boundary or DL/UL switching point, one repetition is transmitted in each UL period in a slot. - Otherwise, the nominal number of repetitions are transmitted, each repetition with the transmission duration indicated in the TDRA. - The TDRA is indicated in the DCI for dynamic grant or type 2 configured grant, or RRC configured for type 1 configured grant. - No DMRS sharing across multiple PUSCH repetitions - No special handling of orphan symbols - The maximum TBS size is not increased compared to Rel-15. - L <= 14 - S+L can be larger than 14 - Note: different repetitions may have the same or different RV. L=4, K=4 Slot boundary L=4, K=2 Examples of option 5 for PUSCH enhancements ### Option 6 One or more PUSCH repetitions in one slot, or two or more PUSCH repetitions across slot boundary in consecutive available slots, is supported using one UL grant for dynamic PUSCH, and one configured grant configuration for configured grant PUSCH. It further consists of: - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates an entry in the higher layer configured table - The number of repetitions, starting symbols of each repetition, length of each repetition, and mapping of the repetitions to slots can be obtained from each entry in the table. - More than one repetition can be mapped to one slot - The resource assignment for each repetition is contained within one slot. Each transmitted repetition is contained within one UL period in a slot. - FFS: increasing the number of bits for TDRA field in DCI - FFS other details - The maximum TBS size is not increased compared to Rel-15.

RAN1 #96bis meeting

Agreement: - Option 5 is not considered further as part of PUSCH enhancements. Agreements: For option 4, dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH is supported for PUSCH enhancements. The dynamic indication can be enabled or disabled by the gNB. - FFS the exact signaling method - FFS the exact DCI format(s) - FFS the exact mechanism to enable or disable - FFS the DCI activating type 2 configured grant PUSCH Agreements: For option 6, - For dynamic PUSCH - For semi-static DL symbol(s), to down-select - Option 1: it is not expected that the resource allocation has conflict with semi-static DL symbol(s). - Option 2: if the resource allocation has conflict with semi-static DL symbol(s), the repetition is not transmitted. - For dynamically indicated DL symbol(s) (via format 2_0), it is not expected at the UE that the resource allocation has conflict with dynamically indicated DL symbol(s). - Note: this is the same as Rel-15 behavior. - For configured grant PUSCH, - For type 1 configured grant PUSCH, and PUSCH other than the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, - If a repetition conflicts with semi-static DL symbol(s), the repetition is not transmitted. - FFS: If a repetition conflicts with dynamically indicated DL symbol(s) (via format 2_0), the repetition is not transmitted. - FFS For the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, follow the same handling as dynamic PUSCH. Agreement: - For option 6, at least for dynamic grants, it is not expected that one repetition (i.e., one SLIV) spans across slot boundary. Agreement: For both option 4 and 6, frequency hopping is supported FFS details

RAN1 #97 meeting

Agreements: - Adopt option 4 with the following update: - The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first “nominal” repetition. - FFS the detailed interaction with the procedure of UL/DL direction determination

RAN1 #98 meeting

Agreements: In terms of how to interpret L and K for all PUSCH transmissions, down-select between the following two: - Alt 1: The time window within which valid symbols are used for transmission is L*K. - FFS the definition of “valid symbols” - Alt 2: The time window within which valid symbols are used for transmission can be longer than L*K symbols, and it is extended at least in case of semi-static DL symbols. - FFS extension of the time window in case of dynamic DL symbols and/or semi-static flexible symbols and/or reserved symbols (if defined) and/or SSB symbols and/or type-0 CSS in CORESET#0 (as indicated by MIB) - FFS the definition of “valid symbols” - FFS whether to define a maximum time window size and if so, details Conclusion: In terms of how to handle the interaction of enhanced PUSCH with DL/UL directions, consider the following options: - For DG PUSCH - If dynamic SFI is not configured, - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols. - If dynamic SFI is configured - Option 1: behavior not dependent on dynamic SFI - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols. - FFS whether the conflict between dynamic SFI and symbols used for PUSCH transmission is considered as an error case, e.g. - Option 1-1a: The UE does not expect any semi-static flexible symbol to be indicated as DL within the PUSCH transmission time window. - Option 1-1b: No error case is defined and in general all semi-static flexible symbols are used for PUSCH within the PUSCH transmission time window. - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols. - Option 1-3: Dynamic indication in UL grant on which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the dynamically indicated invalid symbols. - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules. - Option 2: the UE uses SFI to determine the symbols to transmit - In case SFI is configured and received - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols - Option 2-2: Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols - Option 2-3: Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol. - Option 2-4: A repetition is not transmitted if it conflicts with a dynamic DL/flexible symbol - In case SFI is configured and not received - A repetition is not transmitted if it conflicts with a semi-static flexible symbol. - For CG PUSCH other than the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant - If dynamic SFI is not configured, - Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols. - If dynamic SFI is configured - Option 1: behavior not dependent on dynamic SFI - Option 1-1: Semi-static flexible symbols are used for PUSCH. Segmentation occurs only around semi-static DL symbols. - This does not seem to make much sense for CG. If semi-static flexible symbols are always used for CG PUSCH, the gNB can essentially configure these symbols as UL in semi-static configuration. – no need for this option? - Option 1-2: Semi-static DL/flexible symbols are not used for PUSCH. Segmentation occurs around semi-static DL/flexible symbols. - Option 1-3 from DG is not applicable for CG. - Option 1-4: Pre-defined rules to determine which set of semi-static flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL and the invalid symbols as defined in the rules. - Option 2: the UE uses SFI to determine the symbols to transmit - In case SFI is configured and received - Option 2-1: Segmentation occurs around semi-static DL symbols and dynamic DL/flexible symbols - Option 2-2 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. Segmentation occurs around semi-static DL symbols and dynamic DL symbols) - Option 2-3 does not make sense for CG. (Dynamic flexible symbols are used for PUSCH. A repetition is not transmitted if it conflicts with a dynamic DL symbol.) - Option 2-4: a repetition is not transmitted if it conflicts with a semi-static DL symbol and a dynamic DL/flexible symbol - In case SFI is configured and not received - A repetition is not transmitted if it conflicts with a semi-static flexible symbol. - For the first Type 2 CG PUSCH (including all the repetitions) activated by an UL grant, - Alt 1: same behavior as DG PUSCH - Alt 2: same behavior as CG PUSCH without an associated UL grant - … - FFS: in case of a repetition not being transmitted (as in the above bullets), whether a repetition is a nominal repetition or a repetition after segmentation due to semi-static DL symbol(s)/slot boundary - FFS: whether to postpone or not, and if yes, under what condition(s) - FFS: whether/how guard period is handled - Note that segmentation at slot boundary is always performed, even though it is not explicitly mentioned in the bullets above. - FFS: the handling of conflict with SSB/PRACH symbols, the handling of conflict with semi-statically configured DL reception, etc. Other options are not precluded

# Enhancements to scheduling/HARQ

RAN1 #96 meeting

Rapporteur note: The following agreements are from study item phase, which solution(s) to choose would depend on more discussion in work item phase. Editors may not need to identify the specification impact from these agreements for now, could wait for more agreements from work item phase. Agreements: For a Rel. 16 eURLLC UE and dynamic downlink scheduling, on the active BWP of a given serving cell, the HARQ-ACK associated with the second PDSCH with HARQ process ID x received after the first PDSCH with HARQ process ID y (x != y) can be sent before the HARQ-ACK of the first PDSCH. Specify based on the following solutions: - Solution 1: The UE always processes the second PDSCH. The UE may or may not drop the processing of the first channel. - Solution 2: The UE processes both the first and second PDSCHs as a UE capability with no condition. - Solution 3: The UE processes both the first and second channels under some conditions, e.g. using the CA capability. The conditions are reported as a UE capability. If the conditions are not satisfied, the UE behavior is not defined. - FFS: The details of the UE capability. - Solution 4: - A UE drops (terminates) the processing of the first PDSCH. - Alt1: The UE always drops the first PDSCH. - Alt2: Some scheduling conditions should be defined. If not satisfied, the UE drops the processing of the first channel. - FFS how to define the scheduling conditions, e.g., based on the number of RBs, TBS, number of layers, the gap between the first and second PDSCHs, the gap between the two PUCCHs carrying HARQ-ACK, etc. - The UE behavior, e.g., decision on dropping the first channel and timing capability associated with the second channel, is determined, and is fixed, after decoding the PDCCH associated with the first and the second PDSCH. - When the UE drops the processing of the first channel, increasing the minimum PDSCH processing procedure time (N1) of the second PDSCH by d symbols can be considered. - FFS the value of d. - Dropping the processing of the first PDSCH can be done in one of the two ways: - Alt1: dropping the processing of the first PDSCH on the same serving cell - Alt2: dropping the processing of a PDSCH(s) on the same cell or a different serving cell. - The UE only expects a maximum of one OOO PDSCH-to-HARQ-ACK flow on the active BWP of a given serving cell when applicable - FFS whether or not, out-of-order operation is allowed across PDSCHs with PDSCH-to-HARQ gap compatible with PDSCH processing time (N1) for capability X. Agreements: For a Rel. 16 UE, on the active BWP of a given serving cell, the UE can be scheduled with a second PUSCH associated with HARQ process x starting earlier than the ending symbol of the first PUSCH associated with HARQ process y (x != y) with a PDCCH that does not end earlier than the ending symbol of first scheduling PDCCH. Specify based on the following solutions: - Solution 1: The UE always processes the second scheduled PUSCH. The UE may or may not drop the processing of the first schedeuled PUSCH. - If the first scheduled and second scheduled PUSCHs are not colliding in the time domain: - Solution 2: The UE processes both the first scheduled and second scheduled PUSCHs as a UE capability with no condition. - Solution 3: The UE processes both the first scheduled and second scheduled PUSCHs under some conditions. The conditions are reported as a UE capability. - FFS: The details of the UE capability. - Solution 4: - A UE drops (terminates) the processing of the first scheduled PUSCH. - Alt1: The UE always drops the first scheduled PUSCH. - Alt2: Some scheduling conditions should be defined. If not satisfied, the UE drops the processing of the first scheduled PUSCH. - FFS how to define the scheduling conditions, e.g., based on the number of RBs, TBS, number of layers, the gap between the first and the second PUSCHs, etc. - The UE behavior, e.g., decision on dropping the first scheduled PUSCH and timing capability associated with the second scheduled PUSCH, is determined, and is fixed, after decoding the PDCCH associated with first and the second scheduled PUSCHs. - When the UE drops the processing of the first scheduled PUSCH, increasing the minimum PUSCH preparation procedure time (N2) of the second PUSCH by d symbols can be considered. - FFS the value of d. - Dropping the processing of the first scheduled PUSCH can be done in one of the two ways: - Alt1: dropping the processing of the first scheduled PUSCH on the same serving cell - Alt2: dropping the processing of a PUSCH(s) on the same cell or different serving cell. - The UE only expects a maximum of one OOO PDCCH-to-PUSCH flow on the active BWP of a given serving cell when applicable. - FFS whether or not out-of-order operation is allowed across PUSCHs with PDCCH-to-PUSCH gap compatible with PUSCH processing time (N2) for capability X. - If the first scheduled PUSCH and the second scheduled PUSCH are colliding in the time domain, the UE drops the processing and the transmission of the first scheduled PUSCH. - For dropping, the scheduling limitations do not apply. The UE always drops the first scheduled PUSCH. - Other details of dropping are as those of the solution 4.

RAN1 #96bis meeting

Agreements: In case two unicast PDSCHs for a UE are overlapping, the following scenarios are identified: - Scenario 1-1: Overlapping in the time domain and not in the frequency domain - Scenario 1-2: Overlapping both in the time and frequency domains Working assumption: - When the two unicast PDSCHs for a UE are overlapping, the UE generates HARQ-ACK for both of the PDSCHs.

RAN1 #97 meeting

Conclusion: Study further whether/how to support the following scenarios for the handling of two unicast PDSCHs: - When different DL processing times are associated with different PDSCHs on the same serving cell, and the two PDSCHs are non-overlapping. - Note: The PDSCH-to-PUCCHs can be out-of-order or in-order. - Note: The solution(s) should address the UE processing pipelining issue. - Two PDSCHs follow DL processing timing capability #1 and #2, respectively, on the same serving cell. - FFS if any different solutions are necessary to address different scenarios when the above condition occurs - When the same DL processing time is configured on the same serving cell, and the two PDSCHs are non-overlapping, and the PDSCH-to-PUCCHs are out of order. - Note: There is no UE processing pipelining issue. - Note: the in-order PDSCH-to-PUCCHs are already handled in Rel-15. - The two unicast PDSCHs are overlapping at least in the time domain, regardless of whether the same or different DL processing times is configured on the same serving cell. Note: The solution(s) should address the UE processing pipelining issue in this case.

RAN1 #98 meeting

Proposals: For Rel. 16 NR, the following cases are supported: - Case 1: different minimum processing timeline capabilities can be configured for PDSCHs on the same carrier. The minimum processing timeline capability for each PDSCH is indicated at the PHY layer. - Case 2: additional DMRS and PDSCH processing time capability #2 can be configured simultaneously on the same carrier. Email discussion till 9/30 for the above proposals & other related DL OoO issues – Kianoush (QC)

# Enhanced inter-UE Tx prioritization/multiplexing

## UL cancelation scheme

RAN1 #96bis meeting

Working assumption: - PDCCH is used for UL cancelation indication - The Working assumption can be revisited if the DCI for cancelation indication only carry very small number of information bits, e.g. 1 bit. Agreements: - Upon detecting an UL cancelation indication, at least stop without resuming is supported - FFS whether and how to support stop with resume Agreements: - Further discuss which UL transmissions that can potentially be cancelled by the UL cancelation indication, including - Dynamic scheduled UL transmissions, including PUSCH, PUCCH, SRS - Semi-persistent UL transmissions, including PUSCH, PUCCH, SRS - Periodic UL transmissions, including configured grant PUSCH, PUCCH, SRS - PRACH Agreements: - Further discuss, aiming for down-selection, the group common DCI and UE-specific DCI for UL cancelation indication - For group common DCI (different from Rel-15 SFI) - UE is configured to monitor a group common DCI which indicates the time/frequency region on which the UL cancellation indication applies - For UE specific-DCI - When applicable, UE is configured to monitor a second UL grant for the same TB as an earlier PUSCH indicating UL cancellation before the end of the earlier PUSCH transmission. In this case, the UE follows the UL cancellation indication.

RAN1 #97 meeting

Agreements: - Support at least group common DCI for cancelation indication - FFS whether or not to additionally support UE-specific DCI for cancelation indication

RAN1 #98 meeting

Agreements: - Reuse the existing methods for search space configuration to support UL CI monitoring - FFS possible restrictions - Note: this means both symbol level and slot level monitoring periodicities are possible from specification perspective Agreements: - The UE DCI size budget is not increased by UL CI monitoring - Further discuss methods to reduce the UE monitoring for UL CI, e.g. - The number of aggregation levels and/or candidates for the UL CI monitoring should be limited - Conditions for eMBB UE UL CI monitoring: - For UL transmission with associated PDCCH, - Option 1: UE starts UL CI monitoring after the PDCCH is decoded - Option 2: UE monitors UL CI at least at the latest monitoring occasion ending no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time. - For UL transmission without associated PDCCH, UE monitors UL CI at least at the latest monitoring occasion that ends no later than X symbols before the start of the UL transmission, and X is related to UL CI processing time. - Other conditions? - Others? - FFS the enhancement of UE capability (number of non-overlapping CCE and/or blind decodes) for UL CI monitoring Agreements: - Upon detecting an UL cancelation indication, for the transmission of UL signal/channels, “stop with resuming” is not supported - Except: - SRS can still be transmitted on the non-cancelled symbols (conditioned on if SRS can be pre-empted) - FFS for the PUSCH repetition (Rel-15 & Rel-16) case - FFS for the PUCCH repetition case (conditioned on if PUCCH can be pre-empted) - FFS whether another PUSCH can be scheduled in non-pre-empted resource - FFS impact (e.g. phase continuity issue) to a different carrier due to UL cancelation Agreements: - The following UL channel/signals can be cancelled by UL cancelation indication - PUSCH (including DG-, CG- and SP-) - FFS for SRS - FFS for PUCCH - Option 1: PUCCH (all types) can be cancelled - Option 2: Some PUCCH can be cancelled, e.g. PUCCH carrying CSI - Option 3: PUCCH cannot be cancelled - FFS for PRACH (preamble and/or MSG 3 PUSCH) Agreements: - The UE processing time requirement for UL cancelation indication based on N2 defined in Rel-15 UE cap#2 is supported - FFS whether the processing time requirement for UL cancelation indication larger than N2 as defined in Rel-15 UE cap#2 can also be supported as an UE capability - FFS whether the processing time requirement for UL cancelation indication shorter than N2 as defined in Rel-15 UE cap#2 as can also be supported an UE capability

## Enhanced UL power control scheme

RAN1 #96bis meeting

Conclusion: - Further discuss the following power control enhancements - Increased TPC range - FFS details, e.g. supported value range, number of TPC bits, accumulated and/or absolute TPC, configurability of the TPC tables, applicability to SRS/PUCCH. - Indication of open-loop parameter sets based on scheduling DCI without using SRI - Indication of open-loop parameter sets based on GC-PDCCH

RAN1 #97 meeting

Conclusion: To down-select from the following options for enhanced power control - Option 1: Indication of open-loop parameter sets by DCI - For DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI without using SRI is applied to the scheduled transmission - FFS At least for single active CG-PUSCH, an open-loop parameter set is indicated to the UE by a UE-specific field in group common DCI - FFS for the case of multiple active CG-PUSCH - FFS For a UE, the open-loop parameter sets for DG-PUSCH and CG-PUSCH may be same or different - Option 2: Indication of TPC with increased range by DCI - For DG-PUSCH, a TPC with increased range is indicated to the UE by the TPC field in scheduling DCI - FFS At least for single active CG-PUSCH (and potentially also for DG-PUSCH), a TPC with increased range is indicated to the UE by a UE-specific TPC field in group common DCI - FFS for the case of multiple active CG-PUSCH - At least for DG-PUSCH, for a UE, the number of TPC entries (4 or 8) and power adjustment value for each entry is higher layer configured - FFS For a UE, the TPC configuration for DG-PUSCH and CG-PUSCH may be same or different - Option 3: - For DG-PUSCH, use either the solution from option 1 or option 2 for DG-PUSCH as above - To down-select from option 1 and 2 - FFS At least for single active CG-PUSCH, UE derives the transmissions power based on the time/frequency resource indicated by a group common DCI - If a CG-PUSCH transmission overlaps with the indicated time/frequency resource, UE use one open-loop parameter set with higher power for the transmission - If a CG-PUSCH transmission does NOT overlap with the indicated time/frequency resource, UE use another open-loop parameter set with lower power for the transmission - FFS for the case of multiple active CG-PUSCH - Note: some companies have concern that this was not captured in the TR as one potential solutions

RAN1 #98 meeting

Agreements: - For a DG-PUSCH, an open-loop parameter set indicated to the UE by scheduling DCI using a separate field than SRI is supported. - FFS number of bits for the indication

# Enhanced UL configured grant transmission

RAN1 #AH 1901 meeting

Agreement: - In Rel-16, for both Type 1 and Type 2 configured grant and when multiple active configurations are configured in a BWP, transmission of a TB based on the configured grant is associated with a single active configuration, even if the transmission is repeated

RAN1 #96bis meeting

Agreements: - Support separate RRC parameters for different configured grant configurations (for both type 1 and type 2 configured grants) for a given BWP of a serving cell. - FFS whether or not some parameters can be common among different configured grant configurations Agreements: - Support separate activation for different configured grant Type 2 configurations for a given BWP of a serving cell. - FFS whether or not to support joint activation in a DCI for two or more configured grant Type 2 configurations - Support separate release for different configured grant Type 2 configurations for a given BWP of a serving cell. - FFS whether or not to support joint release in a DCI for two or more configured grant Type 2 configurations Conclusion: RAN1 believes that it is feasible from physical layer perspective to support multiple active configured grant configurations with different Types for a given BWP of a serving cell. However, there is no conclusion in RAN1 whether or not to support it. - No further action in RAN1 until RAN2 has made progress on this topic (whether or not to support, use cases, etc.)

RAN1 #97 meeting

Agreements: - For the maximum number of UL CG configurations per BWP of a serving cell: - 12 Agreements: - Support joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell if the bit-length for indication which configurations released is no more than 4 bits and DCI size is not impacted by adopting joint release. - FFS details.

RAN1 #98 meeting

Agreements: - M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is - Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released - In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication Conclusion: - No support of joint activation in a DCI for two or more configured grant Type 2 configurations in Rel-16 Working assumption: - For activation and release of UL CG, same field(s) is/are used for a DCI format

# Appendix A: objectives of the work item

- Specification of PDCCH enhancements [RAN1] - DCI format(s) with configurable sizes for some fields, with a minimum DCI size targeting a reduction of 10~16 bits relative to Rel-15 DCI format 0_0/1_0 and a maximum DCI size that can be larger than Rel-15 DCI format 0_0/1_0, and provide the possibility to align with the size of the DCI format 0_0/1_0 (including possible zero padding if any) - Increased PDCCH monitoring capability on at least the maximum number of non-overlapped CCEs per slot for channel estimation for at least one SCS subject to restrictions including, but not necessary limited to, those identified in TR 38.824. Enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot (with potential restrictions) can be further considered. - Specification of UCI enhancements [RAN1] - More than one PUCCH for HARQ-ACK transmission within a slot - At least two HARQ-ACK codebooks simultaneously constructed, intended for supporting different service types for a UE - Specification of PUSCH enhancements for both grant-based PUSCH and configured grant based PUSCH [RAN1] - For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots - Specification of enhancements to scheduling/HARQ [RAN1] - Out-of-order HARQ-ACK associated with PDSCHs with different HARQ process IDs - Out-of-order PUSCH scheduling associated with different HARQ process IDs, including overlapping PUSCHs and non-overlapping PUSCHs in time-domain - Methods to handle DL data/data resource conflicts for overlapping PDSCHs in time-domain, scheduled by dynamic DL assignments - Specification of enhanced inter UE Tx prioritization/multiplexing [RAN1] - UL cancelation scheme (see section 7.2.1 in TR 38.824) - Enhanced UL power control scheme (see section 7.2.2 in TR 38.824) - Specification of enhanced UL configured grant transmission [RAN1, RAN2] - Multiple active configured grant type 1 and type 2 configurations for a given BWP of a serving cell - Note: V2X use cases are also considered