3GPP TSG RAN WG1 Meeting #98																R1-1909919

# Prague, CZ, August 26th – 30th 2019

Source:	Intel Corporation

Title:	Summary of RAN1 Agreements on NR Positioning

Agenda item:	7.2.10

Document for:	Information

# Introduction

In this contribution, summary of RAN1 WG agreements on Rel.16 NR Positioning WI is provided. The subsequent sections (Section 2 – Section 4) provide list of agreements made at each RAN1 WG meeting (i.e. from RAN1-96bis and up to RAN1-98).

For completeness and background, we also provide list of the agreements made by RAN1 WG during the study item phase. Agreements from the study item can be found in a backup section of this document (Section 5).

# Work on NR Positioning Support – RAN1#96bis

DL Reference Signals for NR Positioning

Agreement:

A DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID

  - The DL PRS Resources in a DL PRS Resource set are associated with the same TRP

A DL PRS Resource ID in a DL PRS Resource set is associated with a single beam transmitted from a single TRP (A TRP may transmit one or more beams)

Note: This does not have any implications on whether the TRPs and beams from which signals are transmitted are known to the UE.

Agreement:

DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1

  - FFS: cinit value for DL PRS

QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM

FFS: Whether a DL PRS sequence generated using a different mechanism is also specified

Agreement:

Comb-N resource element pattern per DL PRS Resource is supported to map DL PRS sequence to resource elements in frequency domain

  - Comb-N pattern can shift in frequency domain across symbols within DL PRS Resource

  - FFS values for N. The potential values for down-selection are provided in the set {1,2,3,4,6,8,12}

  - FFS relationship between N and number of symbols per DL PRS Resource

  - FFS support of staggered and non-staggered patterns and exact definition of staggered pattern

Agreement:

DL PRS configuration including DL PRS transmission schedule is to be indicated to the UE for DL PRS positioning measurements

  - The UE is not expected to perform any blind detection of DL PRS configurations

Agreement:

Number of DL PRS Sequence IDs is at least 4096

Agreement:

Support of numerologies (CP length and sub-carrier spacing) for the DL PRS is the same as for data transmissions in Rel-15.

Agreement:

DL PRS muting is supported. The UE is expected to be indicated when the DL PRS is muted.

UL Reference Signals for NR Positioning

Agreement:

Support the following configurations of SRS for positioning

  - Semi-persistent configuration

  - Periodic configuration

  - Aperiodic configuration

Agreement:

For positioning, regarding the number of SRS symbols per resource, decide by RAN1#97 whether to increase the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1, 2, 4}.

  - If it is not decided to increase the number of SRS symbols per resource by RAN1#97, increasing the set of allowable number of SRS symbols per resource compared to the NR Rel-15 allowable set {1,2,4} will not be considered further after RAN1#97.

Agreement:

For positioning, regarding the possible SRS symbol locations per slot decide by RAN1#97 whether the SRS can be configured only in the last N symbols of a slot with N>6

  - If the above decision is not made by RAN1#97, configuring SRS only in the last N symbols of a slot with N>6 will not be discussed further after RAN1#97.

Agreement:

Select one or both of the following options to support staggered SRS transmissions for UL SRS positioning

  - Staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource

  - Configuration of the same antenna port for the transmission of SRS symbols in different SRS resources in a SRS resource set.

UE and gNB measurements for NR Positioning

Agreement:

Intra-frequency and inter-frequency DL RSTD measurements (currently supported to be based at least on DL PRS) are supported in RRC_CONNECTED mode

Agreement:

At least intra-frequency DL PRS-RSRP measurements are supported in RRC_CONNECTED mode

  - FFS: Inter-frequency DL PRS-RSRP measurements in RRC_CONNECTED mode

Agreement:

The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements.

  - A DL PRS Resource ID

  - A subset of DL PRS Resource IDs from a single DL PRS Resource set

  - A DL PRS Resource set

Agreement:

The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference

Necessity and details for physical-layer procedures to support UE/gNB measurements

Agreement:

The potential physical layer procedures to support UE/gNB measurements for NR positioning may include:

  - 1.	UE procedures for receiving DL PRS, including

    - a.	DL PRS timing aspects

    - b.	Bandwidth part operation

    - c.	Beam management aspects

    - d.	Measurement/reporting related aspects

  - 2.	UE procedures for transmitting UL PRS, including

    - a.	Transmission timing and timing adjustments

    - b.	Power control

    - c.	Beam management aspects

  - 3.	gNB procedures to support NR positioning measurements, including

    - a.	DL PRS configuration aspects

    - b.	UL PRS configuration aspects

    - c.   Beam management aspects

    - d.	Measurement/reporting related aspects

Agreement:

The DL PRS configuration provided to the UE is independent of the configured DL BWPs of the UE

Agreement:

For intra-frequency measurements:

  - The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap.

    - Select from one of the following options for the measurement bandwidth

      - Option 1: The UE measurement is within the DL BWP configuration

      - Option 2: The UE can measure outside the DL BWP configuration

    - FFS: Scenarios when measurements gaps would need to be configured.

  - When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP.

Agreement:

For positioning purposes, to assist UE to perform Rx beamforming, consider one or more of the following options:

  - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

    - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS).

  - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

  - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter.

    - FFS: Whether this option can be achieved by Option 1

  - FFS on specification impacts.

Agreement:

For positioning purposes, with regard to the UL SRS transmission timing, at least the following options have been identified:

  - Option 1: No adjustment is made to the configured TA. That is, support only a TA configuration which is based only on the serving cell (i.e., the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols).

  - Option 2: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value can be provided by the UE to the network upon request.

    - Note: The adjustment value is not needed to be known at the measuring cell for the purpose of UL-AoA measurement

  - Option 3: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value is provided by the network to the UE.

Agreement:

For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified:

  - Option 1: The UL SRS Tx power is constant (i.e., no power control is supported).

    - FFS: The value of the constant Tx power and how it is determined

  - Option 2: The UL SRS Tx power is based on the existing power control procedure.

  - Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example:

    - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation.

      - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS).

    - FFS: which power control modes are supported (e.g., open loop only, or open loop and closed loop).

Agreement:

For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, consider one or more of at least the following options:

  - Option 1:  Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

    - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS).

  - Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources.

  - Option 3: The Tx beam is fixed for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2.

    - FFS: How the fixed beam is selected

# Work on NR Positioning Support – RAN1#97

DL Reference Signals for NR Positioning

Agreement:

DL PRS Resource is described by at least the following parameters

  - DL PRS Resource ID (previously agreed)

  - Sequence ID (previously agreed)

  - Comb Size-N

  - RE Offset in frequency domain

    - FFS whether this offset is a single value or multiple values

  - Starting slot and symbol of DL PRS Resource

    - FFS whether it can be represented by time offset with respect to some reference

  - Number of symbols per DL PRS Resource (Duration of DL PRS Resource)

  - Quasi-colocation information (QCL with other DL reference signals)

    - FFS QCL type and source reference signals

  - FFS: Number of Tx Ports

  - FFS: Power offset b/w DL PRS and SSB

  - FFS: Transmission bandwidth and starting PRB with respect to Point A

  - FFS: Numerology

Note: RAN1 to discuss further whether some of the above parameters belong to Resource Set configuration and are applied to all Resources within a Resource Set or not.

Agreement:

Number of symbols for DL PRS Resource is configurable from the following set {2, 4, 6}

  - FFS: Inclusion of other values in the set including values in {1, 3, 8, 12}

DL PRS Resource comb-N value is configurable from the set {2, 4, 6}

  - FFS: Inclusion of other values in the set including values in {1, 8, 12}

Note: The dependence between the number of symbols and the comb size should be considered when considering the inclusion of additional values in the sets for these parameters

Agreement:

RAN1 to select one of the following alternatives for periodic DL PRS Resource Allocation at RAN1#98:

  - Alt.1 - Periodicity of DL PRS Resource is configured at DL PRS Resource Set level

    - Common period is used for DL PRS resources within a DL PRS Resource Set

  - Alt.2 - Periodicity of DL PRS Resource is configured at DL PRS Resource level

    - Different periods can be used for DL PRS resources within a DL PRS Resource Set

Agreement:

DL PRS occasion is one instance of periodically repeated time windows (consecutive slot(s)) where DL PRS is expected to be transmitted

  - FFS: If this definition is introduced for muting or DL PRS resource allocation or both

UL Reference Signals for NR Positioning

Agreement:

SRS transmissions for positioning are realized with staggered patterns (a collection of SRS symbols from the same antenna port with different offsets for at least some symbols) in a single SRS resource

  - FFS: construction of the pattern inside the SRS resource structure

Agreement:

For positioning, the number of consecutive OFDM symbols in an SRS resource is configurable with one of the values in the set {1, 2, 4, 8, 12}

  - FFS: Other values including 3,6,14

  - Note: Values of 1, 2 and 4 within an SRS resource can already be configured in Rel-15

Agreement:

For positioning, starting positions in the time domain for the SRS resource can be anywhere in the slot, i.e. an offset loffset  range of {0,1,…,13}.

Agreement:

For positioning, with regard to UL Beam management/alignment towards serving and neighbouring cells, at least the following (option 2 from prior agreement) can be used:

  - UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources

  - FFS: other options

Agreement:

For positioning, the SRS comb size set is extended from {2,4} to {2,4,8}

  - FFS: Additional comb sizes: 1, 6, 12

    - Note: For the comb sizes of 6 and 12, the number of PRBs may be restricted if currently defined sequences are to be used

  - FFS: Maximum number of cyclic shifts for the different comb sizes (cyclic shifts for comb sizes of 2 and 4 already exist in Rel-15)

UE and gNB measurements for NR Positioning

Agreement:

Select one (or more) of the following options for the definition of the RSTD

  - Option 1: RSTD is defined as the time difference with respect to the subframe timings associated with the different TPs;

  - Option 2: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resource sets of different TPs

  - Option 3: RSTD is defined as the time difference with respect to the timings associated with the DL PRS resources of different TPs

  - Note: Definition of the RSTD applies at least to the RSTD measurements obtained from DL PRS. It does not preclude the RSTD obtained from other DL RS.

Agreement:

For FR1, the reference point of the RSTD measurement is the Rx antenna connector of the UE

For FR2, RAN1 has considered the following options as the reference point of the RSTD measurement. The final decision is up to RAN4 on how to define the reference point of the RSTD measurement.

  - Option 1: the combined signal from antenna elements corresponding to a given receiver

  - Option 2: the Rx antenna of the UE

Agreement:

DL PRS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port that carry a DL PRS resource configured for RSRP measurements within the considered measurement frequency bandwidth

  - FFS: DL PRS-RSRP for 2 ports, if 2 port transmission of a DL PRS is agreed.

Agreement:

For FR1, the reference point for the DL PRS-RSRP is the Rx antenna connector of the UE;

For FR2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch

Agreement:

When receiver diversity is in use by the UE to obtain the DL PRS-RSRP measurement, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches

Agreement:

Support inter-frequency DL PRS-RSRP measurements for NR positioning in Rel-16 when the UE is in RRC_CONNECTED state.

Agreement:

UE can be indicated to report the DL PRS Resource ID(s) or the DL PRS Resource set ID(s) associated with the DL PRS Resource ID(s) or the DL PRS Resource set(s), which are used in determining the UE measurements (DL RSTD, UE Tx-Rx time difference, DL PRS-RSRP) in UE measurement reporting.

R1-1907869 LS on Reference Point for Timing Related Measurements (RAN1 LS To: TSG RAN WG4 and Cc: to TSG RAN WG3)

Content of the above RAN1 LS (R1-1907869) is incorporated in table below:

1. Overall Description: RAN1 is currently working on the definition of reference point of UE and gNB measurements for NR positioning for Rel16. For the reference point of RSTD measurements in FR1, RAN1 has decided that the reference point of the RSTD measurement is the Rx antenna connector of the UE. For the reference point of RSTD measurements in FR2, RAN1 has considered the following two options for the definition of the reference point of RSTD measurements: - Option 1: the combined signal from antenna elements corresponding to a given receiver - Option 2: the Rx antenna of the UE In addition, it is also RAN1’s view that it would be better for RAN4 to make the final decision on the reference point of the RSTD measurement in FR2. It is RAN1’s understanding that RAN4 may also consider other options. In addition, it is RAN1’s view that the definition of the reference point of the RSTD measurement, if decided, can be extended to other timing related UE and gNB measurements in FR2. It is RAN1’s understanding that it is part of the work of the RAN3 WG to provide a name for the “transmission measurement function”. For simplicity of the discussion in RAN1 this function is referred to as “gNB” 2. Actions: To RAN4: RAN1 kindly requests RAN4 to provide the definition of a reference point for RSTD measurements in FR2 and inform RAN1 whether it is applicable to other timing related measurements, namely, the UE Rx-Tx timing difference, gNB Rx-Tx timing difference and UL RTOA. If the provided definition for the reference point is not applicable to these other timing related measurements in FR2, RAN1 requests RAN4 to provide definitions for the reference points for these other timing related measurements in FR2.

Agreement:

Support reporting separate metric(s) corresponding to quality of each of the following measurements

  - RSTD

  - UE Rx-Tx time difference

  - UL-RTOA

  - gNB Rx-Tx time difference

  - UL angle of arrival, including the azimuth of arrival (AoA) and the zenith of arrival (ZoA)

FFS: The details of these quality metrics

Necessity and details for physical-layer procedures to support UE/gNB measurements

Agreement:

For intra-frequency measurements, when measurement gaps are configured, the UE is expected to measure the DL PRS resource outside the configured (active and inactive) BWPs (Option 2 in the related Agreement from RAN1 #96-Bis).

  - Note: The UE has flexibility to measure only on a part of the DL PRS configured bandwidth as long as measurement requirements are met.

Agreement:

For positioning purposes, to assist UE to perform Rx beamforming, the following options are supported (options are from previous related agreement in RAN1#96bis):

  - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

    - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS).

  - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

  - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter.

    - FFS: Whether this option can be achieved by Option 1

  - FFS on specification impacts.

Agreement:

For positioning purposes, with regards to the UL SRS transmission power, support at least the following in addition to the existing Rel-15 behaviour:

  - Support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference for the purpose of SRS power control.

    - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS).

  - Study further the UE configuration signalling and procedure.

  - FFS on a fallback procedure if the UE is not able to obtain the pathloss reference.

  - FFS on number of measurements for pathloss.

Agreement:

For positioning purposes, for UL Beam management/alignment towards serving and neighbouring cells, the following is supported (in addition to UE TX beam sweeping):

  - Configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

    - Reference DL RS that can be used include at least SSB. FFS on CSI-RS, DL-PRS.

  - A fixed Tx beam for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2.

    - FFS how the fixed beam is selected.

  - FFS on specification impacts.

  - Note: In Rel-16, UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol (Rel-15 behaviour)

# Work on NR Positioning Support – RAN1#98

R1-1909796	LS on DL/UL Reference Signals and Measurements for NR Positioning (RAN1 LS To: TSG RAN WG2 Cc:	TSG RAN WG3, TSG RAN WG4)

Content of the above RAN1 LS (R1-1909796) is incorporated in table below:

1. Overall Description: RAN1 would like to inform other WGs on the following agreements made by RAN1: Agreement: Inform RAN2 of the following: - It is RAN1 understanding that discussion on mapping of positioning techniques to reference signals and measurements is out of RAN1 scope and therefore RAN1 does not intend to define such mapping. - It is RAN1 understanding that RAN2 will define signalling that can support any RAT dependent positioning technique including hybrid RAT dependent positioning solutions - RAN1 agreed on the tables below that provide mapping between reference signals and measurements based on the latest status of RAN1 discussion - The positioning techniques are indicated for information purpose as examples and do not intend to influence higher layer signalling to be developed by RAN2 - Note: Table will be modified in the future if additional reference signals for UE/gNB measurements are agreed DL/UL Reference Signals; UE Measurements; To facilitate support of the following positioning techniques. Rel.16 DL PRS; DL RSTD; DL-TDOA. Rel.16 DL PRS; DL PRS RSRP; DL-TDOA, DL-AoD, Multi-RTT. Rel.16 DL PRS / Rel.16 SRS for positioning; UE Rx-Tx time difference; Multi-RTT. Rel. 15 SSB / CSI-RS for RRM; SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM); E-CID. DL/UL Reference Signals; gNB Measurements; To facilitate support of the following positioning techniques. Rel.16 SRS for positioning; UL RTOA; UL-TDOA. Rel.16 SRS for positioning; UL SRS-RSRP; UL-TDOA, UL-AoA, Multi-RTT. Rel.16 SRS for positioning, Rel.16 DL PRS; gNB Rx-Tx time difference; Multi-RTT. Rel.16 SRS for positioning,; AoA and ZoA; UL-AoA, Multi-RTT. 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to take the above agreements into account in corresponding work.

DL Reference Signals for NR Positioning

Agreement:

The following parameters describing a DL PRS Resource are defined:

  - Bandwidth of DL PRS Resource

    - FFS granularity of bandwidth configuration which is to be down-selected at the next meeting among the following options:

      - Option 1. One PRB

      - Option 2. Four PRBs

      - Option 3. RBG granularity

      - Option 4. One of values configurable from the set 24, 48, 96 192, 264 PRBs

      - Option 5: Option 2, 3 or 4 combined with a possibility to blank PRBs at the band edges

      - Option 6: Option 2, 3 or 4 combined with the maximum PRBs per carrier bandwidth

  - Start PRB of DL PRS Resource is defined relative to Point A

    - A single Point A for DL PRS resource allocation is provided per frequency layer.

    - UE can be configured with one or multiple frequency layers

      - FFS amount of frequency layers for NR Positioning supported by UE, which is up to UE capability

    - All DL PRS Resources belonging to the same DL PRS Resource Set have common Point A

  - FFS whether additional constraints such as start PRB and center frequency of the bandwidth for the PRS resources are the same within a frequency layer. Resolve FFS at the next meeting.

Agreement:

Periodicity of DL PRS allocation is configured per DL PRS Resource Set

  - i.e. all DL PRS Resources of a given set have the same periodicity

Multiple DL PRS Resource Sets can be configured per TRP

  - FFS how many DL PRS Resources Sets can be configured per TRP

The following periodicity values for periodicity of DL PRS allocation are supported

  - P = {4, 8, 16, 32, 64, 5, 10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480} slots

    - 20480 is not supported for an SCS of 15 kHz

    - FFS: Further restrictions on values applicable to different SCS

Agreement:

A single SCS and CP type are configured for

  - Alt. 1: A DL PRS resource set

  - Alt. 2: A frequency layer

Agreement:

cinit  is a function of PRS sequence ID, slot index and symbol index

  - FFS: Other parameters and initialization details until RAN1#98bis

Agreement:

The RE pattern of a DL PRS is configured with an RE Offset in frequency domain for the first symbol in an DL PRS resource

The relative RE offsets of following symbols are defined relative to the RE Offset in frequency domain of the first symbol in the DL PRS resource

A relative RE offset of each of the following symbols is derived from the configured number of symbols for an DL PRS resource, the comb size for the DL PRS resource and the DL PRS symbol index within the DL PRS resource

UL Reference Signals for NR Positioning

Agreement:

The RE pattern of an SRS resource for positioning is configured with a comb offset for the first symbol in an SRS resource.

The relative RE offsets of following symbols are defined relative to the comb offset of the first symbol in the SRS resource.

A relative RE offset of each of the following symbols is derived from the configured number of symbols for an SRS resource, the comb size for the SRS resource and the SRS symbol index within the SRS resource.

  - FFS: The use of the following for deriving the relative offset in addition to the above

    - A configurable number of consecutive symbols with the same offset

    - Repetition factor

  - No additional parameters will be considered

Conclusion:

No additional comb sizes for SRS for positioning purposes will be introduced in Rel-16 beyond those already agreed.

Conclusion:

No additional number of symbols per SRS resource other than the values already agreed is supported

UE and gNB measurements for NR Positioning

Agreement:

RSTD is defined as the time difference with respect to the received DL subframe timings associated with the different TRPs (Option 1 from agreement made in RAN1#97)

Multiple DL PRS resources can be used to determine the received DL subframe timing of the first arrival path of the TRP.

At least the PRS resource ID(s) or PRS resource set ID(s) used for determining the timing of each TRP in RSTD measurements can be configured for reporting in the measurement report.

Note: This does not preclude the use of any additional reference signals that are being discussed further including existing reference signals

Conclusion:

How to determine the RSTD measurement and UE Rx-Tx time difference measurements that are reported when receiver diversity is used by the UE is up to UE implementation and is not specified by measurement definition.

Agreement:

UE Rx-Tx time difference is defined with respect to the Rx and Tx subframe timing associated with the TRP

- Multiple DL PRS resources can be used to determine the received DL subframe timing of the first arrival path of the TRP.

- At least the PRS resource ID(s) or PRS resource set ID(s) used for determining the timing of each TRP in the UE Rx-Tx time difference measurements can be configured for reporting in the measurement report.

- Note: This does not preclude the use of any additional reference signals that are being discussed further including existing reference signals

Agreement:

For FR1,

- The reference point for Rx time in the definition of the UE Rx-Tx time difference is the Rx antenna connector of the UE.

- The reference point for Tx time in the definition of the UE Rx-Tx time difference is the Tx antenna connector of the UE

Agreement:

UL RTOA is defined with the respect to the subframe timing associated with the UE

- Multiple SRS resources for positioning purposes can be used to determine the received UL subframe timing of the first arrival path of the UE.

- FFS: The resource ID(s) or resource set ID(s) used for determining the timing of the UE and possibly the Rx beam used at the gNB in the UL RTOA measurements can be requested for reporting in the measurement report.

Agreement:

For FR1, the reference point for UL RTOA is the Rx antenna connector of gNB

Conclusion:

How to determine the UL RTOA measurement and gNB Rx-Tx time difference measurements that are reported when receiver diversity is used by the gNB is up to gNB implementation and is not specified by measurement definition.

Agreement:

gNB Rx-Tx time difference is defined with respect to the subframe timing associated with the UE

- Multiple SRS resources for positioning purposes can be used to determine the received UL subframe timing of the first arrival path of the UE.

- FFS: The resource ID(s) or resource set ID(s) used for determining the timing of the UE and possibly the Rx beam used at the gNB in the gNB Rx-Tx time difference measurements can be requested for reporting in the measurement report.

Agreement:

For FR1,

- The reference point for the Rx time of the gNB Rx-Tx time difference is the Rx antenna connector of the gNB

- The reference point for the Tx time of the gNB Rx-Tx time difference is the Tx antenna connector of the gNB

Agreement:

UL SRS-RSRP for NR positioning is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port that carry the UL SRS resource configured for RSRP measurements within the considered measurement frequency bandwidth in the configured measurement occasions

Agreement:

For FR1, the reference point for the UL SRS-RSRP is the Rx antenna connector of the gNB.

For FR2, UL SRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.

When receiver diversity is in use by the gNB to obtain the UL SRS-RSRP measurement, the reported UL SRS-RSRP value shall not be lower than the corresponding UL SRS -RSRP of any of the individual receiver branches

Agreement:

AoA (ϕ) and ZoA (θ) define the estimated angles of a user with respect to a reference direction which are determined at the TRP antenna for an UL channel corresponding to this UE.

The reference directions can be defined according to either one of the following options (both options are supported in specifications)

  - Option 1:

    - For AoA, the geographical North, positive in a counter-clockwise direction.

    - For ZoA, the zenith/vertical

      - Note:  θ=0 pointing to the zenith and θ=900 pointing to the horizon.

  - Option 2:

    - For AoA and ZoA, the reference direction is with respect to the Local Coordinate System (LCS) defined in TR 38.901. The translation of the GCS to LCS uses the set of angles  (bearing angle),  (downtilt angle),   (slant angle) which are reported

## Necessity and details for physical-layer procedures to support UE/gNB measurements

Agreement:

For providing an indication of when the DL PRSs are expected to arrive in time at the UE, at least one of the following options is supported:

  - Option 1:  Provide an expected RSTD value together with uncertainty (search window) to the UE for the TRPs in the assistance data (analogous to LTE).

  - Option 2: Provide a TRP transmission time difference (e.g., time offset between SFN#0, subframe offset, slot-, symbol-, sub-symbol-,  ns-offset, etc.) and expected propagation delay difference together with uncertainty (search window) to the UE for the TRPs in the assistance data.

    - A search window may be needed for both, TRP transmission time difference and propagation time difference.

    - Sub-SFN granularity (e.g, subframe offset, slot-, symbol-, sub-symbol-, ns-offset) may be obtained from the DL-PRS configuration information once time offset between SFN#0 is provided to the UE.

Agreement:

With regards to QCL relations beyond Type-D of a DL PRS resource, support one or more of the following options:

  - Option 1:  QCL-TypeC from an SSB from a TRP.

  - Option 2:  QCL-TypeC from a DL PRS resource from a TRP.

  - Option 3:  QCL-TypeA from a DL PRS resource from TRP.

  - Option 4:  QCL-TypeC from a CSI-RS resource from a TRP.

    - FFS: Which CSI-RS resource type can be used? (e.g., CSI-RS for CSI, CSI-RS for BM, CSI-RS for TRS, CSI-RS for RLM, CSI-RS for RRM).

  - Option 5:  QCL-TypeA from a CSI-RS resource from a TRP.

    - FFS: Which CSI-RS resource type can be used? (e.g., CSI-RS for CSI, CSI-RS for BM, CSI-RS for TRS, CSI-RS for RLM, CSI-RS for RRM).

  - Option 6: No QCL relation beyond Type-D is supported.

- NOTE:

- QCL-TypeA:	Doppler shift, Doppler spread, average delay, delay spread

- QCL-TypeB:	Doppler shift, Doppler spread

- QCL-TypeC:	Average delay, Doppler shift

- QCL-TypeD:	Spatial Rx parameter

Agreement:

For transmission timing of the SRS for positioning purposes, the TA configuration is based only on the serving cell (i.e., the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols). (Option 1 from previous related agreement in RAN1#96bis).

  - Options 2/3 (from previous related agreement in RAN1#96bis) may be further considered if improvements of gNB measurement accuracy can be demonstrated and if it can be shown that the interference to the serving cell can be handled in the system.

  - FFS: whether the application of the TA command at the UE impacts the feasibility of each of the options.

  - FFS: whether UE autonomous TA adjustments impacts the feasibility of each of the options.

Agreement:

For the purpose of power control of the SRS for positioning purposes, support configuring a DL reference signal of a neighbouring cell to be used as DL path loss reference. One of the following can be configured as the DL reference signal.

  - SSB

  - DL-PRS at least for multi-cell RTT

  - FFS: CSI-RS

# Backup (Study Item Agreements)

In this section, we additionally provide agreements made during the study item phase that may serve as a background

## Study on NR positioning support – RAN1#94bis

R1-1811527 endorsed as TR 38.855 v0.0.1

The text proposal in R1-1811614 is endorsed.

### Requirements for NR Positioning

Agreement:

Regulatory requirements are considered as a minimum performance targets for NR Positioning studies

Additional requirements based on commercial use cases can be used as input performance targets that are subject to further analysis in terms of performance / complexity tradeoffs in different evaluation scenarios

Agreement:

For regulatory use cases, the following requirements are considered as a minimum performance targets for NR positioning

  - Horizontal positioning error <= 50m for 80% of UEs

  - Vertical positioning error [<5 m] for [80%] of UEs

    - Note: The regulatory requirements refer to floor level vertical accuracy

  - End to end latency and TTFF < 30 seconds

As a starting point for commercial use cases, the following requirements are considered as performance targets for RAT dependent solutions, which are subject to further analysis in terms of performance / complexity tradeoffs of NR positioning radio-layer solutions

  - Horizontal positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Horizontal positioning error < [10]m for [80]% of UEs in outdoor deployments scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in indoor deployment scenarios

  - Vertical positioning error < [3]m for [80]% of UEs in outdoor deployment scenarios

  - End to end latency < [1]s

Note: This does not eliminate more or less demanding commercial use cases.

Agreement:

At least CDFs of horizontal and vertical (vertical error not necessarily applicable to all solutions and/or scenarios) positioning errors are used as a performance metrics in NR positioning evaluations

  - At least the following percentiles of positioning error are analyzed 50%, 67%, 80%, 90%

Physical layer latency, UE power consumption, scalability/capacity, network deployment complexity, availability, UE and gNB complexity can be considered as important design factors for NR positioning solutions and can be evaluated analytically for proposed solutions

### Evaluation Methodology

Agreement

The following evaluation scenarios are defined for NR positioning studies

  - Scenario 1. Indoor Office for FR1 and FR2 (Open office and Mixed Office)

  - Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

  - Scenario 3. UMa (ISD 500m, TBD: ISD > 500m) for FR1 only (Macro cell only deployment scenario)

FFS if other evaluation scenarios are introduced (e.g. IIoT)

Note: Specific parameters of evaluation scenarios are to be finalized at the RAN1#94bis based on assumptions defined in 3GPP TRs 38.901 / 38.802 / 37.857

Agreement

Common parameters provided in Table 1 and Table 2 below are agreed for NR Positioning evaluations and captured in the 3GPP TR 38.855

Table 1: Parameters common for all evaluation scenarios

| Parameters | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| System parameters |  |  |
| Carrier Frequency, GHz | 4 GHz, 2GHz [TR 38.802] | 30 GHz [TR 38.802] |
| Bandwidth, MHz | 5MHz, 100 and 50MHz for 4 and 2 GHz respectively | 100, 400 MHz |
| Subcarrier spacing, kHz | 15 kHz (for 5 and 50 MHz), 30kHz (for 100 MHz) | 120 kHz |
| gNB model parameters |  |  |
| gNB Noise Figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE Max. TX Power, dBm | 23dBm [TR 38.802] | 23dBm [TR 38.802], EIRP should not exceed 43 dBm (*) |
| UE Noise Figure, dB | 9dB [TR 38.802] | 13dB [TR 38.802] |
| UE Antenna Configuration | Panel model 1 [TR 38.802], Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a [TR 38.802] - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - - Each antenna array has shape dH=dV=0.5λ - - Config a: (M, N, P) = (2, 4, 2), the polarization angles are 0° and 90° - - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE Antenna Radiation Pattern | Omni, 0dBi | Antenna model in Table 2 |
| PHY/Link Level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. For future study whether a common algorithm is to be agreed upon. |  |
| Network Synchronization assumption | Network synchronization error. Network sync error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an gNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, the range of timing errors is [-T2, T2], T2 = 2*T1. T1 = 0 (perfectly synchronized) and T1 =50 ns are evaluated |  |

Table 2: UE antenna radiation pattern model 1 (FR2) (same as Table A.2.1-8 in TR 38.802)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Agreement:

Indoor Office parameters captured in the following Table are agreed for NR positioning evaluations and captured in the 3GPP TR 38.855

  - Note: companies are not precluded to provide analysis for additional scenarios of interest

Modifications to the table for 2 GHz should be further discussed

Summary of Evaluation Parameters for Indoor Office Scenario applicable at least for 4 GHz

| Parameters | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| gNB model parameters |  |  |
| Total gNB TX Power, dBm | 24dBm | 24 dBm, EIRP should not exceed 58 dBm |
| gNB Antenna Configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ, [TR 38.802] | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH = dV = 0.5 λ, [TR 38.802] One TXRU per polarization per panel is assumed |
| gNB Antenna Radiation Pattern | Single sector [TR 38.802] | 3-sector antenna configuration [TR 38.802] |
| Propagation characteristics |  |  |
| Channel Model | According to 3GPP TR 38.901 (Indoor Open Office) | According to 3GPP TR 38.901 (Indoor Open Office) |
| Penetration Loss | 0dB | 0dB |
| Layout considerations |  |  |
| Layout | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, [TR 38.901] Inter-gNB distance = 20m | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, [TR 38.901] Inter-gNB distance = 20m |
| Number of floors, (floor height) | 1 |  |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility | 3 km/h |  |
| UE antenna height | 1.5m |  |
| Min. gNB - UE distance (2D), m | 0m |  |
| gNB antenna height | 3m |  |

Agreement:

UMi street canyon parameters captured in Tables 1 and X below provide the evaluation parameters for NR positioning evaluations and should be captured in the 3GPP TR 38.855

Table 1: Summary of Evaluation Parameters for UMi Street Canyon Scenario

| Parameters | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| gNB model parameters |  |  |
| Total gNB TX Power, dBm | 44 dBm | 37 dBm per panel, EIRP should not exceed 73 dBm |
| gNB Antenna Configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, [TR 38.802] | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ, [TR 38.802] |
| gNB Antenna Radiation Pattern | Directional, 8dBi [TR 38.802], Please refer to Table X | Directional, 8dBi [TR 38.802], Please refer to Table X |
| Propagation characteristics |  |  |
| Channel Model | According to 3GPP TR 38.901 (UMi Street Canyon) | According to 3GPP TR 38.901 (UMi Street Canyon) |
| Penetration Loss | For outdoor UEs: 0 dB, | For outdoor UEs: 0 dB, |
| Layout considerations |  |  |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 200m, [TR 38.802] Wrap-around is applied Note: in case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 200m, [TR 38.802], Wrap-around is applied Note: in case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites |
| Number of floors, (floor height) | All users are on the ground |  |
| UE drop procedure | 100% outdoor uniformly distributed over the horizontal area | 100% outdoor uniformly distributed over the horizontal area |
| UE mobility | Outdoor: 3km/h Only for modelling of Doppler effects | Outdoor: 3km/h Only for modelling of Doppler effects |
| UE antenna height | 1.5 m |  |
| Min. gNB - UE distance (2D), m | 10 m |  |
| gNB antenna height | 10 m by default. Companies can bring results with uniform distribution [5-20]m Default assumption can be revisited if benefits from using uniform distribution is shown |  |

Table X: 3-Sector BS antenna radiation pattern (same as Table A.2.1-6: in TR 38.802)

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8dBi |

Agreement:

UMa parameters captured in the Table below are agreed for NR positioning evaluations and captured in the 3GPP TR 38.855

Table: Summary of Evaluation Parameters for UMa Scenario

| Parameters | FR1 Specific Values |
| --- | --- |
| gNB model parameters |  |
| Total gNB TX Power, dBm | 49 dBm |
| gNB Antenna Configuration | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1), (dH, dV) = (0.5, 0.8)λ, [TR 38.802] Apllicable for 2GHz and 4 GHz carrier frequency. Could be reconsidered for 2GHz |
| gNB Antenna Radiation Pattern | Directional, 8dBi [TR 38.802], Please refer to Table X |
| Propagation characteristics |  |
| Channel Model | According to 3GPP TR 38.901 (UMa scenario) |
| Penetration Loss | For outdoor UEs: 0 dB, For indoor UEs: 20 dB + 0.5 d2D-in, [TR 38.901] |
| Layout considerations |  |
| Layout | Hexagonal grid, 19 or 7 macro sites, 3 sectors per site, ISD = 500m, [TR 38.802] Wrap-around is applied Note: in case if interference considerations are not properly taken into account for 7 sites companies are encouraged to provide results for 19 sites |
| Number of floors, (floor height) | 8, (3m), [TR 37.857] |
| UE drop procedure | 50% indoor and 50% outdoor uniformly distributed over the horizontal area (separate statistic) |
| UE mobility (for modelling Doppler effects) | Indoor: 3km/h / Outdoor: 60km/h |
| UE antenna height | 1.5 m [TR 37.857] |
| Min. gNB - UE distance (2D), m | 35m [TR 37.857] |
| gNB antenna height | Uniformly distributed [20-50] m |

### Potential Techniques for NR Positioning

Agreement:

The RAT dependent solutions considered for study include

  - Downlink based solutions

  - Downlink and uplink based solutions

  - Uplink based solutions

## Study on NR positioning support – RAN1#95

R1-1813589 TR 38.855 Study on NR positioning support v0.0.2 is endorsed

The text proposals in R1-1813869 “Text proposal for deployment scenarios and evaluation methodologies for NR positioning” are agreed for inclusion into the TR

### Remaining Details on Requirements for NR Positioning

Agreement:

For commercial use cases, remove brackets from target performance requirements on horizontal positioning error that were agreed at the RAN1#94bis meeting

Add Note:

  - For commercial use cases, indoor deployment means indoor deployed UEs and gNBs

  - For commercial use cases, outdoor deployment means outdoor deployed UEs and gNBs

Agreement:

The following note is added under the regulatory and commercial NR positioning target requirements

  - Note: It is understood that no single positioning technology has to meet all the requirements for every scenario

### Remaining Details on Evaluation Methodology

Agreement:

The evaluation methodology does not define any baseline reference signals.

The results from the evaluations can potentially be used to select a subset of the reference signals studied as part of the evaluations when considering design options

Agreement:

The existing channel models as provided in the TR 38.901 are used for NR positioning evaluation as a baseline.

  - Note: Companies can provide additional results and describe modifications made for NLOS channel model

### Potential Techniques for NR Positioning

Agreement:

The following candidate techniques are considered for study of DL positioning:

  - Timing based techniques

    - Timing of arrival path(s)

    - Phase difference based techniques

      - Note: feasibility needs to be further assessed

  - Angle-based techniques

    - Downlink angle(s) of departure

    - Downlink angle(s) of arrival

  - Carrier-phase based techniques

    - Note: feasibility needs to be further assessed

  - Received reference signal power based techniques

  - Cell ID and TRP related information (e.g. RS resource and/or resource set ID)

Agreement:

The following candidate techniques are considered for study of UL positioning:

  - Timing based techniques

  - Timing of arrival path(s)

  - Angle-based techniques

  - Uplink angle(s) of departure

  - Uplink angle(s) of arrival

  - Carrier-phase based techniques

  - Note: feasibility needs to be further assessed

  - Received reference signal power based techniques

Agreement:

The following candidate techniques are considered for study of DL and UL positioning:

  - Timing based techniques

    - Round trip time measurement including support for multiple TRPs

  - Combination of DL and UL techniques for NR positioning

    - e.g. E-CID like techniques (including one or multiple cells)

Agreement:

Combination of DL, UL and DL + UL techniques can be used for NR positioning

Combination of RAT-dependent and RAT-independent techniques can be considered for NR positioning

Agreement:

The following candidate reference signals were identified for DL positioning evaluation and are to be further studied

  - NR CSI-RS (including TRS configuration)

  - NR Synchronization Signals (SSBs)

  - New DL positioning reference signals (DL PRS)

Agreement:

The following candidate reference signals were identified for UL positioning evaluation and are to be further studied

  - NR PRACH

  - NR SRS

  - NR UL DMRS

  - NR UL PTRS

  - New UL positioning reference signals (UL PRS)

## Study on NR positioning support – RAN1 # AdHoc - 1901

R1-1901450 3GPP TR 38.855 V1.0.1 Endorse updated version of TR based on agreements from this meeting

### DL only based Positioning

Agreement:

NR should support timing (DL-TDOA) based DL-only positioning techniques in FR1 and FR2

NR should support angle (DL-AoD) based DL-only positioning techniques with at least beam sweeping at least at the gNB in FR1 and FR2

  - Note: This does not necessarily need different reference signals from the timing based DL-only positioning techniques

Agreement:

Enhancements to performance obtainable based on existing NR DL reference signals (e.g., based on extensions to current reference signals or with new reference signals) are necessary to meet accuracy requirements at least in some scenarios

Agreement:

NR DL PRS design for FR1 and FR2 supports:

  - Configurable NR DL PRS signal bandwidth

    - FFS granularity of configuration, relationship with BWPs, whether the configuration is cell and/or UE specific

  - Configurable NR DL PRS signal numerology (SCS)

    - FFS configurability of CP for NR DL PRS

  - Configurable NR DL PRS frequency and time allocation

  - Use of DL beam sweeping / alignment

    - i.e. beam alignment of gNB DL PRS transmission and UE reception of DL PRS

  - Localized in time NR DL PRS transmissions with periodic and/or on-demand resource allocation

    - FFS signaling details

  - Dedicated NR DL PRS resources - time-frequency grid at resource block level

    - PRS transmitted in one cell may or may not collide with PRS transmissions in other cell

      - e.g. frequency vShift/comb-offset is the same or different for two different PRSs in the same RB

    - There is no data/control transmission in time-frequency grid of dedicated NR-DL PRS resources

    - FDM multiplexing with other signals at RE level inside of PRS time-frequency grid is precluded

    - FFS interference randomization techniques across PRS signals

  - FFS shared in time/frequency NR DL PRS resources with other transmissions including data/control

    - FFS which physical channel/signals can share resources with NR DL PRS

    - FFS interference randomization techniques for PRS transmission with other signals

Agreement:

For NR DL PRS resource design:

  - One antenna port is supported.

    - FFS: configurable number of antenna ports (more than one) in addition to support of a single port

- Agreement:

A PRS resource should have a PRS resource ID and a PRS sequence should have a PRS sequence ID

Agreement:

NR DL PRS resource is defined as a set of resource elements used for NR DL PRS transmission that can span multiple PRBs within N (1 or more) consecutive symbol(s) within a slot, where N is FFS.

  - In any OFDM symbol, PRS resource occupies consecutive PRBs

  - FFS if multiple symbols can be non-consecutive

  - FFS on PRS Resource Set

FFS: whether support of PRS frequency hopping is needed

- Agreement:

NR supports RSTD measurements for NR DL PRS

  - FFS: RSTD b/w different configured PRS resources, beams, TRPs, cells, etc.

NR supports RSRP measurement for NR DL PRS

FFS if NR supports RSTD measurement quality metric for NR DL PRS

FFS other measurements

Additional aspects that have been proposed during the study:

  - Receive signal waveform reporting

  - UE-based positioning

  - Multipath measurements

  - SFN transmissions for positioning

  - Techniques to lower UE complexity for OTDOA

  - Broadcast of assistance data

  - LMUs

### UL only based Positioning

Agreement:

NR UL SRS is used as a starting point for design and analysis of UL PRS

  - Further study if and which enhancements are needed

    - FFS: NR UL PRS relationship with UL BWP and component carrier

Agreement:

FFS: At least the following aspects for NR UL PRS design

  - Use of UL beam sweeping at FR2

    - Beam sweeping includes possibility of quasi-omni transmission

  - Use of UL beam alignment at FR2 through DL reception and beam correspondence

  - UL Power control aspects

  - UL timing advance aspects

Agreement:

The following measurements for NR UL PRS at serving and neighbor gNBs should be supported:

  - UL RTOA measurements

  - UL Angle Of Arrival (AOA) measurements (including Azimuth and Zenith Angles)

  - UL RSRP (reference signal received power) measurements

  - gNB RX-TX time difference measurements

### Combination of DL & UL based Positioning

Agreement:

- NR should support timing (UL-TDOA) based UL-only positioning techniques in FR1 and FR2

  - UL-TDOA with two or more neighboring gNBs/TRPs for NR UL positioning should be supported

- NR should support UL-AoA based positioning techniques in FR1 and FR2

  - UL-AoA with one or more neighboring gNBs/TRPs for NR UL positioning should be supported

Round-trip time (RTT) with one or more neighboring gNBs/TRPs for NR DL and UL positioning should be supported for FR1 and FR2

Agreement:

The following measurements for NR DL PRS for serving and neighbor gNBs should be supported:

  - UE RX-TX time difference measurements

Agreement:

Further study whether it is beneficial to support and if so, how to support, NR based positioning for UEs in RRC-IDLE and/or RRC-INACTIVE states

  - Note: This involves RAN2 aspects

### System-level Performance Evaluation for RAT-Dependent Positioning Techniques

Agreement:

The template in Section 3 of R1-1901416 is agreed for inclusion of simulation results in Section 8 of the TR

  - Section 8 will also include a summary of the results

These text proposals for Subsections of TR titled “Results from Company …” should be provided by each company as part of their contribution into RAN1#96

Any analysis of the results can be included in the contribution but should be separate from the above text proposal

  - Note: Rapporteurs to provide any guidance on formatting for the contributions from companies in advance of the contribution deadline

Agreement:

The following parameters should be listed for the set of results provided by each company for each scenario:

| Parameter | [Source 1 Uma, FR1] |  |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) |  |  |
| Carrier frequency |  |  |
| Subcarrier spacing |  |  |
| Reference Signal Transmission Bandwidth |  |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) |  |  |
| Reference signal (type of sequence, number of ports, …) |  |  |
| Number of sites |  |  |
| Number of symbols used per occasion |  |  |
| number of occasions used per positioning estimate |  |  |
| Power-boosting level |  |  |
| Uplink power control (applied/not applied) |  |  |
| interference modelling (ideal muting, or other) |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, …) |  |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) |  |  |
| Network synchronization assumptions |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) |  |  |
| Additional notes, if any |  |  |

### Potential Techniques for NR RAT Independent Positioning

Conclusion:

Results provided by companies for techniques not addressed by the results in Section 8 of the TR can be included in Section 7.2 of the TR

  - Note: No common simulation assumptions have been agreed for RAT independent and hybrid positioning methods

## Study on NR positioning support – RAN1#96

R1-1903534	TR 38.855 v1.0.5 Endorsed v1.0.5

Updated version for email approval till 3/6 (Florent, Ericsson) with the following:

removing Section 9.5, and moving the contents Section 9.3.2 – a new section

removing editor’s comments

removing void sections (section 8) and correct reference

R1-1903775	TR 38.855 v1.0.4 R1-1903775 is endorsed with the following modifications:

In section 7.1, fix typo for the word “Whether”

Add reference at the end of Section 7.2.1

Remove company names in Section 8.

Fix legend in Figure 8.3.3.10-1

Fix Umi and Uma where applicable

Remove “End of text proposal change” in Section 9.3.1

Change “:” to “.” in Section 10.

Add agreed TP for Section 7.2.3

Incorporate any input from RAN3

Enhancements identified in Section 9.4 can be considered for future specification work

R1-1903722	TP for TR 38.855, Section 2, 7.2 and 7.2.4 on RAT independent positioning

Agreement:

The TP for Section 7.2.4 proposed in R1-1903722 excluding the first paragraph is endorsed as Section 7.2.3 in the TR, ensuring that the references are aligned with the TR.

Agreement:

The text proposal in Section 3 of R1-1903485 is endorsed for the Conclusions section of the TR

R1-1903442	TR 38.855 v1.0.2

Agreement:

R1-1903442 with the agreed text proposal for the Conclusions section from Section 3 of R1-1903485 is endorsed as v1.0.2 of the TR in R1-1903487.

R1-1903534	TR 38.855 v1.0.2

TPs in Sections 7.2 and 8.4 capturing observations from the evaluation results

Check all the editorial changes

R1-1903688	TR 38.855 v1.0.3

Agreement:

Include the following in Section 10 of the TR prior to the recommendation to start normative work for NR positioning:

DL-only UE-based positioning is feasible from a RAN1 perspective. System level aspects need further consideration in other working groups.

Agreement:

Capture the following in Section 10 of the TR:

GNSS SSR completion (PPP-RTK support) can be considered for further specification work.

Agreement:

Include the following in Section 10 of the TR:

  - A variety of enhancements for UL SRS were proposed by many sources and were reported to be beneficial by these sources.

Agreement:

Modify a statement in Section 10 of the TR as follows:

  - “It was identified that enhancements are necessary (e.g., based on extensions to current reference signals or with new reference signals) to meet horizontal accuracy requirements in some scenarios. In order to achieve target performance characteristics, …”

R1-1903701	TP for TR 38.855, Section 2 and 7.2 on RAT independent positioning

Agreement:

The TPs in R1-1903701 are endorsed with the following text for Section 7.2.3:

Support for RAT independent positioning in NR was introduced in Rel. 15. Hybridization between RAT independent and NR RAT dependent positioning methods should be supported for NR.

R1-1903702	TP for TR 38.855, Section 8.x on RAT independent positioning evaluations

Agreement:

The TP in R1-1903702 is agreed with the following modifications:

  - Include the figures that have been deleted

  - Change 5G-GNSS to NR-GNSS

  - All mentions of company names or the word company should be changed to “source” with a reference provided to the contribution where the results were included.

  - Insert the statement, “No conclusions can be drawn from the results in this Section”, in Section 8.x.1 after the statement, “No GNSS models or scenarios have been agreed in 3GPP for positioning evaluations.”

Conclusion:

DL-only UE-based positioning is feasible from a RAN1 perspective. System level aspects need further consideration in other working groups.

R1-1903691	Text Proposal on Additional NR Positioning Enhancements for the 3GPP TR 38.855

Agreement:

The text proposal in R1-1903691 is agreed for inclusion in the TR with the following modifications:

  - Phase-based RSTD measurement (PDoA) was proposed in [15] to be applied for DL-TDOA or RTT positioning techniques [16].

  - Add the following statement at the beginning of Section 9.6: “Multiple sources proposed a variety of UL SRS enhancements for NR positioning”.

R1-1903689	Offline session notes for 7.2.10.1.4 on evaluations of positioning solutions (Wednesday)

Agreement:

The TPs in R1-1903689 are endorsed for inclusion in the TR with the following modifications:

  - Replace “polaris” with a reference number in the TP for Section 8.4.1.2

  - Replace “8 out of 12” with “7 out of 12” in the TP for Section 8.4.1.3

  - Fix typo (ources) in the TP for Section 8.4.1.3

  - Change all occurences of UTDOA to UL-TDOA.

  - Replace all occurences “HW” with the appropriate reference number.

  - In the second and fourth bullets in the TP for Section 8.4.3-3 (which mistakenly has been labelled as 8.4.3-1), insert “and outdoor UEs” after “For scenario 3,”.

### DL only based Positioning

Agreement:

The following is agreed for inclusion in TR 38.855:

NR DL PRS resource design with one antenna port should be supported. The use of more than two antenna ports per NR DL PRS resource need not supported.

Agreement:

New DL positioning reference signals should be defined for supporting NR positioning. When and how existing NR DL reference signals can be used for positioning needs further discussion.

### UL only based Positioning

### Combination of DL & UL based Positioning

Agreement:

NR should support E-CID based positioning

  - E-CID downlink measurements should be supported based on at least RRM measurements defined in NR Rel. 15

  - Note:

    - Support of NR Rel-16 signals, measurements and procedures for E-CID are to be discussed when specified

    - NR Rel-15 signals may be used to define additional UE/gNB measurements to facilitate E-CID support

Agreement:

The reference signals agreed for DL-only and UL-only positioning techniques can be reused for DL+UL RTT positioning with multiple TPs/gNBs

### System-level Performance Evaluation for RAT-Dependent Positioning Techniques

Agreement:

The following text proposal for the section summarizing evaluations in TR 38.855 is agreed

---------------------------begin TP for 8.4.1.1 in 38.855:-----------------------

For the Indoor open office scenario, channel models and simulation assumptions, the following can be observed for horizontal accuracy as described in section 5 of the TR for regulatory and commercial requirements:

- For scenario 1, the evaluations from 12 sources showed that DL-TDOA can meet the regulatory requirements.

- For scenario 1, the evaluations from 12 sources showed that DL-TDOA can meet the commercial requirements, when no synchronization error is included in the evaluation. When synchronization error is modeled, the commercial requirements as described in section 5 of the TR are not met.

- For scenario 1, 1 source evaluated DL AoD for FR2, and observed that it can meet the regulatory requirements.

- For scenario 1, 1 source evaluated DL AoD for FR2, and observed that it can meet the commercial requirements.

- For scenario 1, 1 source evaluated DL-TDOA+AoD for FR2, and observed that it can meet the regulatory requirements.

- For scenario 1,  1 source evaluated DL-TDOA+AoD for FR2, and observed that it can meet the commercial requirements, when no synchronization error is included in the evaluation. When synchronization error is modeled, the commercial requirements for horizontal accuracy are not met.

-----End TP------