3GPP TSG RAN WG1 #98		R1-1909929

Prague, CZ, August 26th – 30th, 2019

Source:	ZTE

Title:	RRC parameters for Rel-16 NR 2-step RACH

Agenda Item:	7.2.1

Document for:	Discussion/Decision

# 1 Introduction

This document summarizes the RRC parameters based on RAN1 agreements made for the Rel-16 WI on ‘2-step RACH for NR’ (NR_2step_RACH, latest WID in RP-190711) up to RAN1#98.

# 2 Parameters for RA type selection

Table 1

| Parameter Name (suggested) | Values | New R16 vs extension of R15 | Per (UE, cell, config …) | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- |
| rsrp-Threshold-msgA | TBD | New | RAN2 decide | A RSRP-based criterion for RACH type selection, detailed UE behavior to be described by RAN2. |  |
| msgA-TransMax | TBD | New | RAN2 decide | The maximum number of MsgA transmission. The UE switches to 4-step RACH msg1 transmission if the number of MsgA transmission reach this value. |  |

# 3 Parameters for msgA PRACH

Note 1: It is up to RAN2 whether to comprise some of the below parameters into the RACH-ConfigGeneric, or generate a separate set of parameters in a new IE

Note 2: FFS whether the parameters are broadcast or dedicated signaled.

Note 3: The squared brackets means the parameters have been defined based on agreements, but the existence of a specific parameter in RRC signalling needs further discussions. We will decide based on further agreements either to remove the square brackets of a parameter or to delete the parameter from the list.

## 3.1 separate RO

Table 2

| Parameter Name (suggested) | Values | New R16 vs extension of R15 | Per (UE, cell, config, …) | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- |
| msgA-prach-ConfigurationIndex- | TBD | New | Per BWP | PRACH configuration index for 2-step RACH. If not configured, 2-step RACH reuses the corresponding 4-step RACH parameter. |  |
| msgA-RO-FDM | ENUMERATED {one, two, four, eight}(TBC) | New | Per BWP | The number of msgA PRACH transmission occasions FDMed in one time instance. If not configured, reuse the value of msg1-FDM |  |
| msgA-RO-FrequencyStart | INTEGER (0..maxNrofPhysicalResourceBlocks-1) | New | Per BWP | Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. If not configured, reuse the value of msg1-FrequencyStart |  |

Remaining parameters that have not been discussed/agreed yet whether they are common for 2-step RACH and 4-step RACH):

- totalNumberOfRA-Preambles

- prach-RootSequenceIndex

- preambleTransMax

- msgApreamble-powerRampingStep

- msgApreambleReceivedTargetPower

- zeroCorrelationZoneConfig

- msg1-SubcarrierSpacing

- restrictedSetConfig

- ra-ResponseWindow

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB-msgA or ssb-perRACH-Occasion-msgA

## 3.2 shared RO

Parameters in table 2 but not listed in table 3 are considered as common for both 2-step RACH and 4-step RACH

Table 3

| Parameter Name (suggested) | Values | New R16 vs extension of R15 | Per (UE, cell, config, …) | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- |
| cb-PreamblePerSSB-msgA | TBD | New | Per BWP | Number of preambles used for 2-step RACH, from the non-CBRA preambles associated with each SSB. The number of SSBs per RACH occasion is same as that configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB and the number of preamble for 2-step RACH shall not exceed the number of preambles per SSB minus the number of preambles per SSB for 4-step RACH |  |

Remaining parameters that have not been discussed/agreed yet whether they are common for 2-step RACH and 4-step RACH):

- preambleTransMax

- ra-ResponseWindow

- msgApreamble-powerRampingStep

- msgApreambleReceivedTargetPower

- nrofsharedRO

# 4 Parameters for msgA PUSCH

Table 4

| Parameter Name (suggested) | Values | New R16 vs extension of R15 | Per (UE, cell, config, …) | Description | Configuration restriction (if any) |
| --- | --- | --- | --- | --- | --- |
| [msgA-PUSCH-configList] | [SEQUENCE (SIZE(1..maxmsgA-PUSCH-Configs)) OF MsgA-PUSCH-config] | New | Per BWP | To support multiple msgA PUSCH configurations. [maxmsgA-PUSCH-Configs]is at least 2 for RRC_IDLE/INACTIVE and RRC_CONNECTED state. (FFS other value) MsgA-PUSCH-config includes the following parameters which are noted as “per configuration”. |  |
| [mcs-tbs] | TBD | New | Per configuration per BWP | To be decided if one of them or both to be configured |  |
| [msgAPUSCH-Periodicity] | TBD | New | Per configuration per BWP | msgA PUSCH configuration period. To be decided if PUSCH periodicity can be different from that of PRACH. |  |
| [msgAPUSCH-Offset] | TBD | New | Per configuration per BWP | msgA PUSCH offset relative to [TBD] |  |
| msgAPUSCH-timeDomainOffset | TBD | New | Per configuration per BWP | At least for same configuration periodicity for msgA PRACH and PUSCH, a single time offset with respect to the start of each PRACH slot, counted as the number of slots (based on the numerology of active UL BWP) [msgA PUSCH offset relative to TBD if different periodicity is supported.] |  |
| nrofSlotsMsgAPUSCH | TBD | New | Per configuration per BWP | Number of slots (in active UL BWP numerology) containing one or multiple POs, each slot has the same time domain resource allocation |  |
| nrofMsgAPOperSlot | TBD | New | Per configuration per BWP | Number of time domain PUSCH occasions in each slot. POs including guard period are contiguous in time domain within a slot |  |
| startSymbolAndLengthMsgAPO | TBD | New | Per configuration per BWP | An index giving valid combinations of start symbol, length, and mapping type as start and length indicator (SLIV) for the first msgA PUSCH occasion. The network configures the field so that the allocation does not cross the slot boundary. The list of combinations is provided in PUSCH-Config-Common and/or PUSCH-Config and/or the default table in 38.214. The number of occupied symbols excludes the guard period. |  |
| guardPeriodMsgAPUSCH | TBD | New | Per configuration per BWP | Guard period between PUSCH occasions, in the unit of symbols |  |
| [mappingTypeMsgAPUSCH] | ENUMERATED {typeA, typeB} | New | Per configuration per BWP | PUSCH mapping type A or B, to be decided if this is explicitly signaled, or included in SLIV. |  |
| frequencyStartMsgAPUSCH | INTEGER (0..maxNrofPhysicalResourceBlocks-1) | New | Per configuration per BWP | Offset of lowest PUSCH occasion in frequency domain with respective to PRB 0 |  |
| nrofPRBsperMsgAPO | TBD | New | Per configuration per BWP | Number of PRBs per PUSCH occasion |  |
| nrMsgAPO-FDM | TBD | New | Per configuration per BWP | The number of msgA PUSCH occasions FDMed in one time instance |  |
| msgA-DMRS-Configuration | TBD | New | Per configuration per BWP | DMRS configuration for msgA PUSCH, including DMRS Type, number of DMRS symbols, and number of DMRS ports or including the scramble ID(s) for CP-OFDM waveform. To be decided if this is a list of configurations or if at least DMRS sequences and/or the number of DMRS sequences are given by one or more additional parameters. |  |
| [nrofDMRS-Sequences] | TBD | New | Per configuration per BWP | Number of DMRS sequences for msgA PUSCH at least for CP-OFDM waveform | A condition only port is used. |
| msgADeltaPreamble | INTEGER (-1,6) (TBC) | New | Per BWP | Power offset of msgA PUSCH relative to the preamble received target power. If the offset parameter is absent, the parameter msg3-DeltaPreamble of 4-step RACH is used |  |
| [preamblesPerPRU] |  | New | Per configuration Per BWP | Number of preambles mapped to one PUSCH resource unit. To be decided if it is explicitly signaled. |  |
| [msgA-PUSCH-powerRampingStep] |  | New | Per BWP | Power ramping steps for msgA PUSCH. To be decided if it is separately configured or always follow msgApreamble-powerRampingStep of 2-step PRACH |  |
| msgA-Alpha |  | New | Per configuration Per BWP | Dedicated alpha value for MsgA PUSCH. If value is absent, use msg3-Alpha if configured, else UE applies value 1. |  |

Remaining parameters that have not been discussed/agreed yet:

- msgA-transformPrecoder

- msgA-PUSCH-powerRampingStep

- n_ID