3GPP TSG RAN WG1 Meeting #98								R1-1909933

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.9

Source:	CATT

Title:	List of Higher Layer Parameters  for UE Power Saving in NR

Document for:	Discussion and Decision

- List of Higher Layer Parameters for UE Power Saving in NR

| Parameter Name | RAN1 specification / section | (New) values | New R16 vs extension of R15 | Per (UE, cell, TRP, …) | Broadcast/dedicated | Description | RAN2 specification | Configuration restriction (if any) | Relevant agreement / working assumption |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRXAdaptation | 38.213 | Present | new | Per BWP | dedicated | UE is configured with DRX Adaptation | 38.331 | If present, “searchspacePS and PS-RNTI” have to be present | WID Objectives in RP-191607 - Specify power saving techniques with PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED mode [RAN1, RAN2, RAN4] - Specify procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-based power saving signal/channel for the next occurrence(s) of the drx-onDurationTimer [RAN2, RAN1] |
| CrossSlotScheduling | 38.213 | Present | new | Per BWP | dedicated | UE is configured with cross slot scheduling | 38.331 |  | - Specify the procedure of cross-slot scheduling power saving techniques [RAN1, RAN4] NOTE: The procedure is in addition to Rel-15 cross-slot scheduling procedure |
| SearchSpacePS | 38.213 | TBD | new | Per BWP | dedicated | ID(s) of Search Space set(s) for PDCCH-based power saving signal/channel | 38.331 | Applicable only forPCell in CA and PCell for MCG and PScell for SCG in DC | Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time - Following the principle of Rel-15 search space configuration - FFS: the corresponding UE behavior in monitoring the power saving signal/channel outside Active Time - FFS whether UE can further monitor the search space set(s) inside Active Time |
| PS-RNTI | 38.213 | RNTI-value | new | Per UE | dedicated | Power saving RNTI used for CRC scrambling of PDCCH based power saving signal/channel | 38.331 |  | A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured •The CRC of new DCI format for power saving signal/channel is scrambled by PS-RNTI outside active time |
| minimumSchedulingOffset | 38.213 | TBD | new | Per BWP | dedicated | For a DL BWP, one parameter with up to two configured values (Minimum K0). For an UL BWP, one parameter with up to two configured values (Minimum K2). For a DL BWP, the Minimum K0 parameter denotes minimum applicable value(s) for the TDRA table for PDSCH and for A-CSI RS triggering Offset(s). For an UL BWP, the Minimum K2 parameter denotes minimum applicable value(s) for the TDRA table for PUSCH. | 38.331 |  | To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted, the following is supported: - One or two RRC configured values for restriction to the active TDRA table - RRC configuration is per BWP - If there are one or two RRC configured values for a BWP, 1-bit indication to indicate one value from two candidate values - For the case of one RRC configured value, the 1-bit indication further indicates whether or not there is no restriction to the active TDRA table The 1-bit indication in DCI format 1_1 or format 0_1 is used to jointly determine the minimum applicable K0 for the active DL BWP and the minimum applicable K2 value for the active UL BWP, which are to be applied at least after the application delay. |