3GPP TSG RAN WG1 Meeting #98	R1-1909933

Prague, Czech Republic, August 26-30, 2019

Agenda Item:	7.2.9

Source:	Rapporteur (CATT)

Title:	RAN1 agreements on UE Power Saving in NR

Document for:	Information

Introduction

RAN1 agreements are listed for Rel-16 UE power saving in NR WID in RP-191607 up to RAN1#98. The agreements are categorized into the PDCCH-based power saving signal/channel design in support of triggering UE adaptation, procedure of cross-slot scheduling power saving technique, and UE adaptation to maximum MIMO layer.

PDCCH based Power Saving Signal/Channel

Design Principle of PDCCH-based Power Saving Signal/Channel

The PDCCH based power saving signal/channel was discussed with design principle.  The agreements of the PDCCH-based power saving signal/channel outside and inside Active Time are as follows,

RAN1#96bis agreements Agreements: The assumptions of the DCI design of the PDCCH-based power saving signal/channel include: - No increase of DCI format size budget - FFS whether or not the same or different sets of DCI format sizes for Active time vs. out of Active time - Working assumption: no increase of UE BD/non-overlapping CCE limit Agreements: Possible candidates of DCI format design for the PDCCH-based power saving signal/channel (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - New DCI format(s) - The size of new DCI format may or may not be the same size as the existing DCI size - Enhancement of existing DCI. E.g.,: - Additional new field(s) - Using the existing DCI format for the power saving purpose - Re-purpose field(s) in the DCI - The detection of existing DCI format as the indication for the power saving technique Agreements: The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide: - Indication of one or more power saving associated operations. - Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format Agreements: - The design of the DCI format(s) and size needs to account for one or more of the following aspects - Within or outside Active Time - DCI format size for the power saving signal/channel to fit the DCI format size budget - Including aspects whether or not it is necessary to align it with existing DCI format size - Other aspects are not precluded Agreements: - The DCI format(s) contain information for (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - Alt 1: triggering a single UE only - Alt 2: triggering UE(s) within a group - FFS whether to always trigger all UEs in a group or a subset of it - Alt 3: Alt 1 & Alt 2 Agreements: - Support at least one CORESET with configured search space(s) for the power saving signal/channel - FFS separate vs. shared with a CORESET (and/or search space(s)) configured for other purposes (when applicable) Agreements: - For PDCCH-based power saving signal/channel, - The set of AL(s) is configured - The number of PDCCH candidate(s) for each AL is configured

Design of PDCCH based Power Saving Signal/Channel outside Active Time

The PDCCH based power saving signal/channel outside Active Time is designed to trigger UE adaptation to the DRX operation and the associated power saving techniques during the DRX ON.

Configuration and Monitoring Occasion of Power Saving Signal/Channel outside Active Time

RAN1#96bis agreements Agreements: - The PDCCH-based power saving signal/channel is UE-specifically configured. RAN1#97 agreements Agreements: The monitoring occasion(s) of the power saving signal/channel outside the Active Time is “indicated” to the UE by the gNB with an offset before the DRX ON - “Indicated” implies the explicit signalling by higher layer signalling or implicit through the CORESET/search space - FFS: The value and the range of offset Agreements: One or more PDCCH monitoring occasion of PDCCH-based power saving signal/channel is supported outside Active Time - FFS: Whether the monitoring occasions of the PDCCH-based power saving signal/channel in the same slot or not for multiple monitoring occasions - FFS: whether the monitoring occasions are via search space set(s), CORESET(s), a combination thereof, etc. RAN1#98 agreements Working assumption: - More than one monitoring occasion can be configured within a slot or multiple slots before the DRX ON # Conclusion: For next meeting, down select the following two alternatives: The configuration of the offset of the PDCCH-based power saving signal/channel - Alt1: Dedicated configuration with offset relative to the beginning of DRX ON - Alt2: The Offset is based on search space configuration - FFS: whether this applies to long DRX only or long/short DRX.

DCI format and size for PDCCH-based Power Saving Signal/Channel outside Active Time

RAN1#97 agreements Agreements: For power saving signal/channel configured outside Active Time, introduce a new DCI format for a UE, where the UE is configured to monitor the DCI format, with the power saving information for the UE in the DCI configurable by RRC - FFS whether the DCI is in UESS or CSS or both - FFS detailed configuration of the power saving information - FFS the new DCI format - Note: the same DCI may carry power saving information for one or more UEs Agreements: A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured - FFS how to use the PS-RNTI for scrambling of the PDCCH-based power saving signal/channel RAN1#98 agreements Agreements: - The new DCI format for power saving signal/channel is configured to be monitored at least in CSS. - FFS for UESS Agreements: - The CRC of new DCI format for power saving signal/channel is scrambled by PS-RNTI outside active time

DCI Contents/functions for PDCCH-based Power Saving Signal/Channel outside Active Time

RAN1#96bis agreements on PDCCH-based power saving signal/channel are as follows, Agreements: - The PDCCH-based power saving signal/channel is UE-specifically configured. - The DCI format(s) contain information for (including potential down-selection, which may or may not depend on power saving techniques/scenarios): - Alt 1: triggering a single UE only - Alt 2: triggering UE(s) within a group - FFS whether to always trigger all UEs in a group or a subset of it - Alt 3: Alt 1 & Alt 2 Agreements: Potential DCI contents in DCI format(s), to be further investigated: - Power saving technique associated with C-DRX– - Essential for UE function for the C-DRX - Wakeup – - UE is indicated to transition from outside Active Time to Active Time - UE is indicated to stay at Active Time - Go to sleep– - UE is indicated to transition from Active Time to outside Active Time. - UE is indicated to stay outside Active Time - FFS: The time of receiving the wakeup and go-to-sleep indication inside or outside Active Time. - Cross-slot scheduling - Triggering RS transmission - CSI report - Single vs. multi-cell operation - BWP /SCell - BWP & SCell together - BWP and SCell have separated fields - MIMO layer adaptation/number of Anenna adaptation - May further depend on RAN4’s work - Indication of CORESET/search space/candidate of subsequent PDCCH decoding - PDCCH monitoring periodicity - PDCCH skipping (skipping duration)- - PDCCH skipping – UE is indicated to skip number of the PDCCH monitoring occasions and stays in the Active Time # Note that - For the bullets in italic, there are concerns that some of which may have dependence on the ongoing SI in RAN2. - For the last two bullets, there are additional concerns that these are deemded by some companies to be not in the scope of the power saving WI approved so far The following candidates may be discussed after RAN2’s SI is completed: - Skipping number of DRX monitoring - SPS activation - DRX configuration RAN1#97 agreements PDCCH-based power saving signal/channel Agreements: Outside Active Time, the PDCCH-based power saving signal/channel is configured for triggering UE to or not to monitor the subsequent ON duration(s) - FFS a single vs. multiple durations, particularlly checking consistency with RAN2 agreements Further study in the new DCI how to potentially indicate at least the following techniques (subject to further WID update): - Indicating UE to use the aperiodic RS - Aperiodic CSI-RS - Aperiodic SRS - TRS - Triggering aperiodic CSI report - Cross-slot scheduling - Rel-15 DCI-based BWP switching The power saving techniques can be explicitly included in the DCI contents or implicitly indicated by other techniques (e.g., BWP switching).

CORESET/Search Space configuration

RAN1#96bis agreements Agreements: - Support at least one CORESET with configured search space(s) for the power saving signal/channel - FFS separate vs. shared with a CORESET (and/or search space(s)) configured for other purposes (when applicable) Agreements: - For PDCCH-based power saving signal/channel, - The set of AL(s) is configured - The number of PDCCH candidate(s) for each AL is configured RAN1#97 agreements on CORESET/Search space configuration outside Active Time Agreements: Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time - Following the principle of Rel-15 search space configuration - FFS: the corresponding UE behaviour in monitoring the power saving signal/channel outside Active Time - FFS whether UE can further monitor the search space set(s) inside Active Time Agreements: - The CORESET for the PDCCH-based power saving signal/channel outside Active Time can be configured to index to at least one of the CORESET(S) configured for other PDCCH monitoring - FFS whether the indexed CORESET can be exclusively used by the PDCCH-based power saving signal/channel (i.e., not be used for other PDCCH monitoring) - FFS whether or not to increase the number of CORESETs relative to that in Rel-15 - FFS whether or not a BWP is dedicated for PDCCH-based power saving signal/channel Agreements: - For outside Active Time, up to [3] CORESETs per BWP is supported for the power saving signal/channel outside Active Time with each CORESET associated with its TCI state and QCL assumption - FFS details - FFS whether any other additional handling is necessary for FR2 w.r.t. TCI state, and if so, how RAN1#98 agreements Agreements: - The maximum number of CORESETs for PDCCH-based power saving signal/channel outside Active Time is no larger than the max number that can be configured inside Active Time. Working assumption: - The CORESET for power saving signal/channel outside Active Time can be associated with (in addition to search space set for power saving signal/channel) other search space set(s). Working assumption: UE monitors the PDCCH-based power saving signal/channel outside Active Time being configured on the active BWP in an active cell.

DCI formats and size of PDCCH-based power saving signal/channel Within Active Time

When UE is in the Active Time, UE needs to monitor scheduling DCI and the associated power saving information in PDCCH for triggering adaptation to the power saving techniques.

RAN1#96bis agreements on DCI formats and size of PDCCH-based power saving signal/channel Agreements: - The design of the DCI format(s) and size needs to account for one or more of the following aspects - Within or outside Active Time - DCI format size for the power saving signal/channel to fit the DCI format size budget - Including aspects whether or not it is necessary to align it with existing DCI format size - Other aspects are not precluded RAN1#97 agreements on DCI formats and DCI contents of PDCCH-based power saving signal/channel Agreements: The indication of at least one power saving technique(s) is supported at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time. - It applies to UE-specific search space. - It is FFS for the common search space. - The at least one power saving technique(s) includes at least “Cross-slot scheduling” - FFS: Which existing DCI formats includes the power saving information - Whether power saving information is not included in the fallback DCI(s) (e.g., DCI format 0_0, DCI format 1_0) - Use of non-scheduling DCI formats. - It is FFS which field(s) is used to be repurposed for the indication of the power saving technique if the repurpose of existing field(s) is used. - FFS: New DCI format with size aligned with existing DCI format RAN1#98 agreements Agreements: - Scheduling DCI format(s), 1-1 and/or 0-1 are enhanced to include the configurable additional field of 1-bit dynamic indication of cross-slot scheduling in the Active Time in Rel-16. Agreements: - Power saving information using DCI formats 0_0/1_0 is not supported

Procedure of Cross-Slot Scheduling Power Saving Techniques

The cross-slot scheduling is to set the minimum K0 > 0 and aperiodic CSI-RS triggering offset not within the duration to allow UE switching to micro sleep after PDCCH reception with no addition of PDSCH and CSI-RS signals reception within the given duration (e.g. the same slot).

  - It is known to the UE at PDCCH decoding

  - Extended micro sleep time and reduce the PDCCH processing in reducing UE power consumption

  - Minimum K2 > 0 is essential to avoid the requirements of fast PDCCH processing

  - UE assistance information can be considered

NR supports of cross-slot scheduling

RAN1#96bis agreements Agreements: - Regarding aperiodic CSI-RS triggering, at least if a UE is operated with cross-slot scheduling based power saving, - If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states and the PDCCH SCS is equal to the CSI-RS SCS, specification allows the aperiodic CSI-RS triggering offset to be set to a non-zero value.

Dynamic Adaptation

RAN1#96bis agreements Agreements: - For an active DL and an active UL BWP, a UE can be indicated via signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured) where the signalling type is to be down-selected from: - Alt 1: MAC-CE based - Alt 2: L1 based - FFS: How to determine the minimum applicable value if explicit value is not provided. Agreements: Possible candidate indication methods to adapt the minimum applicable value of K0 (or K2) for an active DL (or UL) BWP, where the indication method is to be selected from: - Alt 1: Indication of a subset of TDRA entries, e.g., bit-map based indication - Alt 2: Indication of one active table from multiple configured TDRA tables - Alt 3: Indication of the minimum applicable value - Note: Other option is not precluded Note: PDCCH monitoring case 1-1 is prioritized for the design. FFS: Whether and how the minimum applicable K0 (or K2) value of the active DL (or UL) BWP is also applied to cross-BWP scheduling Agreements: Possible candidate indication methods to adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, where the indication method is to be selected from: - Alt 1: Implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated - Alt 2: Indication of the minimum applicable value - Note: Other option is not precluded Note: PDCCH monitoring case 1-1 is prioritized for the design. Agreements: - The adaptation on the minimum applicable value of K0 does not apply to at least the following cases: RNTI; PDCCH search space. SI-RNTI; Type0 common. SI-RNTI; Type0A common. RA-RNTI, TC-RNTI; Type1 common. P-RNTI; Type2 common. Conclusion: Companies are encouraged to further investigate whether to apply the adaptation for at least the following cases: - K0 related: RNTI; PDCCH search space. C-RNTI, CS-RNTI. MCS-C-RNTI. - K2 related: RNTI; PDCCH search space. PUSCH scheduled by MAC RAR as described in subclause 8.2 of TS 38.213. C-RNTI, TC-RNTI, CS-RNTI. MCS-C-RNTI. RAN1#97 agreements Agreements: For an active DL and an active UL BWP, a UE can be indicated via L1-based signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured). Agreements: To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP, indication of the minimum applicable value is supported. - FFS: Direct assignment of the minimum application value, indication of one value from one or multiple preconfigured or predetermined value(s), and/or implicit indication. - FFS: How the indicated minimum applicable value is applied to the selection of a DL (UL) TDRA entry. Example directions include at least the following: - Excluding the invalid TDRA entries - Re-interpret the selected K0 (K2) value Agreements: When UE is indicated of the minimum applicable value of K0 (K2) for an active DL (UL) BWP, the application method to the selection of a DL (UL) TDRA entry is to be decided from: - An entry in the active DL (UL) TDRA table with K0 (K2) value smaller than the indicated minimum is not expected by or not valid for the UE for the TDRA indication(s) Agreements: At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to PUSCH scheduled by MAC RAR for at least contention-based RACH procedure. - FFS: Exclude contention-free RACH - FFS: Exclude PUSCH scheduled with TC-RNTI Agreements: - To adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated is supported. Agreements: At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to: - PUSCH scheduled by RAR UL grants for contention-free RACH procedure - PUSCH scheduled with TC-RNTI # Conclusion: Companies are encouraged to check the following proposal for the application delay: For an active DL and an active UL BWP, when UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0 and/or K2, UE is not expected to apply the new indicated minimum applicable value(s) before slot n+X⋅2μPDSCH2μPDCCH for K0, or slot n+X⋅2μPUSCH2μPDCCH for K2, where - X = max(Y, Z) - Y is the minimum applicable K0 value prior to the indicated change - Z = [1] - Z is the smallest feasible non-zero application delay that may depend on DL SCS - FFS: Z > 1 for 60kHz/120kHz SCS or multi-TRP - FFS: Cross-carrier scheduling with different numerologies - FFS: interruption time, if any RAN1#98 agreements Agreements: Scheduling DCI format(s), 1-1 and/or 0-1, to indicate the minimum applicable value of K0 (K2) for active DL (UL) BWP during Active Time is supported - FFS: Whether and how other scheduling DCI format(s) during Active Time can be used - FFS whether to have joint or separate indication for DL & UL Agreements: To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted, the following is supported: - One or two RRC configured values for restriction to the active TDRA table - RRC configuration is per BWP - If there are one or two RRC configured values for a BWP, 1-bit indication to indicate one value from two candidate values - For the case of one RRC configured value, the 1-bit indication further indicates whether or not there is no restriction to the active TDRA table Agreements: For an activated BWP without the 1-bit indication received in DCI for adapting the minimum applicable value of K0(K2) for the BWP when there are one or two RRC configured values for the BWP, e.g., due to BWP switching triggered by BWP timer expiration, etc., the value applied for the BWP is determined by (to down-select one of them): - Option 1: No restriction if one value is RRC configured; The lowest-indexed RRC configure value if two values are RRC configured - Option 2: The configured value if one value is RRC configured; The lowest-indexed RRC configured value if two values are RRC configured; - Option 3: No restriction Agreements: - The 1-bit indication in DCI format 1_1 or format 0_1 is used to jointly determine the minimum applicable K0 for the active DL BWP and the minimum applicable K2 value for the active UL BWP, which are to be applied at least after the application delay. Agreements: - For an active DL and/or an active UL BWP, after UE is indicated to change the minimum applicable values of K0 and/or K2 and before the change indication takes effect, - UE can be scheduled data with restriction based on current active minimum applicable values of K0 and/or K2

UE adaptation to maximum number of MIMO layers

RAN1#98 agreements Agreements: - Support per-DL-BWP configuration of maximum number of DL MIMO layers - Signalling details up to RAN2